3GPP TSG-SA3 LI Meeting #50	S3-13xxxx

# Sophia-Antipolis, France, June 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LI to GBA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia Corporation, Nokia Siemens Networks) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2013-06-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | To decrypt the communication secured by GBA-provided application specific keys, the LEAs need access to the application specific keys. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of LI to GBA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No decryption of GBA secured communication services. Potential regulatory implications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4, 14 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 33.108 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Telecommunications security; Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671: "Handover Interface for the lawful interception of telecommunications traffic".

[4]	GSM 01.33: "Lawful Interception requirements for GSM".

[5]	GSM 02.33: "Lawful Interception - stage 1".

[6]	GSM 03.33: "Lawful Interception - stage 2".

Editor's note: 
References 4, 5 & 6 are not cited in the body of the present TS, and anyway they do not exist in this Release.

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A: "Lawfully Authorised Electronic Surveillance".

[9]	IETF RFC 2806: "URLs for Telephone Calls ".

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122: "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123: "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	(void)

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: “Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2”

[31]	3GPP TS 22.220:  “Service Requirements for Home NodeBs and Home eNodeBs”.

[32]	3GPP TS 36.300:  Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2”.

[33]	3GPP TS 25.467:  “UTRAN architecture for 3G Home Node B (HNB); Stage 2”

[34]	3GPP TS 33.320:  “Security of Home Node B (HNB) / Home evolved Node B (HeNB)”.

[xy]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

*** SECOND MODIFICATION ***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [13] and the following apply:

3GMS	3rd Generation Mobile Communications System

3G GGSN	3rd Generation Gateway GPRS Support Node

3G GSN		3rd Generation GPRS Support Node (GGSN/SGSN)

3G MSC		3rd Generation Mobile Switching Center

3G SGSN	3rd Generation Serving GPRS Support Node

3G UMSC	3rd Generation Unified Mobile Switching Centre

AAA	Authentication, Authorization, and Accounting

ADMF	Administration Function

AN	Access Network

AP	Access Provider

BM-SC	Broadcast-Multicast Service Centre

BSF	Bootstrapping Serving Function

CC	Content of Communication

CS	Circuit Switched

CSCF	Call Session Control Function

CSG	Closed Subscriber Group

DF	Delivery Function

DSMIP		Dual Stack Mobile IP

ECT	Explicit Call Transfer

EPC	Evolved Packet Core

ePDG	Evolved PDG

EPS	Evolved Packet System

E-UTRAN	Evolved UTRAN

FTP	File Transfer Protocol

GBA	Generic Bootstrapping Architecture

GGSN	Gateway GPRS Support Node

GPRS	General Packet Radio Service

GSM	Global System for Mobile communications

GSN	GPRS Support Node (SGSN or GGSN)

HA	Home Agent

HeMS	HeNB Management System

HeNB	Home enhanced NodeB

HeNB GW	HeNB Gateway

H(e)NB	Home and Home enhanced NodeB

HI	Handover Interface

HMS	HNB Management System

HNB	Home NodeB

HNB GW	HNB Gateway

HRPD	High Rate Packet Data

HSS	Home Subscriber Server

IA	Interception Area

ICEs	Intercepting Control Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, HLR, AAA Server, PDG, MME, S-GW, PDN-GW, HSS)

IETF	Internet Engineering Task Force

IMEI	International Mobile station Equipment Identity

IMS	IP Multimedia Core Network Subsystem

IMSI	International Mobile Subscriber Identity

INEs	Intercepting Network Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, MGW, HLR, AAA Server, PDG)

IP	Internet Protocol

IRI	Intercept Related Information

I-WLAN	Interworking WLAN (3GPP WLAN interworking subnetwork)

LAN	Local Area Network

LDI	Location Dependent Interception

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LIPA	Local IP Access

MBMS	Multimedia Broadcast/Multicast Service

ME	Mobile Entity

MIP	Mobile IP

MME	Mobility Management Entity

MN	Mobile Node

MSISDN	Mobile Subscriber ISDN Number

NAF	Network Application Function

NAI	Network Access Identifier

NO	Network Operator

PDG	Packet Data Gateway

PDN 	Packet Data Network

PDN-GW	PDN Gateway

PMIP	Proxy Mobile IP

PoC	Push to talk over Cellular

PS	Packet Switched

RA	Routing Area

RAI	Routing Area Identity

SAI	Service Area Identity

SeGW	Security Gateway

SGSN	Serving GPRS Support Node

SIP	Session Initiation Protocol

SMS	Short Message Service

S-GW	Serving Gateway

TEL URL	"tel" URL, as defined in RFC 2806 [9]

TLS	Transport Layer Security

UE	User Equipment

UMTS	Universal Mobile Telecommunication System

URI	Universal Resource Identifier

URL	Universal Resource Locator

VoIP	Voice over IP

WLAN	Wireless LAN

*** NEXT MODIFICATION *** NEW CLAUSE ***

# 14	Interception of Generic Bootstrapping Architecture (GBA) Secured Communications

## 14.1	Introduction

The Generic Bootstrapping Architecure (GBA) is defined in the 3GPP TS 33.220 [xy]. This section details the stage 2 Lawful Interception architecture and functions that are needed to provide the GBA based application specific encryption keys from the GBA architecture towards the DF2 for a subscriber that is target of interception.

Figure 14.1 shows the LI architecture for the GBA where the BSF provides the events and associated information towards the DF2 over the X2 interface.

Figure 14.1: GBA Intercept Configuration

## 14.2	Provision of Content of Communications

The GBA interception provides the application specific cryptographic keys (aka GBA application specific keys) which are used to decrypt the intercepted communication secured using those GBA application specific keys. Interception of the content of communications for GBA secured services is not part of this section and can be achieved via other methods outlined in this specification.

## 14.3	Provision of Intercept Related Information

### 14.3.1	Provision of Intercept Related Information Data Flow

Figure 14.2 shows the transfer of intercept related information to the DF2. If an event related to a target subscriber occurs, the BSF shall send the relevant data to the DF2.

intercepted

subscriber

Delivery

Function 2

LEMF

BSF

Figure 14.2: Provision of Intercept Related Information

### 14.3.2	X2-interface

The following information needs to be transferred from the BSF to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in clauses 14.3.3 may be provided;

The IRI should be sent to DF2 using a reliable transport mechanism.

### 14.3.3	GBA LI Events and Event Information

Intercept Related Information (Events) are necessary for the following;

- Bootstrapping

- Query from NAF

- Start of interception with GBA key

A set of possible elements as shown in Table 14.3.1 are used to generate the events.

Table 14.3.1: Information Events for GBA Event Records

Element

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Observed Other Identity Other Identity of the target subscriber (monitored subscriber).

Event type Description which type of event is delivered: Bootstrapping, Query from NAF, Start of interception with GBA key

Event date Date of the event generation in the BSF

Event time Time of the event generation in the BSF.

Network Element Identifier Unique identifier for the element reporting the BSF.

B-TID Bootstrapping transaction identifier, TS 33.220 [xy].

Key lifetime The lifetime of the key material is set according to the local policy of the BSF, TS 33.220 [xy].

Bootstrapping time The timestamp of the bootstrapping event.

Ks_int_NAF GBA application specific key (internal), if GBA_U has been used, TS 33.220 [xy].

Ks_ext_NAF GBA application specific key (external), if GBA_U has been used, TS 33.220 [xy].

Ks_NAF GBA application specific key, if GBA_ME has been used, TS 33.220 [xy].

Ua protocol id Ua interface security protocol id defined in clause Annex H in TS 33.220 [xy].

NAF_Id The FQDN of the NAF, concatenated with the Ua security protocol identifier, TS 33.220 [xy].

## 14.4	Structure of GBA Events

### 14.4.1	Bootstrapping

This event will be generated when the UE triggers a bootstrapping procedure towards the BSF when the UE wants to interact with a NAF. The actual bootstrapping procedure is defined in the TS 33.220 [xy], in sections 4.5.2 and in 5.3.2. The information elements shown in Table 14.4.1 table, if available, will be delivered to the DF2, by the BSF.

Table 14.4.1: Bootstrapping

Observed IMSI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

B-TID

Key lifetime

Bootstrapping time

### 14.4.2	Query from NAF

The Query from NAF event is generated when the BSF receives an application specific key query from a NAF in order to retrieve GBA based application specific keys and related information. A new event is generated for each individual query events. The information elements shown in Table 14.4.2 will be delivered to the DF2, if available, by the BSF.

Table 14.4.2: Query from NAF

Observed IMSI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

Ks_ext_NAF

NAF_Id

Ks_int_NAF

Ks_NAF

Key lifetime

Bootstrapping time

Ua protocol id

### 14.4.3	Start of Interception with GBA key

For start of interception where GBA application specific key is already in use a Start of Interception with GBA key event is generated. The elements, shown in Table 14.4.3 will be delivered to the DF2, if available, by the BSF. A new Start of Interception with GBA key event shall be generated for each individual B-TID the interception target has associated to in the BSF.

Table 14.4.3: Start of Interception with GBA key

Observed IMSI

Observed Other Identity

Event Type

Event Time

Event Date

Network Element Identifier

B-TID

NAF_Id

Ks_ext_NAF

Ks_int_NAF

Ks_NAF

Key lifetime

Bootstrapping time

Ua protocol id

*** END OF MODIFICATION ***