3GPP SA3LI#72 BIS	s3i190110

Dusseldorf, DE (Germany), 26-28 February 2019

Source:	OTD

Title:	Toggle Contribution

Document for:	Agreement

Agenda Item:	 10

Work Item / Release:	LI15

Abstract of the contribution: LI Toggle contribution

## 5.5	Protocols for LI_HI2 and LI_HI3

### 5.5.1 	General

Functions having an LI_HI2 or LI_HI3 interface shall support the use of ETSI TS 102 232-1 [9] and ETSI TS 102 232-7 [10] to realise the interface.

In the event of a conflict between either specification and the present document, the terms of the present document shall apply.

### 5.5.1bis LI Toggles

The MDF2 and MDF3 shall support the following toggles on a per intercept (per LIID) basis in order to deliver the interception related data that an LEA is authorised to receive.  These toggles are not exclusive, i.e., more than one item in any given category can be turned on for any given intercept (LIID).

The LIPF shall be able to provision the following toggles in the MDF2 and MDF3 per intercept (per LIID). These toggles are not exclusive, i.e., more than one item in any given category can be turned on for any given intercept (LIID).

#### 5.5.1bis.1 Service Type

- Voice Service

- Broadband Data

- Messaging (e.g., SMS/MMS)

- Push-to-Talk (including MCPTT)

#### 5.5.1bis.2 Interception Type

- IRI

- CC

#### 5.5.1bis.3 Location

- Do NOT report location

- Report location at the beginning and end of a session

- Report location every time the network detects a change in target location

If an xIRI PDU is received by the MDF2 for a particular LIID containing location information and the toggle for that LIID allows for delivery of the location information (e.g., Report Location at the beginning and end of a session), the MDF2 shall include the received location information along with the other information from the xIRI PDU in the corresponding LI_HI2 message.

If an xIRI PDU is received by the MDF2 for a particular LIID containing location information and the toggle for that LIID does not allow for delivery of the location information (e.g., toggle set to Do NOT report Location), the MDF2 shall remove any location information from the xIRI PDU before its inclusion in any LI_HI2 message.

#### 5.5.1bis.4 Dialled Digit Extraction

- Report Dialled Digits

- Do not report Dialled Digits

#### 5.5.1bis.5 CSP Changes / Roaming

- Continue interception when target is roaming

- Suspend interception when the target is roaming

### 5.5.2 	Usage for realising LI_HI2

The LI_HI2 messages are structured as a header and a payload. The header contains general information like LIID, Timestamp, correlation information (as for example defined in ETSI TS 102 232-1 [9]). The payload contains intercept related information based on information that the MDF2 has received from sources in the network, such as the IRI-POI as described in section 6 and 7 of the present document. Details of the LI_HI2 messages can be found in Annex B of the present document. Messages defined as passing over the LI_HI2 interface shall be passed as the payload of the threeGPP-IRI field (see TS ETSI 102 232 -7 [10] clause 15)

Editor’s Note – ETSI TS 102 232-7 Clause 15 does not yet exist, and will either be introduced by a CR or the text above will be revised.

### 5.5.3 	Usage for realising LI_HI3

The LI_HI3 PDUs are structured as a header and a payload. The header contains general information like LIID, Timestamp, correlation information (as for example defined in ETSI TS 102 232-1 [9]). The payload contains content of communication based on information that the MDF3 has received from sources in the network, such as the CC-POI as described in section 6 and 7 of the present document. Details of the LI_HI3 messages can be found in Annex B of the present document. Messages defined as passing over the LI_HI3 interface shall be passed as the payload of the threeGPPCC field (see ETSI TS 102 232 – 7 [10] clause 15)

Editor’s Note – ETSI TS 102 232-7 Clause 15 does not yet exist, and will either be introduced by a CR or the text above will be revised.