3GPP SA3LI#72 BIS	s3i190140 (was s3i190110)

Dusseldorf, DE (Germany), 26-28 February 2019

Source:	OTD

Title:	Toggle Contribution

Document for:	Agreement

Agenda Item:	 10

Work Item / Release:	LI15

Abstract of the contribution: LI Toggle contribution modified due to comments from Tuesday, Feb 26th meeting.

#### <<<<<<<<<<<<<<<<<<<< FIRST CHANGE >>>>>>>>>>>>>>>>>>>>

## 5.2	Protocols for LI_X1 and LI_T interface

### 5.2.1 	General usage of ETSI TS 103 221-1

Functions having an LI_X1, LI_T2 or LI_T3 interface shall support the use of ETSI TS 103 221-1 [7] to realise the interface.

In the event of a conflict between ETSI TS 103 221-1 [7] and the present document, the terms of the present document shall apply.

The LIPF and MDF2/3 shall maintain a mapping between internal interception identifiers (XIDs) and external interception identifiers (LIIDs), as defined by TS 103 221-1 [7] clause 5.1.2. In the case of multiple interceptions for a single target identifier, it is an implementation decision for the LIPF/TF whether multiple XIDs are used (i.e. a one-to-one mapping between XID and LIID is maintained) or whether the single XID is used and mapped to multiple LIIDs at the MDF2/3.Clauses 6 and 7 give further details for specific networks or services (e.g. minimum supported Target Identifier Formats).

In the event that a request issued over the interface fails, or an error is reported, the LIPF should raise an alert in the appropriate LI O&M system. Further procedures (e.g. retrying a failed request) are left to CSP policy to define.

### 5.2.2 	Usage for realising LI_X1

For the purposes of realising LI_X1 between the LIPF and a POI or TF, the LIPF plays the role of the “ADMF” as defined in the TS 103 221-1 reference model (clause 4.2), and the POI or TF plays the role of the “NE.”

### 5.2.3	Usage for realising LI_X1 (Management)

For the purposes of realising LI_X1 between the LIPF and a Triggered POI, the LIPF plays the role of the “ADMF” as defined in the TS 103 221-1 reference model (clause 4.2), and the Triggered POI plays the role of the “NE.”

### 5.2.4 	Service Scoping

#### 5.2.4.1 General

The CSP shall support the following specified options on a per intercept (per LIID) basis, to ensure strict delivery of only the interception data that an LEA is authorised to receive. These options are not exclusive, i.e., none, one, or more than one item in any given category may be turned on for any given intercept (LIID).

#### 5.2.4.2 CSP Service Type

- Voice

- Data

- Messaging (e.g., SMS/MMS)

- Push-to-Talk (including MCPTT)

#### 5.2.4.3 Interception Type

- IRI

- CC

#### 5.2.4.4 Location

- Report location at the beginning and end of a session

- Report location every time the network detects a change in target location (including location update with no physical change of location)

- LALS

#### 5.2.4.5 Dialled Digit Extraction

- Report Dialled Digits

#### 5.2.4.6 Roaming

- Stop interception for non HPLMN RAN

### 5.2.5	Usage for realising LI_T2

For the purposes of realising LI_T2 between a TF and a Triggered POI, the TF plays the role of the “ADMF” as defined in the TS 103 221-1 reference model (clause 4.2), and the Triggered POI plays the role of the “NE”.

### 5.2.6	Usage for realising LI_T3

For the purposes of realising LI_T3 between a TF and a Triggered POI, the TF plays the role of the “ADMF” as defined in the TS 103 221-1 reference model (clause 4.2), and the Triggered POI plays the role of the “NE”.

<<<<<<<<<<<<<<<<<<<< END of FIRST CHANGE >>>>>>>>>>>>>>>>>>>>