3GPP SA3LI#73	S3i190208

# La Jolla (California), US, 9-12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 009 | rev | 0 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PTC for 33.127 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (OTD) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Rel 16 |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The additon of stage two requirements in 33.127 for the CSP to meet their obligatory requirements for LI of Push to Talk over Cellular (PTC) type services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of stage two PTC LI events and messages for LI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No method for the CSP to deliver LI of PTC services. The CSP cannot meet their obligatory requirements for LI for PTC type services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** Start of First MODIFICATION ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful interception requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	OMA MLP: "Mobile Location Protocol V3.3".

NOTE:	Available at http://www.openmobilealliance.org.

[7]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[8]	ETSI TS 103 221-1: "Lawful Interception (LI); Part 1: Internal Network Interface X1 for Lawful Interception".

[9]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[10]	ETSI GR NFV-SEC 011: "Network Functions Virtualisation (NFV); Security; Report on NFV LI Architecture".

[11]	3GPP TS 33.107: "3G Security; Lawful interception architecture and functions".

[12]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[13]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[AD]	3GPP TS 23.280: "Common functional architecture to support Mission Critical (MC) services; Stage 2"

[AE]	OMA-AD-PoC-V2_1-20110802-A

[AF]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2"

[AG]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT) over LTE; Stage 1"

*** End of First MODIFICATION ***

*** Start of Second MODIFICATION ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

ADMF	LI Administration Function

AMF	Access Management Function

AUSF	Authentication Server Function

CC	Content of Communication

CSI	Cell Supplemental Information

CSP	Communication Service Provider

CUPS	Control and User Plane Separation

DN	Data Network

GPSI	Generic Public Subscription Identifier

IP	Interception Product

IRI	Intercept Related Information

LALS	Lawful Access Location Services

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LICA	Lawful Interception Certificate Authority

LICF	Lawful Interception Control Function

LI_HI1	Lawful Interception Handover Interface 1

LI_HI2	Lawful Interception Handover Interface 2

LI_HI3	Lawful Interception Handover Interface 3

LI_HI4	Lawful Interception Handover Interface 4

LIPF	Lawful Interception Provisioning Function

LIR	Location Immediate Request

LI_SI	Lawful Interception System Information Interface

LI_Xk	Lawful Interception Handover Interface Key Management System

LI_X0		Lawful Interception Internal Interface 0

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X3	Lawful Interception Internal Interface 3

LMF	Location Management Function

LTF	Location Triggering Function

MCPTT	Mission Critical Push To Talk

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

N3IWF	Non 3GPP Inter Working Function

NPLI	Network Provided Location Information

NR	New Radio

NRF	Network Repository Function

NSSF	Network Slice Selection Function

PCF	Policy Control Function

PEI	Permanent Equipment Identifier

PoC	Push to talk Over Cellular

POI	Point Of Interception

PTC	Push to Talk over Cellular

SIRF	System Information Retrieval Function

SMF	Session Management Function

SMSF	SMS-Function

SUCI	Subscriber Concealed Identifier

SUPI	Subscriber Permanent Identifier

TF	Triggering Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UPF	User Plane Function

xCC	LI_X3 Communications Content

xIRI	LI_X2 Intercept Related Information

*** End of Second MODIFICATION ***

*** Start of Third MODIFICATION ***

# X	Invocation of Lawful Interception for Push to Talk over Cellular services.

## X.1	General

In the present clause, "PTC" will be used to reference events or services that occur in either of the two different architectures unless specified otherwise, e.g., MCPTT or PoC.

The LI_HI2 and LI_HI3 interfaces in clause 5.4.1 of the present document as shown in Figure 5.4-1, represent the interfaces between the LEA and two delivery functions.

In Figure 5.4-1 the MDF2 delivery function is used:

-	to convert the information on the LI_X2-interface to the corresponding information on the LI_HI2 interface;

-	to distribute the intercept related information to the relevant LEAs.

In Figure 5.4-1 the MDF3 is responsible:

-	to convert the information on the LI_X3-interface to the corresponding information on the LI_HI3 interface;

-	to distribute the Content of Communication (CC) to the relevant LEAs.

The following servers support PTC architecture;

- MCPTT servers (Common services core [3gPP TS 23.280] [AD] to support mission critical services):

- Group management, Configuration, Migration, Identity, Key, Location.

- PoC servers (Shared XDMS [OMA-AD-PoC-V2_1-20110802-A][AE]):

- Shared List, Group, Policy, Presence, NW PoC Box, Aggregation Proxy, External Media Content server.

- SIP/IP Core servers:

- Presence.

If two or more different parties involved in a PTC communication are each a target of interception, each interception shall operate independently of the others and the results of each intercept shall be delivered to the respective LEMF in accordance with the applicable warrant.

## X.2	Provision of IRI – PTC Service

### X.2.1	Introduction

PoC and MCPTT use similar architectures for service delivery, and LI shall primarily occur at the PoC or MCPTT server for that service. PTC architecture is based on PTC clients, PTC application server, Shared XML Document Management Server (XDMS) or the Common Core Servers (CCS) as shown in Figure X.2.1. These servers can be considered as a means of application configuration setting management as it stores application-specific configuration settings, and these PTC servers handle application-specific tasks such as talk burst control (the reservation of talk burst for one user at a time) and PTC session control. They also provide interfaces to the operator's provisioning and network management systems and create application-specific Charging Detail Records (CDRs). The IMS portion of the network takes care of common functions, such as user authentication for PTC, session routing and generic charging based on SIP. The PTC client is usually software in the User Equipment (UE) but it could be an application also in the Personal Computer (PC).

Figure X.2.1 shows the transfer of xIRI to the MDF2 and delivery of IRI over the LI_HI2 to the LEMF. If an event originating/terminating to a PTC MS occurs, the Shared XDMS/MCPTT common core servers or an event is detected in the Unified Data Management (UDM) function shall send the relevant data to the MDF2 for delivery to the LEA.

For a PTC MS, delivery shall occur in the following cases:

-	when the PoC/MCPTT server detects a  PTC session event (e.g. when receiving a PTC signalling message or sends a PTC signalling message to the PTC target;

-	when the Shared XDMS/CCS servers detects a PTC event (e.g. when receiving a PTC signalling message from the target MS, or when sending a PTC signalling message  to the PTC target.

Figure X.2.1: Provision of Intercept Related Information - PTC

### X.2.2	Decryption for PTC services

This clause describes how the CSP can provide the session encryption keys generated by the key Management System (KMS) to the LEMF when the CSP has PTC services with Security options. If a POI allows interception of IRI or Content of Communication (CC) in clear then this clause does not apply.

Within this document clause 8.X specifies LI architecture and functions needed to provide session encryption keys generated by the SPs KMS to protect IMS media for a subscriber who is a target for interception in the IMS nodes. Similarly, the CSP can provide the session encryption keys generated by the KMS to the LEMF when the CSP has PTC services with Security options. The LI architecture and functions needed for delivery of the encryption parameters are shown below to provide session encryption keys and the specific parameters generated by the KMS.  Once this information is retrieved this can be delivered to the LEMF using the PTC Encryption Message found in clause X.5.23 in order for the LEMF to decrypt the intercepted traffic.

Figure X.2.2.1 shows the LI architecture for the case in which decryption is performed by the LEMF and a KMS is used to support PTC security with a LI_Xk interface defined between the MDF2 and the KMS. Once this security information is retrieved, MDF2 may deliver this security information to the LEMF as IRI.

Figure X.2.2.1: KMS Intercept configuration

## X.3	Provision of Content – PTC Service

The access method for the delivery of PTC Intercept Product is based on duplication of packets without modification at PoC/MCPTT servers. The duplicated packets are sent to the MDF3 for delivery to the LEA, as shown in figure X.3.1, .

Figure X.3.1: Configuration for interception of Content- PTC

## X.4	Provision of Interception

### X.4.1	X3-Interface

In addition to the intercepted content of communications, the following information needs to be transferred from the PTC POI to the MDF3 in order to allow the MDF3 to perform its functionality:

-	target identity;

-	correlation number;

-	time stamp;

-	direction (indicates whether T-PDU is from the target or to the target) - optional;

-	identity of source of media (communications content) for group calls;

-	the target location (if available) or the Interception Area (IA)s in case of location dependent interception;

-	date/time of Location (if target location provided).

### X.4.2	X2-interface

The following information needs to be transferred from the SIP Core, or the UDM to the MDF2 in order to allow a MDF2 to perform its functionality:

-	target identity;

-	events and associated parameters as defined in table X.4.4.1 and X.4.4.2 shall be provided;

-	the target location (if available) or the IAs in case of location dependent interception;

-	date/time of Location (if target location provided);

-	correlation number;

-	encryption parameters (keys and associated parameters for decrypting CC), if available and necessary.

### X.4.3	LI Defined Events

#### X.4.3.1	IRI Defined Events

The LI event information sent to the MDF2 is triggered by different PTC session related and non-call related events/reports. Details for each event are described in the following clauses.

The following events are applicable to both the PoC and MCPTT service POI:

-	PTC Session Initiation;

-	PTC Session Abandon;

-	PTC Session Start;

-	PTC Session End;

-	PTC Start of Interception;

-	PTC Pre-Established Session;

-	PTC Instant Personal Alert;

-	PTC Party Join;

-	PTC Party Drop;

-	PTC Party Hold;

-	PTC Party Retrieve;

-	PTC Media Type Notification;

-	PTC Group Advertisement;

-	PTC Floor Control;

-	PTC Target Presence;

-	PTC Associate Presence;

-	PTC List Management;

-	PTC Access Policy;

-	PTC Group Request;

- 	PTC Encryption.

The following events are applicable to the UDM:

-	PTC Serving System;

-	 subscriber record change;

-	 Cancel location;

-	 Register, reregister or deregister location;

-	 Location information request.

The following events are applicable to the SIP Core:

-	 Service Registration.

A set of elements as shown below can be associated with the events above. The events trigger the transmission of the information from the PTC server POI, or UDM to MDF2. Available IEs from this set of elements as shown below can be extended in the UDM.

#### X.4.3.2	Communication Content (CC) Event

The following event information is sent to the MDF3 when CC is authorized for delivery.  The following are applicable to both the PoC and MCPTT service POI:

PTC Communication Content (CC) Delivery.

### X.4.4	Events Elements

#### X.4.4.1	IRI Event Elements

The LI IRI event information is triggered by PTC session related and non-call related events/reports and sent to the MDF2/MDF3. Details for each event are described in the following clauses.

Within Table X.3.4.1 a provisioned target identity can be a SIP URI, TEL URI, MCPTT ID, Permanent Equipment Identity (PEI) or an IMEI.

A PTC Client may support multiple PTC Addresses and be involved in one or more PTC Sessions at the same time using the same or different PTC Addresses.

A set of elements as shown below can be associated with an event. The events trigger the transmission of the information from a PTC POI to the MDF.

Table X.4.4.1: IRI Information Elements for PTC Event Records

AbandonCause - Identifies the reason for the abandoned PTC Session.

Access_PolicyFailure - Reports the error code or reason for failure when Access Policy Request is unsuccessful.

Access_Policy_Type – Identifies the type of access policy list being managed or queried by the PTC Intercept target.

Access_Policy_event - Identifies the choice the target or an associate makes within access policy selection which can be by user or group to allow or block the incoming PTC session, Auto Answer, or allow or block a conference type.

Ad-hoc PTC Group Session - A PoC Group Session established by a PoC User to PoC Users listed on the invitation. The list includes PoC Users or PoC Groups or both.

Alert indicator - Indicates an emergency alert sent, received or cancelled.

AssociatePresenceStatus - Provides the Associate Presence Status, which is a list of: PresenceID: Identity of PTC Client(s) or PTC group, when known PresenceType: Identifies type of ID [PTC Client(s) or PTC group]. PresenceStatus: Presence state of each ID.

BearerCapability - Identifies the re-negotiated media characteristics (e.g., SDP information, media format, vocoder type) for the PoC User.

Broadcast indicator - Indicates that the group call request is for a broadcast group call.

Charging Correlation ID – PTC supports both subscription based charging and traffic based charging. Provide any charging events, e.g., service activation, correlation ID between the PoC service charging data and the packet data services.

Cipher - The name of the cipher.

Contact_Identity - Identity of the contact in the list, one contact per Contact List or Group List.

CryptoContext - If further information is needed to associate the encryption information with a specific session or stream, this parameter shall be included to identify the context to which this encryption message applies.

Emergency indicator - Indicates the request is an MCPTT emergency call.

Event Date - Date of the event generation in the PTC Server.

Event Time - Time of the event generation in the PTC Server

Event Type - Description of which type of event is delivered: PTC Session Initiation, Abandon, Start, End, End Cause, Start of Interception, Pre-Established Session, Instant Personal Alert, Floor Control, Target Presence, Associate Presence, a PTC Party Join, Party Drop, Party Hold, Party Retrieve, PTC Media Modification, PTC Group Advertisement. Group Request, Group Response, Group Interrogate, PTC Media Type Notification, Bearer Capability, MCPTT Emergency Group Call, Cancel, Alert, State, MCPTT Imminent Peril Group Call. PTC Serving System, PTC List Management and Access Policy.

Failure_Code – The reason or code for the failure or closing of the session.

Floor_Request - Indicates that the originating client requests the floor.

FloorSpeakerIdentity - Identification of the PTC Client that has the Talk Burst.

Group_Ad_Receiver – The group administrator who was the receiver of the group call.

Group_Ad_Sender – The group administrator who was the originator of the group call.

Group_Identity - Identifies the PTC Group Identity, Nick Name, and characteristics.

GroupAuthorizationRules - Identifies the action requested by the target to the PTC Group authorization rules.

Hold_ Indication – Identifies the PTC Session was placed on hold (deactivate Media Bursts).

Hold_user – Identifies the PTC user who placed their PTC Session on hold.

Implicit floor request - When originating client requests the floor.

InitationCause - The network receives an invitation from the PTC Intercept Target to initiate a PTC session.

Invited_PTC_Client - A PTC Client that is invited to a PTC Session

Inviting_PTC_user – The PTC User who has been invited to a PTC Session.

Key - The key needed to decipher.

KeyEncoding - Shall be included to provide the encoding of the key if the encoding is other than binary

IPAPartyIdentity - Instant Personal Alert - Identifies the party that receives the Instant Personal Alert from the PTC Intercept target or the associate that sends the Instant Personal Alert to the PTC Intercept target.

Join_PTC_user – Identity of the PTC User who has joined the session, i.e., associate identity or targets.

ListManagmentAction –Identifies the action requested by the PTC Intercept target to the Contact Lists or Group Lists. Identifies the PTC-specific documents stored in the network that the target attempts to modify or that changes were made to the targets PTC-specific documents stored in the network and identifies what action was taken by the target or the associate i.e., create, modify, retrieve, delete, notify.

List_ManagementFailure - Reports the error code or reason for failure when List Management modifications should fail, when known i.e., not authorized, time out, etc.

ListManagementType – Different PTC Group lists: ContactListManagementAttempt, GroupListManagementAttempt, and GroupListManagementResult. Identifies the PTC-specific documents stored in the network that the target attempts to modify or that changes were made to the targets PTC-specific documents stored in the network and identifies which list was modified i.e., list or group.

Location - Include when reporting of the PTC Intercept Target’s location. Indicates the location of the target.

Time of Location - Date/Time of location. The time when location was obtained by the location source node.

Max_TB_Time - Include the maximum duration value for the talkburst before the permission is revoked, provide when known.

MCPTT group ID - The Mission Critical Push To Talk group Identity.

MCPTT ID - Mission Critical Push To Talk identity.

MCPTT Imminent_peril_indicator - Indicates that the PTC call is an imminent peril type call.

MCPTT indicator – Indicates direction of the received request as either from the client or from the group to the client.

MCPTT Organization name – Name of the organization that the Mission Critical device belongs to.

MediaStream_Availability – Indicates if the PTC intercept target’s PTC Client is not able/willing to receive media streams immediately. Provide when Pre-established session is established.

Network Element Identifier - Unique identifier for the network element reporting the event.

Observed IMEI - The provisioned International Mobile Equipment Identity target identity.

Observed SIP URI - The provisioned target identity can be a SIP URI

Observed TEL URI - The provisioned target identity can be a TEL URI

Party_Drop - Member of a PTC Group Session and leaves the PTC Session, provide when known.

PreEstablishedSessionID – Identifies the PTC Pre-established Session.

PreEstablishedStatus – Indicates if the Pre-Established Session is established (setup completed), modified, or released.

PTCCorrelationId - Uniquely identifies the PTC Session, correlates CII messages, and correlates CII and CC messages.

PTC group ID - The PTC group ID of the group on which the call is initiated.

PTCHost - Indentifies the PTC participant who has authority to initiate and administrate an active PTC Group Session. Provide when known.

PTC_ID_List - The list of PTC IDs of the PTC group members.

PTCOriginatingId - Identifies the originating party. Provided when known.

PTCOther - Other information that is required to decrypt the data.

PTCParticipants - Identifies the invited PTC participants, when known, if other than the PTC Intercept Target.

PTC Party - Join/Drop/Hold/Retrieve, MCPPT emergency/imminent group/peril etc.

PTCSessionInfo - Provides PTC Session information such as PTC Session URI, PTC Session type, and Nickname.

PTCUserAccessPolicy – Identifies the action requested by the PTC Intercept Target related to the PTC user access policy.

Queued_FloorControl - Indicates if queuing is supported by the PTC Server and the intercept target’s PTC Client.

Queued_Position_Status - If queued floor control is supported, indicates the queue position.

RegistrationRequest - Identifies the type of registration request (e.g., register, reregister, deregister).

RegistrationOutcome - Identifies success or failure of registration and the failure reason.

Retrieve_Indication - Identifies if the PTC Session is retrieved from hold.

RTP_Setting – The IP address and the port number at the PTC Server for the RTP Session.

Salt - Include to provide the initial salt value if the cipher requires a salt value.

SDP – Answer, offer and SDP parameter negotiations. Report when known.

Serving System Address - Identifies the serving system when the PoC Intercept Subject is authorized for access network service provided by the TSP.

SIP_message_header - Answer, offer and SIP parameter negotiations. Report when known.

TalkburstControl_Setting – The offered Talk Burst Control Protocol, e.g., Talk Burst parameter(s) and the port numbers. Provide when Pre-established session is established.

TargetIdentity - The PTC identifier for the PTC Intercept Target.

TargetPresenceStatus - PTC-related presence information of the PTC intercept target.

Talk_burst_priority - If more than one level of priority is supported, indicates the talk burst priority level of the PTC Client.

Talk_burst_reason_code – Identifies the reason code for the denial or revoke of a talk burst.

TBCP_Deny - Indicates that the PTC Server has notified a PTC Client that it has been denied permission to send a Talk Burst.

TBCP_Granted - Indicates that the PTC Server has notified the PTC Client that it has been granted permission to send a Talk Burst.

TBCP_Idle - Used by the PTC Server to notify all PTC Clients that no one has the permission to send a Talk Burst at the moment and that it may accept the TBCP Talk Burst Request message.

TBCP_Queued - Indicates the request to talk is queued, if queued floor control is supported. Include identification of the PTC Client that has the queued Talk Burst, if known.

TBCP_Release - Indicates the request to talk has completed.

TBCP_Request - Indicates that the PTC Client has requested permission from the PTC Server to send a Talk Burst.

TBCP_Revoke - Indicates that the PTC Server has revoked the media resource from a PTC Client and can be used for preemption functionality, but is also used by the system to prevent overly long use of the media resource.

TBCP_Taken - Indicates that the PTC Server has notified all PTC Clients, except the PTC Client that has been given permission to send a Talk Burst, that another PTC Client has been given permission to send a Talk Burst.

Time of Location - Date/Time of location. The time when location was obtained by the location source node.

#### X.4.4.2	CC Event Elements

The following event information is sent to the MDF3 when CC is authorized for delivery. The following events are applicable to both the PoC and MCPTT service POI.

Table X.4.4.2: CC Information Elements

Observed IMEI - The provisioned International Mobile Equipment Identity target identity.

Observed SIP URI - The provisioned target identity can be a SIP URI

Observed TEL URI - The provisioned target identity can be a TEL URI

MCPTT ID - Mission Critical Push To Talk identity.

Event Date - Date of the event generation in the PTC Server.

Event Time - Time of the event generation in the PTC Server.

Event Type - Description of which type of event is delivered: PTC Communication Content.

Network Element Identifier - Unique identifier for the network element reporting the event.

PTC group ID - The PTC group ID of the group on which the call is initiated.

MCPTT group ID - The Mission Critical Push To Talk group Identity.

PTCSessionInfo - Provides PTC Session information such as PTC Session URI, PTC Session type, and Nickname.

PTCCorrelationId - Uniquely identifies the PTC Session, correlates CII messages, and correlates CII and CC messages.

Location – Indicates the location of the target (if authorized)

Time of Location - Date/Time of location. The time when location was obtained by the location source node.

PTC CC Payload – The intercepted communications content encapsulated packet of a PTC session.

#### X.4.4.3	Location information provided for PTC Service

The CSP shall report the most accurate location available for the target when location is provided for all PTC events. The following is a list of additional conditions applicable when Location information is to be provided:

- If a Cell Identity is provided for the UE's location only, the SP is obligated to provide any Cell Site information available for the Cell serving the target.

- When location information is available, or modified, in the Access and Mobility Management function (AMF)/Unified Data Management (UDM) for the PTC target, this location information shall be delivered to the MDF.

## X.5	PTC Surveillance Events

### X.5.1	PTC General

PTC Service events defined below are using the PTC Surveillance Events from Table X.4.4.1. These events and associated event information are delivered from the POI to the MDF2 for either type of service provided by the SP, i.e. PoC or MCPTT.

### X.5.2	PTC Service Registration

The PTC Service Registration event occurs when the target registers, reregisters, deregisters for a PTC service, or there is a change to the targets access network, regardless of whether it is successful or unsuccessful.

Table X.5.2: PTC Service Registration

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event Type

Event Time

Event Date

Network Element Identifier

Any other IMPU or IMPI of the target (if available)

RegistrationRequest

RegistrationOutcome

### X.5.3	PTC Serving System

A PTC Serving System event is generated when there is a change to the SP serving the PTC target access network (i.e. for mobility).

Table X.5.3: PTC Serving System

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

Serving System Address (Attribute-Value Pairs (AVP) name such as Visited-PLMN-Id)

Any other IMPU or IMPI of the target (if available)

### X.5.4	PTC Session Initiation

A PTC Session Initiation event occurs when the PTC target initiates a session or the target receives an invitation to join a session regardless of the success or the final disposition of the invitation. The PTC client may use the pre-established session for PTC session Initiation.

Table X.5.4: PTC Session Initiation

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

PTCHost

PTCOriginatingID

PTCParticipants

AssociatePresenceStatus

Location

Time of Location

InitationCause

BearerCapability

### X.5.5	PTC Session Abandon

The PTC Session Abandon event is triggered when the PTC Session is not established and the request is abandoned before the PTC Session established end to end connectivity.

Table X.5.5: PTC Session Abandon

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

PTCOriginatingId

PTCParticipants

AssociatePresenceStatus:

Location

Time of Location

AbandonCause

### X.5.6	PTC Session Start

This event occurs when a PTC Session (e.g. One-to-One, One-to-Many, or One-to-Many-to-One) is answered, and voice communication begins. The PTC client may use the pre-established session for PTC session Start.

Table X.5.6: PTC Session Start

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

PTCHost

PTCParticipants

AssociatePresenceStatus:

Location

Time of Location

BearerCapability

### X.5.7	PTC Session End

The PTC Session End event occurs when the PTC Session is released for any reason (i.e. normal or abnormal release) and voice communications ends. The PTC client may have a pre-established session, which the Participating PTC Server responds to the release request.

Table X.5.7: PTC Session End

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

PTCParticipants

Location

Time of Location

Cause

### X.5.8	PTC Start of Interception

This event occurs when interception is started and there is an on-going PTC session. The PTC client may have a pre-established session when interception is started.

Table X.5.8: PTC Start of Interception

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

PTCOriginatingID

PTCHost

PTCParticipants

BearerCapability

PTCCorrelationID

### X.5.9	PTC Pre-Established Session

This event occurs when a pre-established session is setup between the PTC service client present within the target’s UE and the PTC server associated with the PTC client. The PTC client may use the pre-established session for originating or terminating calls after pre-established session establishment, regardless of whether the PTC target is actively transmitting or receiving talk bursts.

Table X.5.9: PTC Pre-Established Session

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

PTC_Server_URI

RTP_Setting

PTCHost

PTC_Media_Capability

PreEstablishSessionId

PreEstablishedStatus

TalkburstControl_Setting

MediaStream_Availability

Location

Time of Location

BearerCapability

PTCCorrelationID

### X.5.10	PTC Instant Personal Alert

This event occurs when an Instant Personal Alert (i.e. a request for one PTC subscriber to initiate a PTC Session is initiated or at a later time) or sent to the PTC target.

Table X.5.10: PTC Instant Personal Alert

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

IPAPartyIdentity

### X.5.11	PTC Party Join event

This event occurs when a request to join (or re-joins) a PTC Group Session (i.e. Chat Group) that is already in progress is received from the PTC target or an associate joins (or re-joins) and the PTC Intercept target or the PTC service is made aware of the action.

Table X.5.11: PTC Party Join event

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationId

PTCHost

PTCSessionInfo

TargetIdentity

Inviting_PTC_user

Join_PTC_user

AssociatePresenceStatus

BearerCapability

### X.5.12	PTC Party Drop

This occurs when the target is a participating member of a PTC Group Session and leaves the PTC Session in which the PTC target is also participating.

Table X.5.12: PTC Party Drop

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

Party_Drop

AssociatePresenceStatus:

### X.5.13	PTC Party Hold

A PTC Party Hold event occurs when the target places an on-going PTC Session on hold in on-going PTC Session.

Table X.5.13: PTC Party Hold

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCHost

PTCSessionInfo

TargetIdentity

Hold_ Indication

Hold_User

### X.5.14	PTC Party Retrieve

A PTC Party Retrieve event occurs when the target retrieves an on-going PTC Session.

Table X.5.14: PTC Party Retrieve

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCHost

PTCSessionInfo

TargetIdentity

Retrieve_Indication

Retrieve_User

### X.5.15	PTC Media Modification

During the PTC Session, a PTC Client may modify the voice frame packetization or voice codec mode by Out-of-band signalling using SDP payload within SIP messages. The Media Modification event is generated when a re-negotiation of the media parameters occurs during a PTC Session involving the target MS.

Table X.5.15: PTC Media Modification

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTCCorrelationID

PTCSessionInfo

TargetIdentity

BearerCapability

### X.5.16	PTC Group Advertisement

This event is generated when a PTC Intercept Target sends Group Advertisement information to a single PTC user, a list of PTC users or to all members of the Group using the Group Identity.

Table X.5.16: PTC Group Advertisement

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

Group_Identity

Group_Ad_Sender

Group_Ad_Receiver

PTCHost

Group_Characteristics

### X.5.17	PTC Floor Control

Floor Control arbitrates requests from the PTC Clients for the right to send media (i.e. the right to speak).  Note, the term “Floor Control” is used to mean the same as term “Talk Burst Control”. When the PTC target is participating in a PTC Session, a Floor Control event is generated when the target requests to speak (e.g., presses the PTT mechanism) or the target is given permission to speak in response to a request (e.g. the network responds positively to the PTC Subscriber’s request) or is refused the request to speak until the associate is finished speaking (e.g. the PTC Intercept associate releases the PTT mechanism).

Table X.5.17: PTC Floor Control

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

PTCCorrelationID

PTCHost

PTCSessionInfo

FloorActivity Choice of: TBCP_Request TBCP_Granted TBCP_Deny TBCP_Queued TBCP_Release TBCP_Revoke

FloorSpeakerIdentity

Queued_FloorControl

Queued_Position_Status

Max_TB_Time

Talk_burst_priority

Talk_burst_reason_code

### X.5.18	PTC Target Presence

If the Presence functionality is supported by the PTC Server and the PTC Server assumes the role of a Presence Source, this event is generated when the PTC Server publishes network presence information to the Presence server on behalf of PTC target.

Table X.5.18: PTC Target Presence

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

TargetPresenceStatus

### X.5.19	PTC Associate Presence

This event is generated when the PTC Server receives presence status notifications from the Presence Servers after having subscribed to the PTC presence status of other PTC Clients (i.e. Associates of the PTC Intercept target).

Table X.5.19: PTC Associate Presence

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

AssociatePresenceStatus:

### X.5.20	PTC List Management Events

This event is generated when the target PTC Client attempts to change their own contact list or their own PTC Group list(s). This event is also generated when the network notifies the Intercept target’s PTC Client of changes made to their PTC-specific documents stored in the network (i.e. contact lists or PTC Group lists).

Table X.5.20: PTC List Management Events

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

SIP message header offer

SIP message header answer

SDP

TargetIdentity

ListManagementType Choice of: ContactListManagementAttempt GroupListManagementAttempt ContactListManagementResult GroupListManagementResult Request unsuccessful

ListManagmentAction Choice of: Create Modify Retrieve Delete Notify

Contact_Identity

Group_Identity

PTCHost

List_ManagementFailure

### X.5.21	PTC Access Policy event

This event is generated when the PTC Intercept target attempts to change the access control lists (e.g. PTC user access policy and PTC Group authorization rules).  In addition this event is generated when the network responds to a modification or query by the PTC Intercept target to the access control lists (e.g. PTC user access policy and PTC Group authorization rules).

Table X.5.21: PTC Access Policy event

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

TargetIdentity

Access_Policy_Type Choice of: PTCUserAccessPolicyAttempt GroupAuthorizationRulesAttempt PTCUserAccessPolicyQuery GroupAuthorizationRulesQuery PTCUserAccessPolicyResult GroupAuthorizationRulesResult Request unsuccessful

PTCUserAccessPolicy Choice of: Allow_Incoming_PTC_Session_request Block_Incoming_PTC_Session_request Allow_Auto_Answer_Mode Allow_Override_Manual_Answer_Mode

Access_Policy_event

GroupAuthorizationRules Choice of: Allow_Initiating_Conference Block_Initiating_Conference Allow_Joining_Conference Block_Joining_Conference Allow_Add_Participants Block_Add_Participants Allow_Subscription_Conference_State Block_Subscription_Conference_State Allow_Anonymity Forbid_Anonymity

Contact_Identity

Group_Identity

PTCHost

Access_PolicyFailure

### X.5.22	PTC Media Type Notification

This event is generated for media detected at the POI for media types other than PTC speech (e.g. video, images, text, and files) directed to/from the targets PTC client. Media Types are either real-time or non-real time, i.e., Audio (e.g. music), Video Discrete Media (e.g. still image, formatted and non-formatted text, file), or Real Time Streaming Media (RTSP). Media parameters are SIP/SDP based information exchanged between the PTC server and the targets PTC client, between the PTC server and the PoC Box and between PTC servers that specify the characteristics of the Media for a PTC session being established or a PTC session that already exists.

Table X.5.22: PTC Media Type Notification

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

SIP message header offer

SIP message header answer

SDP

PTCCorrelationID

PTCSessionInfo

TargetIdentity

BearerCapability

### X.5.23	PTC Encryption Message

The CSP shall provide the encryption method, specific parameters and the encryption keys to LE when a CSP service provides encryption that is provided or managed by the CSP.

The Encryption message is sent by the MDF2 to the LEMF when there is a need to pass the decryption information associated with intercepted content. If rekeying is deployed, one or more new Encryption messages are sent coincident with the change in keys.

Table X.5.23 Encryption Message Parameters

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event Type

Event Time

Event Date

CryptoContext

Cipher

Key

Salt

KeyEncoding

PTCOther

## X.6	PTC Group Calls

### X.6.1	General

A PTC Group Session supports a One-to-One, One-to-Many, or One-to-Many-to-One with the following events; Session initiation request/response, Session modification, joining/leaving, termination, voice communication begins, ends, or forced disconnected. When detected at the POI, these events can originate from the targets PTC Client to the PTC Server or from the PTC Server to the targets PTC Client or PTC server to PTC Server on the behalf of the target.

### X.6.2	Group Call Request

This event is generated when received at the PTC server serving the target or sent to the targets PTC client for a PTC Group Call request to join, re-join, or release of the group call. This can be a Group Call Request event received at the PTC Server serving the target from a separate PTC server (outside the SP architecture) to the target. This event would be generated for each instance as described.

Table X.6.2: Group Call Request

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTCHost

PTC_ID_list

SDP

Floor Request

Broadcast indicator

### X.6.3	Group Call Response

A Group Call Response event is generated upon sending a group call response to the target or received at the PTC Server from the target or on behalf of the target to a separate PTC server (outside the SP architecture).

Table X.6.3: Group Call Response

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

SDP

### X.6.4	PTC Group Interrogate

This event is generated when a group interrogate request or a response is received at the PTC Server serving the target.

Table X.6.4: Group Interrogate

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTC ID list

### X.6.5	Ad Hoc PTC Group Session

As Ad Hoc PTC sessions have no fixed membership and exist only for the duration of the PTC Session, this event shall be reported when the PTC server supports Ad Hoc group sessions.  An Ad Hoc PTC Session is established when a PTC Subscriber selects and invites more than one other PTC Subscriber, a one-to-many session.  A temporary Ad Hoc PTC Group is formed when a PTC Subscriber selects more than one person from his contact list, or one or more Groups, or any combination thereof. The Ad Hoc Group exists only for the duration of the particular PTC Session for which the group was formed. This event is generated when received at the PTC server serving the target or sent to the targets PTC client to join, re-join, or release of an Ad Hoc group call. Any participant in an Ad Hoc PTC Group Session may invite other PTC Subscribers to join that PTC Session, and thus would require a PTC Party Join message clause X.5.11 to be delivered to the MDF identifying the associate who has joined the call when this occurs.

The PTC Server, maintains and caches the list of attendants for the duration of the Ad-hoc PoC Group Session and will cache the list for a time period according to local policy after the Ad-hoc PoC Group Session release.

Table X.6.5: Ad-hoc PTC Group Session

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC group ID

PTC ID list

SDP

Floor Request

Broadcast indicator

## X.7	MCPTT Priority Calls and Alerts Messages

### X.7.1	Background

MCPTT emergency group calls are defined by TS 23.379 [AG] and TS 22.179 [AG]. Group calls are enabled in both on-network and off-network but interception of these events are at the on-network MCPTT Server serving the target. Off-network MCPTT interception is for future study (FFS). If there are multiple events for or to the target for each of these separate events detected an event report is generated.

### X.7.2	General

A MCPTT Emergency Group Session can support a One-to-One, One-to-Many, or One-to-Many-to-One with the following events; Group Call initiation request/response, Group Call Session modification, joining/leaving, termination, voice communication begins, ends, or forced disconnected, an Imminent Peril Group Call or alerts. When detected at the POI, these events can originate from the targets MCPTT Client to the MCPTT Server or from the MCPTT Server to the targets MCPTT Client or the targets serving MCPTT server to another domain MCPTT Server on the behalf of the target.

### X.7.3	MCPTT Emergency Group Call

When the MCPTT Emergency Group Call Request or Response is detected at the MCPTT Server, it can originate from the targets MCPTT client or be sent to the targets MCPTT client from a MCPTT Group.

Table X.7.3: Emergency Group Call

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

Emergency indicator

Alert indicator

MCPTT group ID

MCPTT indicator

PTCCorrelationID

Location

Time of Location

### X.7.4	MCPTT Emergency Group Call Cancel

When a MCPTT Emergency Group Call Cancel is detected at the MCPTT Server, it can originate from the targets MCPTT client or towards the targets MCPTT client from a MCPTT Group.

TableX.7.4: Emergency Group Call Cancel

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

Emergency indicator

Alert indicator

MCPTT group ID

MCPTT indicator

PTCCorrelationID

Location

Time of Location

### X.7.5	MCPTT Emergency Group Alert

When a MCPTT Emergency Alert Notification, Response, Request or Cancel is detected at the MCPTT Server, it can originate from the targets MCPTT client or towards the targets MCPTT client from a MCPTT Group.

Table X.7.5: Emergency Group Alert

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

MCPTT indicator

MCPTT Organization name

PTCCorrelationID

Location

Time of Location

### X.7.6	MCPTT Emergency Group State

When the MCPTT Emergency State event to Inform, Response, Request or Cancel is detected at the MCPTT Server, it can originate from the targets MCPTT client or to the targets MCPTT client from a MCPTT Group.

Table X.7.6: Emergency Group State

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

MCPTT indicator

Client emergency state response (See NOTE)

Client emergency state cancel inform (See NOTE))

Client emergency state cancel response (See NOTE)

Group emergency condition inform (See NOTE)

Group emergency condition response (See NOTE)

Group emergency condition cancel request (See NOTE)

Group emergency condition cancel response (See NOTE)

NOTE: At least one of these information elements shall be present

### X.7.7	MCPTT Imminent Peril Group Call

When the MCPTT Imminent Peril Group Call Request, Response or Cancel is detected at the MCPTT Server, it can originate from the targets MCPTT client or to the targets MCPTT client from a MCPTT Group.

Table X.7.7: Imminent Peril Group Call

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

MCPTT group ID

MCPTT indicator

MCPTT Imminent_peril_indicator

PTCCorrelationID

Location

Time of Location

## X.8	PTC Communication Content (CC)

### X.8.1	General

When communication content (CC) delivery is authorized the CSP shall access and deliver communication content for the target for the duration of any of the different types of PTC sessions (i.e., One-to-One, One-to-Many, or One-to-Many-to-One, MCPTT or Private Calls). Any CC that are originated by, redirected by and terminated to the surveillance target's equipment, facilities, or service when the surveillance target is part of the PTC session or the target is connected to the PTC Session under surveillance shall be delivered to the MDF3 as identified parameters in table X.4.4.2.

### X.8.2	Communication Content (CC)

The Communication Content event is used to encapsulate communications content packets for transfer over the interface to the MDF3, in accordance with this standard.

Table X.8.2: Communication Content

Observed SIP URI

Observed TEL URI

Observed IMEI

PEI

MCPTT ID

Event type

Event Time

Event Date

Network Element Identifier

PTC Group ID

MCPTT group ID

PTCSessionInfo

PTCCorrelationID

Location

Time of Location

PTC CC Payload

*** End of Third MODIFICATION ***

*** End of All MODIFICATIONS ***