3GPP SA3LI#97	s3i250177

# 29 April – 02 May 2025, Washington DC (US)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0288 | rev | - | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications and corrections on MCPTT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (ZITiS) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2025-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specification there are some restrictions and descriptions which may lead to a wrong interpretation. This CR is intended to provide clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction and clarification within MCPTT specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete or even wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.4, 7.6.6, 7.6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF FIRST CHANGE ***

### 7.6.4	Common IRI parameters

Each xIRI shall include at the minimum the following information:

-	Target identity.

-	Time stamp.

-	Correlation information.

-	Location information.

-	PTC related information (e.g., PTC group ID, PTC party).

-	Encryption parameters (if available).

-	Direction.

*** END OF FIRST CHANGE ***

*** SECOND CHANGE ***

### 7.6.6	Common CC parameters

In addition to the intercepted content of communications, the following information needs to be transferred from the CC-POI to the MDF3 in order to allow the MDF3 to perform its functionality:

-	Target identity.

-	Correlation information.

-	Time stamp.

-	Identity of source of media.

*** END OF SECOND CHANGE ***

*** THIRD CHANGE ***

### 7.6.8	Network topologies

Currently, interception within the VPLMN is not considered.

*** END OF THIRD CHANGE ***

*** END OF LAST CHANGE ***