3GPP TSG-SA3 Meeting #97-LI	s3i250197

# Washington, United States, 29th Apr 2025 - 2nd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0284 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI for MCData services (stage 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Define LI for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | LI for MCData Stage 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory issue for CSPs with MCData services |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 7.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | CR 0283 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i250138; s3i250175 |  |  |  |  |  |  |  |  |

*** START OF FIRST CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful interception requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	OMA-TS-MLP-V3_5-20181211-C: "Open Mobile Alliance; Mobile Location Protocol, Candidate Version 3.5", https://www.openmobilealliance.org/release/MLS/V1_4-20181211-C/OMA-TS-MLP-V3_5-20181211-C.pdf".

[7]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[8]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1 ".

[9]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[10]	ETSI GR NFV-SEC 011: "Network Functions Virtualisation (NFV); Security; Report on NFV LI Architecture".

[11]	3GPP TS 33.107: "3G Security; Lawful interception architecture and functions".

[12]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[13]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[14]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[15]	3GPP TS 33.128: "Protocol and Procedures for Lawful Interception; Stage 3".

[16]	ETSI TS 103 221-2: " Lawful Interception (LI); Internal Network Interfaces; Part 2: X2/X3".

[17]	MMS Architecture OMA-AD-MMS-V1_3-20110913-A.

[18]	Multimedia Messaging Service Encapsulation Protocol OMA-TS-MMS_ENC-V1_3-20110913-A.

[19]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[20]	ETSI GS NFV-IFA 026: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Architecture enhancement for Security Management Specification".

[21]	3GPP TS 33.108: "Handover Interface for Lawful Interception (LI)".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for 
Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[25]	OMA-AD-PoC-V2_1-20110802-A: "Push to talk over Cellular (PoC) Architecture".

[26]	GSMA IR.92: "IMS Profile for Voice and SMS".

[27]	GSMA NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[28]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[29]	ETSI GS NFV-SEC 012: "Network Functions Virtualisation (NFV) Release 3; Security; System architecture specification for execution of sensitive NFV components".

[30]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[31]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage3".

[32]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[33]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[34]	OMA-AD-CPM-V2_2-20170926-C: "Open Mobile Alliance, OMA Converged IP Messaging System Description", http://www.openmobilealliance.org/release/CPM/V2_2-20200907-C/OMA-AD-CPM-V2_2-20170926-C.pdf.

[35]	GSMA RCC.07: "Rich Communication Suite – Advanced Communications Services and Client Specification".

[36]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[37]	IETF RFC 6714: "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[38]	IETF RFC 3862: "Common Presence and Instant Messaging (CPIM): Message Format".

[39]	3GPP TS 24.229: "IP Multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[40]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[41]	IETF RFC 8946: "Personal Assertion Token (PASSporT) Extension for Diverted Calls".

[42]	IETF draft-ietf-stir-passport-rcd-26, "PASSporT Extension for Rich Call Data".

NOTE:	The above document cannot be formally referenced until it is published as an RFC.

[43]	IETF RFC 7095: "jCard: The JSON Format for vCard".

[44]	3GPP TS 24.196: "Enhanced Calling Name (eCNAM)".

[45]	IETF RFC 8816: "Secure Telephone Identity Revisited (STIR) Out-of-Band Architecture and Use Cases".

[46]	IETF RFC 9475: "Messaging Use Cases and Extensions for Secure Telephone Identity Revisited (STIR)".

[47]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[48]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[49]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[50]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[51]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[52]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[53]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[54]	3GPP TS 29.272: "Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[55]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[56]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[57]	3GPP TS 36.455: "LTE Positioning Protocol A (LPPa)".

[58]	3GPP TS 23.140: "Multimedia Messaging Protocol. Functional Description. Stage 2".

[59]	3GPP TS 32.303: "Proximity-based Services (ProSe); Stage 2".

[60]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[61]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[Y]	3GPP TS 23.282: " Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

*** END OF FIRST CHANGE ***

*** START OF SECOND CHANGE ***

## 7.Y	MCData service

### 7.Y.1	General

MCData is a Mission Critical (MC) service. MCData service architecture consists of MCData server, Common Services Core (CSC) servers and MCData Content server.

The MCData server provides support for one-to-one communication and group communication for the following services for on-network:

-	Short data service (SDS) as defined in TS 23.282 [Y] clause 6.5:

-	Standalone SDS using signaling control plane.

-	Standalone SDS using user media plane.

-	SDS session.

-	File distribution (FD) as defined in TS 23.282 [Y] clause 6.6:

-	FD using HTTP.

-	FD using media plane.

-	Data streaming as defined in TS 23.282 [Y] clause 6.7.

-	IP connectivity as defined in TS 23.282 [Y] clause 6.8.

The CSC servers provide configuration and management functions for MCData and have been introduced in 7.X.1

The MCData Content server supports file upload, URI generation, and file download for FD using HTTP.

MCData service has many similarities with the RCS service.

### 7.Y.2	Architecture

The MCData server shall have LI capabilities to generate xIRI records and xCC when MCData events related to the target UE are handled. The CSC servers shall have LI capabilities to generate xIRI records when CSC events related to target UE are handled. The MCData Content server shall have LI capabilities to generate xIRI records and xCC when the target UE uploads or downloads a file and when any user downloads a file previously uploaded by a target. Figure 7.Y.2-1 provides a reference point representation of the LI architecture for MCData interception.

Figure 7.Y.2-1: LI architecture for MCData

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides it to the LIPF.

The LIPF present in the ADMF provisions the IRI-POI and the IRI-TF in the MCData server, the IRI-POI in any CSC server, the MDF2 and the MDF3 over the LI_X1 interfaces.

To enable the interception of the target's message contents (e.g. when the warrant requires the interception of communication contents), the CC-POI and the CC-TF present in the MCData server are also provisioned with the intercept data.

NOTE 1:	The IRI-POI, CC-POI, IRI-TF and CC-TF represented in figure 7.Y.2-1 are logical functions and require correlation information be shared between them; they may be handled by the same process within the NF.

NOTE 2:	The IRI-POI, CC-POI represented in figure 7.Y.2-1 with LI_T2, LI_T3, and LI_X1 (Management) interfaces are the triggered POIs and are logically separate from the non-triggered IRI-POI, CC-POI represented in figure 7.Y.2-1 with LI_X1 interface.

The IRI-POI present in the MCData server detects SIP signalling exchange for MCData services, generates and delivers the related xIRI to the MDF2 over LI_X2. The MDF2 delivers the IRI messages to the LEMF over LI_HI2.

When the IRI-TF present in the MCData server detects a URI for FD using HTTP, the IRI-TF present in the MCData server sends a trigger to the IRI-POI in the MCData Content server over the LI_T2 interface.

The IRI-POI present in the MCData Content server detects file uploads and downloads relating to FD using HTTP, generates and delivers the related xIRI to the MDF2 over LI_X2. The MDF2 delivers the IRI messages to the LEMF over LI_HI2.

When interception of communication contents is required, the CC-TF present in the MCData server sends a trigger to the CC-POI present in the MCData Content server over the LI_T3 interface.

The trigger sent from the IRI-TF to the IRI-POI or the CC-TF to the CC-POI includes the following information:

-	File detection rules.

-	Correlation information.

-	MDF3 address.

The CC-POI present in the MCData Content server generates the xCC from the uploaded file and delivers the xCC (that includes the correlation number and the target identity) to the MDF3. The MDF3 delivers the CC to the LEMF over LI_HI3.

### 7.Y.3	Target identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI and CC-POI present in the MCData server:

-	MCData ID.

-	Instance Identifier URN.

-	IMPU.

-	IMPI.

-	MCData group ID.

The interception performed on the above identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the CSC servers:

-	MC ID.

-	MCData ID.

-	Instance Identifier URN.

-	IMPU.

-	MCData group ID.

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI and CC-POI present in the MCData Content server:

-	MCData ID.

-	MCData group ID.

### 7.Y.4	IRI events

The IRI-POI present in the MCData server shall generate xIRI when it detects the following specific events or information:

-	Encapsulated SIP message.

-	Start of interception with already registered UE.

-	Start of interception with already established session.

The IRI-POI present in each CSC server shall generate xIRI when it detects the following specific events or information:

-	Management message.

The IRI-POI present in the MCData Content server shall generate xIRI when it detects the following specific events or information:

-	File distribution.

The encapsulated SIP message xIRI is generated when the IRI-POI present in an MCData server detects that a SIP message is received from, or sent to, a target or processed on behalf of a target at the MCData server.

The start of interception with already registered UE xIRI is generated when the IRI-POI present in an MCData server detects that interception is activated on the target UE that is already registered for MCData services.

The start of interception with already established session xIRI is generated when the IRI-POI present in an MCData server detects that interception is activated on a target UE that has an already established MCData session. When a target UE has multiple MCData sessions, this xIRI shall be sent for each MCData session with a different value of correlation information.

The management message xIRI is generated when the IRI-POI present in any CSC server detects that messages are exchanged between the target MCData UE and CSC server.

The file distribution xIRI is generated when the IRI-POI present in the MCData Content server detects that a target UE has uploaded a file, or a target has downloaded a file or when any UE has downloaded a file previously uploaded by a target UE.

### 7.Y.5	Network topologies

LI for the MCData service is provided by the MCData service provider which manages the MCData server.

*** END OF SECOND CHANGE ***