3GPP SA3LI#97	s3i250201

# 29 April – 02 May 2025, Washington DC (US)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0726 | rev | 1 | Current version: | 17.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AMFIdentifierDeassociation and MMEIdentifierDeassociation xIRIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ericsson, EVE Compliancy Solutions) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TS AMFIdentifierAssociation and MMEIdentifierAssociation xIRIs are specified for the IRI POI respectively in AMF and MME. In scenarios in which a deassociation between temporary and permanent identifies occurs without any new association, there is no proper xIRI specified to report the deassociation to the LEA. A clarification is needed on a specific traffic case triggering the Identifier association (x)IRI. A wrong internal reference needs correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New AMFIdentifierDeassociation and MMEIdentifierDeassociation xIRIs are specified. It is clarified that the (xIRI) shall be triggered by the AMF regardless of whether the CONFIGURATION UPDATE (5G-GUTI) or REGISTRATION ACCEPT procedure is subsequently successfully completed or not. The internal reference is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would not be possibile to update the LEA when a deassociation between temporary and permanent identifies occurs without any new association. Misalignment in the association known at the LEA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1; 6.2.2.2.1; 6.2.2.2.7; 6.2.2.3; 6.3.1; 6.3.2.1; 6.3.2.2.1; 6.3.2.2.2; 6.3.2.3.2; 6.3.2.3.3; Annex TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 33.127 CR 0280 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !321 Commit hash: 9c4392950dd6dfb0184a22b785f3b9dca632a5e2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i250112 |  |  |  |  |  |  |  |  |

*** FIRST MAIN DOCUMENT CHANGE ***

#### 6.2.2.1	Provisioning over LI_X1

The IRI-POI present in the AMF is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the AMF shall support the following target identifier formats in the ETSI TS 103 221-1 [7] messages (or equivalent if ETSI TS 103 221-1 [7] is not used):

-	SUPIIMSI.

- 	SUPINAI.

-	PEIIMEI.

-	PEIIMEISV.

-	GPSIMSISDN.

-	GPSINAI.

Table 6.2.2-0A shows the minimum details of the LI_X1 ActivateTask message used for provisioning the IRI-POI in the AMF.

Table 6.2.2-0A: ActivateTask message for the IRI-POI in the AMF

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One of the target identifiers listed in the paragraph above. | M |
| DeliveryType | Set to "X2Only". | M |
| ListOfDIDs | Delivery endpoints for LI_X2 for the IRI-POI in the AMF. These delivery endpoints are configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to the task activation. | M |
| TaskDetailsExtensions/ IdentifierAssociationExtensions | This field shall be included if the IRI POI is required to generate AMFIdentifierAssociation and AMFIdentifierDeassociation records (see clause 6.2.2.2.1). If the field is absent, AMFIdentifierAssociation and AMFIdentifierDeassociation records shall not be generated. | C |
| ListOfServiceTypes | Shall be included when the explicit identification of specific CSP service types to be intercepted by the task as described in clause 5.2.4 is required. This parameter is defined in ETSI TS 103 221-1 [7], clause 6.2.1.2, table 4. | C |

Table 6.2.2-0B: IdentifierAssociationExtensions Parameters

| Field Name | Description | M/C/O |
| --- | --- | --- |
| EventsGenerated | One of the following values: - IdentifierAssociation - All See clause 6.2.2.2.1 for the interpretation of this field. | M |

*** NEXT MAIN DOCUMENT CHANGE ***

#### 6.2.2.2	Generation of xIRI over LI_X2

##### 6.2.2.2.1	General

The IRI-POI present in the AMF shall send the xIRIs over LI_X2 for each of the events listed in TS 33.127 [5] clause 6.2.2.4, the details of which are described in the following clauses.

If the AMF receives one or more cell IDs in an N2 message (as specified in TS 38.413 [23]), the IRI-POI in the AMF shall report all of them.

The IRI-POI in the AMF shall only generate xIRI containing AMFIdentifierAssociation and AMFIdentifierDeassociation records when the IdentifierAssocationExtensions parameter has been received over LI_X1 (see clause 6.2.2.1). The IRI-POI in the AMF shall generate records according to the value of the EventsGenerated sub-parameter (see table 6.2.2-0B) as follows:

-	IdentifierAssociation: AMFIdentifierAssociation, AMFIdentifierDeassociation and AMFLocationUpdate records shall be generated. No other record types shall be generated for that target.

-	All: All AMF record types shall be generated.

*** NEXT MAIN DOCUMENT CHANGE ***

##### 6.2.2.2.7	AMF identifier association/deassociation

The IRI-POI present in the AMF shall generate an xIRI containing an AMFIdentifierAssociation record when the IRI-POI present in the AMF detects a new identifier association for a UE matching one of the target identifiers provided via LI_X1. Generation of this record is subject to this record type being enabled for a specific target (see clause 6.2.2.2.1).

Table 6.2.2-6: Payload for AMFIdentifierAssociation record

| Field name | Description | M/C/O |
| --- | --- | --- |
| sUPI | SUPI associated with the procedure (see NOTE 1). | M |
| sUCI | SUCI used in the procedure, if applicable and if available. | C |
| pEI | PEI used in the procedure, if available (see NOTE 1). | C |
| gPSI | GPSI used in the procedure, if available (see NOTE 1). | C |
| gUTI | 5G-GUTI used in the procedure, see TS 24.501 [13] clause 9.11.3.4. | M |
| location | Location information available when identifier association occurs. Encoded as a userLocation parameter (location>locationInfo>userLocation) and, when Dual Connectivity is activated, as an additionalCellIDs parameter (location>locationInfo>additionalCellIDs), see Annex A. | M |
| fiveGSTAIList | List of tracking areas associated with the registration area within which the UE is current registered, see TS 24.501 [13], clause 9.11.3.9. (see NOTE 2) | C |
| additionalUserIdentifiers | Provides additional user identifiers known at the AMF or stored in AMF context, e.g. additional GPSI. | C |
| NOTE 1: SUPI shall always be provided, in addition to the warrant target identifier if different to SUPI. Other identifiers shall be provided if available. NOTE 2: List shall be included each time there is a change to the registration area. |  |  |

The IRI-POI present in the AMF generating an xIRI containing an AMFIdentifierAssociation record shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

In the case where the IRI-POI present in the AMF detects that a REGISTRATION ACCEPT message or a CONFIGURATION UPDATE (5G-GUTI) message as defined in TS 24.501 [13] has been sent by the AMF towards a target UE, the IRI-POI present at the AMF shall immediately generate an AMFIdentifierAssociation xIRI. This xIRI shall be generated regardless of whether the REGISTRATION ACCEPT or CONFIGURATION UPDATE (5G-GUTI) procedure is subsequently successfully completed or not.

The IRI-POI present in the AMF shall generate an xIRI containing an AMFIdentifierDeassociation record when the IRI-POI present in the AMF detects an identifier deassociation without a new association within the same UE context inside the same AMF for a UE matching one of the target identifiers provided via LI_X1. Generation of this record is subject to this record type being enabled for a specific target (see clause 6.2.2.2.1).

Table 6.2.2-x: Payload for AMFIdentifierDeassociation record

| Field name | Description | M/C/O |
| --- | --- | --- |
| sUPI | SUPI associated with the procedure (see NOTE). | M |
| sUCI | SUCI used in the procedure, if applicable and if available. | C |
| pEI | PEI used in the procedure, if available (see NOTE). | C |
| gPSI | GPSI used in the procedure, if available (see NOTE). | C |
| gUTI | 5G-GUTI used in the procedure, see TS 24.501 [13] clause 9.11.3.4. | M |
| location | Location information available when identifier deassociation occurs, sent only if it is the current location. Encoded as a userLocation parameter (location>locationInfo>userLocation) and, when Dual Connectivity is activated, as an additionalCellIDs parameter (location>locationInfo>additionalCellIDs), see Annex A. | C |
| additionalUserIdentifiers | Provides additional user identifiers known at the AMF or stored in AMF context, e.g. additional GPSI. | C |
| NOTE: SUPI shall always be provided, in addition to the warrant target identifier if different to SUPI. Other identifiers shall be provided if available. |  |  |

The IRI-POI present in the AMF generating an xIRI containing an AMFIdentifierDeassociation record shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

*** NEXT MAIN DOCUMENT CHANGE ***

#### 6.2.2.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in AMF, the MDF2 shall generate the corresponding IRI message and deliver over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received in the xIRI over LI_X2. This record may be enriched with any additional information available at the MDF (e.g. additional location information).

The timestamp field of the PSHeader structure shall be set to the time at which the AMF event was observed (i.e. the timestamp field of the X2 PDU).

The IRI type parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 6.2.2-7.

Table 6.2.2-7: IRI type for IRI messages

| IRI message | IRI type |
| --- | --- |
| AMFRegistration | REPORT |
| AMFDeregistration | REPORT |
| AMFLocationUpdate | REPORT |
| AMFStartOfInterceptionWithRegisteredUE | REPORT |
| AMFUnsuccessfulProcedure | REPORT |
| AMFIdentifierAssociation | REPORT |
| AMFIdentifierDeassociation | REPORT |
| AMFPositioningInfoTransfer | REPORT |
| AMFUEConfigurationUpdate | REPORT |

These IRI messages shall omit the CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The threeGPP33128DefinedIRI field in ETSI TS 102 232-7 [10] clause 15 shall be populated with the BER-encoded IRIPayload.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the AMFStartOfInterceptionWithRegisteredUE record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the AMFStartOfInterceptionWithRegisteredUE record is specified in table 6.2.2-4.

If the MDF2 did not receive from the IRI-POI the value of timeOfRegistration parameter in a previous corresponding AMFStartOfInterceptionWithRegisteredUE for the same registration, the MDF2 shall include in that parameter the time provided in the timestamp previously received in the header of the related AMFRegistration xIRI.

*** NEXT MAIN DOCUMENT CHANGE ***

## 6.3	4G

### 6.3.1	General

The present document allows three options for EPC LI stage 3 interfaces for 4G / LTE:

-	Option A: Use LI_X1, LI_X2 and LI_X3 interfaces specified below in clauses 6.3.2 and 6.3.3 for the events listed in TS 33.127 [5] clauses 6.3.2.3 and 6.3.3.3, and the events related to SMS over NAS as specified in TS 33.107 [36] clause 18.2.4.

-	Option B: Use LI_X1, LI_X2 and LI_X3 interfaces as specified in clause 6.3.2 and 6.3.3 for the events listed in TS 33.107 [36] clause 12.2.1.2 and for the events related to the MMEIdentifierAssociation and MMEIdentifierDeassociation records described in clause 6.3.2.2.2.

-	Option C: Use TS 33.107 [36] clause 12 natively as defined in that document.

For implementations that include EPS/5GS interworking, Option A shall be used.

In all cases, the present document specifies the stage 3 for the LI_HI1, LI_HI2 and LI_HI3 interfaces.

*** NEXT MAIN DOCUMENT CHANGE ***

### 6.3.2	LI at MME

#### 6.3.2.1	Provisioning over LI_X1

The IRI-POI present in the MME is provisioned over LI_X1 by the LIPF using the X1 protocol as described in clause 5.2.2.

The POI in the MME shall support the following target identifier formats:

-	IMSI (using the IMSI target identifier format from ETSI TS 103 221-1 [7]).

-	MSISDN (using the E164Number target identifier format from ETSI TS 103 221-1 [7]).

-	ME Identity (using the IMEI target identifier format from ETSI TS 103 221-1 [7]).

Table 6.3.2-0A shows the minimum details of the LI_X1 ActivateTask message used for provisioning the IRI-POI in the MME.

Table 6.3.2-0A: ActivateTask message for the IRI-POI in the MME

| ETSI TS 103 221-1 [7] field name | Description | M/C/O |
| --- | --- | --- |
| XID | XID assigned by LIPF. | M |
| TargetIdentifiers | One of the target identifiers listed in the paragraph above. | M |
| DeliveryType | Set to "X2Only". | M |
| ListOfDIDs | Delivery endpoints for LI_X2 for the IRI-POI in the MME. These delivery endpoints are configured using the CreateDestination message as described in ETSI TS 103 221-1 [7] clause 6.3.1 prior to the task activation. | M |
| TaskDetailsExtensions/ IdentifierAssociationExtensions | This field shall be included if the IRI-POI is required to generate MMEIdentifierAssociation and MMEIdentifierDeassociation records (see clause 6.3.2.2.1). If the field is absent, MMEIdentifierAssociation and MMEIdentifierDeassociation records shall not be generated. | C |
| ListOfServiceTypes | Shall be included when the task should only intercept specific CSP service types as described in clause 5.2.4. This parameter is defined in ETSI TS 103 221-1 [7], clause 6.2.1.2, table 4. | C |

Table 6.3.2-0B: IdentifierAssociationExtensions Parameters

| Field Name | Description | M/C/O |
| --- | --- | --- |
| EventsGenerated | One of the following values: - IdentifierAssociation - All See clause 6.3.2.2.1 for the interpretation of this field. | M |

*** NEXT MAIN DOCUMENT CHANGE ***

#### 6.3.2.2	Generation of xIRI over LI_X2

##### 6.3.2.2.1	General

If the MME receives one or more cell IDs in an S1 message (as specified in TS 36.413 [38]), the POI associated with the MME shall report all of them.

The IRI-POI in the MME shall only generate xIRI containing the MMEIdentifierAssociation and MMEIdentifierDeassociation records in the following scenarios:

-	IdentifierAssociation: MMEIdentifierAssociation, MMEIdentifierDeassociation and Tracking Area/EPS Location Update (see TS 33.107 [36] clause 12.2.1.2) records shall be generated. No other record types shall be generated for that target.

-	All: All MME record types shall be generated.

When Option A specified in clause 6.3.1 is used:

-	The IRI-POI present in the MME shall send the xIRIs over LI_X2 for each of the events listed in TS 33.127 [5] clause 6.3.2.3, the details of which are described in the following clauses.

-	In addition to the xIRI events listed in TS 33.127 [5] clause 6.3.2.3, the MME shall support xIRI generation in case of SMS over NAS as specified in TS 33.107 [36] clause 18.2.4. For records related to SMS over NAS in EPS:

-	The IRI-POI present in the MME shall set the payload format to EpsHI2Operations.EpsIRIContent (value 14), see clause 5.3 and ETSI TS 103 221-2 [8] clause 5.4. The payload field shall contain an EpsHI2Operations.EpsIRIContent structure encoded according to TS 33.108 [12] clauses 10.5, 15.2 and B.9.

-	As the LIID may be not available at the MME but is mandatory in EpsHI2Operations.EpsIRIContent according to TS 33.108 [12] Annex B.9, its value in the lawfulInterceptionIdentifier field of the encoded PDU shall be set to the fixed string "LIIDNotPresent".

When Option B specified in clause 6.3.1 is used:

-	The IRI-POI present in the MME shall send the xIRIs over LI_X2 for each of the events listed in TS 33.107 [36] clause 12.2.1.1, the details of which are specified in clause 12.2.3 of the same TS, and in case of SMS over NAS as specified in TS 33.107 [36] clause 18.2.4.

-	For all records except MMEIdentifierAssociation (see clause 6.3.2.2.2), the IRI-POI present in the MME shall set the payload format to EpsHI2Operations.EpsIRIContent (value 14), see clause 5.3 and ETSI TS 103 221-2 [8] clause 5.4. The payload field shall contain an EpsHI2Operations.EpsIRIContent structure encoded according to TS 33.108 [12] clauses 10.5, 15.2 and B.9.

-	As the LIID may be not available at the MME but is mandatory in EpsHI2Operations.EpsIRIContent according to TS 33.108 [12] Annex B.9, its value in the lawfulInterceptionIdentifier field of the encoded PDU shall be set to the fixed string "LIIDNotPresent".

-	In addition to the xIRI events listed in TS 33.107 [36], the MME shall support xIRI containing the MMEIdentiferAssociation and MMEIdentifierDeassociation records in clause 6.3.2.2.2.

*** NEXT MAIN DOCUMENT CHANGE ***

##### 6.3.2.2.2	MME identifier association/deassociation

The IRI-POI present in the MME shall generate an xIRI containing an MMEIdentifierAssociation record when the IRI-POI present in the MME detects a new identifier association for a UE matching one of the target identifiers provided via LI_X1. Generation of this record is subject to this record type being enabled for a specific target (see clause 6.3.2.2.1).

Table 6.3.2-1: Payload for MMEIdentifierAssociation record

| Field name | Description | M/C/O |
| --- | --- | --- |
| iMSI | IMSI associated with the procedure. (see NOTE 1). | M |
| iMEI | IMEI used in the procedure, if available (see NOTE 1). | C |
| mSISDN | MSISDN used in the procedure, if available (see NOTE 1). | C |
| gUTI | LTE GUTI used in the procedure. | M |
| location | Location information available when identifier association occurs. Shall include all location information for the target UE available at the MME encoded as one of the following (see NOTE 3): - ePSUserLocationInformation parameter (location>fourGLocationInfo>ePSUserLocationInformation). - ePSLocationInformation parameter (location>fourGLocationInfo>ePSLocationInformation). When Dual Connectivity is activated, the additionalCellIDs parameter (location>fourGLocationInfo>ePSLocationInformation>mMELocationInformation>additionalCellIDs) shall also be populated, see clause 7.3.3 and Annex A. | M |
| tAIList | List of tracking areas associated with the registration area within which the UE is current registered. (see NOTE 2). | C |
| NOTE 1: IMSI shall always be provided, in addition to the warrant target identifier if different to IMSI. Other identifiers shall be provided if available. NOTE 2: List shall be included each time there is a change to the registration area. NOTE 3: The location information was sent as a userLocation (location>locationInfo>userLocation) between versions 17.0.0 and 17.7.0 of the present document. The location information may also be present in this field for backwards compatibility. |  |  |

The IRI-POI present in the MME generating an xIRI containing an MMEIdentifierAssociation record shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

When transmitting the xIRI, the IRI-POI present in the MME shall set the payload format to 2, and provide the payload as a BER-encoded TS33128Payloads.XIRIPayloads structure.

In the case where the IRI-POI present in the MME detects that an ATTACH ACCEPT, TRACKING AREA UPDATE ACCEPT (including a GUTI) or GUTI REALLOCATION COMMAND message as defined in TS 24.301 [51] has been sent by the MME towards a target UE, the IRI-POI present at the MME shall immediately generate a MMEIdentifierAssociation xIRI. This xIRI shall be generated regardless of whether the ATTACH ACCEPT, TRACKING AREA UPDATE ACCEPT or GUTI REALLOCATION COMMAND procedure is subsequently successfully completed or not.

The IRI-POI present in the MME shall generate an xIRI containing an MMEIdentifierDeassociation record when the IRI-POI present in the MME detects an identifier deassociation without a new association within the same UE context inside the same MME for a UE matching one of the target identifiers provided via LI_X1. Generation of this record is subject to this record type being enabled for a specific target (see clause 6.3.2.2.1).

Table 6.3.2-x: Payload for MMEIdentifierDeassociation record

| Field name | Description | M/C/O |
| --- | --- | --- |
| iMSI | IMSI associated with the procedure. (see NOTE 1). | M |
| iMEI | IMEI used in the procedure, if available (see NOTE 1). | C |
| mSISDN | MSISDN used in the procedure, if available (see NOTE 1). | C |
| gUTI | LTE GUTI used in the procedure. | M |
| location | Location information available when identifier deassociation occurs, sent only if it is the current location. Shall include all location information for the target UE available at the MME encoded as one of the following (see NOTE 2): - ePSUserLocationInformation parameter (location>fourGLocationInfo>ePSUserLocationInformation). - ePSLocationInformation parameter (location>fourGLocationInfo>ePSLocationInformation). When Dual Connectivity is activated, the additionalCellIDs parameter (location>fourGLocationInfo>ePSLocationInformation>mMELocationInformation>additionalCellIDs) shall also be populated, see clause 7.3.3 and Annex A. | C |
| NOTE 1: IMSI shall always be provided, in addition to the warrant target identifier if different to IMSI. Other identifiers shall be provided if available. NOTE 2: The location information was sent in the MMEIdentifierAssociation record as a userLocation (location>locationInfo>userLocation) between versions 17.0.0 and 17.7.0 of the present document. The location information may also be present in this field for backwards compatibility. |  |  |

The IRI-POI present in the MME generating an xIRI containing an MMEIdentifierDeassociation record shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

When transmitting the xIRI, the IRI-POI present in the MME shall set the payload format to 2, and provide the payload as a BER-encoded TS33128Payloads.XIRIPayloads structure.

*** NEXT MAIN DOCUMENT CHANGE ***

##### 6.3.2.3.2	Option A

The IRI message the MDF2 generates shall contain a copy of the relevant record received in the xIRI over LI_X2 and provide it over LI_HI2 without undue delay.

The timestamp field of the PSHeader structure shall be set to the time at which the MME event was observed (i.e. the timestamp field of the X2 PDU).

The IRI type parameter (see ETSI TS 102 232-1 [9] clause 5.2.10) shall be included and coded according to table 6.3.2-8.

Table 6.3.2-8: IRI type for IRI messages

| IRI message | IRI type |
| --- | --- |
| MMEAttach | REPORT |
| MMEDetach | REPORT |
| MMELocationUpdate | REPORT |
| MMEStartOfInterceptionWithEPSAttachedUE | REPORT |
| MMEUnsuccessfulProcedure | REPORT |
| MMEIdentifierAssociation | REPORT |
| MMEIdentifierDeassociation | REPORT |
| MMEPositioningInfoTransfer | REPORT |

These IRI messages shall omit the CIN (see ETSI TS 102 232-1 [9] clause 5.2.4).

The threeGPP33128DefinedIRI field in ETSI TS 102 232-7 [10] clause 15 shall be populated with the BER-encoded IRIPayload.

When an additional warrant is activated on a target UE and the LIPF uses the same XID for the additional warrant, the MDF2 shall be able to generate and deliver the IRI message containing the MMEStartOfInterceptionWithEPSAttachedUE record to the LEMF associated with the additional warrant without receiving a corresponding xIRI. The payload of the MMEStartOfInterceptionWithEPSAttachedUE record is specified in table 6.3.2-6.

For records related to SMS over NAS in EPS, the process detailed in clause 6.3.2.3.3 shall be used.

*** NEXT MAIN DOCUMENT CHANGE ***

##### 6.3.2.3.3	Option B and Option C

For all messages except MMEIdentifierAssociation and MMEIdentifierDeassociation, the IRI messages shall include an IRI payload encoded according to TS 33.108 [12] Annex B.9.

The MDF2 shall encode the correct value of LIID in the IRI message, replacing the value "LIIDNotPresent" given in the xIRI (see clause 6.3.2.2).

For MMEIdentifierAssociation and MMEIdentifierDeassociation messages, the IRI message shall be encoded as an IRIEvent structure according to Annex A and used to populate the threeGPP33128DefinedIRI field in ETSI TS 102 232-7 [10] clause 15.

*** END OF MAIN DOCUMENT CHANGES ***

START OF ATTACHMENT CHANGE 1

---a/33128/r17/TS33128Payloads.asn
+++b/33128/r17/TS33128Payloads.asn

@@ -199,7 +199,13 @@ XIRIEvent ::= CHOICE

199	199

200	200		    -- MMS-related events continued from choice 35

201	201		    mMSConvertedFromEmail                               [133] MMSConvertedFromEmail,

202		-	    mMSConvertedToEmail                                 [134] MMSConvertedToEmail

202	+	    mMSConvertedToEmail                                 [134] MMSConvertedToEmail,

203	+

204	+	    -- Tags 135 to 185 are not used in this version of the specification

205	+

206	+	    -- Additional Identifier Association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

207	+	    aMFIdentifierDeassociation                          [186] AMFIdentifierDeassociation,

208	+	    mMEIdentifierDeassociation                          [187] MMEIdentifierDeassociation

203	209		}

204	210

205	211		-- ==============

@@ -392,7 +398,13 @@ IRIEvent ::= CHOICE

392	398

393	399		    -- MMS-related events continued from choice 35

394	400		    mMSConvertedFromEmail                               [133] MMSConvertedFromEmail,

395		-	    mMSConvertedToEmail                                 [134] MMSConvertedToEmail

401	+	    mMSConvertedToEmail                                 [134] MMSConvertedToEmail,

402	+

403	+	    -- Tags 135 to 185 are not used in this version of the specification

404	+

405	+	    -- Additional Identifier Association events, see clauses 6.2.2.2.7 and 6.3.2.2.2

406	+	    aMFIdentifierDeassociation                          [186] AMFIdentifierDeassociation,

407	+	    mMEIdentifierDeassociation                          [187] MMEIdentifierDeassociation

396	408		}

397	409

398	410		IRITargetIdentifier ::= SEQUENCE

@@ -3877,6 +3889,17 @@ AMFIdentifierAssociation ::= SEQUENCE

3877	3889		    additionalUserIdentifiers  [8] UserIdentifiers OPTIONAL

3878	3890		}

3879	3891

3892	+	AMFIdentifierDeassociation ::= SEQUENCE

3893	+	{

3894	+	    sUPI                       [1] SUPI,

3895	+	    sUCI                       [2] SUCI OPTIONAL,

3896	+	    pEI                        [3] PEI OPTIONAL,

3897	+	    gPSI                       [4] GPSI OPTIONAL,

3898	+	    gUTI                       [5] FiveGGUTI,

3899	+	    location                   [6] Location OPTIONAL,

3900	+	    additionalUserIdentifiers  [7] UserIdentifiers OPTIONAL

3901	+	}

3902	+

3880	3903		MMEIdentifierAssociation ::= SEQUENCE

3881	3904		{

3882	3905		    iMSI        [1] IMSI,

@@ -3887,6 +3910,15 @@ MMEIdentifierAssociation ::= SEQUENCE

3887	3910		    tAIList     [6] TAIList OPTIONAL

3888	3911		}

3889	3912

3913	+	MMEIdentifierDeassociation ::= SEQUENCE

3914	+	{

3915	+	    iMSI        [1] IMSI,

3916	+	    iMEI        [2] IMEI OPTIONAL,

3917	+	    mSISDN      [3] MSISDN OPTIONAL,

3918	+	    gUTI        [4] GUTI,

3919	+	    location    [5] Location OPTIONAL

3920	+	}

3921	+

3890	3922		-- =================================

3891	3923		-- Identifier Association parameters

3892	3924		-- =================================

END OF ATTACHMENT CHANGE 1