SA3LI#98	 S3i250335

July 15-18, 2025, Florence (Italy)

Source:	Nokia

Title:	pCR #6 to TR 33.929-6: RCS topology Part IV – RCS servers in one third party domain, CSP RCS users in the different CSP domains

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	LI19-GUIDE

Abstract of the contribution: This pCR provides input text to TR 33.929-6 LI for RCS. Provides some RCS topologies when the RCS servers are in the same third party RCS domain. Here, the CSP RCS users are in the different CSP domains.

BACKGROUND

The TR 33.929-6 provides the implementation guidance for the LI aspects related to the RCS. This pCR gives provides the RCS network topologies for RCS when the RCS servers are in the same third party domain. The CSP RCS users are in different CSP domains.

*** Related text from the previous pCR ****

# 1	Scope

# 2	References

# 3	Definitions of terms, symbols and abbreviations

# 4	Illustration of LI for IMS-based RCS

## 4.1	Overview

## 4.2	RCS topology scenarios

### 4.2.1	General

### 4.2.2	Few conventions used

In all the illustrations, when the RCS user is a subscriber of a CSP domain, 5G radio and packet core is assumed. Within the drawings, (A) represents the ingress side and (B) represents the egress side. When ingress and egress have to be shown in multiple network domains, (A1), (A11), (B1), B (11), etc., are used.

The figure 4.2.2-1 shows some of the conventions used within the topology drawings (in a general way):

Figure 4.2.2-1: Few conventions used within the drawings

### 4.2.3	RCS servers in CSP domain

### 4.2.4	Third party RCS service providers

#### 4.2.4.1	General

#### 4.2.4.2	One third party RCS service provider

##### 4.2.4.2.1	General

This clause illustrates the case when RCS Servers are within the same third party RCS service provider domain with various scenarios depending on where the RCS users are.

Scenario 1: RCS users are not in CSP domain.

Scenario 2: RCS users are in the same CSP domain that does not have IMS.

Scenario 3: RCS users are in the same CSP domain that has IMS.

Scenario 4: RCS users are in the different CSP domains that do not have IMS.

Scenario 5: RCS users are in the different CSP domains that have IMS.

Not all the scenarios listed above may be deployed in practice, The present document only provides the illustrations in case those scenarios are deployed.

##### 4.2.4.2.2	Scenario 1: RCS users are not in CSP domain

##### 4.2.4.2.3	Scenario 2: RCS users are in the same CSP domain that does not have IMS

##### 4.2.4.2.4	Scenario 3: RCS users are in the same CSP domain that has IMS

PROPOSAL

Incorporate the following to the TR 33.929-6.

***** New text ****

##### 4.2.4.2.5	Scenario 4 : RCS users are different CSP domains that do not have IMS

This clause illustrates the case when RCS Servers are in the same third party RCS service provider domain and the RCS users are different CSP domains that do not have IMS.

Only non-roaming scenario is illustrated here. The roaming scenarios are similar to the illustrations shown in clause 4.2.4.2.3 except that two CSP domains are involved here.

User (A) is a subscriber in CSP domain A1 and User (B) is a subscriber in the CSP domain B1. The CSP domains provide just the access when those users establish RCS sessions and as such both signalling and media pass through the CSP domain over the packet data connection.

The figure 4.2.4.2.5-1 illustrates the case when RCS User (A) and RCS User (B) are targets in the third party RCS service provider domain.

Figure 4.2.4.2.5-1: RCS users in different CSP domains that do not have IMS

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when RCS User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the RCS User (B) is a target for RCS in the third party RCS service provider domain.

The RCS Servers provide both IRI-POI and CC-POI functions.

##### 4.2.4.2.6	Scenario 5: RCS users are in different CSP domains that have IMS

This clause illustrates the case when RCS Servers are in the same third party RCS service provider domain and the RCS users are different CSP domains that have IMS.

The illustrations focus RCS LI and non-roaming scenarios. Roaming scenarios and LI for other IMS-based services are similar to the illustrations to the case 3 (clause 4.2.4.2.4).

User (A) is a subscriber in the CSP domain A1 and User (B) is a subscriber in the CSP domain B1. Since the RCS Servers used for the RCS sessions are in third party RCS service provider domain, in this deployment option, both CSPs provide just the access when those users establish RCS sessions and as such both signalling and media pass through the CSP domains over the packet data connection.

The signalling and media for non-RCS related IMS sessions do not pass through the third party RCS service provider domain.

The figure 4.2.4.2.6-1 illustrates the CSP deployment option 1 when User (A) and User (B) are targets for RCS in the third party service provider domain.

Figure 4.2.4.2.6-1: RCS users in different CSP domains that have IMS

The RCS Server (A) present in the third party RCS service provider domain provides the LI functions when User (A) is a target for RCS in the third party RCS service provider domain and RCS Server (B) present in the third party RCS service provider domain provides the LI functions when the User (B) is a target for RCS in the third party RCS service provider domain.

The RCS Servers provide both IRI-POI and CC-POI functions.

***** End of the new text ****