3GPP TSG-SA3 Meeting #98-LI	s3i250421

# Florence, Italy, 15th Jul 2025 - 18th Jul 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0290 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI for MC Data (Stage 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-07-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Define LI for MCData |  |  |  |  |  |  |  |  |
| Summary of change: |  | LI for MCData Stage 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory issue for CSPs with MCData services |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR 0289 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i250309 |  |  |  |  |  |  |  |  |

*** START OF FIRST CHANGE ***

### 7.X.4	MCData service

#### 7.X.4.1	Background

MCData is a Mission Critical (MC) service. MCData service architecture consists of MCData Server, Common Services Core (CSC) Servers and MCData Content Server.

The MCData Server provides support for one-to-one communication and group communication for the following services for on-network:

-	Short Data Service (SDS) as defined in TS 23.282 [Xc] clause 6.5:

-	Standalone SDS using signaling control plane.

-	Standalone SDS using user media plane.

-	SDS session.

-	File Distribution (FD) as defined in TS 23.282 [Xc] clause 6.6:

-	FD using HTTP.

-	FD using media plane.

-	Data streaming as defined in TS 23.282 [Xc] clause 6.7.

-	IP connectivity as defined in TS 23.282 [Xc] clause 6.8.

The CSC Servers provide configurations and management functions for MCData and have been introduced in 7.X.3.1.

The MCData Content Server supports file upload, URI generation, and file download for FD using HTTP.

MCData service has many similarities with the RCS service.

#### 7.X.4.2	LI Architecture

The MCData Server shall have LI capabilities to generate xIRI records and xCC when MCData events related to the target UE are handled. The CSC Servers shall have LI capabilities to generate xIRI records when CSC events related to target UE are handled. The MCData Content Server shall have LI capabilities to generate xIRI records and xCC when the target UE uploads or downloads a file and when any user downloads a file previously uploaded by a target. Figure 7.X.4.2-1 provides a reference point representation of the LI architecture for MCData interception.

Figure 7.X.4.2-1: LI architecture for MCData

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides it to the LIPF.

The LIPF present in the ADMF provisions the IRI-POI in the MCData Server, the IRI-POI in any CSC Server, the IRI-POI in the MCData Content Server, the MDF2 and the MDF3 over the LI_X1 interfaces.

The IRI-POI present in the MCData Server detects SIP signaling exchange for MCData services, generates and delivers the related xIRI to the MDF2 over LI_X2. The MDF2 delivers the IRI messages to the LEMF over LI_HI2.

The IRI-POI present in the MCData Content Server detects file uploads and downloads relating to FD using HTTP, generates and delivers the related xIRI to the MDF2 over LI_X2. The MDF2 delivers the IRI messages to the LEMF over LI_HI2.

When interception of communication contents is required, the CC-POI present in the MCData Server generates the xCC from MCData messages and delivers the xCC (that includes the correlation number and the target identity) to the MDF3. The MDF3 delivers the CC to the LEMF over LI_HI3.

The CC-POI present in the MCData Content Server generates the xCC from the uploaded file and delivers the xCC (that includes the correlation number and the target identity) to the MDF3. The MDF3 delivers the CC to the LEMF over LI_HI3.

#### 7.X.4.3	Target identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI and CC-POI present in the MCData Server:

-	MCData ID.

-	Instance Identifier URN.

-	IMPU.

-	IMPI.

-	MCData group ID.

The interception performed on the above identities are mutually independent, even though, an xIRI may contain the information about the other identities when available.

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the IdMS:

-	MC ID.

-	MCData ID.

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the CMS:

-	MCData ID.

-	Instance Identifier URN.

-	IMPU.

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the GMS:

-	MCData ID.

-	Instance Identifier URN.

-	IMPU.

-	MCData group ID.

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the KMS and LMS:

-	MCData ID.

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI and CC-POI present in the MCData Content Server:

-	MCData ID.

-	MCData group ID.

#### 7.X.4.4	IRI events

The IRI-POI present in the MCData Server shall generate xIRI when it detects the following specific events or information:

-	Encapsulated SIP message.

-	Start of interception with registered UE.

-	Start of interception with established session.

The IRI-POI present in each CSC Server shall generate xIRI when it detects the following specific events or information:

-	Management message.

The IRI-POI present in the MCData Content Server shall generate xIRI when it detects the following specific events or information:

-	File distribution.

The encapsulated SIP message xIRI is generated when the IRI-POI present in an MCData Server detects that a SIP message is received from, or sent to, a target or processed on behalf of a target at the MCData Server.

The start of interception with registered UE xIRI is generated when the IRI-POI present in an MCData Server detects that interception is activated on the target UE that is already registered for MCData services.

The start of interception with established session xIRI is generated when the IRI-POI present in an MCData Server detects that:

-	interception is activated on a target MCData UE that has an already established MCData session. When a target UE has multiple MCData sessions, this xIRI shall be sent for each MCData session with a different value of correlation information.

-	interception is activated on a target MCData group and an MCData session has already been established for this MCData group.

The management message xIRI is generated when the IRI-POI present in any CSC Server detects that messages are exchanged between the target MCData UE and CSC Server.

The file distribution xIRI is generated when the IRI-POI present in the MCData Content Server detects that a target UE has uploaded a file, or a target UE has downloaded a file or when any UE has downloaded a file previously uploaded by a target UE.

#### 7.X.4.5	Network topologies

As the MCData platform is operated by MCData service provider, the interception is only managed by such operator.

*** END OF FIRST CHANGE ***