3GPP TSG-SA3 Meeting #99-LI	s3i250606

# Kansas City, United States, 4th Nov 2025 - 7th Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0780 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI on MCPTT stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI(Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2025-09-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Absence of LI for MCPTT in Stage 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add LI for MCPTT in Stage 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LI for MCPTT in Stage 3 would still be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 7.20.1.1; 7.20.2.5.1; 7.20.2.7; 7.20.3; 7.20.4; 7.20.5.3, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Schema changes for this CR can be found on the Forge: Merge Request: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/347 Commit Hash: https://forge.3gpp.org/rep/sa3/li/-/commit/e531a270b5fa4b805a29bd6a0326864257ca347f |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF  FIRST CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful Interception Requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 33.127: "Lawful Interception (LI) Architecture and Functions".

[6]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[7]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1".

[8]	ETSI TS 103 221-2: "Lawful Interception (LI); Internal Network Interfaces; Part 2: X2/X3".

[9]	ETSI TS 102 232-1: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 1: Handover specification for IP delivery".

[10]	ETSI TS 102 232-7: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 7: Service-specific details for Mobile Services".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[12]	3GPP TS 33.108: "3G security; Handover interface for Lawful Interception (LI)".

[13]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[14]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General Aspects".

[15]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[16]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[19]	3GPP TS 23.003: "Numbering, addressing and identification ".

[20]	OMA-TS-MLP-V3_5-20181211-C: "Open Mobile Alliance; Mobile Location Protocol, Candidate Version 3.5", https://www.openmobilealliance.org/release/MLS/V1_4-20181211-C/OMA-TS-MLP-V3_5-20181211-C.pdf.

[21]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[22]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[23]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[24]	3GPP TS 29.572: "Location Management Services; Stage 3".

[25]	3GPP TS 29.503: "5G System; Unified Data Management Services".

[26]	IETF RFC 815: "IP datagram reassembly algorithms".

[27]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[28]	IETF RFC 793: "Transmission Control Protocol".

[29]	IETF RFC 768: "User Datagram Protocol".

[30]	IETF RFC 4340: "Datagram Congestion Control Protocol (DCCP)".

[31]	IETF RFC 4960: "Stream Control Transmission Protocol".

[32]	IANA (www.iana.org): Assigned Internet Protocol Numbers, "Protocol Numbers".

[33]	IETF RFC 6437: "IPv6 Flow Label Specification".

[34]	IETF RFC 791: "Internet Protocol".

[35]	Open Geospatial Consortium OGC 05-010: "URNs of definitions in ogc namespace".

[36]	3GPP TS 33.107: "3G security; Lawful interception architecture and functions".

[37]	3GPP TS 37.340: "Evolved Universal Radio Access (E-UTRA) and NR-Multi-connectivity; Stage 2".

[38]	3GPP TS 36.413: "S1 Application Protocol (S1AP)".

[39]	OMA-TS-MMS_ENC-V1_3-20110913-A: "Multimedia Messaging Service Encapsulation Protocol".

[40]	3GPP TS 23.140: "Multimedia Messaging Protocol. Functional Description. Stage 2".

[41]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IETF RFC 4566: "SDP: Session Description Protocol".

[44]	3GPP TS 24.193: "Stage 3: Access Traffic Steering, Switching and Splitting (ATSSS)".

[45]	3GPP TS 29.509: "5G System; Authentication Server Services; Stage 3".

[46]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[47]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[48]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[49]	3GPP TS 29.505: "5G System; Usage of the Unified Data Repository services for Subscription Data; Stage 3".

[50]	3GPP TS 23.401 "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[51]	3GPP TS 24.301 "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS), Stage 3".

[52]	3GPP TS 23.271 "Functional stage 2 description of Location Services (LCS)".

[53]	3GPP TS 29.172 "Evolved Packet Core (EPC) LCS Protocol (ELP) between the Gateway Mobile Location Centre (GMLC) and the Mobile Management Entity (MME); SLg interface".

[54]	3GPP TS 29.171 "LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[55]	3GPP TS 24.379: "Mission Critical Push to Talk (MCPTT) call control; protocol specification".

[56]	OMA-TS-PoC-System_Description-V2_1-20110802-A: "OMA PoC System Description".

[57]	3GPP TS 29.541: "5G System; Network Exposure (NE) function services for Non-IP Data Delivery (NIDD); Stage 3".

[58]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[59]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs); Stage 3".

[60]	3GPP TS 29.337: "Diameter-based T4 interface for communications with packet data networks and applications".

[61]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[62]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[63]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[64]	3GPP TS 29.598: "5G System; Unstructured Data Storage Services; Stage3".

[65]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[66]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[67]	GSMA IR.88: "IR.88 LTE and EPC Roaming Guidelines".

[68]	GSMA NG.114 "IMS Profile for Voice, Video and Messaging over 5GS".

[69]	IETF RFC 8225: "PASSporT: Personal Assertion Token".

[70]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[71]	IETF RFC 8588: "Personal Assertion Token (PaSSporT) Extension for Signature-based Handling of Asserted information using toKENs (SHAKEN)".

[72]	3GPP TS 24.196: "Enhanced Calling Name (eCNAM)".

[73]	IETF RFC 9795: " Personal Assertion Token (PASSporT) Extension for Rich Call Data".

[74]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP)and Session Description Protocol (SDP); Stage 3".

[75]	IANA Session Initiation Protocol (SIP) Parameters: https://www.iana.org/assignments/sip-parameters/sip-parameters.xhtml.

[76]	IETF RFC 8946: "Personal Assertion Token (PASSporT) Extension for Diverted Calls".

[77]	3GPP TS 23.204: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[78]	GSMA RCC.07: "Rich Communication Suite – Advanced Communications Services and Client Specification".

[79]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[80]	IETF RFC 3862: "Common Presence and Instant Messaging (CPIM): Message Format".

[81]	IETF RFC 5438: "Instant Message Disposition Notification (IMDN)".

[82]	OMA-TS-CPM_System_Description-V2_2-20170926-C: "OMA Converged IP Messaging System Description".

[83]	Void.

[84]	3GPP TS 36.455: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa) ".

[85]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[86]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[87]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[88]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping".

[89]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[90]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[91]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[92]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[93]	3GPP TS 24.558: "Enabling Edge Applications; Protocol specification".

[94]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification".

[95]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[96]	3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[97]	ETSI TS 103 280: "Lawful Interception (LI); Dictionary for common parameters".

[98]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[99]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[100]	3GPP TS 29.563: "5G System; Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[101]	3GPP TS 29.562: "5G System; Home Subscriber Server (HSS) Services; Stage 3".

[102]	3GPP TS 24.341 "Support of SMS over IP networks, Stage 3".

[103]	3GPP TS 38.473 "NG-RAN;F1 application protocol (F1AP)".

[104]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[105]	ITU-T Recommendation Q.763 (1999): "Specifications of Signalling System No.7; Formats and codes".

[106]	3GPP TS 29.272: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[107]	IETF RFC 6442: "Location Conveyance for the Session Initiation Protocol".

[108]	Void.

[109]	OMA-TS-CPM_Conv_Function: "OMA CPM Conversation Functions".

[110]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[111]	3GPP TS 32.299: " Telecommunication management; Charging management; Diameter charging applications".

[112]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[113]	3GPP TS 38.414: "NG-RAN; NG data transport".

[114]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[115]	IETF RFC 5322: "Internet Message Format".

[116]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[117]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[118]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[119]	W3C Recommendation: "XML Path Language (XPath)".

[120]	IETF RFC 2046: "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[121]	3GPP TR 33.928: "ADMF Logic for Provisioning Lawful Interception (LI) ".

[122]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System".

[123]	3GPP TS 23.038: "Alphabets and language-specific information".

[124]	ITU-T Recommendation X.680 (2021): "Information technology—Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[125]	IETF RFC 4282: "The Network Access Identifier".

[126]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[127]	IEEE "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)", https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[128]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN)".

[129]	3GPP TS 33.503: "Security aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[130]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[131]	3GPP TS 24.174: "Support of multi-device and multi-identity in the IP Multimedia Subsystem (IMS)".

[132]	OMA-TS-CPM_Message_Storage: "OMA CPM Message Storage".

[133]	3GPP TS 29.520: "Network Data Analytics Services".

[134]	3GPP2 C.S0015-A: "Short Message Service (SMS) for Wideband Spread Spectrum Systems".

[135]	ETSI TS 102 232-5: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 5: Service-specific details for IP Multimedia services".

[136]	ETSI TS 102 232-3: "Lawful Interception (LI); Handover Interface and Service-Specific Details (SSD) for IP delivery; Part 3: Service-specific details for internet access services".

[137]	3GPP TS 29.176: "IP Multimedia Subsystem (IMS); Media Function (MF); Services Stage 3".

[138]	3GPP TS 29.175: "IP Multimedia Subsystem (IMS) Application Server (AS) Services Stage 3".

[139]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[140]	3GPP TS 24.554: " Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[141]	3GPP TS 29.555: "5G Direct Discovery Name Management Services; Stage 3".

[142]	3GPP TS 24.583: "Application layer support for Personal IoT Network (PINAPP); Stage 3".

[143]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[144]	IETF RFC 6347: "Datagram Transport Layer Security Version 1.2".

[145]	IETF RFC 9146: "Connection Identifier for DTLS 1.2".

[146]	IETF RFC 9147: "The Datagram Transport Layer Security (DTLS) Protocol Version 1.3".

[147]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[148]	3GPP TS 32.291: "Charging management; 5G system, charging service; Stage 3".

[149]	3GPP TS 32.297: "Charging management; Charging Data Record (CDR) file format and transfer".

[150]	3GPP TS 32.298: "Charging management; Charging Data Record (CDR) parameter description".

[151]	3GPP TS 24.581: "Mission Critical Video (MCVideo) media plane control; Protocol specification".

[152]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management; Protocol specification".

[153]	3GPP TS 24.481: "Mission Critical Services (MCS) group management; Protocol specification".

[154]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

[155]	3GPP TS 24.283: "Mission Critical Location Management (MCLoc); Protocol specification".

[156]	IETF RFC 7525: "Recommendations for Secure Use of Transport Layer Security (TLS) and Datagram Transport Layer Security (DTLS)".

[157]	IETF RFC 6125: "Representation and Verification of Domain-Based Application Service Identity within Internet Public Key Infrastructure Using X.509 (PKIX) Certificates in the Context of Transport Layer Security (TLS)".

[158]	ETSI TS 104 000: "Lawful Interception (LI); Internal Network Interface X0".

[159]	IETF RFC 4279: "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)".

[160]	IETF RFC 2818: "HTTPS over TLS".

[161]	IETF RFC 8032: "Edwards-Curve Digital Signature Algorithm (EdDSA)".

[162]	3GPP TS 33.210: "Aspects; Network Domain Security (NDS); IP network layer security".

[163]	3GPP TS 24.526: "User Equipment (UE) policies for 5G system (5GS); Stage 3".

[Xa]	3GPP TS 24.380: "Mission Critical Push To Talk (MCPTT) media plane control; Protocol specification".

*** END OF FIRST CHANGE ***

*** START OF SECOND CHANGE ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADMF	LI Administration Function

CC	Content of Communication

CDF	Charging Data Function

CHF	Charging Function

CSC	Common Services Core

CSP	Communication Service Provider

CUPS	Control and User Plane Separation

DC	Data Channel

DCSF	Data Channel Signalling Function

DNAI	Data Network Access Identifier

ICF	Identity Caching Function

IEF	Identity Event Function

IPPR	Internet Protocol Packet Reporting

IQF	Identity Query Function

IRI	Intercept Related Information

LAF	Location Acquisition Function

LALS	Lawful Access Location Services

LARF	Location Acquisition Requesting Function

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LICF	Lawful Interception Control Function

LI_HI1	LI_Handover Interface 1

LI_HI2	LI_Handover Interface 2

LI_HI3	LI_Handover Interface 3

LI_HI4	LI_Handover Interface 4

LI_HILA	Lawful Interception Handover Interface Location Acquisition

LI_HIQR	Lawful Interception Handover Interface Query Response

LIPF	Lawful Interception Provisioning Function

LIR	Location Immediate Request

LI_SI	Lawful Interception System Information Interface

LISSF	Lawful Interception State Storage Function

LI_ST	Lawful Interception State Transfer Interface

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X2_LA	Lawful Interception Internal Interface 2 Location Acquisition

LI_X3	Lawful Interception Internal Interface 3

LI_XEM1	Lawful Interception Internal Interface Event Management Interface 1

LI_XER	Lawful Interception Internal Interface Event Record

LI_XLA	Lawful Interception Internal Interface Location Acquisition

LI_XQR	Lawful Interception Internal Interface Query Response

LTF	Location Triggering Function

MC	Mission Critical

MCData	Mission Critical Data

MCPTT	Mission Critical Push-To-Talk

MCVideo	Mission Critical Video

MCX	Mission Critical Service

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

MDT	Minimization of Drive Test

MM	Multimedia Message

MMS	Multimedia Message Service

N3AEC	Non-3GPP Access Establishment Cause

N3AF	Non-3GPP Access Function

NAT	Network Address Translation

NPLI	Network Provided Location Information

O&M	Operations and Management

PKMF	ProSe Key Management Function

POI	Point Of Interception

RCS	Rich Communication Suite

SDP	Session Description Protocol

SIP	Session Initiation Protocol

SIRF	System Information Retrieval Function

SOI	Start Of Interception

TF	Triggering Function

TNGF	Trusted Non-3GPP Gateway Function

TWIF	Trusted WLAN Interworking Function

xCC	LI_X3 Communications Content.

xIRI	LI_X2 Intercept Related Information

*** END OF SECOND CHANGE ***

*** START OF THIRD CHANGE ***

#### 7.20.1.1	General

If the warrant is for IRI only, the IRI-POIs in the MC Service Servers (or MCX Servers), in CSC Servers (i.e. . Identity Management Server, Configuration Management Server, Group Management Server, Key Management Server and Location Management Server) and in MCData Content Server shall be provisioned in accordance with clause 7.20.1.2.

If the warrant is for IRI and CC, then the IRI-POIs and CC-POIs in the MC Service Servers, in CSC Servers and in MCData Content Server shall be provisioned in accordance with clause 7.20.1.2.

In both cases, the MDF2 shall be provisioned in accordance with clause 7.20.1.3, and the MDF3 shall be provisioned in accordance with clause 7.20.1.4.

*** END OF THIRD CHANGE ***

*** START OF FOURTH CHANGE ***

##### 7.20.2.5.1	MCVideoTransmissionControlMessage record

The IRI-POI present in the MCVideo Server shall generate an xIRI containing an MCVideoTransmissionControlMessage record when the IRI-POI present in the MCVideo Server detects when the target MC service user sends or receives a transmission request message. In addition, when the MC service group is the target, the IRI-POI present in the MCVideo Server hosting the MC service group shall generate an xIRI containing an MCVideoTransmissionControlMessage record when the IRI-POI present in the MCVideo Server detects when a transmission control message is sent or received by any MC service participant participating in the MC service group session. Accordingly, the IRI-POI in the MCVideo Server generates the xIRI when any of the following events is detected:

-	when the MCVideo Server receives a transmission control message from a target MC service user.

-	when the MCVideo Server hosting the MC service group, where the MC service group is the target, receives a transmission control message from an MC service participant.

-	when the MCVideo Server sends a transmission control message to a target MC service user.

-	when the MCVideo Server hosting the MC service group, where the MC service group is the target, sends a transmission control message to an MC service participant.

Table 7.20.2.5.1-1: Payload for MCVideoTransmissionControlMessage record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| mCServiceIDorGroupID | MCServiceIDOrGroupID | 1 | MC service ID of the target user or group ID of the target MC service group | M |
| transmissionControlMessage | OCTET STRING | 1 | Contains the transmission control message being exchanged as an RTCP APP packet. See TS 24.581 [151] clauses 9.1 and 9.2. | M |
| transmissionControlMessageDirection | Direction | 1 | Indicates the direction of the transmission control message: "toTarget" or "fromTarget". | M |

*** END OF FOURTH CHANGE ***

*** START OF FIFTH CHANGE ***

#### 7.20.2.7	Floor control message

##### 7.20.2.7.1	MCPTTFloorControlMessage record

The IRI-POI present in the MCPTT Server shall generate an xIRI containing an MCPTTFloorControlMessage record when the IRI-POI present in the MCPTT Server detects when the target MC service user sends or receives a floor request message. In addition, when the MC service group is the target, the IRI-POI present in the MCPTT Server hosting the MC service group shall generate an xIRI containing an MCPTTFloorControlMessage record when the IRI-POI present in the MCPTT Server detects when a floor control message is sent or received by any MC service user participating in the MC service group session. Accordingly, the IRI-POI in the MCPTT Server generates the xIRI when any of the following events is detected:

-	when the MCPTT Server receives a floor control message from a target MC service user.

-	when the MCPTT Server hosting the MC service group, where the MC service group is the target, receives a floor control message from an MC service participant.

-	when the MCPTT Server sends a floor control message to a target MC service user.

-	when the MCPTT Server hosting the MC service group, where the MC service group is the target, sends a floor control message to an MC service participant.

Table 7.20.2.7.1-1: Payload for MCPTTFloorControlMessage record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| mCServiceIDorGroupID | MCServiceIDOrGroupID | 1 | MC service ID of the target user or group ID of the target MC service group | M |
| floorControlMessage | OCTET STRING | 1 | Contains the floor control message being exchanged as an RTCP APP packet. See TS 24.380 [Xa] clause 8.1.2. | M |

*** END OF FIFTH CHANGE ***

*** START OF SIXTH CHANGE ***

### 7.20.3	Generation of IRI over LI_HI2

When an xIRI is received over LI_X2 from the IRI-POI in the MCX Server or any CSC Server, the MDF2 shall send the IRI message over LI_HI2 without undue delay. The IRI message shall contain a copy of the relevant record received from LI_X2. The record may be enriched by other information available at the MDF (e.g. additional location information).

The timestamp field of the ETSI TS 102 232-1 [9] PSHeader structure shall be set to the time at which the MCX Server or any CSC Server event was observed (i.e. the timestamp field of the xIRI).

Table 7.20.2.3-1 shows the IRI type (see ETSI TS 102 232-1 [9] clause 5.2.10) to be used for each record type.

Table 7.20.2.3-1: IRI type for IRI messages

| Record type | IRI Type |
| --- | --- |
| MCXSIPMessage | REPORT |
| MCXStartOfInterceptionWithAlreadyRegisteredUE | REPORT |
| MCXStartOfInterceptionWithAlreadyEstablishedSession | REPORT |
| MCXManagementMessage | REPORT |
| MCVideoTransmissionControlMessage | REPORT |
| MCDataFileDistribution | REPORT |
| MCPTTFloorControlMessage | REPORT |

*** END OF SIXTH CHANGE ***

*** START OF SEVENTH CHANGE ***

### 7.20.4	Generation of xCC over LI_X3

#### 7.20.4.1	General

As described in TS 33.127 [5], content for MC service may be present in the MCX Server.

The interception of content at the MCCPTT Server and MC Video Server is covered in clause 7.20.4.2. The interception of content at the MC Data Server is covered in clause 7.20.4.3. The interception of content at the MC Data Content Server is covered in clause 7.20.4.4.

The CC-POI shall generate the xCC for the MC service media. The CC-POI shall then deliver the xCC to the MDF3.

#### 7.20.4.2	Generation of xCC at the CC-POI in the MCPTT Server and MCVideo Server

The CC-POI present in the MCPTT Server and MC Video Server shall send xCC over LI_X3 for any of the MC service events listed in TS 33.127 [5] where content is available.

The CC-POI shall set the payload format to indicate the appropriate payload type (5 for IPv4 packet, 6 for IPv6 packet) as described in ETSI TS 103 221-2 [8] (clauses 5.4 and 5.4.13).

#### 7.20.4.3	Generation of xCC at the CC-POI in the MCData Server

The CC-POI present in the MCData Server shall send xCC over LI_X3 for any of the MCData events listed in TS 33.127 [5] where content is available.

When the MCData contents consist of a SIP Message, the xCC payload shall consist of the MCData contents given as a SIP message as described in ETSI TS 103 221-2 [8] clause 5.4.10. The payload format shall be set to "SIP Message" (value 9).

When the MCData contents consist of an MSRP Message, the xCC payload shall consist of the MCData contents given as an MSRP Message as described in ETSI TS 103 221-2 [8] clause 5.4.14. The payload format shall be set to "MSRP Message" (value 13).

When the MCData contents consist of a MIME Entity, the xCC payload shall consist of the MCData contents given as a MIME encoded document as described in ETSI TS 103 221-2 [8] clause 5.4.16. The payload format shall be set to "MIME Message" (value 15).

#### 7.20.4.4	Generation of xCC at the CC-POI in the MCData Content Server

The CC-POI present in the MCData Content Server shall send xCC over LI_X3 for the File distribution event where content is available.

The file content consists of a MIME Entity. The xCC payload shall consist of the MCData content given as a MIME encoded document as described in ETSI TS 103 221-2 [8] clause 5.4.16. The payload format shall be set to "MIME Message" (value 15).

*** END OF SEVENTH CHANGE ***

*** START OF EIGHT CHANGE ***

### 7.20.5	Generation of CC over LI_HI3

#### 7.20.5.1	Generation of CC for MCVideo Service

When xCC is received over LI_X3 from the CC-POI in the MCVideo Server, the MDF3 shall populate the threeGPP33128DefinedCC field with a CCPPDU structure containing MCVideoCCPDU and send it over LI_HI3 interface to LEMF without undue delay. The MCVideoCCPDU shall contain the IPv4 or IPv6 packet received over LI_X3.

#### 7.20.5.2	Generation of CC for MCData Service

When xCC is received over LI_X3 from a CC-POI in the MCData Server or MCData Content Server, the MDF3 shall deliver the CC over LI_HI3 to the LEMF according to the clause 5.5.3 without undue delay.

The MDF3 shall populate the threeGPP33128DefinedCC field with a CCPDU structure containing MCDataCCPDU.

When the payload format of the xCC received over LI_X3 is "SIP Message" (value 9), the CC shall be mediated as MCDataCCPDU.encapsulatedMCDataPayload.sIP.

When the payload format of the xCC received over LI_X3 is "MSRP Message" (value 13), the CC shall be mediated as  MCDataCCPDU.encapsulatedMCDataPayload.mSRP.

When the payload format of the xCC received over LI_X3 is "MIME Message" (value 15), the CC shall be mediated as MCDataCCPDU.encapsulatedMCDataPayload.mIME.

The MDF3 shall populate the timeStamp field of the ETSI TS 102 232-1 [9] PSHeader structure of CC with the xCC timeStamp and the Payload Direction of the CCPayload structure to reflect the value received on xCC. The LIID and CID fields shall correctly reflect the target identity and communication session to which the CC belongs.

#### 7.20.5.3	Generation of CC for MCPTT Service

When xCC is received over LI_X3 from the CC-POI in the MCPTT Server, the MDF3 shall populate the threeGPP33128DefinedCC field with a CCPPDU structure containing MCPTTCCPDU and send it over LI_HI3 interface to LEMF without undue delay. The MCPTTCCPDU shall contain the IPv4 or IPv6 packet received over LI_X3.

*** END OF EIGHT CHANGE ***

START OF CHANGE 1

---a/33128/r19/TS33128Payloads.asn
+++b/33128/r19/TS33128Payloads.asn

@@ -341,7 +341,8 @@ XIRIEvent ::= CHOICE

341	341		    mCXStartOfInterceptionWithEstablishedSession        [193] MCXStartOfInterceptionWithEstablishedSession,

342	342		    mCXManagementMessage                                [194] MCXManagementMessage,

343	343		    mCVideoTransmissionControlMessage                   [195] MCVideoTransmissionControlMessage,

344		-	    mCDataFileDistribution                              [196] MCDataFileDistribution

344	+	    mCDataFileDistribution                              [196] MCDataFileDistribution,

345	+	    mCPTTFloorControlMessage                            [197] MCPTTFloorControlMessage

345	346		}

346	347

347	348		-- ==============

@@ -666,7 +667,8 @@ IRIEvent ::= CHOICE

666	667		    mCXStartOfInterceptionWithEstablishedSession        [193] MCXStartOfInterceptionWithEstablishedSession,

667	668		    mCXManagementMessage                                [194] MCXManagementMessage,

668	669		    mCVideoTransmissionControlMessage                   [195] MCVideoTransmissionControlMessage,

669		-	    mCDataFileDistribution                              [196] MCDataFileDistribution

670	+	    mCDataFileDistribution                              [196] MCDataFileDistribution,

671	+	    mCPTTFloorControlMessage                            [197] MCPTTFloorControlMessage

670	672		}

671	673

672	674		IRITargetIdentifier ::= SEQUENCE

@@ -705,7 +707,8 @@ CCPDU ::= CHOICE

705	707		    iMSCCPDU            [6] IMSCCPDU,

706	708		    rCSCCPDU            [7] RCSCCPDU,

707	709		    mCVideoCCPDU        [8] MCVideoCCPDU,

708		-	    mCDataCCPDU         [9] MCDataCCPDU

710	+	    mCDataCCPDU         [9] MCDataCCPDU,

711	+	    mCPTTCCPDU          [10] MCPTTCCPDU

709	712		}

710	713

711	714		-- ===========================

@@ -6174,6 +6177,13 @@ MCDataFileDistribution ::= CHOICE

6174	6177		    mCDataFileDownload [2] MCDataFileDownload

6175	6178		}

6176	6179

6180	+	-- See clause 7.20.2.7 for details of this structure

6181	+	MCPTTFloorControlMessage ::= SEQUENCE

6182	+	{

6183	+	    mCServiceIDOrGroupID                [1] MCServiceIDOrGroupID,

6184	+	    floorControlMessage                 [2] OCTET STRING

6185	+	}

6186	+

6177	6187		-- ====================

6178	6188		-- MCService parameters

6179	6189		-- ====================

@@ -6350,6 +6360,8 @@ MCDataCCPDU ::= CHOICE

6350	6360

6351	6361		MCVideoCCPDU ::= OCTET STRING

6352	6362

6363	+	MCPTTCCPDU ::= OCTET STRING

6364	+

6353	6365		-- ===================

6354	6366		-- 5G LALS definitions

6355	6367		-- ===================

END OF CHANGE 1

END OF LAST CHANGE