Source:	Philips International B.V., Qualcomm Incorporated

Title:	IVAS-4 updates on Control Data For Binaural Audio Rendering

Document for:	Agreement

Agenda Item:	1.3

# Introduction

At the SA4#120-e meeting, the IVAS-4 document [1] was updated by, among others, addition of provisioning of HRIR / BRIR filter sets and the default sets thereof to the “Control Data For Binaural Audio Rendering” section. Based on the discussion during the meeting an editor’s note was added, inviting inputs of modelling combined direct/early-reflections/late-reverb binauralization.

In response to this invite for input, this contribution proposes an addition of reverberation to the rendering control data constraints.

Reverberation rendering can be essential for producing realistic binaural audio, especially for AR/XR applications. It influences the perception of an auditory scene by providing room acoustics information and essentially increases the sense of sound externalization [3].

Assuming that the sound source and a listener are in the same room, the sound generated by a source arrives at a listener in three portions, as direct sound, early reflections, and late reverb [2]. Early reflections contain relatively sparse echoes that individually have perceptual relevance. As reflection density builds up, the perceptual relevance of the individual reflections decreases and transitions into the late reverb portion where densely populated echoes create a diffuse response. In the late reverb phase, individual echoes have no particular perceptual relevance but rather the combined effect gives rise to perceptual cues for identifying room size, source distance and the room’s material properties.

While early reflections can be efficiently modeled using impulse responses and have a temporally and spatially relevant component, late reverb can be sufficiently, and more efficiently, characterized by statistic means. In acoustics, rooms’ reverberation properties are typically represented by a number of generic parameters such as decay times and relative energy parameters that can be derived from room impulse responses. Early reflections can also be modeled parametrically, based on physical room properties.

With such a compact description, artificial reverberators are typically controlled to realistically simulate the reverberation characteristics in time and frequency domain.

This contribution proposes updating the IVAS-4 document [1] on design constraints to reflect the abovementioned aspects. Furthermore, it proposes adding section B.3 Reverb parameters providing the artificial reverb parameters description.

# Proposal

The following changes are proposed to be implemented in IVAS-4, table in section IVAS Codec Design Constraints.

| Control Data For Binaural Audio Rendering | The IVAS decoder/renderer shall support the provision of HRIR / BRIR filter sets and reverb parameters as control data for binaural audio rendering. The format of HRIR / BRIR filter sets and reverb parameters shall be documented in IVAS candidate deliverables. Editor’s Note: potential requirements on conversion tools from HRIRs/BRIRs in SOFA format into the internal format are to be addressed in IVAS-6 The IVAS decoder/renderer shall support the default HRIR / BRIR set for binaural rendering specified in Annex B. In addition, IVAS decoder/renderer shall support reverb parameters as described in Annex B. The IVAS decoder/renderer shall support head-tracking data as control data for binaural audio rendering. The format of the head-tracking data shall be either in quaternion or Euler angle domain [with other components TBD] every [5ms subframe / 20ms frame] Editor’s Note: head-tracking data format to be defined Annex of IVAS-4 |
| --- | --- |

# References

- Pdoc IVAS-4: IVAS Design Constraints, v. 0.6.0, August 2022

- Julius O. Smith III, Artificial Reverberation and Spatialization (MUS420/EE367A Lecture 3), June 13, 2018 (https://ccrma.stanford.edu/~jos/Reverb/Reverb.pdf)

- Isaac Engel and Lorenzo Picinali, Reverberation and its Binaural Reproduction: The Trade-off between Computational Efficiency and Perceived Quality, (https://www.intechopen.com/chapters/80155)