SA4-e (AH) Audio SWG post 131-bis-e	S4aA250027

Online, 5th May 2025

Source:	Dolby Laboratories Inc., Novamint, Viasat, Inmarsat, Iridium, Sateliot, EDF, EchoStar, Aalyria

Title:	Application scenario for IMS voice call using LEO and MEO satellites

Spec:	3GPP TR 26.940 v0.0.2

Agenda item:	1.9

Document for:	Agreement

1. Introduction

At the recent SA#107 meeting, the "Study on Ultra Low Bitrate Speech Codec" has been approved. The WID [1] primarily focuses on use cases like IMS Voice Call over GEO satellites and the resulting transmission parameters. However, as indicated in clause 5.4 of TR 22.887 [2], the future of satellite communications is moving toward hybrid satellite networks utilizing the services from LEO, MEO and GEO satellites and hence use cases like IMS voice call over LEO and MEO satellite and the resulting transmission parameters should/shall also be studied as part of the ULBC study.

Some of the hybrid use case of IMS voice call using LEO, MEO and GEO satellite accesses is specified in clause 5.4 of TR 22.887 [2], and corresponding requirements and KPIs for LEO, MEO and GEO satellite accesses are provided in Tables 7.4.1-1 and 7.4.2-1 in TS 22.261 [3].

#### Reference architecture in 3GPP

5G Satellite network-based architectures are mentioned in Annex A.2 of TR 22.822 [4].

#### Conversation communication between two-parties

As stated in Clause 5 of TR22.887, the use case focuses on conversational communication between two parties:

- Case I: Mobile-originated satellite-to-terrestrial call and Mobile-terminated satellite-from-terrestrial call

- Case II: Satellite-to-satellite call

The IMS platform is utilized due to its ability to ensure interoperability with other operators (e.g., VoLTE/VoNR) and call systems (e.g., fixed-line networks). This means that on the terrestrial side, all originating/termination IMS-supported phone calls with satellite terminal are allowed, leading to interoperability requirements for ULBC to transcode with other operator/system-supported codecs such as AMR and EVS.

Observation#1: If the other party in the conversation is on a terrestrial network, transcoding between ULBC and other IMS-supported codecs available in SDP negotiation should be supported. However, if both parties are using satellite phones that support the ULBC , transcoding may be avoided.

#### Transmission data rate

Transmissions data rates for satellite access are highlighted in Table 7.4.2-1 of TS 22.261 [3].

#### Propagation delay

The propagation delay via LEO, MEO and GEO satellites are highlighted in Table  7.4.1-1 of TS 22.261 [3].

2. Reason for Change

The present document provides the applied scenarios of ULBC in the context of voice communication via Low Earth Orbit (LEO) and Medium Earth Orbit (MEO) satellites based on the use case captured in TR 22.887 [2] and requirements captured in TS 22.261 [3].

3. Proposal

It is proposed to agree the following changes to draft 3GPP TR 26.940 [5]

* * * First Change * * * *

# 4	Application scenarios

### […]

## 4.3	Scenario 2: IMS Voice Call over LEO/MEO

### 4.3.1	General

This is an application scenario of ULBC in the context of voice communication via Low Earth Orbit (LEO) and Medium Earth Orbit (MEO) satellites.

### 4.3.2 	Background

Satellite communication plays a vital role in extending terrestrial network coverage, ensuring seamless connectivity for users anytime, anywhere. This technology unlocks new opportunities across various sectors, including smartphones and IoT. While Geostationary Earth Orbit (GEO) satellites may play an important role in delivering ultra-low-data-rate services on a global scale, LEO and MEO based satellite access may represent alternatives with attractive KPIs in terms of propagation delay and data transmission throughput.

### 4.3.3	Required KPIs

KPIs for LEO, MEO and GEO satellite accesses are provided in Tables 7.4.1-1 and 7.4.2-1 in TS 22.261 [3]. The following Table lists the maximum propagation delays for the three cases:

Table 4.3.3-1: Propagation delay via satellite access

|  | UE to ground max propagation delay [ms] [NOTE1] | End-to-end latency requirement [ms] [NOTE2] |
| --- | --- | --- |
| LEO | 26 | 35 |
| MEO | 198 | 95 |
| GEO [NOTE3] | 272 | 285 |

NOTE1:	Delay between UE and ground station via satellite link; Inter satellite links are not considered.

NOTE2:	 5ms network latency is assumed and added to satellite one-way delay.

NOTE3: 	Already described in clause 4.2.

According to Table 7.4.2-1 in TS 22.261, for the pedestrian scenario with handheld UE it is required to support experienced data rates on DL of [1] Mbit/s on DL and [100] kbps on UL. This applies for satellite access in general without distinction between GEO and NGSO. This corresponds to an area traffic capacity of 1.5 Mbits/s/km2 on DL and 150 kbit/s/km2 on UL when assuming an overall user density of at least [100]/km2 and an activity factor of [1,5]%. For GEO satellite access specific requirements apply as discussed in clause 4.2.

NOTE: 	The provided requirements are a working assumption in TS 22.261. These requirements may be subject to change.

### 4.3.4 	Scenario Description

From user perspective, the typical UE supporting LEO or MEO satellite access is not different compared to a UE supporting GEO satellite access. Hence, the scenario description of clause 4.2.2 applies.

Editor's note:	it is assumed for the time being that a codec supporting scenario described in clause 4.2 will allow to support this scenario too - it will be revised if there are contradictory evidences.

### 4.3.5	Interfaces and interface properties

#### 4.3.5.1	General

This clause identifies the relevant interfaces between architectural entities such as UEs, satellites, ground stations, MGW, eNBs. Relevant interface properties are described.

#### 4.3.5.2	Interfaces

Editor’s note: identify all interfaces such as UE1 -> satellite (UL), UE1 <- satellite (DL), satellite -> ground station (DL), satellite <- ground station (UL), ground station -> eNB1, ground station <- eNB1, eNB1 -> UE2, eNB1 <- UE2, eNB1 -> MGW -> UE2, eNB1 <- MGW <- UE2

#### 4.3.5.3	Interface properties

Editor’s note: Describe basic parameters like min latency, bit rate limits, traffic characteristics, transparent vs regenerative payload options.

### 4.3.6	Derived high-level prerequisites

Editor’s note: The high-level prerequisites will be derived once the full set of requirements and key interface properties are set.

* * * End of Changes * * * *

[1] Tdoc SP-250378, "Study on Ultra Low Bitrate Speech Codec"

[2] TR 22.887, "Study on satellite access - Phase 4"

[3] TS 22.261, "Service requirements for the 5G system"

[4] TR 22.822, "Study on using satellite access in 5G"

[5] 3GPP TR 26.940, "Study on Ultra Low Bitrate Speech Codecs"