Source:	Orange

Title:	On design constraints for ULBC

## Document for:	Agreement

## Agenda Item:	4.4

- Introduction

The “Study on Ultra Low Bitrate Speech Codec” (FS_ULBC) has been approved at SA#107 [1], and the WID has been further revised in [2].

In the present Tdoc we address the following study objectives from the WID description [2]:

3.	Identify the relevant design constraints for such a codec, in coordination with other WGs, including

-	Bit rates

-	Sample rate and audio bandwidth

-	Frame length

-	Complexity and memory demands

-	Algorithmic delay

-	Packet loss concealment (PLC)

-	Potential use of noise suppression as part of the codec

-	Discontinuous transmission including voice activity detection and comfort noise

-	Speech quality

-	Robustness to non-speech input

The present Tdoc is based on the template proposed in [3] and reflects views from the Source expressed in previous email discussions.

- Proposed design constraints

| Parameter | Recommended Design Constraint | Note |
| --- | --- | --- |
| Bit rates | tbd | Dependency: RAN simulations and system considerations |
| Sample rate and audio bandwidth | Support of [8, 16, 32, 48] kHz / [NB, WB, SWB, FB] required Editor’s Notes: - Support of 8 kHz justified for interoperability. Would NB be just tested and supported “externally” based on some external resampling? - Support of 48 kHz may be considered at higher bitrate operation - Support at least a single model (e.g.in SWB)? | Many neural codecs operate at 24 kHz, this specific sampling rate should be discussed |
| Frame length | Multiple of 20 ms | Dependency: RAN simulations and system considerations |
| Complexity and memory demands | tbd | Expected deliverable (implementation of ULBC) to be clarified (e.g., C, Python), realistic measurements vs. simplified model to be clarified |
| Algorithmic delay | tbd | The algorithmic delay is defined as the frame size buffering delay plus any other delays inherent in the codec algorithm (e.g., look-ahead, sample-rate conversion, and decoder post-processing) |
| Packet loss concealment (PLC) | Support required Editor’s Note: typical loss profiles/characteristics to be clarified | Support of JBM to be added |
| Potential use of noise suppression as part of the codec | If noise suppression is supported, it is expected that it is possible to disable it Editor’s Notes: - if noise suppression is supported in the codec, there is also a potential issue of "tandeming", if noise suppression is also integrated in the audio front-end (either in sending or even receiving) as well as in the codec. - Clarifications needed: - need to support in UBLC? (typically vendor specific, defined outside the codec) - noise suppression supported at the encoder and/or decoder? - impacts on test methodology, DTX operation/performance | Motivation: disable to be able to test feature apart avoid tandeming in real operation |
| Discontinuous transmission including voice activity detection and comfort noise | Support required Editor’s Note: typical radio characteristics and optimizations (SPS, DRX, bitrate) to be clarified |  |
| Robustness to non-speech input | tbd | Editor’s note: May need to be in performance requirement |

## References

- 3GPP SP-250378, SID on Ultra Low Bitrate Speech Codec, Source: China Mobile Com. Corporation, vivo, Fraunhofer IIS, Qualcomm Incorporated, Spreadtrum, Dolby Laboratories Inc., Xiaomi, Huawei

- 3GPP SP-250635, Revised Study on Ultra Low Bitrate Speech Codec, Source : SA4

- 3GPP Tdoc S4-250976, Template for ULBC design constraints, Source: Fraunhofer IIS