3GPP TSG SA WG4 Audio SWG AH	S4aA250275

Erlangen, Germany, 23-25 September 2025	revision of S4aA250257

Source:	Fraunhofer IIS

Title:	pCR on Design Constraints of ULBC: Static and Dynamic Configuration

Spec:	3GPP TR 26.940

Agenda item:	4.4

Document for:	Agreement

1. Introduction

TR 26.940 V.2.0 (S4-251152) does not cover aspects relevant for designing ULBC, particularly the establishment of common principles for distinguishing static and dynamic configuration elements. These principles will influence both bitstream structure and session signalling. This pCR proposes design constraint templates to support efficient transmission over constrained links such as NTN-IoT.

2. Reason for Change

To introduce clear design constraints for ULBC related to static and dynamic configuration, ensuring efficient signalling and minimal overhead.

3. Background

3GPP’s speech codecs, just as almost all other media codecs, need to be properly configured on both ends of the communication. To configure there are typically static configuration elements (usually hardcoded or exchanged before the communication session starts) and dynamic configuration elements (usually transmitted inside or together with the coded bit stream, or by other control mechanisms), and can change for every speech frame. A general rule of thumb has often been that static configuration data is transmitted “out-of-band”, i.e. on CP (Control Plane) or via SDP, and dynamic configuration data is transmitted “in-band”, i.e. on UP (User Plane).

This document provides an analysis of the configuration elements of 3GPP’s existing speech codecs and how they’d be applicable to ULBC communications.

It should also be noted that this document is from an SA4 perspective and thus looking at codec necessities. Depending on the architecture defined, e.g. in SA2, certain adaptations may become necessary.

4. Configuration Elements in 3GPP speech codecs

In the following some common configuration elements are taken as examples, specifically the codec identification, bit rate / frame type, and sample rate / audio bandwidth. Other configuration elements exist, see e.g. IVAS.

Codec Identification

In MTSI-based communications one needs to identify the codec in use, which is typically one of “AMR”, “AMR-WB”, or “EVS” for speech/audio. The identification is performed using SDP where the codec string is present in the ‘rtpmap’ attribute and mapped to a dynamic RTP payload type, e.g. “a=rtpmap:97 AMR/8000/1”.

Bit Rate / Frame Type

The frame type (FT) defines the bit rate of the speech frames. For AMR the FT is 4 bits, for EVS this is already 5 bits to distinguish between EVS primary and EVS-IO modes while further audio bandwidth information is embedded in the EVS speech frames. This information is necessary to successfully decode a speech frame due to the “adaptive multi-rate” functionality of the existing 3GPP speech codecs. This information is available in the headers of the RTP payload of the coded frames but the range may be negotiated using SDP for e.g. setting rate limits. A special frame type case is “SID”, which indicates signal-adaptive background noise characteristics during silence periods in DTX operation; a master flag whether DTX is used is conveyed via SDP.

Sample rate and audio bandwidth

In former times the sample rate of speech codecs has always been selected to cover the supported bandwidth, e.g. 8kHz sample rate for NB to cover speech components up to 3.5kHz and was fixed for a codec and thus implicitly known. With EVS this limitation was lifted since EVS is an “adaptive multi-rate multi-bandwidth” codec. Thus the sample rate can differ from the audio bandwidth the codec operates on. This flexibility in EVS comes at the cost of additional signalling: decoders must explicitly know the bandwidth, which is therefore part of the EVS bitstream header.

5. Impact on ULBC

It is understood from previous ULBC discussions that bits are a scarce resource for ULBC communication on CP and UP, and excessive configuration signalling can negatively impact call setup and cause repeated overhead. Since dynamic configurations may need to be repetitive (and thus consume in-band bits that should rather be spent on the coded bit stream) as much of the dynamic configuration as possible should be made static. In addition to link capacity limitations, handshake procedures for parameter negotiation introduce latency (especially over GEO) and may negatively impact user experience by causing long call setup times. To avoid those latencies it is preferable to avoid the handshake for negotiating parameters and instead advertise them only.

Depending on discussions in other WGs certain limitations may apply, but the general principle will remain to keep the number of bits used for configuration as low as possible.

While other WGs are exploring signalling methods (e.g., IP vs. non-IP, SDP usage, negotiated vs. advertised parameters), SA4 must focus on ensuring that codec configuration needs are efficiently supported, regardless of the signalling path chosen. Their decisions are of high importance but don’t change the principle, rather how it can be achieved in coordination with the other WGs.

Conclusion

Given the analysis of existing codecs and ULBC application scenarios, the general principle is to minimize configuration bits and favour static configuration whenever feasible.

SA4 should ensure, in collaboration with other WGs, that all essential configuration elements are conveyed efficiently without compromising the extremely low bit rates targeted by ULBC.

It is proposed to agree the following changes to 3GPP TR 26.940.

* * * First Change * * * *

## 6.3	Additional Design Coniderations

### 6.3.1	Codec Parameters and Configuration

Codec parameters, including codec identification, audio bandwidth, bit rate, functionalities such as DTX, are commonly aligned between encoder and decoder configurations. Those can be be characterized as static and dynamic parameters. Dynamic parameters may need to be repetitive and are typically include in each packet or frame. Static parameters are typically not or only rarely changed within a session and are commonly exchanged by SDP in IMS or predefined.

Editor’s Note: Common static and dynamic parameters are to be identified, reflecting the necessary parameter set for the envisioned scenarios.

* * * End of Changes * * * *