Source:	Xiaomi Technology.

Title:	Update proposals on “conclusion and recommendations” clause  in TR26.933

## Document for:	Discussion and agreement

## Agenda Item:	1.6

# Introduction

At SA4-129-e closing plenary meeting, a TR version was agreed in S4-241787, however, the "10 Conclusions and Recommendations" of the version is still in brackets for further discussion. There was an offline discussion on August 27 which focused on discussion of the conclusion and recommendation clause, the outputs of the offline discussion are in the following, they are for further discussion and agreement.

# Content

There are two items from the outputs of the offline discussion.

Item one: adding following “NOTE1” after clause 8.5.

NOTE1:  Higher-capability headphones often come with built-in processing capabilities, such as active noise cancellation (ANC), and other audio enhancement features. These features are processed directly within the headphones, which can reduce the need for additional processing in UE terminals. On the other hand, many other headphones, particularly those with lower capabilities, may not support or process these enhancements independently. As a result, the UE terminal may still need to perform some audio processing tasks to ensure that the audio quality meets certain requirements. So, a minimum quality may need to be defined for ensuring suitable captured signals.

Item two: updated clause 10 are as follows:

The TR covers various UE categories including mobile phones, headphones, tablets, laptops, watches, XR devices and cars. On playback side, most UE categories typically support stereo, and/or binaural audio through on-device loudspeakers or headphones. On capture side, some example issues for immersive audio capture are summarized below:

1)Before supplying the raw multi-microphone captured signal, support for equalization filter design may be needed to achieve balanced frequency response characteristics across all microphones.

2)As device orientation can change during audio capture, provision of microphone selection and/or device orientation tracking may be needed for appropriate compensation.

3)Multi-channel AEC, noise reduction, microphone array signal processing and beamforming may be used to improve quality of the immersive audio signal.

4)As devices are available in different capability levels, different immersive audio format may be supported.

NOTE1: When a binaural microphone is used for capturing, support might be needed for binaural to stereo conversion if it for loudspeaker consumption at the receiver side by appropriate signaling information.

NOTE2: Latency is a critical element but was not studied. Latency requirements would need to be defined.

NOTE3: The need to negotiate the best appropriate format depending on the capturing and rendering capabilities is for further study.

Widespread on-device support of immersive audio capture functionality may help to accelerate the deployment of 3GPP IVAS. To address the needs for immersive audio capture, including the identified key issues, it is recommended to launch work to specify minimum performance requirements or performance objectives on immersive audio capture and example designs meeting these.

# Conclusion

It is proposed to discuss the above 2 items and make consensus on the contents. Then to integrate the agreed contents into S4-241787 to the finalize the TR 26.933.

## References

- S4-241787. S4-241787-TR26.933-v1.1.0