3GPP TSG-SA WG4 Meeting post 132	S4aA250046

online, June 18th 2025

Source:	vivo

Title:	Simulation results for verifying TBS feasibility

Spec:	3GPP TR 26.940 v0.2.0

Agenda item:	1.9

Document for:	Discussion/Agreement

1. Introduction

At the recent SA4#132 post meeting, some TBS and simulation assumption such as the channel model are agreed.

However, some simulation related details are still need more discussions to be determined, this contribution will provide more detail analysis on following aspects:

- DL/UL link budget analysis to determine the target SNR

Link budget is calculated based on SET-1 GEO parameters and handheld UE parameters with different UE transmit power considerations and latitude considerations, and then the CNR results under different assumptions are categorized to different cases to be regarded as the target SNR for RAN simulation.

- HD-FDD scheduling structure

SPS structure is considered in RAN simulation for simplification. The DL and UL transmission are considered be transmitted within the bundling time by TDM manner.

- Methodology of evaluations

Methodology of evaluations are provided to verify the TBS validation based on the SPS assumption. All the TBS values with black colour as listed in Table 1,2,3 in TR 26.940 are valid based on simulation results, extra larger TBS should not be supported.

2. Reason for Change

The present document provides a methodology to verify the TBS validation for RAN simulation that used to derive the error trace to reflect the realistic GEO channel.

- Link budget analysis

In the TR 38.821[1], link budget is represented by Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE or carrier-to-noise ratio (CNR) if interference is not considered. For link level simulation, CNR can be used as target SNR to determine whether the target BLER can be achieved, as shown in Figure 1, if the corresponding BLER at the target SNR is higher than target BLER, the TB would be failed to be decoded.

Figure 1 Relationship of target SNR and target BLER based on the SNR-BLER curve of link-level simulation

Based on the link budget analysis in TR38.821, the formula for CNR calculation is

CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ]

In the following Table 1, some candidate values for parameters of CNR calculation are provided. For UE parameters, both 1Rx and 2 Rx are considered and different UE transmit power should also be taken into account since high-power UE other than PC3 (23dBm) has been agreed to be feasible for Rel-19 IoT-NTN in RAN4, e.g., PC2 (26dBm), PC1 (31dBm).

Different assumptions[2] in Table 1 are categorized to different cases to make all the possible combination of different parameter assumptions clearly, and corresponding CNR results are provided in Table 2 and Table 3 for UL and DL, respectively.

Table 1 Link budget parameters

|  | Parameter | Value |
| --- | --- | --- |
|  | EIRP density | 59 dBW/MHz |
| Satellite parameters | G/T | 19 dB K-1 |
|  | Antenna polarization | Circular polarization |
|  | Satellite altitude | 35786km |
| UE parameters | Antenna polarization | Linear polarization |
|  | Antenna configuration | 1Tx1Rx, 1Tx2Rx |
|  | Antenna gain | 0dBi |
|  | Elevation angle | 12.5 degree |
|  | UE power | 23dBm, 26dBm, 31dBm |
|  | Noise Figure | 7dB |
| System parameters | Carrier frequency | 2GHz |
|  | SCS | 3.75kHz, 15kHz |
|  | BW | 15kHz |
|  | PLFS | 190.6 dB |
|  | PLA | 0 dB |
|  | PLSM | 3 dB |
|  | PLSL (high latitude, low latitude) | 0 dB, 2.2dB |
|  | PLAD | DL: 3dB Polarization loss UL: 3dB Polarization loss+3dB additional loss |

Table 2 UL CNR results under different parameter assumption

|  | SCS/BW | UE power | PLSL | UL CNR |
| --- | --- | --- | --- | --- |
| Case 1 | 3.75kHz/3.75kHz | 23dBm | 0 | 5.28 dB |
| Case 2 | 15kHz/15kHz | 23dBm | 0 | -0.74 dB |
| Case 1-1 | 3.75kHz/3.75kHz | 23dBm | 2.2 | 3.08 dB |
| Case 2-1 | 15kHz/15kHz | 23dBm | 2.2 | -2.94 dB |
| Case 3 | 3.75kHz/3.75kHz | 26dBm | 0 | 8.28dB |
| Case 4 | 15kHz/15kHz | 26dBm | 0 | 2.26dB |
| Case 3-1 | 3.75kHz/3.75kHz | 26dBm | 2.2 | 6.08dB |
| Case 4-1 | 15kHz/15kHz | 26dBm | 2.2 | 0.06dB |
| Case 5 | 3.75kHz/3.75kHz | 31dBm | 0 | 13.28dB |
| Case 6 | 15kHz/15kHz | 31dBm | 0 | 7.26dB |
| Case 5-1 | 3.75kHz/3.75kHz | 31dBm | 2.2 | 11.08dB |
| Case 6-1 | 15kHz/15kHz | 31dBm | 2.2 | 5.06dB |

Table 3 DL CNR results under different parameter assumption

|  | SCS/BW | UE Rx number | PLSL | DL CNR |
| --- | --- | --- | --- | --- |
| Case 1 | 15kHz/180kHz | 2Rx | 0 | 2.39dB |
| Case 1-1 | 15kHz/180kHz | 2Rx | 2.2 | 0.19dB |
| Case 2 | 15kHz/180kHz | 1Rx | 0 | -0.61dB |
| Case 2-1 | 15kHz/180kHz | 1Rx | 2.2 | -2.81dB |

- HD-FDD framework

The issue of HD-FDD framework for IoT-NTN has been draw the attention. For the scheduling assumption, there are two following options,

- Dynamic scheduling

In current NB-IoT specification, DL/UL should be scheduled by dynamic DCI format N1 and DCI format N0, respectively as shown in Figure 1. The two DCI format should have a scheduling gap for the consideration of UE processing time. Besides, NPDCCH repetition should also be considered but it is hard to suggested a proper value for the duration of NPDCCH for different cases. DL/UL switching time is regarded as 1ms according to the guard period for half-duplex FDD operation defined in TS36.211[3].

Figure 1 Dynamic scheduling scheme for IoT NTN

- Semi- Persistent Scheduling

SPS can be used to save the scheduling overhead of PDCCH, only the duration of NPDSCH and NPDSCH are need to be considered within the bundling time as shown in Figure 2, which can make the evaluation work be simplified either.

Figure 2 Semi- Persistent scheduling scheme for IoT NTN

- Methodology of evaluations

To further evaluate the validation of the candidate TBS, a methodology is provided as following

Step 0: Regarding the scheduling scheme as SPS, only the TBS that resulted with suitable DL/UL allocation would be chosen, i.e. duration time of NPDSCH + duration time of NPUSCH of same/similar TBS cannot be larger than bundling time.

Step 1： Maximum allowed repetition number of NPUSCH is computed as nRepNPUSCH=maxnRepNPUSCH* Number of RU*RU duration<bundling time.

Step 2: Link level simulation to find the corresponding required SNR @2% BLER under the repetition number of NPUSCH in Step 1.

Step 3: Compare UL link budget result CNR with the required SNR in Step 2, if CNR is larger than the required SNR in Step 2, corresponding NPUSCH duration time with that repetition number is calculated as TNPUSCH=nRepNPUSCH* Number of RU*RU duration, then the corresponding reserved NPDSCH duration time is calculated as TNPDSCH=Bundling time- TNPUSCH-DL_UL_switchingtime.

Step 4: Maximum allowed repetition number of NPDSCH is computed as nRepNPDSCH=maxnRepNPDSCH* Number of SF*SF duration<TNPDSCH.

Step 5: Link level simulation to find the corresponding required SNR @2% BLER under the repetition number of NPDSCH in Step 4.

Step 6: Compare DL link budget result CNR with the required SNR in Step 5, The TBS is regarded as valid if CNR is larger than the required SNR in Step 5.

Step 7: For the valid TBS, data rate is calculated by

Data rate=TBSBundling time*(1-target_BLER)

- Link level Simulation parameters

Link level simulation parameters are summarized as following Table 4.

Table 4 Link level simulation assumptions

|  | Parameter | Value |
| --- | --- | --- |
| Channel | Carrier frequency | 2GHz |
|  | Channel model | NTN-TDL-C |
|  | Frequency error | 0 |
|  | Timing error | 0 |
|  | Doppler spread | 1Hz, 5 Hz |
| Transmitter | SCS | 3.75kHz and 15kHz for NPUSCH 15kHz for NPDSCH |
|  | Number of tones | 1 for NPUSCH 12 for NPDSCH |
|  | Waveform | DFT-s-OFDM |
|  | IRU/ISF | Ref. to table 16.5.1.2-2 for NB-IoT for NPUSCH and table 16.4.1.5.1-1 for NB-IoT for NPDSCH in TS36.213 |
|  | Modulation order | QPSK |
|  | ITBS (MCS) | Ref. to table 16.5.1.2-2 for NB-IoT for NPUSCH and table 16.4.1.5.1-1 for NB-IoT for NPDSCH in TS36.213 |
|  | Repetition number | 1,2,4,8,16, 32 |
| Receiver | Receiver Algorithm | MMSE |
|  | Channel estimation | Real channel estimation |
| KPI | BLER target | 2% |

- Link level simulation results

Based on the evaluation methodology in above section, for TBS of 144 with 80ms bundling time, the suitable case of UL and DL and corresponding parameters are listed in Table 5. And the results of all TBS are summarized in Table 6. It can be observed that all the TBS in listed in Table 1,2,3 of TR 26.940 can achieve target BLER of 2% with different CNR assumption. In terms of the bundling time with same TBS, the larger the bundling time is, the more cases can be supported. Specially, for all the TBS of the smallest bundling time of 80ms, PC3 of UE transmit power cannot support any NPUSCH/ NPDSCH transmission successfully, PC1 of UE transmit power assumption should be used to achieve the target BLER of 2%.  In terms of the TBS with same bundling time, the larger TBS is, the less cases can be supported. And it seems that extra higher TBS cannot be supported since the largest TBS in Table 1,2,3 in TR 26.940 ( i.e. 424, 808, 1544 for 80ms bundling time, 160ms bundling time and 320ms bundling time, respectively.) are already on the basis of PC1 UE transmit power assumption , maximum antenna gain assumption and minimum loss.

Observation#1: In terms of the bundling time with same TBS, the larger the bundling time is, the more cases can be supported.

Observatin#2: In terms of the TBS with same bundling time, the larger TBS is, the less cases can be supported.

Observation#3: The largest TBS for each bundling time can only be supported on the basis of maximum UE power, maximum UE antenna gain and minimum loss.

Proposal#1: At least taking HPUE of PC1 into consideration for the error trace generation simulation assumption.

Proposal#2: Based on the above analysis, the TBS values with black colour as listed in Table 1,2,3 in TR 26.940 are valid, extra larger TBS should not be supported.

Table 5 Performance of TBS 144bit w/ 80ms bundling time

| TBS | NPUSCH 3.75kHz Single tone |  |  |  |  |  |  | NPDSCH 15kHz Multi-tone |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IRU | NRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @2% BLER | TNPUSCH | UL Support case | TNPDSCH | ISF | NSF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @2% BLER | DL Support case |
| 144 | 3 | 4 | 1 | / |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 2 | 5 | 1 | 6.57 dB | 64 ms | Case3; Case5; Case5-1 | 15ms | 3 | 4 | 1 | 2 | 1.82 dB | Case A |
|  |  |  |  |  |  |  |  |  | 1 | 2 | 5 | 4 | 1.31 dB | Case A |
|  | 0 | 1 | 10 | 2 | 6.49 dB | 64 ms | Case3; Case5; Case5-1 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 0 | 1 | 10 | 8 | -1.52 dB | Case A; Case A-1; Case B |
|  | NPUSCH 15kHz Single tone |  |  |  |  |  |  | NPDSCH 15kHz Multi-tone |  |  |  |  |  |  |
|  | IRU | NRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @2% BLER | TNPUSCH | UL Support case | TNPDSCH | ISF | NSF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @2% BLER | DL Support case |
|  | 3 |  | 1 | 2 | 1.82 dB | 64 ms | Case 4; Case 6; Case 6-1; | 15ms | 3 | 4 | 1 | 2 | 1.82 dB | Case A |
|  | 1 |  | 5 | 4 | 1.31 dB | 64 ms | Case 4; Case 6; Case 6-1; |  | 1 | 2 | 5 | 4 | 1.31 dB | Case A |
|  | 0 |  | 10 | 8 | 1.44 dB | 64 ms | Case 4; Case 6; Case 6-1; |  | 0 | 1 | 10 | 8 | -1.52 dB | Case A; Case A-1; Case B |

Table 6 TBS Validation with different cases

| Bundling Time | TBS | UL | DL |
| --- | --- | --- | --- |
| 80ms | 144 | Case 3; Case 5; Case 5-1 | Case A; Case A-1; Case B |
|  |  | Case 4; Case 6; Case 6-1 | Case A; Case A-1; Case B |
|  | 256 | Case 5; Case 5-1 | Case A; Case A-1; Case B |
|  |  | Case 4; Case 6; Case 6-1 | Case A; Case A-1; Case B |
|  |  | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 328 | Case 5 | Case A; Case A-1 |
|  |  | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 424 | Case 6 | Case A; Case A-1; Case B |
| 160ms | 208 | Case 1; Case 3; Case 3-1; Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 3; Case 3-1; Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 4; Case 4-1; Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 2; Case 4; Case 4-1; Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 424 | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 600 | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 808 | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
| 320ms | 328 | Case 1; Case 3; Case 3-1; Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 1; Case 1-1; Case 3; Case 3-1; Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 2; Case 4; Case 4-1; Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 4; Case 4-1; Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 776 | Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 1000 | Case 3; Case 5; Case 5-1 | Case A; Case A-1 |
|  |  | Case 5 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 1544 | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |

Note: For the case of NPUSCH with 1000TBS, we use the value of 1032 for NPDSCH assumption to balance the MCS of each channel.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.940 [4].

* * * First Change * * *

5.2.3 Evaluating TBS feasibility

5.2.3.1 Methodology of evaluation

Step 0: Regarding the scheduling scheme as SPS, only the TBS that resulted with suitable DL/UL allocation would be chosen, i.e. duration time of NPDSCH + duration time of NPUSCH of same/similar TBS cannot be larger than bundling time.

Step 1： Maximum allowed repetition number of NPUSCH is computed as nRepNPUSCH=maxnRepNPUSCH* Number of RU*RU duration<bundling time.

Step 2: Link level simulation to find the corresponding required SNR @2% BLER under the repetition number of NPUSCH in Step 1.

Step 3: Compare UL link budget result CNR with the required SNR in Step 2, if CNR is larger than the required SNR in Step 2, corresponding NPUSCH duration time with that repetition number is calculated as TNPUSCH=nRepNPUSCH* Number of RU*RU duration, then the corresponding reserved NPDSCH duration time is calculated as TNPDSCH=Bundling time- TNPUSCH-DL_UL_switchingtime.

Step 4: Maximum allowed repetition number of NPDSCH is computed as nRepNPDSCH=maxnRepNPDSCH* Number of SF*SF duration<TNPDSCH.

Step 5: Link level simulation to find the corresponding required SNR @2% BLER under the repetition number of NPDSCH in Step 4.

Step 6: Compare DL link budget result CNR with the required SNR in Step 5, The TBS is regarded as valid if CNR is larger than the required SNR in Step 5.

5.2.3.2 Simulation results

Table 1 UL CNR results under different parameter assumption

|  | SCS/BW | UE power | PLSL | UL CNR |
| --- | --- | --- | --- | --- |
| Case 1 | 3.75kHz/3.75kHz | 23dBm | 0 | 5.28 dB |
| Case 2 | 15kHz/15kHz | 23dBm | 0 | -0.74 dB |
| Case 1-1 | 3.75kHz/3.75kHz | 23dBm | 2.2 | 3.08 dB |
| Case 2-1 | 15kHz/15kHz | 23dBm | 2.2 | -2.94 dB |
| Case 3 | 3.75kHz/3.75kHz | 26dBm | 0 | 8.28dB |
| Case 4 | 15kHz/15kHz | 26dBm | 0 | 2.26dB |
| Case 3-1 | 3.75kHz/3.75kHz | 26dBm | 2.2 | 6.08dB |
| Case 4-1 | 15kHz/15kHz | 26dBm | 2.2 | 0.06dB |
| Case 5 | 3.75kHz/3.75kHz | 31dBm | 0 | 13.28dB |
| Case 6 | 15kHz/15kHz | 31dBm | 0 | 7.26dB |
| Case 5-1 | 3.75kHz/3.75kHz | 31dBm | 2.2 | 11.08dB |
| Case 6-1 | 15kHz/15kHz | 31dBm | 2.2 | 5.06dB |

Table 2 DL CNR results under different parameter assumption

|  | SCS/BW | UE Rx number | PLSL | DL CNR |
| --- | --- | --- | --- | --- |
| Case 1 | 15kHz/180kHz | 2Rx | 0 | 2.39dB |
| Case 1-1 | 15kHz/180kHz | 2Rx | 2.2 | 0.19dB |
| Case 2 | 15kHz/180kHz | 1Rx | 0 | -0.61dB |
| Case 2-1 | 15kHz/180kHz | 1Rx | 2.2 | -2.81dB |

Editor’s Note: Whether higher transmit power should be considered is dependent on RAN4.

Table 3 TBS Validation with different cases

| Bundling Time | TBS | UL | DL |
| --- | --- | --- | --- |
| 80ms | 144 | Case 3; Case 5; Case 5-1 | Case A; Case A-1; Case B |
|  |  | Case 4; Case 6; Case 6-1 | Case A; Case A-1; Case B |
|  | 256 | Case 5; Case 5-1 | Case A; Case A-1; Case B |
|  |  | Case 4; Case 6; Case 6-1 | Case A; Case A-1; Case B |
|  |  | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 328 | Case 5 | Case A; Case A-1 |
|  |  | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 424 | Case 6 | Case A; Case A-1; Case B |
| 160ms | 208 | Case 1; Case 3; Case 3-1; Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 3; Case 3-1; Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 4; Case 4-1; Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 2; Case 4; Case 4-1; Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 424 | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 600 | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 808 | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
| 320ms | 328 | Case 1; Case 3; Case 3-1; Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 1; Case 1-1; Case 3; Case 3-1; Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 2; Case 4; Case 4-1; Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 4; Case 4-1; Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 776 | Case 5; Case 5-1 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 1000 | Case 3; Case 5; Case 5-1 | Case A; Case A-1 |
|  |  | Case 5 | Case A; Case A-1; Case B; Case B-1 |
|  |  | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |
|  | 1544 | Case 6; Case 6-1 | Case A; Case A-1; Case B; Case B-1 |

* * * End of Changes * * *

[1] TR 38.821, " Solutions for NR to support non-terrestrial networks (NTN)".

[2] TR 38.811, " Study on New Radio (NR) to support non-terrestrial networks ".

[3] TS 36.211, " Physical channels and modulation"