3GPP TSG-SA WG4 Meeting post 132	S4aA250082

online, July 11th 2025

Source:	vivo

Title:	Updates on end to end simulation for error trace generation

Spec:	3GPP TR 26.940 v0.2.0

Agenda item:	1.9

Document for:	Discussion/Agreement

1. Introduction

At the recent SA4#132 post meeting, simulation parameters are mainly agreed, and an LS would be sent to RAN to make a confirmation about the parameters. However, TBS for evaluation is still noted with ‘companies reported’ and the methodology is not determined yet, this contribution will try to provide some analysis from following aspects:

- Provide preliminary link level simulation results (BLER vs SNR curves, in Annex A) and analysis

- Provide Methodology to generate error trace

- Confirm the TBS values

2. Reason for Change

#### Preliminary simulation results provision and analysis

In this section, the link level simulation results and BLER vs SNR curves of the first step of above methodology are presented. Detailed step-by-step calculation is listed here, but the results are provided in the Annex.

- Link budget results

Table 1 UL CNR results under different parameter assumption

|  | SCS/BW | UE power | PLSL | UL CNR |
| --- | --- | --- | --- | --- |
| Case 1 | 3.75kHz/3.75kHz | 23dBm | 0 | 5.28 dB |
| Case 2 | 15kHz/15kHz | 23dBm | 0 | -0.74 dB |
| Case 1-1 | 3.75kHz/3.75kHz | 23dBm | 2.2 | 3.08 dB |
| Case 2-1 | 15kHz/15kHz | 23dBm | 2.2 | -2.94 dB |
| Case 3 | 3.75kHz/3.75kHz | 26dBm | 0 | 8.28dB |
| Case 4 | 15kHz/15kHz | 26dBm | 0 | 2.26dB |
| Case 3-1 | 3.75kHz/3.75kHz | 26dBm | 2.2 | 6.08dB |
| Case 4-1 | 15kHz/15kHz | 26dBm | 2.2 | 0.06dB |
| Case 5 | 3.75kHz/3.75kHz | 31dBm | 0 | 13.28dB |
| Case 6 | 15kHz/15kHz | 31dBm | 0 | 7.26dB |
| Case 5-1 | 3.75kHz/3.75kHz | 31dBm | 2.2 | 11.08dB |
| Case 6-1 | 15kHz/15kHz | 31dBm | 2.2 | 5.06dB |

Table 2 DL CNR results under different parameter assumption

|  | SCS/BW | UE Rx number | PLSL | DL CNR |
| --- | --- | --- | --- | --- |
| Case A | 15kHz/180kHz | 2Rx | 0 | 2.39dB |
| Case A-1 | 15kHz/180kHz | 2Rx | 2.2 | 0.19dB |
| Case B | 15kHz/180kHz | 1Rx | 0 | -0.61dB |
| Case B-1 | 15kHz/180kHz | 1Rx | 2.2 | -2.81dB |

- Link level Simulation parameters

Link level simulation parameters are summarized as following Table 3.

Table 3 Link level simulation assumptions

|  | Parameter | Value |
| --- | --- | --- |
| Channel | Carrier frequency | 2GHz |
|  | Channel model | NTN-TDL-C5 [TS36.102] |
|  | Frequency error | 0 |
|  | Timing error | 0 |
|  | Doppler spread | 5 Hz |
| Transmitter | SCS | 3.75kHz and 15kHz for NPUSCH 15kHz for NPDSCH |
|  | Number of tones | 1 for NPUSCH 12 for NPDSCH |
|  | Waveform | DFT-s-OFDM |
|  | IRU/ISF | Ref. to below Tables |
|  | Modulation order | QPSK |
|  | ITBS (MCS) | Ref. to below Tables |
|  | Repetition number | Ref. to below Tables |
| Receiver | Receiver Algorithm | MMSE |
|  | Channel estimation | Real channel estimation (w/o DMRS bundling) |
| KPI | BLER target | 1%, 2%, 6%, 10% |

- Link level simulation results

Based on the above simulation parameters and SPS scheduling scheme, preliminary simulation results are provided in the following Tables, which are in the form of corresponding required SNR of target BLER with 1%, 2%, 6% and 10%, respectively. The corresponding BLER-SNR curves are presented in the Appendix with a typical CNR in each figure.

- 80ms bundling time

Table 4 NPUSCH performance of TBS 144bit w/ 80ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 64ms | 3.75kHz | 1 | 5 | 1 | 5.95 dB | 6.43 dB | 7.43 dB | 8.19 dB |
| 2 |  |  | 0 | 10 | 2 | 5.96 dB | 6.42 dB | 7.49 dB | 8.00 dB |
| 3 |  | 15kHz | 3 | 1 | 2 | 1.36 dB | 1.89 dB | 2.81 dB | 3.28 dB |
| 4 |  |  | 1 | 5 | 4 | 1.09 dB | 1.49 dB | 2.25 dB | 2.49 dB |
| 5 |  |  | 0 | 10 | 8 | 1.26 dB | 1.73 dB | 2.53 dB | 3.13 dB |

Table 5 NPDSCH performance of TBS 144bit w/ 80ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 8ms | 15kHz | 3 | 1 | 2 | -3.47 dB | -2.72 dB | -1.27 dB | -0.32 dB |
| 7 |  |  | 1 | 5 | 4 | -3.50 dB | -2.70 dB | -1.25 dB | -0.22 dB |
| 8 |  |  | 0 | 10 | 8 | -3.48 dB | -2.71 dB | -1.23 dB | -0.43 dB |

Table 6 NPUSCH performance of TBS 256bit w/ 80ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 64ms | 3.75kHz | 1 | 8 | 1 | 9.40 dB | 10.04 dB | 11.18 dB | 11.99 dB |
| 10 |  | 15kHz | 6 | 1 | 1 | 3.41 dB | 3.91 dB | 4.87 dB | 5.34 dB |
| 11 |  |  | 3 | 4 | 2 | 2.52 dB | 3.10 dB | 3.98 dB | 4.48 dB |
| 12 |  |  | 1 | 8 | 4 | 2.64 dB | 3.09 dB | 4.11 dB | 4.46 dB |
| 13 | 48ms |  | 2 | 6 | 2 | 3.74 dB | 4.25 dB | 5.15 dB | 5.60 dB |
| 14 | 40ms |  | 4 | 3 | 1 | 4.55 dB | 5.12 dB | 6.31 dB | 6.94 dB |

Table 7 NPDSCH performance of TBS 256bit w/ 80ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 8ms | 15kHz | 6 | 1 | 1 | -1.72 dB | -0.92 dB | 0.43 dB | 1.38 dB |
| 16 |  |  | 3 | 4 | 2 | -1.70 dB | -0.96 dB | 0.48 dB | 1.37 dB |
| 17 |  |  | 1 | 8 | 4 | -1.62 dB | -0.86 dB | 0.55 dB | 1.55 dB |
| 18 | 10ms |  | 4 | 3 | 2 | -2.57 dB | -1.78 dB | -0.38 dB | 0.62 dB |
| 19 | 12ms |  | 2 | 6 | 4 | -3.16 dB | -2.43 dB | -1.10 dB | 0.04 dB |
| 20 | 16ms |  | 6 | 1 | 2 | -4.17 dB | -3.40 dB | -2.15 dB | -1.20 dB |
| 21 |  |  | 3 | 4 | 4 | -4.16 dB | -3.45 dB | -2.13 dB | -1.16 dB |
| 22 | 20ms |  | 4 | 3 | 4 | -4.94 dB | -4.25 dB | -2.88 dB | -2.07 dB |
| 23 | 24ms |  | 2 | 6 | 8 | -5.60 dB | -4.87 dB | -3.58 dB | -2.77 dB |
| 24 | 32ms |  | 6 | 1 | 4 | -6.57 dB | -5.96 dB | -5.02 dB | -4.26 dB |
| 25 |  |  | 3 | 4 | 8 | -6.54 dB | -5.85 dB | -4.91 dB | -4.26 dB |
| 26 |  |  | 1 | 8 | 16 | -6.49 dB | -5.75 dB | -4.79 dB | -4.29 dB |

Table 8 NPUSCH performance of TBS 328bit w/ 80ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27 | 64ms | 3.75kHz | 1 | 10 | 1 | 12.57 dB | 13.30 dB | 14.78 dB | 15.67 dB |
| 28 |  | 15kHz | 3 | 5 | 2 | 3.30 dB | 3.86 dB | 4.84 dB | 5.30 dB |
| 29 |  |  | 1 | 10 | 4 | 3.57 dB | 4.08 dB | 4.97 dB | 5.37 dB |
| 30 | 48ms |  | 2 | 7 | 2 | 4.46 dB | 5.04 dB | 6.07 dB | 6.51 dB |
| 31 |  |  | 5 | 3 | 1 | 4.52 dB | 5.04 dB | 6.03 dB | 6.44 dB |
| 32 | 40ms |  | 4 | 4 | 1 | 5.34 dB | 5.94 dB | 7.16 dB | 7.77 dB |

Table 9 NPDSCH performance of TBS 328bit w/ 80ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | 8ms | 15kHz | 3 | 5 | 2 | -0.89 dB | -0.10 dB | 1.26 dB | 2.24 dB |
| 34 |  |  | 1 | 10 | 4 | -0.86 dB | -0.12 dB | 1.33 dB | 2.34 dB |
| 35 | 10ms |  | 4 | 4 | 2 | -1.73 dB | -0.97 dB | 0.37 dB | 1.34 dB |
| 36 | 12ms |  | 5 | 3 | 2 | -2.39 dB | -1.66 dB | -0.24 dB | 0.60 dB |
| 37 |  |  | 2 | 7 | 4 | -2.35 dB | -1.55 dB | -0.13 dB | 0.77 dB |
| 38 | 16ms |  | 3 | 5 | 4 | -3.48 dB | -2.68 dB | -1.39 dB | -0.57 dB |
| 39 |  |  | 1 | 10 | 8 | -3.42 dB | -2.54 dB | -1.28 dB | -0.55 dB |
| 40 | 20ms |  | 4 | 4 | 4 | -4.24 dB | -3.53 dB | -2.14 dB | -1.28 dB |
| 41 | 24ms |  | 5 | 3 | 4 | -4.86 dB | -4.16 dB | -2.85 dB | -2.27 dB |
| 42 |  |  | 2 | 7 | 8 | -4.83 dB | -4.03 dB | -2.99 dB | -2.10 dB |
| 43 | 32ms |  | 3 | 5 | 8 | -5.84 dB | -5.17 dB | -4.24 dB | -3.72 dB |
| 44 |  |  | 1 | 10 | 16 | -5.85 dB | -5.09 dB | -4.26 dB | -3.47 dB |

Table 10 NPUSCH performance of TBS 424bit w/ 80ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45 | 40ms | 15kHz | 4 | 4 | 1 | 5.34 dB | 5.94 dB | 7.16 dB | 7.77 dB |

Table 11 NPDSCH performance of TBS 424bit w/ 80ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | 20ms | 15kHz | 4 | 4 | 4 | -4.24 dB | -3.53 dB | -2.14 dB | -1.28 dB |

- 160ms bundling time

Table 12 NPUSCH performance of TBS 208bit w/ 160ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47 | 128ms | 3.75kHz | 3 | 3 | 1 | 3.85 dB | 4.25 dB | 4.95 dB | 5.36 dB |
| 48 |  | 15kHz | 3 | 3 | 4 | -0.64 dB | -0.30 dB | 0.31 dB | 0.62 dB |
| 49 | 96ms | 3.75kHz | 2 | 4 | 1 | 5.15 dB | 5.54 dB | 6.46 dB | 6.93 dB |
| 50 |  | 15kHz | 5 | 1 | 2 | 0.88 dB | 1.34 dB | 2.11 dB | 2.62 dB |
| 51 |  |  | 2 | 4 | 4 | 0.39 dB | 0.79 dB | 1.55 dB | 2.04 dB |

Table 13 NPDSCH performance of TBS 208bit w/ 160ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52 | 16ms | 15kHz | 3 | 3 | 4 | -4.81 dB | -4.03 dB | -2.64 dB | -1.88 dB |
| 53 | 24ms |  | 5 | 1 | 4 | -6.13 dB | -5.52 dB | -4.24 dB | -3.36 dB |
| 54 |  |  | 2 | 4 | 8 | -6.16 dB | -5.43 dB | -4.13 dB | -3.27 dB |
| 55 | 32ms |  | 3 | 3 | 8 | -7.13 dB | -6.50 dB | -5.50 dB | -4.81 dB |
| 56 | 48ms |  | 5 | 1 | 8 | -8.69 dB | -8.17 dB | -7.21 dB | -6.78 dB |
| 57 |  |  | 2 | 4 | 16 | -8.69 dB | -8.16 dB | -7.22 dB | -6.69 dB |

Table 14 NPUSCH performance of TBS 424bit w/ 160ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58 | 80ms | 15kHz | 4 | 4 | 2 | 2.34 dB | 2.78 dB | 3.67 dB | 4.16 dB |

Table 15 NPDSCH performance of TBS 424bit w/ 160ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59 | 40ms | 15kHz | 4 | 4 | 8 | -6.77 dB | -6.14 dB | -5.08 dB | -4.57 dB |

Table 16 NPUSCH performance of TBS 600bit w/ 160ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 96ms | 15kHz | 5 | 6 | 2 | 3.54 dB | 4.05 dB | 4.82 dB | 5.18 dB |

Table 17 NPDSCH performance of TBS 600bit w/ 160ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61 | 48ms | 15kHz | 5 | 6 | 8 | -5.72 dB | -5.21 dB | -4.14 dB | -3.68 dB |

Table 18 NPUSCH performance of TBS 808bit w/ 160ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62 | 128ms | 15kHz | 6 | 6 | 2 | 3.36 dB | 3.72 dB | 4.45 dB | 5.09 dB |
| 63 | 96ms |  | 5 | 8 | 2 | 4.49 dB | 5.01 dB | 5.92 dB | 6.33 dB |

Table 19 NPDSCH performance of TBS 808bit w/ 160ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 64 | 16ms | 15kHz | 6 | 6 | 2 | -0.38 dB | 0.39 dB | 1.68 dB | 2.49 dB |
| 65 | 24ms |  | 5 | 8 | 4 | -1.86 dB | -1.11 dB | -0.01 dB | 0.86 dB |
| 66 | 32ms |  | 6 | 6 | 4 | -3.07 dB | -2.30 dB | -1.38 dB | -0.94 dB |
| 67 | 48ms |  | 5 | 8 | 8 | -4.68 dB | -4.05 dB | -3.12 dB | -2.62 dB |

- 320ms bundling time

Table 20 NPUSCH performance of TBS 328bit w/ 320ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 68 | 256ms | 3.75kHz | 3 | 5 | 2 | 2.55 dB | 2.85 dB | 3.52 dB | 3.87 dB |
| 69 |  |  | 1 | 10 | 4 | 2.77 dB | 2.99 dB | 3.76 dB | 3.98 dB |
| 70 |  | 15kHz | 3 | 5 | 8 | -1.74 dB | -1.52 dB | -0.91 dB | -0.64 dB |
| 71 |  |  | 1 | 10 | 16 | -1.51 dB | -1.18 dB | -0.72 dB | -0.57 dB |
| 72 | 192ms | 3.75kHz | 5 | 3 | 1 | 3.51 dB | 3.93 dB | 4.64 dB | 4.91 dB |
| 73 |  |  | 4 | 4 | 1 | 4.40 dB | 4.79 dB | 5.47 dB | 5.94 dB |
| 74 |  |  | 2 | 7 | 2 | 3.62 dB | 3.98 dB | 4.66 dB | 4.99 dB |
| 75 |  | 15kHz | 5 | 3 | 4 | -0.82 dB | -0.52 dB | 0.03 dB | 0.32 dB |
| 76 |  |  | 2 | 7 | 8 | -0.82 dB | -0.53 dB | -0.03 dB | 0.24 dB |
| 77 | 160ms |  | 4 | 4 | 4 | -0.30 dB | -0.04 dB | 0.78 dB | 1.20 dB |

Table 21 NPDSCH performance of TBS 328bit w/ 320ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 78 | 32ms | 15kHz | 3 | 5 | 8 | -5.84 dB | -5.17 dB | -4.24 dB | -3.72 dB |
| 79 |  |  | 1 | 10 | 16 | -5.85 dB | -5.09 dB | -4.26 dB | -3.47 dB |
| 80 | 40ms |  | 4 | 4 | 8 | -6.77 dB | -6.14 dB | -5.08 dB | -4.57 dB |
| 81 | 48ms |  | 5 | 3 | 8 | -7.50 dB | -6.97 dB | -6.13 dB | -5.40 dB |
| 82 |  |  | 2 | 7 | 16 | -7.48 dB | -7.02 dB | -5.99 dB | -5.52 dB |
| 83 | 64ms |  | 3 | 5 | 16 | -8.65 dB | -8.02 dB | -7.09 dB | -6.74 dB |
| 84 | 80ms |  | 4 | 4 | 16 | -9.45 dB | -9.08 dB | -8.20 dB | -7.91 dB |
| 85 | 96ms |  | 5 | 3 | 16 | -10.17 dB | -9.56 dB | -8.81 dB | -8.37 dB |
| 86 |  |  | 2 | 7 | 32 | -10.12 dB | -9.65 dB | -8.89 dB | -8.51 dB |
| 87 | 128ms |  | 1 | 10 | 64 | -11.06 dB | -10.66 dB | -9.98 dB | -9.45 dB |
| 88 |  |  | 3 | 5 | 32 | -11.08 dB | -10.62 dB | -10.06 dB | -9.48 dB |

Table 22 NPUSCH performance of TBS776bit w/ 320ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 89 | 160ms | 3.75kHz | 4 | 9 | 1 | 10.18 dB | 10.63 dB | 11.63 dB | 12.04 dB |
| 90 |  | 15kHz | 4 | 9 | 4 | 2.49 dB | 2.82 dB | 3.44 dB | 4.06 dB |

Table 23 NPDSCH performance of TBS 776bit w/ 320ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 91 | 80ms | 15kHz | 4 | 9 | 16 | -6.92 dB | -6.58 dB | -5.81 dB | -5.43 dB |

Table 24 NPUSCH performance of TBS1096bit w/ 320ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 92 | 256ms | 3.75kHz | 6 | 8 | 1 | 8.49 dB | 8.85 dB | 9.66 dB | 9.94 dB |
| 93 |  | 15kHz | 6 | 8 | 4 | 1.79 dB | 2.07 dB | 2.51 dB | 2.79 dB |

Table 25 NPDSCH performance of TBS 1096bit w/ 320ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 94 | 32ms | 15kHz | 6 | 8 | 4 | -1.89 dB | -1.12 dB | -0.24 dB | 0.42 dB |

Table 26 NPUSCH performance of TBS1544bit w/ 320ms bundling time

| Case | TNPUSCH | SCS | IRU | ITBS (MCS) | nRepNPUSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 95 | 160ms | 15kHz | 7 | 9 | 2 | 4.82 dB | 5.13 dB | 5.71 dB | 6.23 dB |

Table 27 NPDSCH performance of TBS 1544bit w/ 320ms bundling time

| Case | TNPDSCH | SCS | ISF | ITBS (MCS) | nRepNPDSCH | Corresponding required SNR @10% BLER | Corresponding required SNR @6% BLER | Corresponding required SNR @2% BLER | Corresponding required SNR @1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 96 | 80ms | 15kHz | 7 | 9 | 8 | -4.68 dB | -4.22 dB | -3.47 dB | -3.08 dB |

Observation#1: Higher UE transmit power results in a higher UL achievable CNR (based on Table 1), which in turn allows for a larger TBS for a certain bundling time (based on Table 4-27), and further results in higher PHY data rate and higher codec bit rate.

Observation#2: For a given TBS value, the transmission time (refer to the TNPDSCH, TNPUSCH in above tables) varies depending on the specific TBS configuration parameters (SCS, ISF, ITBS, nRepnpusch). Configurations with shorter transmission times are observed to yield lower mouth-to-ear delays and higher uplink capacity/larger number of multiplexed UEs.

Observation#3: For a given TBS, the smaller the bundling time is, the higher SNR is required to achieve the target BLER. For a give bundling time, the larger the TBS is, the higher SNR is required to achieve the target BLER. However, higher SNR generally means higher requirements for the system (e.g. power class of UE, antenna gain, GEO satellite capabilities)

Proposal 1: it is proposed that TNPUSCH and TNPDSCH should be considered as a key factor when selecting TBS configurations for error trace generation, in order to better reflect realistic delay and capacity trade-offs.

Proposal 2: It is proposed that the selection of TBS values for simulation and error-trace generation should carefully consider the trade-offs between bundling time, system requirements (e.g., UE power class, antenna gain, GEO satellite capabilities), and system capabilities (e.g., downlink and uplink capacity, number of multiplexed UEs), i.e. smaller TBS is preferable than larger TBS.

#### Error-trace generation methodology

In S4aA250072, several approaches/methodologies for error trace generation are listed. Based on the preliminary simulation results, the observations, proposal#1 and Approach 3 as listed in S4aA250072, a detail methodology is proposed as below:

Step 1: Generate the BLER vs SNR curves per bundling time per TBS with different scheduling scheme configurations (SCS, ISF, ITBS, nRepnpusch). The preliminary simulation results are captured in Annex A.

Step 2: Select some SNR values for some TBS values based on the generated BLER-SNR curves:

-	Calculate link budget result achievable UL/DL CNR, and view it as the selected SNR values

-	Compare selected SNR values with the required SNR corresponding to x% BLER in the BLER vs SNR curves, where x=1, 2, 6, 10. If the required SNR is smaller than the selected SNR, the configuration for the TBS is viable.

-	Find the viable TBS configuration that leads to minimum transmission time (i.e. TNPUSCH and TNPDSCH with different TBS configurations, i.e. SCS, ISF, ITBS, nRepnpusch) for each TBS and regard them as the selected TBS values.

Step 3: Generate traces: Generate loss traces for the selected TBS values and SNR values.

#### TBS determination

There is EN left to determine TBS. To progress the discussion on simulation parameters, it is necessary to finalize the TBS parameters. Based on the above preliminary simulation results, the observations and proposal#2, the EN is proposed to be deleted and the following TBS are proposed:

| Bundling time | 80ms | 160ms |
| --- | --- | --- |
| New TBS | 120 | 176 |
| PHY bitrate (kbps) | 1.5 | 1.1 |
| Codec bitrate (kbps) | 0.8 | 0.75 |

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.940 [4].

* * * First Change * * *

#### 5.2.2.2	Uplink simulation parameters

The following parameters are for the uplink of the service link.

Channel model: NTN-TDL-C [38811]

Modulation: QPSK

Subcarrier Spacing (SCS): 3.75kHz, 15kHz

Number of tones: 1 for 3.75kHz SCS and 15kHz SCS

Number of repetitions: companies will report the number of repetitions for each simulation

Voice bundling period: 80ms, 160ms, 320ms

NOTE: the 40ms bundling is not considered because for SCS 3.75kHz the minimum time-domain allocation is 32ms and it leaves insufficient time for downlink data (NPDSCH) and control (NPDCCH) transmissions in the same 40ms time interval.

Doppler spread: 1Hz, 5 Hz

Target BLER: 1%, 2%, 6%, 10%

Maximum Achievable SNR values: (3GPP SET-1 UL SNR) – 10*log10(B/3.75) + (P - 23dBm) + G + [X] dB, where

- 3GPP SET-1 UL SNR (=2.6dB) is the UL SNR for a single tone at 3.75kHz, 23dBm UE power, 0dBi UE antenna gain, as considered in TR36.763.

- B is the bandwidth, taking values 3.75kHz, 15kHz

- P is the maximum UE transmission power, taking value 23 dBm, 26 dBm, 31 dBm,

- G is the difference between the UE antenna gain and that assumed in TR36.763, and it is from 0 up to -5.5dBi,

- X is TBD (to be reported by companies) to account for lower loss (e.g., lower scintillation loss), and/or better performance of commercial satellites.

TBS values and PHY bitrates: The TBS values are selected from table 16.5.1.2-2 for NB-IoT for NPUSCH in TS36.213 and the corresponding PHY bitrates and codec bitrate (assuming 7 bytes of packet header) are calculated for each bundling period.

NOTE 1: The final size of packet header depends on the conclusions reached by SA2 and RAN, including whether 1-byte MAC header is feasible.

NOTE 2: The packet header is only counted once, regardless of how many voice frames are bundled together.

NOTE 3: The precise relationship between the voice frame duration and the bundling time depends on the RTP payload design. In the case of multiple voice frames bundled together, the loss of a single Transport Block (TB) means the loss of multiple consecutive voice frames.

Table 1 TBS and PHY bitrate for 80ms bundling

| TBS (bits) | 120 | 144 | 256 | 328 | 424 |
| --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.5 | 1.8 | 3.2 | 4.1 | 5.3 |
| Codec bitrate (kbps) | 0.8 | 1.1 | 2.5 | 3.4 | 4.6 |

Table 2 TBS and PHY bitrate for 160ms bundling

| TBS (bits) | 176 | 208 | 424 | 600 | 808 |
| --- | --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.1 | 1.30 | 2.65 | 3.75 | 5.05 |
| Codec bitrate (kbps) | 0.75 | 0.95 | 2.30 | 3.40 | 4.70 |

Table 3 TBS and PHY bitrate for 320ms bundling

| TBS (bits) | 328 | 776 | 1096 | 1544 |
| --- | --- | --- | --- | --- |
| PHY bitrate (kbps) | 1.025 | 2.425 | 3.425 | 4.825 |
| Codec bitrate (kbps) | 0.850 | 2.250 | 3.250 | 4.650 |

Channel consistency: The same set of channel realizations are used across all combinations.

* * * Second Change * * *

### 5.2.3 Methodology to generate error trace

Step 1: Generate the BLER vs SNR curves per bundling time per TBS with different configurations (SCS, ISF, ITBS, nRepnpusch). The preliminary simulation results are captured in Annex A.

Step 2: Select some SNR values for some TBS values based on the generated BLER-SNR curves:

-	Calculate link budget result achievable UL/DL CNR, and view it as the selected SNR values

-	Compare selected SNR values with the required SNR corresponding to x% BLER in the BLER vs SNR curves, where x=1, 2, 6, 10. If the required SNR is smaller than the selected SNR, the configuration for the TBS is viable.

-	Find the configuration that leads to minimum transmission time (i.e. TNPUSCH and TNPDSCH with different TBS configurations, i.e. SCS, ISF, ITBS, nRepnpusch) for each TBS and regard them as the selected TBS values.

Step 3: Generate traces: Generate loss traces for the selected TBS values and SNR values.

### 5.2.4 Methodology to derive codec bitrate

Editor’s Note: The methodology is FFS.

* * * Third Change * * *

## Annex A: BLER vs SNR curves for error trace generation

- 80ms bundling time

| - 160ms bundling time |  |
| --- | --- |
| - 320ms bundling time |  |

* * * End of Changes * * *