3GPP TSG-SA WG4 Audio SWG Telco on FS_ULBC	S4aA250115

23 October 2025, Online

Source:	Dolby Laboratories Inc., Nokia, Ericsson LM

Title:	[FS_ULBC] Updated proposal on methodology for error trace generation

Spec:	3GPP TR 26.940 v0.2.0

Agenda item:	1.9

Document for:	Discussion/Agreement

### Introduction

Various inputs to the methodology for generating error traces have been discussed in previous Audio SWG meetings on FS_ULBC. While the recent FS_ULBC ad hoc meeting in Erlangen made progress, time constraints prevented resolution of all differing viewpoints. One topic that has recently gained prominence is ULBC system capacity—specifically, the number of simultaneous ULBC connections supported on a 180 kHz NB-IoT carrier.

This contribution builds upon the latest proposal from the source regarding error trace generation methodology [1]. It broadens the scope to address system capacity as a central consideration and proposes using it as the starting point for the error trace generation procedure.

Below is an updated proposal on methodology for error trace generation to be included in the ULBC TS as clause 5.2.3.

### 5.2.3 Methodology to generate error trace

Loop: For each of the potential bundling times of 40, 80, 160 and 320 ms do the following steps:

Step 0: Agree on a set of scheduling scheme configurations (number of tones, SCS, ISF, IRU, Nrep) that maximize capacity in UL and DL.

Note: For each bundling time, one or more scheduling scheme configurations may achieve maximum capacity. Configurations with identical products of NRU∙ NRep result in equivalent capacity. With reference to Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH of TS 36.213, each TBS in the column identified by a specific IRU value is equivalent in terms of resource utilization e.g., NPUSCH Format 1 single-tone using (ITBS = 7 , IRU = 0) along with 2 repetitions is equivalent to using  (ITBS = 3 , IRU = 1) along 1 repetition in terms of resource utilization.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Step 1: Agree on a set of candidate TBS (from Table 16.5.1.2-2 in TS 36.213) associated with the agreed set of scheduling scheme configurations to be evaluated.

Note: The set of candidate TBS from Table 16.5.1.2-2 in TS 36.213 corresponds to a set of potential net bit rates. Accordingly, they may be confined within reasonable upper and lower limits. The set may preferably be contiguous, i.e. containing all TBSs of a column of the TBS table within the limits. See example with green cells in above table. When confining the set of candidate TBS, also to be considered are possible limitations in terms of the available modulation schemes for NTN NB-IoT, where only those entries can be used that rely on BPSK or QPSK modulation. Such limitations are indicated as greyed out in the above TBS/MCS table. Details on such constraints are given in [2].

Step 2: Generate the BLER vs SNR curves per candidate TBS for the different agreed scheduling scheme configurations.

Step 3: Calculate link budget results with different UE parameters (e.g. tx power classes, number of rx in DL), i.e., achievable UL/DL CNRs, and view them as the selected SNR values (e.g. SNR1, SNR2)

Figure 1: Example with 2 selected SNR values according to step 3

SNR1

SNR2

TBS1

TBS2

SNR

BLER

Step 4: For each candidate TBS, agree on the scheduling scheme configuration that leads to best possible transmission performance minimizing BLER at given SNR. Regard them as the selected TBS configurations (e.g. select solid curves, see Figure 2)

Step 5: For each selected TBS configuration lookup target BLER corresponding to selected SNR values (e.g. BLER11 and BLER12 for TBS1 and BLER11 and BLER12 for TBS2)

Step 6: Discard TBSs that lead to a target BLER > [20%].

Figure 2: Example with selected TBS configurations showing target BLER corresponding to selected SNR values according to step 5

SNR1

SNR2

TBS1

TBS2

BLER12

BLER22

BLER11

BLER21

SNR

BLER

Step 7: Generate traces: Generate packet loss patterns for the selected TBS configurations for identified target BLERs.

Note: BLER11 with TBS1 and BLER21 with TBS2 are equivalent in terms of capacity and link budget (i.e. SNR) and can be compared on a fair basis.

### References

[1] S4aA250271: [FS_ULBC] Consolidated methodology for error trace generation

[2] S4aA25xxxx: Multiuser SPS Capacity Analysis