3GPP Audio Telco Post TSG SA WG4 #133-e	       S4aA250118

October 23, 2025                                                                                Revision of 4aA250114

# Title:	RAN simulation results for ULBC

Source:	Qualcomm Innovation Center Inc.

Agenda item:	1.9

# Document for:	Discussion

# 1. Background

This paper presents the simulation results for the 80ms bundling period with TBS 144 and 1 UE RX following the methodology in S4aA250111 [1] for step 3.1.

# 2. Simulation parameters

The simulation parameters in the table below:

| Simulation parameter | Range |
| --- | --- |
| direction | U: uplink D: downlink |
| TBS | See TBS tables |
| Bundling period | 80, 160, 320 (ms) |
| Doppler spread | 1Hz, 5Hz |
| Number of RX | UL: 1 DL: 1, 2 |
| SCS | 3.75, 15 (kHz) |
| Number of tones | UL: 1 for 3.75kHz SCS; 1, 3, 6, 12 for 15kHz SCS DL: 15 |
| Length of NPUSCH or NPDSCH | UL: determined by ITBS, IRU, Nrep, and bundling period DL: determined by ITBS, ISF, Nrep, and bundling period |
| seed | 10 |
| The BLER of this loss trace | 1%, 2%, 6%, 10% |

The allowed lengths of NPUSCH are determined by the bundling period, TBS value, SCS, number of tones, the number of RUs per repetition and the number of repetitions as specified in TS36.213, while the allowed lengths of NPDSCH are determined by the bundling period, the number of subframes in each repetition and the number of repetitions as specified in TS36.213.

# 3. Optimal configurations for the scenario: 80 bundling period, TBS 144, 1 UE RX

# We first look at DL. For DL, the frequency-domain resource allocation is fixed at 180 kHz and the only variable is the duration of the resource allocation determined by the number of subframes (NSF) and the number of repetitions (Nrep). For simulation, 5 DL durations are selected without skipping any intermediate duration for TBS 144 allowed in TS36.213. DMRS bundling and long NRS for channel estimation [2] are assumed. The BLER’s are shown in Figure 1. Note that -3.3 dB is the maximum achievable SNR for all 5 durations.

Figure 1 BLER vs SNR for NDPSCH with TBS 144 bits and 1 UE RX

In step 3.1 of the proposed simulation methodology in [1], the goal is to find the best configuration to maximize the system capacity at the given per-UE performance which for the simulation study is denoted by the BLER. The bitrate is another important aspect, but since the TBS is given, BLER is the only metric for the per-UE performance.

1% BLER

# For 1% BLER, it is met only by the 16ms NPDSCH (NSF=4, Nrep=4). Given the bundling period 80ms, the network can schedule at most 80/16=5 UEs.

# For UL, the maximum allowed duration is 48ms (resulting from NRU=6, SCS 15kHz and one tone). This configuration has a coding rate 0.1458, already lower than the rate (1/3) of the Turbo code mother code. Therefore, increasing the number of tones while keeping the UE transmission power fixed does not improve the performance. This 48ms NPUSCH configuration is optimal and is chosen for the UL simulation.

2% BLER

# For 2% BLER, it is met by the 16ms NPDSCH and 12ms NPDSCH (NSF=3, Nrep=4). Because the objective is to optimize the system capacity, the 12ms NPDSCH is chosen. Given the bundling period 80ms, the network can schedule at most floor(80/12)=6 UEs.

# For UL, the maximum allowed duration is 64ms. It may result from a first configuration (NRU=8, SCS 15kHz and one tone) or a second configuration (NRU=2, SCS 3.75kHz and one tone). The first configuration is better than the second configuration because the coding rate of the first is less than 1/3 while the coding rate of the second is larger than 1/3. Also, because the coding rate of the first configuration is already less than 1/3, there is no need to consider other configurations that differ only in the number of tones. Thus, this 64ms NPUSCH configuration (NRU=8, SCS 15kHz and one tone) is chosen for the UL simulation.

# 6% BLER

# For 6% BLER, for similar reason given for the 2% BLER, the 8ms NPDSCH (NSF=4, Nrep=2) is chosen. Given the bundling period 80ms, the network can schedule at most 80/8=10 UEs.

# For UL, following the argument for the case of 2% BLER, the 64ms NPUSCH configuration (NRU=8, SCS 15kHz and one tone) is chosen for the simulation.

10% BLER

# For 10% BLER, for similar reason given for the 2% BLER, the 6ms NPDSCH (NSF=3, Nrep=2) is chosen. Given the bundling period 80ms, the network can schedule at most floor(80/6)=13 UEs. However, to meet the 10% BLER requirement, the UL must use the 64ms NPUSCH configuration (NRU=8, SCS 15kHz and one tone) because the 64ms NPUSCH configuration (NRU=2, SCS 3.75kHz and one tone) cannot meet the BLER requirement. The resulting system capacity is 12 UEs/cell.

# For UL, following the argument for the case of 2% BLER, the 64ms NPUSCH configuration (NRU=8, SCS 15kHz and one tone) is chosen for the simulation.

# The UL simulation results for the selected configurations are shown in Figure 2. Figure 2 (a) shows the BLER vs SNR, which marks the SNRs 2.6dB and -3.4dB for 3.75kHz SCS and 15kHz SCS respectively, both for 1 tone and 23dBm UE TX power. Figure 2 (b) gives the BLER but as function of the UE TX power.

| # (a) | # (b) |
| --- | --- |

Figure 2 For TBS 144 bits, (a) BLER vs SNR and (b) BLER vs UE TX power

# The optimal configurations are shown in the table below:

Table 1 Optimal configurations for TBS 144 and 1 UE RX for step 3.1 in [1]

|  | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| UL config | NRU=8, Nrep=1, SCS 15kHz, 1 tone (64ms) | NRU=8, Nrep=1, SCS 15kHz, 1 tone (64ms) | NRU=8, Nrep=1, SCS 15kHz, 1 tone (64ms) | NRU=6, Nrep=1, SCS 15kHz, 1 tone (48ms) |
| Required UE TX power (dBm) | 22.7 | 23.2 | 24.2 | 26.4 |
| Required SNR (dB) | -3.7 | -3.2 | -2.2 | 0.0 |
| DL config | NSF=3, Nrep=2 (6ms) | NSF=4, Nrep=2 (8ms) | NSF=3, Nrep=4 (12ms) | NSF=4, Nrep=4 (16ms) |
| DL required SNR (dB) | -3.4 | -4.0 | -4.1 | -4.6 |
| System capacity | 12 | 10 | 6 | 5 |

# From the table we see that:

- For the UE power class 23dBm, only 10% BLER is achievable (the green column in the table) with a system capacity 13 UEs per cell, and other BLERs are not achievable and thus the system capacities at those BLERs are zero.

- For the UE power class 26dBm, only 10%, 6% and 2% BLERs are achievable (the green columns and the blue column in the table).

- For the UE power class 31dBm, all BLERs (10%, 6%, 2%, 1%) are achievable (the green columns, the blue column and the yellow column in the table).

# 4. Proposal

# It is important to give a workable example on how to determine the configurations to be simulated that are based on the best tradeoff between per-UE performance and system capacity. Therefore, we propose the following.

# Proposal: include clauses 2 and 3 above to the PD.

# References

# [1] S4aA250117, “RAN simulation methodology for ULBC” Qualcomm, 3GPP SA4 Audio Ad Hoc meeting post #133-e, Oct 23, 2025.

[2] S4-251303, “Scenarios for RAN simulation calibration for ULBC over NTN”, Qualcomm, vivo, Xiaomi, Huawei Technologies Co. Ltd, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.