Source:	Apple Inc.

Title:	ULBC Performance Requirements

Document for:	Discussion and Agreement

Agenda item:	1.7

- Discussion

Given the envisioned application scenarios of ULBC codec in Clause 4, TR 26.940 targeting IMS voice service over GEO and NGSO, the codec must deliver a level of quality consistent with what end users have come to expect based on the deployed VoLTE IMS voice services. AMR-WB 12.65kbps and EVS-SWB 13.2kbps are among the commonly deployed VoLTE codec operating points. The ongoing TBS discussions in SA4 for the ULBC codec is centered around bitrates potentially in the range of 1-3 kbps. Based on the above, it is recommended to consider the following “minimum” performance requirements:

- at the lowest operating range, e.g., around 1kbps, the ULBC codec shall provide speech quality that is no worse than that of AMR-WB @12.65kbps in clean speech, noisy speech, and in packet loss conditions,

- at the operating range, e.g., around 3kbps, the ULBC codec shall provide speech quality that is no worse than that of EVS-SWB @13.2kbps in clean speech, noisy speech, and in packet loss conditions

  - Reference codecs

The source recommends including, at a minimum, the following deployed VoLTE codecs and their corresponding operating points in the ToR comparison testing.

- AMR:  12.2kbps

- AMR-WB:  8.85kbps, 12.65kbps, 23.85kbps

- EVS AMR-WB-IO:  8.85kbps, 12.65 kbps, 23.85kbps

- EVS-SWB:  9.6kbps, 13.2kbps, 13.2kbps CA, 24.4kbps

- Proposal

It is proposed to update Clause 8 in TR 26.940 to reflect the changes below.

* * * First Change * * * *

# 8	Performance requirements

## 8.1	General

Given the envisioned application scenarios of ULBC codec in Clause 4, TR 26.940 targeting IMS voice service over GEO and NGSO, the codec must deliver a level of quality consistent with what end users have come to expect based on the deployed VoLTE IMS voice services. AMR-WB 12.65kbps and EVS-SWB 13.2kbps are among the commonly deployed VoLTE codec operating points.  The ongoing TBS discussions in SA4 for the ULBC codec is centered around bitrates potentially in the range of 1-3 kbps. Based on the above, it is recommended to consider the following “minimum” performance requirements:

- at the lowest operating range, e.g., around 1kbps, the ULBC codec shall provide speech quality that is no worse than that of AMR-WB @12.65kbps in clean speech, noisy speech, and in packet loss conditions,

- at the operating range, e.g., around 3kbps, the ULBC codec shall provide speech quality that is no worse than that of EVS-SWB @13.2kbps in clean speech, noisy speech, and in packet loss conditions

### 8.1.1	A List of Reference Codecs

The source recommends including, at a minimum, the following deployed VoLTE codecs and their corresponding operating points in the ToR comparison testing.

- AMR:  12.2kbps

- AMR-WB:  8.85kbps, 12.65kbps, 23.85kbps

- EVS AMR-WB-IO:  8.85kbps, 12.65 kbps, 23.85kbps

- EVS-SWB:  9.6kbps, 13.2kbps, 13.2kbps CA, 24.4kbps

* * * End of Changes * * * *