SA4-e (AH) MBS SWG post 111-e	S4aI201098

# 3rd December 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Procedures and APIs for Downlink and Uplink Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-12-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are a number of bugs in the terminology and description of the procedures and APIs for downlink streaming in certain sub-clauses of clause 4. More significant bugs and shortcomings exist in the sub-clauses of clauses 7 and 11 in terms of missing consideration for and related descriptions on, in the M1 and M5 APIs, uplink streaming services support by the 5GMS system. In addition, a few bugs are present in the described procedures for uplink streaming in sub-clause 5.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed corrections to the cited bugs in these sections of TS 26.512. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Remaining inaccuracies in TS 26.512 regarding normative descriptions of procedures and APIs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 4.3, 5.2, 7, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE: Correction to sub-clauses 4.2 and 4.3

## 4.2	APIs relevant to Downlink Streaming

Table 4.2-1 summarises the APIs used to provision and use the various downlink streaming features specified in TS 26.501 [2].

Table 4.2-1: Summary of APIs relevant to downlink streaming features

| 5GMSd feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Content Hosting | Content is ingested, hosted and distributed by the 5GMSd AS according to a Content Hosting Configuration associated with a Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Server Certificates Provisioning API | 7.3 |
|  |  |  | Content Preparation Templates Provisioning API | 7.4 |
|  |  |  | Content Protocols Discovery API | 7.5 |
|  |  |  | Content Hosting Provisioning API | 7.6 |
|  |  | M2d | HTTP-pull based content ingest protocol | 8.2 |
|  |  |  | DASH-IF push based content ingest protocol | 8.3 |
|  |  | M4d | DASH [4] or 3GP [37] | 10 |
|  |  | M5d | Service Access Information API | 11.2 |
| Metrics reporting | The 5GMSd Client uploads metrics reports to the 5GMSd AF according to a Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Configuration API | 7.8 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
| Consumption Reporting | The 5GMSd Client provides feedback reports on currently consumed content according to a provisioned Consumption Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Consumption Reporting Provisioning API | 7.7 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Consumption Reporting API | 11.3 |
| Dynamic Policy invocation | The 5GMSd Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1d | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API | 7.9 |
|  |  | M5d | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSd client requests bitrate recommendations and delivery boosts from the 5GMSd AF. | M5d | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |

## 4.3	Procedures of the M1d (5GMS Provisioning) interface

---- <snipped> ----

### 4.3.3	Content Hosting Configuration procedures

#### 4.3.3.1	General

These procedures are used by the 5GMSd Application Provider and the 5GMSd AF on M1d to configure the content hosting feature for downlink streaming.

---- <snipped> ----

### 4.3.5	Content Preparation Template procedures

#### 4.3.5.1	General

The 5GMSd AS is able to process content ingested at interface M2d before serving it on interface M4d, as described in clause 7.4.1. The content processing operations are specified in a Content Preparation Template resource, as specified in clause 7.4.2.

---- <snipped> ----

### 4.3.9	Metrics Reporting Configuration procedures

#### 4.3.9.1	General

##### These procedures are used by the 5GMSd Application Provider to configure QoE metrics reporting functionality associated with downlink streaming. This clause defines the basic procedures. More details are provided in clause 7.8.

##### A given instance of a Metrics Reporting Configuration is identified by the metricsReportingConfigurationId property of the MetricsReportingConfiguration resource, The properties of that resource, as described in clause 7.8.3.1, pertain to metrics collection and reporting by the Media Session Handler to the 5GMS AF.

---- <snipped> ----

END OF 1st CHANGE

2nd CHANGE: Corrections to sub-clauses 5.2

## 5.2	APIs relevant to Uplink Streaming

Table 5.2-1 summarises the APIs used to provision and use the various uplink streaming features specified in TS 26.501 [2].

Table 5.2-1: Summary of APIs relevant to uplink streaming features

| 5GMSd feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Metrics Reporting | The 5GMSu Client uploads metrics reports to the 5GMSd AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1u | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Provisioning API | 7.8 |
|  |  | M5u | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
| Dynamic Policy invocation | The 5GMSd Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1u | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API | 7.9 |
|  |  | M5u | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSd client requests bitrate recommendations and delivery boosts from the 5GMSd AF. | M5u | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.5 |

END OF 2nd CHANGE

3rd CHANGE: Corrections to sub-clauses of clause 7

# 7	Provisioning (M1) APIs

## 7.1	General

This clause defines the provisioning API used by a 5GMS Application Provider to configure downlink and uplink 5G Media Streaming services.

## 7.2	Provisioning Sessions APIs

### 7.2.1	Overview

The Provisioning Sessions APIs are used by the 5GMS Application Provider to instantiate and manipulate Provisioning Sessions associated with downlink streaming and uplink streaming sessions in the 5GMS System, as described in clause 4.3.2. Having created a Provisioning Session for a given instance of downlink or uplink streaming session, the 5GMS Application Provider can then go on to provision other 5GMS features in the context of that Provisioning Session, using the APIs specified in clause 7.3 et seq.

### 7.2.2	Resource structure

The Provisioning Sessions APIs for downlink streaming and uplink streaming sessions are accessible, respectively, through the following URL base paths:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/

{apiRoot}/3gpp-m1u/v1/provisioning-sessions/

Table 7.4.2-1 specifies the operations and the corresponding HTTP methods that are supported by each API. In each case, the sub-resource path specified in the second column of the table shall be appended to the appropriate (downlink or uplink streaming) URL base path.

Table 7.2.2-1: Operations supported by the Provisioning Sessions API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Provisioning Session |  | POST | Used to create a new Provisioning Session resource. If the operation succeeds, the URL of the created Provisioning Session resource shall be returned in the Location header of the response. |
| Retrieve Provisioning Session | {provisioningSessionId} | GET | Used to retrieve a Provisioning Session resource for inspection. |
| Destroy Provisioning Session |  | DELETE | Used to destroy an existing Provisioning Session resource. |

### 7.2.3	Data model

#### 7.2.3.1	Downlink ProvisioningSession resource

The data model for the ProvisioningSession resource in downlink streaming is specified in Table 7.2.3.1-1 below:

Table 7.2.3.1-1: Definition of ProvisioningSession resource for downlink streaming

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | C: R R: RO | A unique identifier for this Provisioning Session. |
| aspId | AspId | 0..1 | C: W R: RO | The identity of the Application Service Provider responsible for this Provisioning Session, as specified in clause 5.6.2.3 of TS 29.514 [34]. |
| serverCertificateIds | Array(String) | 0..1 | C: – R: RO | A (possibly empty) array of Server Certificate identifiers currently associated with this Provisioning Session. |
| contentPreparation‌TemplateIds | Array(String) | 0..1 | C: – R: RO | A (possibly empty) array of Content Preparation Template identifiers currently associated with this Provisioning Session. |
| contentProtocols‌DiscoveryIds | Array(String) | 1..1 | C: – R: RO | An array of Content Protocols Discovery identifiers currently associated with this Provisioning Session. |
| contentHosting‌ConfigurationId | String | 0..1 | C: – R: RO | The Content Hosting Configuration identifier currently associated with this Provisioning Session, if any. |
| consumptionReporting‌ConfigurationId | String | 0..1 | C: – R: RO | The Consumption Reporting Configuration identifier currently associated with this Provisioning Session, if any. |
| metricsReporting‌ConfigurationIds | Array(String) | 0..1 | C: – R: RO | A (possibly empty) array of Metrics Reporting Configuration identifiers currently associated with this Provisioning Session. |
| policyTemplateIds | Array(String) | 0..1 | C: – R: RO | A (possibly empty) array of Policy Template identifiers currently associated with this Provisioning Session. |

#### 7.2.3.2	Uplink ProvisioningSession resource

The data model for the ProvisioningSession resource in uplink streaming is specified in Table 7.2.3.2-1 below:

Table 7.2.3.2-1: Definition of ProvisioningSession resource for uplink streaming

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | C: R R: RO | A unique identifier for this Provisioning Session. |
| aspId | AspId | 0..1 | C: W R: RO | The identity of the Application Service Provider responsible for this Provisioning Session, as specified in clause 5.6.2.3 of TS 29.514 [34]. |
| metricsReporting‌ConfigurationIds | Array(String) | 0..1 | C: – R: RO | A (possibly empty) array of Metrics Reporting Configuration identifiers currently associated with this Provisioning Session. |
| policyTemplateIds | Array(String) | 0..1 | C: – R: RO | A (possibly empty) array of Policy Template identifiers currently associated with this Provisioning Session. |

## 7.3	Server Certificates Provisioning API

### 7.3.1	Overview

The Server Certificates Provisioning API, specific to downlink streaming, is used to provision X.509 [8] server certificates that can be referenced by a Content Hosting Configuration and subsequently presented by the 5GMSd AS when it distributes content to 5GMSd Clients at interface M4d using Transport Layer Security [12]. Server Certificate resources are provisioned within the scope of an enclosing Provisioning Session.

---- <snipped> ----

## 7.4	Content Preparation Templates Provisioning API

### 7.4.1	Overview

Content Preparation Templates, specific to downlink streaming, are used to specify manipulations applied by a 5GMSd AS to media resources ingested at interface M2d for distribution at interface M4d. The Content Preparation Templates API is used to provision a Content Preparation Template within the scope of a Provisioning Session that can subsequently be referenced from a Content Hosting Configuration.

---- <snipped> ----

## 7.5	Content Protocols Discovery API

### 7.5.1	Overview

The Content Protocols Discovery API, specific to downlink streaming, is used by a 5GMSd Application Provider to find out which content ingest protocols are supported by the 5GMSd AS(s) associated with a 5GMSd AF. One of the supported ingest protocols is subsequently indicated in a Content Hosting Configuration for downlink streaming.

---- <snipped> ----

## 7.6	Content Hosting Configuration API

### 7.6.1	Overview

This clause specifies the API, specific to downlink streaming, that a 5GMSd Application Provider uses at interface M1d to provision and manage 5GMSd AS Content Hosting Configurations by interacting with a 5GMSd AF. Each such configuration is represented by a ContentHostingConfiguration, the data model for which is specified in clause 7.6.3 below. The RESTful resources for managing Content Hosting Configurations are specified in clause 7.6.2 and the operations on these resources are further elaborated in clause 7.6.4.

---- <snipped> ----

## 7.7	Consumption Reporting Provisioning API

### 7.7.1	Overview

The Consumption Reporting Provisioning API, specific to downlink streaming, is a RESTful API that allows a 5GMSd Application Provider to configure the Consumption Reporting Procedure for a particular Provisioning Session at interface M1d. The different procedures are described in clause 4.2.5. The Consumption Reporting Configuration is represented by a ConsumptionReportingConfiguration, the data model for which is specified in clause 7.7.3 below. The RESTful resources for managing the Consumption Reporting Configuration is specified in clause 7.7.2.

---- <snipped> ----

## 7.8	Metrics Reporting Configuration APIs

### 7.8.1	Overview

The Metrics Reporting Configuration APIs for downlink streaming and uplink streaming sessions allow an 5GMS System operator or a 5GMSd Application Provider to configure the Metrics Collection and Reporting procedure for a particular Provisioning Session at interfaces M1d and M1u, respectively.

### 7.8.2	Resource structure

The Metrics Reporting Configuration APIs for downlink streaming and uplink streaming sessions are accessible, respectively, through the following URL base paths:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

{apiRoot}/3gpp-m1u/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.8.2-1 below specifies the operations and the corresponding HTTP methods that are supported by each API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the appropriate (downlink or uplink streaming) URL template and the sub-resource path specified in the second column of the table shall be appended to that URL base path.

Table 7.8.2-1: Metrics Reporting Configuration resource

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Metrics Reporting Configuration | metrics-reporting-configuration | POST | Create and optionally provide a configuration. If the operation succeeds, the URL of the created Metrics Reporting Configuration resource shall be returned in the Location header of the response. |
| Read Metrics Reporting Configuration | metrics-reporting-configuration/ {metricsReportingConfigurationId} | GET | Retrieve the values of an existing Metrics Reporting Configuration. |
| Update Metrics Reporting Configuration |  | PUT, PATCH | Provide initial upload of a new configuration, or either the modification of, or replacement to an existing configuration. |
| Delete Metrics Reporting Configuration |  | DELETE | Delete a configuration, disables reporting. |

### 7.8.3	Data model

#### 7.8.3.1	MetricsReportingConfiguration resource

The data model for the MetricsReportingConfiguration resource is specified in Table 7.8.3-1 below:

Table 7.8.3-1: Definition of MetricsReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| metricsReportingConfigurationId | String | 1..1 | An identifier for this Metrics Reporting Configuration that is unique within the scope of the enclosing Provisioning Session. |
| scheme | Array(URI String) | 0..1 | The scheme associated with this Metrics Reporting Configuration. A scheme may be associated with 3GPP or with a non-3GPP entity. For downlink streaming, if not specified, the 3GPP metrics scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 from TS 26.247 shall apply. |
| dataNetworkName | String | 0..1 | The Data Network Name (DNN) which shall be used when sending metrics reports. If not specified, the default DNN shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval between successive metrics reports. If not specified, a single final report shall be sent after the streaming session has ended. |
| samplePercentage | Percentage | 0..1 | The proportion of streaming sessions for which metrics shall be reported. If not specified, reports shall be sent for all sessions. |
| urlFilters | Array(String) | 0..1 | For downlink streaming, a list of content URL patterns for which metrics shall be reported. If not specified, reporting shall be done for all URLs. |
| metrics | Array(String) | 0..1 | A non-empty list of metrics which shall be collected and reported. In the case of downlink streaming and for the 3GPP scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 the listed metrics shall correspond to one or more of the metrics as specified in clauses 10.3 and 10.4, respectively, of TS 26.247 [7], and the quality reporting scheme and quality reporting protocol as defined in clauses 10.5 and 10.6, respectively, of [7] shall be used. If not specified, the complete (or default if applicable) set of metrics associated with the specified scheme shall be collected and reported. |

## 7.9	Policy Templates Provisioning APIs

### 7.9.1	Overview

The Policy Templates Provisioning APIs allow a 5GMS Application Provider to configure a set of Policy Templates within the scope of a Provisioning Session, associated with an instance of downlink streaming or uplink streaming session, that can subsequently be applied to media streaming sessions belonging to that Application Provider using the Dynamic Policies API specified in clause 11.5. A Policy Template is used to specify the traffic shaping and charging policies to be applied to these media streaming sessions.

A Policy Template, identified by its policyTemplateId, represents a set of PCF/NEF API parameters which defines the service quality and associated charging for the corresponding downlink or uplink media streaming sessions. The Policy Template is configured as part of the provisioning procedures with the 5GMS AF and is then used by the 5GMS AF to request specific QoS and charging policies for that session from the PCF or NEF.

The state of a Policy Template can be:

-	pending: The Policy Template is awaiting validation, potentially because not all required parameters have yet been provided. This is the default state after Policy Template creation.

-	invalid: One or more of the Policy Template's properties failed validation by the 5GMS AF.

-	ready: After successful validation by the 5GMS AF the Policy Template moves into this state.

-	suspended: The 5GMS AF may move a Policy Template into this state under certain conditions defined within the Service Level Agreement.

When the Policy Template is used for QoS Flows associated with downlink streaming or uplink streaming session, the qoSSpecification object (of type M1QoSSpecification) shall be present:

-	The qosReference value is obtained with the Service Level Agreement. See TS 23.502 for detailed usage.

-	The maxBtrUl and maxBtrDl properties define the maximal bit rate which can be used for QoS Flows. This value is defined by the 5G System.

-	The maxAuthBtrUl and MaxAuthBtrDl properties define the maximal authorized bit rate values which can be requested by a Media Session Handler. Higher bit rate values are not authorized for use by the 5GMS Application Provider.

-	The minPacketLossRateDl and minPacketLossRateUl properties define the minimal authorized packet loss rate, which can be requested by a Media Session Handler.

When the Policy Template is used for differential changing the chargingSpecification property shall be present.

The ApplicationSessionContext Object is a mandatory object, which contains at least the aspId property.

-	The aspId identifies the API invoker.

-	The dnn property contains the Data Network Name of the data network, in which the 5GMS AF is hosted.

-	When Network Slicing is used, the sliceInfo property contains information about the network slice, which is serving the UE.

### 7.9.2	Resource structure

The Policy Template Provisioning APIs for downlink streaming and uplink streaming sessions are accessible, respectively, through the following URL base paths:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

{apiRoot}/3gpp-m1u/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.9.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.9.2-1: Operations supported by the Policy Template Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template | policy-templates | POST | Used to create a new Policy Template resource. If the operation succeeds, the URL of the created Policy Template resource shall be returned in the Location header of the response. |
| Fetch a Policy Template | policy-templates/‌{policyTemplateId} | GET | Used to retrieve an existing Policy Template resource. |
| Update a Policy Template |  | PUT, PATCH | Used to modify the configuration of an existing Policy Template. |
| Delete a Policy Template |  | DELETE | Used to delete an existing Policy Template resource. |

### 7.9.3	Data model

#### 7.9.3.1	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in Table 7.9.3-1 below:

Table 7.9.3-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Usage | Visibility | Description |
| --- | --- | --- | --- | --- | --- |
| policyTemplateId | String | 1..1 | C: RO R: RO U: RO |  | Unique identifier of this Policy Template within the scope of the Provisioning Session for the associated instance of downlink or uplink streaming session. |
| state | Enumeration of Strings | 1..1 | C: RO R: RO U: RO |  | A Policy Template may be in the pending, ready, or suspended state. Only a Policy Template in the ready state may be instantiated as a Dynamic Policy Instance and applied to the corresponding downlink or uplink streaming sessions. |
| apiEndPoint | String | 1..1 | C: RW R: RO U: RW | MNO Admin | The API endpoint that should be invoked when activating a Dynamic Policy Instance based on this Policy Template. |
| apiType | Enumeration of Strings | 1..1 | C: RW R: RO U: RW | MNO Admin | N5: Npcf Policy Authorization Service. N33: AsSessionWithQoS or CHargableParty. |
| externalReference | String | 1..1 | C: RW R: RO U: RW |  | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the corresponding downlink or uplink streaming session. |
| qoSSpecification | M1QoSSpecification | 0..1 | C: RW R: RO U: RW |  | Specifies the network quality of service to be applied to the downlink or uplink streaming sessions at this Policy Template. |
| ApplicationSession‌Context | Object | 1..1 |  |  | Specifies information about the downlink or uplink application session context to which this Policy Template can be applied. |
| afAppId | AfAppId | 0..1 |  | Read-Only | As defined in clause 5.6.2.3 of TS 29.514 [34]. |
| sliceInfo | Snssai | 0..1 |  |  |  |
| dnn | Dnn | 0..1 |  |  |  |
| aspId | AspId | 0..1 |  |  |  |
| chargingSpecification | ChargingSpecification | 0..1 |  |  | Provides information about the charging policy to be used for this Policy Template. |

Editor's Note: The parameter externalReference is for further study. It may be a provisioning parameter of the Media Player and/or a Media Session Handler to assist mapping of external references to a policyTemplateId.

Editor's Note: The ChargingSpecification object may contain any charging related information, such as sponId or afChargeId.

END OF 3rd CHANGE

4th CHANGE: Corrections to sub-clauses of clause 11

# 11	Media Session Handling (M5) APIs

## 11.1	General

This clause defines the Media Session Handling APIs used by the Media Session Handler to access resources exposed by the 5GMS AF at interface M5.

NOTE:	While the entirety of M5 APIs applies to downlink streaming, only a subset is applicable to uplink streaming. Specifically, the Consumption Reporting API is not applicable to uplink streaming and is explicitly indicated in the sub-clause 11.3.

## 11.2	Service Access Information APIs

### 11.2.1	General

The Service Access Information APIs are used by the Media Session Handler to obtain configuration information associated with downlink streaming and uplink streaming sessions from the 5GMS AF that enables it to use the other Media Session Handling APIs specified in clause 11.3 et seq.

### 11.2.2	Resource structure

The Service Access Information APIs for downlink streaming and uplink streaming sessions are accessible, respectively, through the following URL base paths:

{apiRoot}/3gpp-m5d/v1/service-access-information/

{apiRoot}/3gpp-m5u/v1/service-access-information/

The operations and the corresponding HTTP methods in Table 11.2.2-1 are supported. In each case, the sub-resource path specified in the second column shall be appended to the appropriate (downlink or uplink streaming) URL base path.

Table 11.2.2-1: Operations supported by the Service Access Information API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Fetch Service Access Information | {provisioningSessionId} | GET | Used to acquire the Service Access Information resource for the specified Provisioning Session associated with a downlink or uplink streaming . The {provisioningSessionId} uniquely identifies the Service Access Information Resource and is allocated by the 5GMSd AF during creation of a Provisioning Session. |

### 11.2.3	Data model

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformtion resource is specified in Table 11.2.3.1-1 below. It should be noted that while the entirety of resource/sub-resources as shown in the table applies to downlink streaming, only a subset is applicable to uplink streaming. Those resources/sub-resources not applicable to uplink streaming are explicitly indicated in Table 11.2.3.1-1.

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | RO | Unique identification of the M1d Provisioning Session. |
| StreamingAccess (n/a to uplink streaming) | Object | 0..1 | RO |  |
| mediaPlayerEntry | URL String | 0..1 | RO | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| ClientConsumptionReporting‌Configuration (n/a to uplink streaming) | Object | 0..1 | RO |  |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| serverAddresses | Array(URL String) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| locationReporting | Boolean | 1..1 | RO | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |
| samplePercentage | Percentage | 1..1 | RO | The percentage of streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |
| DynamicPolicyInvocationConfiguration | Object | 0..1 | RO |  |
| serverAddresses | Array(URL String) | 1..1 | RO | A list of 5GMSd or 5GMSu AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| validPolicyTemplateIds | Array(String) | 1..1 | RO | A list of Policy Template identifiers which the 5GMSd or 5GMSu Client is authorized to use. |
| sdfMethods | Array(SdfMethod) | 1..1 | RO | A list of recommended service data flow description methods (descriptors), e.g. 5-Tuple, ToS, 2-Tuple, etc, which should be used by the Media Session Handler to describe the service data flows for the traffic to be policed. |
| externalReferences | Array(String) | 0..1 | RO | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. Example: "HD_Premium". |
| ClientMetricsReportingConfigurations | Array(Object) | 0..1 | RO |  |
| serverAddresses | Array(URL String) | 1..1 | RO | A list of 5GMSd or 5GMSu AF addresses, depending on whether the Service Access Information pertains to a downlink streaming or uplink streaming service, to which metrics reports shall be sent. (Opaque URL, following the 5GMS URL format.) |
| dataNetworkName | String | 0..1 | RO | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| samplePercentage | Percentage | 1..1 | RO | The percentage of streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. |
| urlFilters (n/a to uplink streaming) | Array(String) | 1..1 | RO | A list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. |
| metrics | Array(String) | 1..1 | RO | A list of metrics which shall be reported. |
| NetworkAssistanceConfiguration | Object | 0..1 | RO |  |
| serverAddress | URL String | 1..1 | RO | Address of the 5GMSd or 5GMSu AF that offers, respectively, the APIs for 5GMSd or 5GMSu AF-based Network Assistance, for access by the 5GMSd Media Session Handler. This address shall be an opaque URL, following the 5GMS URL format. |

### 11.2.4	Operations

This clause defines the behaviour that is expected from the 5GMSd or 5GMSu AF when a downlink or uplink Service Access Information resource, respectively, is acquired by the Media Session Handler. The main operation that is performed is to look up or generate the corresponding Service Access Information.

## 11.3	Consumption Reporting API (n/a to uplink streaming)

### 11.3.1	General

The Consumption Reporting API allows the Media Session Handler to report media consumption to the 5GMSd AF. The API defines data models, resources and the related procedures for the creation and management of the consumption reporting procedures. This procedure is configured by the ServiceAccessInformation resource, as defined in clause 11.2.3.

---- <snipped> ----

## 11.4		Metrics Reporting APIs

### 11.4.1	General

The Metrics Reporting API allows the Media Session Handler to send metrics reports to the 5GMSd AF or the 5GMSu AF, for a downlink streaming service or an uplink streaming service, respectively. This procedure is configured by the ServiceAccessInformation resource, as defined in clause 11.2.3. Note that multiple metrics configurations can be active at the same time, each identified by a unique metricsReportingConfigurationId.

### 11.4.2	Reporting procedure

Metrics reports related to a specific metricsReportingConfigurationId shall be submitted to one of the URLs selected from the ClientMetricsReportingConfiguration.serverAddresses array of the ServiceAccessInformation resource (see clause 11.2.3). The path of the URL should conform to one of the following general formats:

{apiRoot}/3gpp-m5d/v1/metrics-reporting/{provisioningSessionId}/{metricsReportingConfigurationId}

{apiRoot}/3gpp-m5u/v1/metrics-reporting/{provisioningSessionId}/{metricsReportingConfigurationId}

depending on whether the metrics reporting functionality is associated with downlink or uplink streaming, and where {provisioningSessionId} shall be substituted by the 5GMS Client with the relevant Provisioning Session identifier and {metricsReportingConfigurationId} shall be substituted with the relevant Metrics Reporting Configuration identifier.

The only HTTP method supported by this endpoint is POST.

### 11.4.3	Report format

Metrics reports shall be submitted by the Media Session Handler in a format specified by the metrics reporting scheme in question. The Content-Type HTTP request header shall be set in accordance with the relevant metrics reporting scheme specification.

NOTE:	For downlink streaming, TS 26.247 [7] clauses 10.6.1 and 10.6.2 specify the required MIME content type and metrics report format for the 3GPP urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 metrics reporting scheme.

## 11.5	Dynamic Policies APIs

### 11.5.1	Overview

The Dynamic Policies API allows the Media Session Handler to request a specific policy and charging treatment to be applied to a particular application data flow of a downlink or uplink streaming session by invoking RESTful operations on the 5GMSd AF at interface M5d or M5u, respectively. The API defines a set of data models, resources and the related procedures for the creation and management of the dynamic policy request.

### 11.5.2	Resource structure

The Dynamic Policies APIs for downlink streaming and uplink streaming sessions are accessible, respectively, through the following URL base paths:

{apiRoot}/3gpp-m5d/v1/dynamicpolicies/

{apiRoot}/3gpp-m5u/v1/dynamicpolicies/

Table 11.5.2-1 below specifies the operations and the corresponding HTTP methods that are supported by these APIs for either downlink or uplink streaming. The sub-resource path specified in the second column shall be appended to the appropriate (downlink or uplink streaming) URL base path.

Table 11.5.2-1: Operations supported by the Dynamic Policies APIs

| Resource name | Sub-resource path | Allowed HTTP methods | Description |
| --- | --- | --- | --- |
| Dynamic Policies | policies | POST | Create a new Dynamic Policy resource. If the operation succeeds, the URL of the created Dynamic Policy Instance resource shall be returned in the Location header of the response. |
| Dynamic Policy | policies/{dynamicPolicyId} | GET | Read a Dynamic Policy resource. |
|  |  | PUT | Replace an existing Dynamic Policy resource. |
|  |  | PATCH | Modify an existing Dynamic Policy resource. |
|  |  | DELETE | Delete an existing Dynamic Policy resource. |

### 11.5.3	Data model

#### 11.5.3.1	DynamicPolicy resource

Table 11.5.3.1-1: Definition of Dynamic Policy resource

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| dynamicPolicyId | String | 1..1 | RO | Unique identifier for this Dynamic Policy for the associated downlink or uplink streaming session. |
| policyTemplateId | String | 1..1 | C: RW R: RO U: RW | Identifies the Policy Template which should be applied to the application flow(s) of the associated downlink or uplink streaming session. |
| serviceDataFlowDescriptions | Array(ServiceDataFlowDescription) | 1..1 | C: RW R: RO U: RW | Describes the service data flows of the associated downlink or uplink streaming session managed by this Dynamic Policy. |
| provisioningSessionId | String | 1..1 | C: RW R: RO U: RW | Uniquely identifies Provisioning Session, which is linked to the Application Service Provider. |
| qosSpecification | M5QoSSpecification | 0..1 | C: RW R: RO U: RW | Describes the network Quality of Service properties of this Dynamic Policy. |
| enforcementMethod | String | 0..1 | C: RO R: RO U: RO | Description of the Policy Enforcement Method. The parameter is set by the 5GMSd or 5GMSu AF, depending on whether this Dynamic Policy is associated with a downlink or uplink streaming session. |
| enforcementBitRate | Integer | 0..1 | C: RO R: RO U: RO | Description of the enforcement bit rate. |

### 11.5.4	Operations

This clause defines the behaviour that is expected when activating a Dynamic Policy Instance for the associated downlink or uplink streaming session. The policyTemplateId uniquely identifies the Policy Template, to which the Dynamic Policy Instance is associated. The provisioningSessionId associates the Dynamic Policy Instance to a Provisioning Session associated with the downlink or uplink streaming session.

The Dynamic Policy resource contains a serviceDataFlowDescription property which contains the service data flow template according to TS 23.503. The ServiceDataFlowDescription shall contain one of:

-	a flowDescription Object (incl. 5-Tuples, Type of Service, Security Parameter Index, etc.).

-	a domainName.

When the Media Session Handler activate a QoS-related Dynamic Policy Template, then the qosSpecifcation property shall be present and it shall contain the following properties:

-	marBwDlBitRate or marBwUlBitRate, indicating the maximum requested downlink or uplink bit rate by the Media Session Handler.

-	mirBwDlBitRate or mirBwUlBitRate, indicating the minimum requested downlink or uplink bit rate by the Media Session Handler.

-	minDesBwDlBitRate or minDesBwUlBitrate, indicating the minimum downlink or uplink bit rate desired by the Media Session Handler.

When the 5G System employs a traffic enforcement function to ensure that the traffic is complying a certain traffic policy, the Dynamic Policy resource may contain the following two properties

-	 an enforcementMethod, indicating the type of enforcement method (like leaky bucket).

-	 an enforcementBitrate property, indicating the maximal permitted downlink or uplink bit rate.

## 11.6	Network Assistance APIs

### 11.6.1	Overview

If AF-based Network Assistance is supported, then the Network Assistance API component of interface M5d or M5u, as defined in the present sub-clause, is first used to provision. respectively, a Network Assistance Session resource associated with downlink streaming or uplink streaming session. The Network Assistance Resource can then be used to obtain bit rate recommendations and to issue delivery boost requests during the ongoing downlink or uplink media streaming session.

### 11.6.2	Resource structure

The Network Assistance APIs for downlink streaming and uplink streaming sessions are accessible, respectively, via the following URL base paths:

{apiRoot}/3gpp-m5d/v1/network-assistance/

{apiRoot}/3gpp-m5u/v1/network-assistance/

Table 11.6.2-1 below specifies the operations and the corresponding HTTP methods that are supported by these APIs. In each case, the sub-resource path specified in the second column of the table shall be appended to the appropriate (downlink or uplink streaming) URL base path.

Table 11.6.2-1: Operations supported by the Network Assistance API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Network Assistance Session resource |  | POST | Provision a new Network Assistance Session. If the operation succeeds, the URL of the created Network Assistance Session resource shall be returned in the Location header of the response. |
| Fetch a Network Assistance Session resource | {naSessionId} | GET | Fetch the properties of an existing Network Assistance Session. |
| Update a Network Assistance Session resource | {naSessionId} | PUT, PATCH | Update the properties of an existing Network Assistance Session. |
| Request a bit rate recommendation | {naSessionId}/recommendation | GET | Obtain a bit rate recommendation for the next recommendation window. |
| Request a delivery boost | {naSessionId}/boostRequest | POST | Request a delivery boost for the next recommendation window. |
| Terminate Network Assistance Session | {naSessionId} | DELETE | Terminate a Network Assistance session. |

### 11.6.3	Data model

#### 11.6.3.1	NetworkAssistanceSession resource

For either a downlink or uplink streaming session, the NetworkAssistanceSession resource is specified in Table 11.6.3.1-1 below.

Table 11.6.3.1-1: Definition of NetworkAssistanceSession resource

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| naSessionId | String | 1..1 | C: RO R: RO U: RO | Unique identifier for this Network Assistance Session. |
| serviceDataFlowInformation | Array(ServiceDataFlowDescription) | 0..1 | C: RW R: RO U: RW | Identification of the application flows for the corresponding downlink or uplink streaming session for which Network Assistance is to be used, e.g. 2-tuple (IP addresses) or 5-tuple (IP Addresses, protocol and ports). |
| policyTemplateId | String | 0..1 | C: RW R: RO U: RW | Identification of the policy that is in force for the streaming session. |
| requestedQoS | M5QoSSpecification | 0..1 | C: RW R: RO U: RW | The requested QoS parameters. |
| recommendedQoS | M5QoSSpecification | 0..1 | C: RO R: RO U: RO | The QoS parameters currently recommended by the 5GMS AF. |
| notficationURL | String | 0..1 | C: RO R: RO U: RO | A URL to the MQTT channel over which notifications are to be sent by the 5GMS AF for this session. When set, the Media Session Handler shall subscribe to this channel. The notification messages shall be in the form of the M5QoSSpecification data type. |

### 11.6.4	Operations

The 5GMSd or 5GMSu client uses the POST method to create, respectively, a downlink or uplink streaming Network Assistance session with the 5GMS AF. The AF returns the Network Assistance session identifier if session setup was successful, otherwise an error code is returned without a Network Assistance session identifier.

The 5GMSd or 5GMSu Client uses, respectively, the downlink or uplink Network Assistance session resource identifier (naSessionId) provided by the AF to refer all subsequent API calls to the AF applicable to that Network Assistance session.

The 5GMSd or 5GMSu Client populates the Network Assistance session resource with, respectively, the downlink or uplink service data flow information and optionally the policy template id that are valid for the streaming session for which Network Assistance operations are to be performed. The AF uses this information to execute Network Assistance operations in the 5GC.

The 5GMSd or 5GMSu Client Client uses the GET method with the Network Assistance Session resource identifier to retrieve, respectively, a downlink or uplink Network Assistance Session resource from the 5GMS AF. The AF returns the Network Assistance Session resource if retrieval was successful, otherwise an appropriate error code is returned without the session resource in case of failure.

The 5GMSd or 5GMSu Client Client uses the GET method with the sub-resource path specified in Table 11.6.2-1 to request, respectively, a downlink or uplink bit rate recommendation from the 5GMS AF. The 5GMS AF shall return the recommended bit rate in an HTTP response body of type M5QoSSpecification if a bit rate recommendation could be obtained, otherwise an appropriate HTTP error code shall be returned with no response body. For a downlink streaming session, the recommended minimum and maximum bit rates shall be indicated in the properties mirBwDlBitRate and marBwDlBitRate, respectively. For an uplink streaming session, the recommended minimum and maximum bit rates shall be indicated in the properties mirBwUlBitRate and marBwUlBitRate, respectively.If a unique recommendation is given by the 5GMSd AF then this recommended bit rate shall be set in both of these properties. The optional properties minDesBwDlBitRate, minDesBwUlBitRate, desLatency and desLoss shall not be included in the response.

The 5GMSd or 5GMSu Client uses the POST method with the sub-resource path specified in Table 11.6.2-1 to request, respectively, a downlink or uplink delivery boost from the 5GMS AF. The 5GMS AF shall respond with the OperationSuccessResponse data type indicating whether or not the delivery boost will be attempted by the network within an upcoming nominal time period.

The 5GMSd or 5GMSu Client uses the PUT or PATCH methods to replace the existing downlink or uplink streaming session parameters, respectively, with new settings. The AF returns the NetworkAssistanceSession resource with settings resulting from the PUT or PATCH update operation.

The 5GMSd or 5GMSu Client uses the DELETE method to terminate the indicated downlink or uplink Network Assistance session, respectively. The 5GMS AF returns an appropriate response code. If the termination was successful then any subsequent calls referring to the terminated session will result in the error 404 (Not Found).

END OF 4h CHANGE