3GPP TSG-S4 ad hoc post Meeting #115-e	S4aI211236

# Online,  , 31 August–2 November 2021	revision of S4aI211228

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.531 | CR | – | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Domain model for data collection and reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVEX |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Documentation of static domain model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Domain model figure and explanatory text. - Updates to baseline provisioning information to include data exposure restrictions. - Placeholder security model clause and example to define the above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 4.5, 4.6 (renumbered), 4.7 (renumbered) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 4.2	Functional entities for data collection and reporting

(SNIPPED)

7.	The NWDAF is the primary consumer of processed UE data. This is exposed to the NWDAF by the Data Collection AF in the form of data reporting event notifications via reference point R5 using the Naf_EventExposure service (as specified in TS 29.517 [5]) after any processing by the Data Collection AF has been performed according to its provisioned processing instructions.

(SNIPPED)

NEXT CHANGE

## 4.5	Information security model

An encrypted data transfer protocol shall be employed at reference point R2 to protect the secrecy and integrity of collected UE data in transit between the Direct Data Collection Client and the Data Collection AF.

Editor’s Note: Provide normative description of stage 2 information security model, including the concept of “access level”.

For example, the Application Service Provider, as provider of the UE Application, is granted complete and fine-grained access to UE data reports pertaining to that application, whereas the NWDAF is granted partial and coarse-grained access to that data, possibly in aggregated and anonymized form, under Application Service Provider control. The Application Service Provider is able, through provisioning, to authorise event exposure for each of the specified access levels.

## 4.6	Domain model

### 4.6.1	General

Figure 4.6.1-1 depicts the static data model for the data collection and reporting domain. It is further described below.

Figure 4.6.1-1: Static domain model

The Application Service Provider provisions zero or more sets of provisioning information in the Data Collection AF at reference point R1. The baseline set of information provisioned is described in clause 4.6.2. Each set of provisioning information pertains to one application, identified by its external application identifier, and one type of exposed event, uniquely identified in the 5G System by its Event ID, as defined in clause 4.15.1 of TS 23.502 [3]. There may be more than one set of provisioning information for a particular external application identifier, but the combination of the external application identifier and Event ID shall be unique for a given Data Collection AF instance.

Each set of provisioning information is manifested as a data collection client configuration that the Data Collection AF makes available to Direct Data Collection Client instances at reference point R2, to Indirect Data Collection Client instances at R3 and to AS instances at R4.

These data collection and reporting clients then send data reports to the Data Collection AF associated with the data collection client configuration. It is the responsibility of the data collection and reporting client to discover its external application identifier by means outside the scope of the present document.

### 4.6.2	Provisioning information for data collection and reporting

A separate set of provisioning information shall be provided to the Data Collection AF at reference point R1 for each Event ID it is to expose. This provisioning information embodies the Service Level Agreement between the network operator and the Application Service Provider envisaged in clause 6.2.8.1 of TS 23.288 [4]. The provisioning information shall include at least the parameters defined in table 4.6.2-1 below:

Table 4.6.2-1: Baseline provisioning information for data collection and reporting

| Parameter | Cardinality | Description |
| --- | --- | --- |
| External Application Identifier | 1..1 | The identifier to be used in reports sent to the Data Collection AF by data collection and reporting clients. (This needs to be mapped to the Internal Application Identifier when exposing events to the NWDAF.) |
| Internal Application Identifier | 1..1 | The identifier to be used by event consumers (including the NWDAF and the Event Conumer AF) when subscribing to events in the Data Collection AF. |
| Event ID | 1..1 | The identifier of an AF event that will be exposed to event consumers as a result of the provisioning. |
| Valid targets | 1..1 | A parameter to control whether event consumers are permitted to filter events by External UE identifier or External Group Identifier when subscribing, instead of receiving events relating to all UEs. |
| Parameters to be collected | 1..* | The subset of domain-specific parameters associated with the specified Event ID to be collected by the Data Collection AF (subject to user consent). |
| Data processing instructions | 1..* | A set of operations to be performed by the Data Collection AF on the parameters reported according to clause 4.6.4 prior to exposure as an event at a particular access level. The set of supported operations shall include at least: COUNT, MEAN, MEDIAN, MINIMUM, MAXIMUM and SUM. |
| Data exposure restrictions | 1..* | A set of restrictions on the exposure of the collected data after any data processing, each corresponding to a different access level. |

### 4.6.3	Configuration information for data collection and reporting clients

All clients of the Data Collection AF wishing to report data shall first obtain a data collection and reporting configuration from the Data Collection AF at reference point R2, R3 or R4 (as appropriate). The data collection and reporting configuration shall include at least the parameters defined in table 4.5.3-1 below:

Table 4.6.3-1: Baseline information for data collection and reporting configuration

| Parameter | Cardinality | Description |
| --- | --- | --- |
| External Application Identifier | 1..1 | Identifies the UE Application to which this data collection and reporting configuration pertains. Quoted in reports sent to the Data Collection AF. |
| Parameters to be collected | 1..* | The subset of domain-specific parameters associated with the specified Event ID to be collected by the Data Collection AF (subject to user consent). |

Editor’s Note: When requesting configuration, additional information needs to be passed across the R7 client API (then R2) to realise the "authentication information that enables the [Data Collection] AF to verify the authenticity of the UE's Application that provides data" as envisaged in clause 6.2.8.1 of TS 23.288 [4].

### 4.6.4	Information included in data reports to the Data Collection AF

Table 4.6.4-1: Baseline information for data reporting

| Parameter | Cardinality | Description |
| --- | --- | --- |
| External Application Identifier | 1..1 | Identifies the UE Application to which this data report pertains. |
| Collected parameters | 1..* | The set of parameters collected by the data collection and reporting client. |

Editor’s Note: When reporting data, additional information needs to be passed across the R7 client API (then R2) to realise the "authentication information that enables the [Data Collection] AF to verify the authenticity of the UE's Application that provides data" as envisaged in clause 6.2.8.1 of TS 23.288 [4].

END OF CHANGES