3GPP TSG-SA WG4 ad hoc meeting post SA4#132	S4aI250100

Electronic, 5th June–10th July 2025

Title:	Gap analysis of CMCD configuration with MPEG-DASH 6th Edition

Source:	BBC

Agenda Item:	2.5 (AMD_PRO-MED)

Document for:	Discussion and agreement

Contact:	Richard Bradbury <richard dot bradbury at bbc dot co dot uk>

# Abstract

The present document analyses the configuration parameters for CMCD [1] client data reporting specified in the sixth edition of MPEG-DASH Part 1 [2] in comparison with the latest proposed design in the AMD_PRO-MED Work Item for provisioning the 5G Media Streaming (5GMS) System and for configuring the 5GMS Client [3] and recommends some modifications based on this gap analysis.

# Introduction

MPEG WG3 recently delivered a Final Draft International Standard (FDIS) revision of the MPEG-DASH Part 1 specification to ISO/IEC JTC 1/SC 29 for publication approval [2]. This sixth edition of the specification specifies for the first time the signalling of client data from a DASH client to a server following the syntax specified by the Common Media Client Data (CMCD) specification [1].

According to clauses K.3.7 and K.4.2 of [2], the DASH client may be configured to perform CMCD reporting by the inclusion of a CMCDParameters element in the Service Description of a Media Presentation Description (MPD) document. The XML element hierarchy is as shown in the table overleaf.

NOTE:	A ServiceDescription element may be declared at any of the following scopes in an MPD document: MPD, Period, EventStream.

# Gap analysis

| XML elements and attributes |  |  |  |  | Summary | Default value | Remarks | Endorsed baseline in 26512-CR0089r2 | Proposed way forward for revision of 26512-CR0089 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ServiceDescription |  |  |  |  |  |  | Table K.4.2.1-1. |  |  |
|  | … |  |  |  |  |  | (Other attributes and elements omitted for brevity.) |  |  |
|  | ClientDataReporting |  |  |  | Parameters for client data reporting. |  | Table K.4.2.7.1-1 |  |  |
|  |  | @schemeIdUri |  |  | Reporting scheme URI. |  | urn:mpeg:dash:cta-5004:2023 | Controlled vocabulary of terms in a 3GPP name space defining terms for each of the four different scopes of CMCD information | No gap detected. |
|  |  | @serviceLocations |  |  | Identifier(s) of Service Locations for which client data reporting is performed. | Report for all Service Locations |  | Not currently supported; assumes reporting performed for all Service Locations in the presentation. | Explicitly specify that the Media Player Entry may restrict the set of Service Locations for which CMCD reporting is performed by the Media Player. |
|  |  | @adaptationSets |  |  | Identifier(s) of Adaptation Sets for which client data reporting is to be performed. | Report for all Adaptation Sets |  | Not currently supported; assumes reporting performed for all Adaptation Sets in the current Period. | Explicitly specify that the Media Player Entry may restrict the set of Adaptation Sets for which CMCD reporting is performed by the Media Player. |
|  |  |  | CMCDParameters |  | CMCD-specific parameters for client data reporting. |  | Table K.4.2.7.2-1. |  |  |
|  |  |  |  | @version | Highest CMCD specification version. | 1 | Later versions of the specification are not yet in scope. | Version 1 only. | No gap detected. |
|  |  |  |  | @mode | Either query or header. | query |  | header by default; query can be specified by 5GMSd-Aware Application via client API at reference point M6/M11. | Specify a client-data-reporting-mode query parameter to provide the equivalent functionality in 3GPP Service URL. This parameter takes precedence over any client data reporting mode declared in the selected Media Entry Point. |
|  |  |  |  | @includeInRequests | One or more of: segment, init, xlink, mpd, callback, chaining, fallback, sbd, steering, mpdpatch, altmpd, mpdlink, *. | Include in requests for all types of object |  | Not currently supported; assumes reporting performed for all requested object types. | Explicitly specify that the Media Player Entry may restrict the set of object types for which CMCD reporting is performed by the Media Player. |
|  |  |  |  | @keys | The subset of CMCD keys (per CTA-5004 [1]) to be reported. | None | Keys not defined in CTA-5004 [1] are not reported. | Controlled vocabulary of URN-based terms in a 3GPP name space comprising all the CTA-5004 key names. | No gap detected at reference point M1 or M5. Specify that keys specified in the MPD are in addition to those specified in the Service Access Information. Probably no need to additionally specify a new keys query parameter for the 3GPP Service URL as well. |
|  |  |  |  | @contentID | The value of the CMCD cid key to be used when reporting CMCD information. | None | Restricted to 64 characters. May be derived automatically by the DASH client from the up:ExtUrlQueryInfo element in the MPD. | Not currently supported. | Specify a client API enabling this value to be set by a 5GMS-Aware Application. Also specify a content-id query parameter to provide the equivalent functionality in 3GPP Service URL. Specify that any value explicitly declared in the MPD is overridden with a value supplied via one of the above APIs. |
|  |  |  |  | @sessionID | The value of the CMCD sid key to be used when reporting CMCD information. | None | Restricted to 64 characters. May be derived automatically by the DASH client from the up:ExtUrlQueryInfo element in the MPD. May be a UUID (preferably UUIDv7) generated by the DASH client per RFC 9562 and used consistently for a given DASH streaming session. |  | Specify that any value explicitly declared in the MPD is overridden with the media delivery session identifier. |

# Proposal

Based on the gap analysis in section 3, it is proposed that SA4 agrees to specify the following precedence rules in TS 26.512 [2]:

- Specify additional query parameters in the 3GPP Service URL for signalling:

  - content-id to be inserted into client data reports.

    - (In the case of 5GMS Consumption Reporting, this needs to sit alongside the DASH Representation ID.)

  - client-data-reporting-mode to select between CMCD query or CMCD headers.

    - (Achieves parity with the endorsed design for Configuration and Settings API at M6/M11.)

- Explicitly specify that the Media Player Entry may restrict certain aspects of client data reporting such as which subset of Service Locations, Adaptation Sets and media object types are reported on, and that such restrictions should be honoured by the Media Player.

- Explicitly specify that the Media Player Entry may specify additional client data keys to be reported, in which case these should be provided by the Media Player and exposed to downstream event consumers.

- Explicitly specify that any fixed session identifier declared in a Media Player Entry is always overridden by the media delivery session identifier.

# References

CTA-5004: "Web Application Video Ecosystem: Common Media Client Data (CMCD)", September 2020.

ISO/IEC JTC 1/SC 29/WG 3 FDIS 23009-1:2025 "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats". March 2025.

3GPP TSG SA WG4: TS 26.512 CR0089 rev 2 []: "5G Media Streaming (5GMS); Protocols".

3GPP TR 26.804: "Study on 5G media streaming extensions", Release 19.