Annex E (normative):
HTTP-based CMMF content delivery protocol instantiation

# E.0	Introduction

This annex defines a content delivery protocol instantiation when CMMF is used to deliver resources that are expected to be retrieved by a generic application service using HTTP.

The HTTP-based CDP Instantiation is defined as follows:

Typical procedures for a HTTP-based CMMF CDP Instantiation are provided in clause E.1.

A Delivery Session comprises of one or more Transport Sessions. Each Transport Session is comprised of one or more Transport Objects where each Transport Object is identified by a unique Transport Object Identifier (TOI), for example a uniquely identifiable URL.

A Transport Object shall be a compliant CMMF bitstream as specified in clauses 5 and 6.

The retrieval of an application service resource/source object (identifiable by a unique URL that an application service would have been expected to retrieve using HTTP) constitutes a Delivery Session.

The retrieval of an application service resource/source object shall be possible from some combination of Transport Objects created for a Delivery Session.

The availability of Transport Objects created for a Delivery Session shall be the same as the availability of the application service resource/source object.

CMMF Transport Objects shall be retrieved by a CMMF Receiver using HTTP (e.g., HTTP/1.1 [i.4], HTTP/2 [i.6], HTTP/3 [i.7], etc.).

CMMF Configuration Information required by a CMMF Receiver may apply to a single Delivery Session or multiple Delivery Sessions.

The CMMF Configuration Information can be expressed as a manifest, configuration file, or API providing information such as a mapping between application resources/source objects (identifiable by a unique URL) and Transport Sessions/Transport Objects contained within a Delivery Session. Further details are provided in clause E.2.

# E.1	Procedures for HTTP-based CMMF CDP Instantiation

## E.1.1	Introduction

This clause provides procedures for CMMF content preparation, provisioning and delivery for the HTTP-based CMMF CDP instantiation. A standard HTTP-based content delivery model is assumed where:

An Application Provider makes its content available to clients via one or more service locations exposed by one or more Application Servers (e.g., CDNs, CDN points of presence (PoP), etc.).

Clients obtain content via one or more of the exposed service locations (e.g., using HTTP GET requests).

## E.1.2	Procedure for HTTP-based CMMF provisioning, content ingest, and CMMF Transport Object preparation

A baseline procedure describing the network provisioning, content ingest, and CMMF Transport Object preparation required to support a HTTP-based CDP Delivery Session is shown in Figure E.1.

Figure E.1: High-level procedure for HTTP-based CMMF provisioning and preparation

The procedure makes the following assumptions:

A CMMF-Aware Application Provider is responsible for network provisioning and CMMF Delivery Session configuration/provisioning which may include:

Provisioning content hosting.

Preparation of CMMF Transport Objects and/or provisioning of compute resources to create CMMF Transport Objects.

Preparation of necessary CMMF CI.

An Application Server (e.g., CDN, CDN PoP, etc.) is used for distribution of source and/or CMMF Transport Objects to CMMF-Aware Clients. This Application Server may:

Cache objects for later retrieval.

Encode ingested source objects or re-encode ingested CMMF Transport Objects.

Prepare CMMF Transport Objects.

Prepare or update the CMMF CI used by a CMMF-Aware Client.

A HTTP pull-based architecture is assumed where a Delivery Session and the retrieval of CMMF Transport Objects is initiated by a CMMF-Aware Client.

Steps:

1.	The CMMF-Aware Application Provider provisions content hosting within the Application Server (e.g., CDN, CDN PoP, etc.) and establishes one or more service locations where source and/or CMMF Transport Objects are made available to CMMF-Aware Clients. In the case where the Application Server prepares the CMMF CI, the Application Server may provide the CMMF-Aware Application Provider with a URL to the CMMF CI.

2.	In the case were the CMMF-Aware Application Provider generates the CMMF CI, the CMMF-Aware Application Provider prepares the CMMF CI used by CMMF-Aware Clients to access CMMF Transport Objects during a Delivery Session. This CMMF CI should include information about the available service locations provisioned in step 1.

3.	The CMMF-Aware Client triggers the Service and Content Discovery procedure. A URL pointing to where the CMMF CI is available may be provided to the CMMF-Aware Client during this procedure or the CMMF CI may be provided to the CMMF-Aware Client by other means.

4.	The CMMF-Aware Client requests the CMMF CI. The CMMF CI may be available from the Application Server, the CMMF-Aware Application Provider, etc.

5.	In the case where the CMMF CI is not cached by the Application Server and the CMMF CI is prepared by the CMMF-Aware Application Provider, the Application Server propagates the request for the CMMF CI to the CMMF-Aware Application Provider.

6.	The CMMF-Aware Application Provider provides the CMMF CI to the Application Provider.

7.	In the case where the Application Server generates the CMMF CI, the Application Server prepares the CMMF CI used by CMMF-Aware Clients to access CMMF Transport Objects during a Delivery Session. If the CMMF-Aware Application Provider originally prepared the CMMF CI, the Application Server may be configured to update the CMMF CI before delivery to a CMMF-Aware Client.

8.	The CMMF CI for a Delivery Session is provided to a CMMF-Aware Client.

9.	The CMMF-Aware Client selects a CMMF Transport Object to download.

10.	The CMMF-Aware Client requests a CMMF Transport Object from the Application Server.

11.	In the case where the CMMF Transport Object is not cached by the Application Server, the Application Server propagates the request for either the CMMF Transport Object or its associated source object (in the case where the Application Server generates CMMF Transport Objects) to the CMMF-Aware Application Provider.

12.	In the case where the CMMF-Aware Application Provider receives a request for a CMMF Transport Object, it may use the source object associated with the CMMF Transport Object to prepare one.

NOTE:	CMMF Transport Objects may be prepared prior to receiving a request or in response to receiving a request.

13.	The CMMF-Aware Application Provider provides either the requested CMMF Transport Object or its associated source object to the Application Server. In either case, the received object may be cached for later use.

14.	In the case where the Application Server is provisioned to prepare CMMF Transport Objects, it may use either a source object or a cached CMMF Transport Object to generate a new CMMF Transport Object.

15.	The CMMF Transport Object is provided to the CMMF-Aware Client.

## E.1.3	Procedure for HTTP-based CMMF delivery

A baseline procedure describing the establishment of a HTTP-based CDP Delivery Session is shown in Figure E.2.

Figure E.2: High-level procedure for HTTP-based CMMF delivery

The procedure makes the following assumptions:

CMMF Transport Objects used to support a Delivery Session are hosted at one or more service locations. These Transport Objects may be prepared so that each service location hosts a different encoded representation or variation of the source content/object. These Transport Objects may be prepared by the CMMF-Aware Application Provider prior to provisioning of a service location(s) or at each service location depending on the network configuration.

CMMF CI may apply to all Delivery Sessions or be specific to a single Delivery Session. The procedure assumes the former.

The CMMF Receiver in the Client has the necessary functionality to establish multiple Transport Sessions and retrieve multiple Transport Objects simultaneously from multiple service locations.

Prerequisites:

1.	The CMMF-Aware Application Provider has provisioned the Application Server to distribute CMMF Transport Objects to CMMF-Aware Applications.

2.	The CMMF-Aware Application Provider provides the URL of the CMMF CI to the CMMF-Aware Application.

Steps:

3.	The CMMF-Aware Application configures the CMMF Receiver to be used for obtaining content/source objects. This includes providing the CMMF Receiver with the URL to the CMMF CI.

4.	The CMMF Receiver obtains the CMMF CI from either the CMMF-Aware Application Provider or one of the provisioned service locations and informs the CMMF-Aware Application that the CMMF CI has been received and processed.

5.	Content or a source object is selected by the CMMF-Aware Application.

6.	The CMMF-Aware Application provides the URL to the selected content/source object to the CMMF Receiver.

7.	The CMMF Receiver identifies the service locations and CMMF Transport Object URLs corresponding to the received source object URL.

8.	The CMMF Receiver requests and downloads the necessary CMMF Transport Objects required to recover the content/source object. CMMF Transport Objects may be downloaded in parallel from multiple service locations; and each CMMF Transport Object may be obtained in full or in part depending on the Delivery Session configuration and/or network conditions.

9.	The CMMF Receiver decodes the information received from the obtained full or partial CMMF Transport Objects and extracts the content/source object.

10.	The content/source object is provided to the CMMF-Aware Application Provider.

# E.2	CMMF Configuration Information for HTTP-based CMMF CDP

## E.2.1	Introduction

This clause provides a template for the CMMF Configuration Information that can be used by a CMMF Receiver to translate between source objects needed by a CMMF-Aware Application and the CMMF Transport Objects used to deliver those source objects.

## E.2.2	Source Object URL to CMMF Transport Object URL mapping procedures

### E.2.2.1	General

Procedures to translate URLs of application service resources/source objects (i.e., URLs for resources/source objects that would have been expected to be retrieved using HTTP) into URLs of the CMMF Transport Objects containing encoded representations or variants of those resources are defined in this clause.

### E.2.2.2	Mapping complete object URLs

An application service resource/source object and its corresponding URL is first identified or provided by the application. The URL is then mapped into one or more CMMF Transport Object URLs which are used to request those CMMF Transport Objects from one or more service locations by:

Adding a scheme and domain name to the URL of the service location for which the CMMF Transport Object is hosted or replacing the scheme and domain name of the application service resource URL with a scheme and domain name of the service location for which the CMMF Transport Object is hosted; and/or

Inserting a pre-defined sub-path into the URL’s path that is associated with the CMMF Transport Object. This sub-path may also indicate the CMMF representation or variant of the CMMF-encoded application service resource to retrieve from the identified service location.

For example, an application service resource with the URL https://example.com/path-to-media/segment-1.mp4 is mapped to two CMMF Transport Objects, each containing a different CMMF representation or variant of the application service resource.

The first CMMF Transport Object is available at the service location with base URL https://host-a.example.com. The sub-path cmmf-a, which also is used to identify which CMMF-encoded representation or version of the application service resource to download from the service location, is inserted into the path of the application service resource URL.

Likewise, the second CMMF Transport Object is available at https://host-b.example.com and the sub-path cmmf-b is inserted into the path of the media presentation URL.

The mapped request URLs for both CMMF Transport Objects are then:

https://host-a.example.com/path-to-media/cmmf-a/segment-1.mp4

and

https://host-b.example.com/path-to-media/cmmf-b/segment-1.mp4

respectively.

### E.2.2.3	Mapping sub-object URLs using byte range requests

In the case where an application service resource/source object is identified by both a URL to an object and a byte range within that object (e.g., chunked CMAF), the mapping is similar to that described in clause E.2.2.2 above. The scheme and domain name to a service location hosting a CMMF Transport Object containing a CMMF-encoded representation or variant of the application service resource is either added or replaced in the application service resource’s URL and a sub-path is inserted. The byte range, which is usually communicated using the Byte-Range HTTP request header, is translated into an additional sub-path element and is also inserted into the mapped URL’s path.

For example, an application service resource with the URL https://example.com/path-to-media/segment-1.mp4 and byte range startByte to endByte is mapped to two CMMF Transport Objects containing different CMMF-encoded representations or variants of the application service resource.

-	The first CMMF Transport Object is available at the service location with base URL https://host-a.example.com. The sub-path cmmf-a, which is also used to identify which CMMF-encoded representation or variant of the application service resource to download from the service location, is inserted into the path of the application service resource URL. The byte range is also inserted into the path URL as byte-range-<startByte>-<endByte>.

-	Likewise, the second CMMF Transport Object is available at https://host-b.example.com and the sub-paths cmmf-b and byte-range-<startByte>-<endByte> are inserted into the path of the application service URL.

The mapped request URLs for both CMMF Transport Objects are:

https://host-a.example.com/path-to-media/cmmf-a/byte-range-<startByte>-<endByte>/segment-1.mp4

and

https://host-b.example.com/path-to-media/cmmf-b/byte-range-<startByte>-<endByte>/segment-1.mp4

respectively.

### E.2.2.4	Mapping HTTP headers and query parameters

Besides the mapping of the Byte-Range HTTP request header discussed in clause E.2.2.3, any additional HTTP headers and/or query parameters that would be added to an HTTP request for the source object should be directly mapped to the HTTP requests for the CMMF Transport Objects. The CMMF Receiver may also add or modify HTTP headers and/or query parameters as necessary.

## E.2.3	HTTP-based CMMF configuration information

### E.2.3.1	General

A template for communicating necessary CMMF configuration information required for HTTP-based content delivery is described in this clause. It may be used by a CMMF Receiver to configure a Delivery Session and obtain selected content/source objects using the HTTP-based CDP.

### E.2.3.2	Semantics

The semantics of the CMMF configuration information for HTTP-based content delivery are provided in table E.1.

Table E.1: Semantics of the HTTP-based CMMF configuration information manifest

| Property Name |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- |
| version |  |  |  |  | M | The version of CMMF configuration information manifest. The present document defines CMMF configuration information manifest version 0. A CMMF Receiver shall refuse to use a CMMF configuration information manifest with an unrecognized value for this key. |
| ttl |  |  |  |  | O | Specifies how many seconds the CMMF Receiver shall wait before reloading the CMMF configuration information manifest. If absent, the CMMF configuration information manifest is refresh rate is determined by the Media Application/Client. |
| applicationResourceLocators |  |  |  |  | O | An array of application resource locator objects providing one or more URLs to an application service resource for which this CMMF configuration information applies. An application resource locator object may consist of: - A single file or source object (e.g., media segment), or - An object comprising of a list of files or source objects (e.g., MPEG-DASH MPD, HLS playlist, etc.) in which case this CMMF configuration information is applied to all files or source objects contained in the list. If present, the array shall contain at least one application resource locator object. If omitted, the application resources this CMMF configuration information applies to are supplied via a process external to this configuration. |
|  | locator |  |  |  | M | Absolute URL pointing to an application service resource such as an MPD, HLS master playlist, file, etc. |
|  | contentType |  |  |  | M | The MIME media type of the resource at locator. |
|  | isManifest |  |  |  | M | A Boolean indicated whether the application resource at locator is a single file or source object or an object comprising of a list of files or source objects. If TRUE, the application resource located at locator is an object comprising of a list of files or source objects. This CMMF configuration information shall be applied to all files or source objects contained in the list. If FALSE, the CMMF configuration information shall only be applied to the application resource located at locator. |
| applicationResourceConfigurations |  |  |  |  | M | An array of CMMF Transport Object configurations for the identified application service resources. At least one shall be defined. In the case where multiple CMMF Transport Object configurations apply to an application resource, the first takes precedence. |
|  | admissionRules |  |  |  | O | An object providing rules to determine if the CMMF Transport Object configuration applies to the application resource. The CMMF Transport Object configuration applies if any application resource satisfies applicationResourcePathPattern and/or applicationResourceContentType. If both applicationResourcePathPattern and applicationResourceContentType are defined, the application resource must satisfy both for this CMMF Transport Object configuration to apply. If present, either applicationResourcePathPattern or applicationResourceContentType shall be defined. If absent, the CMMF Transport Object configuration applies to all application resources. |
|  |  | applicationResourcePathPattern |  |  | CM | An array of regular expressions against which the path of the application service resource URL shall be compared. If any of the path patterns listed within the array are found within the URL, this CMMF Transport Object configuration applies. If present, the array shall contain at least one path pattern. If absent, path patterns are not used to determine if the CMMF Transport Object configuration applies. |
|  |  | applicationResourceContentType |  |  | CM | An array of MIME media types of the application service resource for which this CMMF Transport configuration applies. If present, the array shall contain at least one MIME media type. If absent, this CMMF Transport Object configuration applies to all application service resource MIME media types. |
|  | ExclusionRules |  |  |  | O | An object providing rules to determine if the CMMF Transport Object configuration does not apply to the application resource. The rules provided within this object take precedence over those provided in admissionRules. The CMMF Transport Object configuration does not apply if an application resource satisfies either applicationResourcePathPattern or applicationResourceContentType. If present, either applicationResourcePathPattern or applicationResourceContentType shall be defined. If absent, the CMMF Transport Object configuration applies to all application resources. |
|  |  | applicationResourcePathPattern |  |  | CM | An array of regular expressions against which the path of the application service resource URL shall be compared. If any of the path patterns listed within the array are found within the URL, this CMMF Transport Object configuration does not apply. If present, the array shall contain at least one path pattern. If absent, path patterns are not used to determine if the CMMF Transport Object configuration applies. |
|  |  | applicationResourceContentType |  |  | CM | An array of MIME media types of the application service resource for which this CMMF Transport configuration does not apply. If present, the array shall contain at least one MIME media type. If absent, MIME media types are not used to determine if the CMMF Transport Object configuration applies. |
|  | serviceLocations |  |  |  | M | An array of service location objects from which CMMF Transport Objects for this CMMF Transport Object configuration are available. At least one shall be defined. |
|  |  |  | baseUrl |  | M | Base URL of the service location from which CMMF Transport Objects are made available. baseUrl overwrites any base URLs provided by the application resource locator objects (if they exist). baseUrl shall contain a scheme and the service location’s domain name (e.g., https://example.com) at a minimum. |
|  |  |  | requestPathMaps |  | O | An array of request path mappings to convert received application service resource URLs into CMMF Transport Object URLs. |
|  |  |  |  | requestPathPattern | M | A regular expression against which the path of the application service resource URL, beginning from the leading "/", shall be compared. |
|  |  |  |  | mappedPath | M | A replacement for the portion of the application service resource path URL that matches requestPathPattern. |
|  |  |  | additionalHttpHeaders |  | O | A list of key-value pairs indicating HTTP headers that should be added when accessing CMMF Transport Objects from the service location. HTTP headers provided in additionalHttpHeaders either adds to or, in the case of duplicate HTTP header key values, overwrites any HTTP headers provided by the Media Application/Client. |
|  |  |  | additionalHttpQueryParameters |  | O | A list of key-value pairs indicating HTTP query parameters that should be added when accessing CMMF Transport Objects from the service location. HTTP query parameters provided in additionalHttpQueryParameters either adds to or, in the case of duplicate HTTP query key values, overwrites any HTTP query parameters provided by the Media Application/Client. |
|  | cmmfCodingConfiguration |  |  |  | O | The CMMF coding configuration for this CMMF Transport Object configuration. If present, application resources for which this CMMF Transport Configuration applies will be encoded within CMMF Transport Objects that are retrieved by the CMMF Receiver from one or more serviceLocations. If absent, the application resources for which this CMMF Transport Configuration applies will not be encoded within CMMF Transport Objects. Rather, the CMMF Receiver shall retrieve the source object for those application resources from one of the provided serviceLocations. |
|  |  |  | cmmfVersion |  | M | Version of the CMMF specification for which the CMMF Transport Objects conform. |
|  |  |  | cmmfCodeType |  | O | The CMMF code_type used to create the CMMF Transport Objects (see clause 6.1.4.5). |
|  |  |  | cmmfProfile |  | O | The CMMF profile_type used to create the CMMF Transport Objects (see clause 6.1.4.11). |
|  |  |  | cmmfProfileDescription |  | O | The CMMF profile_description used to create the CMMF Transport Objects (see clause 6.1.4.12). |
| Key M=mandatory, O=optional, OD=optional with default value, CM=conditionally mandatory |  |  |  |  |  |  |

### E.2.3.3	Schema

Table E.2 specifies a JSON schema for a document format that may be used to define the parameters of the mappings between application service resources/objects and CMMF Transport Objects containing CMMF-encoded representations or variants of those application service resources hosted at an application provider’s service locations. It also allows for different mappings to be defined depending on the application service resource type (e.g., video, audio, etc.) and/or the pattern of the application service resource URL.

Table E.2: JSON schema of the HTTP-based CMMF configuration information manifest

{ "type": "object", "properties": { "version": { "description": "Version of the CMMF configuration information manifest.", "type": "integer" }, "ttl": { "description": [ "The number of seconds a CMMF Receiver shall wait before reloading the CMMF", "configuration information manifest." ], "type": "number" }, "applicationResourceLocators": { "description": "Array of application resource locator objects.", "type": "array", "items": { "type": "object", "properties": { "locator": { "description": "Absolute URL pointing to an application service resource.", "type": "string" }, "contentType": { "description": [ "MIME media type of the application service resource located at locator." ], "type": "string" }, "isManifest": { "description": [ "Boolean to indicate if the application service resource located at locator is a "list of files/source objects or a single file/source object." ], "type": "boolean" }, "required": ["locator", "contentType", "isManifest"] }, "minItems": 1 }, "applicationResourceConfigurations": { "description": "Array of CMMF Transport Object configurations.", "type": "array", "items": { "type": "object", "properties": { "admissionRules": { "description": [ "Rules used to determine if this CMMF transport configuration should be", "applied to the application resource." ]; "type": "object", "properties": { "applicationResourcePathPattern": { "description": [ "Array of regular expressions against which the path of the application", "service resource URL shall be compared to determine if this CMMF", "transport configuration should be applied to the application resource." ], "type": "array", "items": { "type": "string", "minItems": 1 } }, "applicationResourceContentType": { "description": [ "Array of MIME media type of the application service resource for which", "this CMMF transport configuration applies." ], "type": "array", "items": { "type": "string", "minItems": 1 } } }, "minProperties": 1 }, "ExclusionRules": { "description": [ "Rules used to determine if this CMMF transport configuration should not be", "applied to the application resource. The rules provided within this object", "take precedence over those provided in admissionRules." ]; "type": "object", "properties": { "applicationResourcePathPattern": { "description": [ "Array of regular expressions against which the path of the application", "service resource URL shall be compared to determine if this CMMF", "transport configuration should be applied to the application resource." ], "type": "array", "items": { "type": "string", "minItems": 1 } }, "applicationResourceContentType": { "description": [ "Array of MIME media type of the application service resource for which", "this CMMF transport configuration applies." ], "type": "array", "items": { "type": "string", "minItems": 1 } } }, "minProperties": 1 }, "serviceLocations": { "description": [ "Array of service location objects from which CMMF Transport Objects", "are available." ], "type": "array", "items": { "type": "object", "properties": { "baseUrl": { "description": [ "Base URL of the service location from which CMMF Transport", "Objects are made available." ], "type": "string" }, "requestPathMaps": { "description": "", "type": "array", "items": { "type": "object", "properties": { "requestPathPattern": { "description": [ "A regular expression against which the path of the application", "resource URL shall be compared to determine the location where", "the URL sub-path for the CMMF Transport Object will be inserted." ], "type": "string" }, "mappedPath": { "description": [ "A replacement URL sub-path for the portion of the application", "resource URL that matches requestPathPattern." ], "type": "string" }, }, "required": ["requestPathPattern", "mappedPath"] }, "minItems": 1 }, "additionalHttpHeaders": { "description": [ "A list of key-value pairs indicating HTTP headers that should be", "added when accessing CMMF Transport Objects from the service", "location." ], "type": "object", "properties": {} }, "additionalHttpQueryParameters": { "description": [ "A list of key-value pairs indicating HTTP query parameters that", "should be added when accessing CMMF Transport Objects from the", "service location." ], "type": "object", "properties": {} }, } }, "required": ["baseUrl"] }, "cmmfCodingConfiguration": { "description": [ "CMMF coding configuration for this CMMF Transport Object", "configuration." ], "type": "object", "properties": { "cmmfVersion": { "description": "Version of the CMMF bitstream used.", "type": "number" }, "cmmfCodeType": { "description": "The CMMF code type used.", "type": "integer" }, "cmmfProfile": { "description": "The CMMF profile used.", "type": "string" }, "cmmfProfileDescription": { "description": "The CMMF profile description used.", "type": "string" }, }, "required": ["cmmfVersion"] } }, "required": ["serviceLocations"], } }, "required": ["version", "applicationResourceConfigurations"] }

### E.2.3.3	IANA registration of CMMF configuration information document for HTTP-based content delivery

This clause provides the formal MIME type registration for the HTTP CMMF Configuration Manifest compliant with the JSON schema specified in clause E.2.3.2. It is referenced from the registry at http://www.iana.org/assignments/media-types.

The MIME Type and Subtype are defined in table E.3.

Table E.3: MIME media type registration of CMMF configuration information document for HTTP-based content delivery

| Parameter | Value |
| --- | --- |
| MIME media type name | application |
| MIME subtype name | cmmf-http-cm + json |
| Required parameters | None |
| Optional parameters | None |
| Encoding considerations | This is a JSON document, and the encoding considerations are the same as for media type application/json defined in IETF RFC 8259. |
| Security considerations | This media format is a JSON document and inherits the security considerations provided in RFC 8259. The format does not incorporate any active or executable content. However, other forms of material from outside resources can be referenced by the format, and this material can contain active or executable content. Such material is expected to be identified by its own MIME type, and the security considerations of that format should be taken into account. This format is highly susceptible to manipulation or spoofing for attacks designed to inject fake information. Both integrity protection and source authentication are recommended to prevent misleading of the recipient. |
| Interoperability considerations | The present document defines a platform-independent expression of a document, and it is intended that wide interoperability can be achieved. |
| Published specification | ETSI TS 103 973 (the present document) |
| Applications which use this media type | Various |
| Additional information | File extension(s): json Intended usage: COMMON |
| Other information/general comment | None |
| Person & email address to contact for further information | Jason Cloud (jason.cloud@dolby.com) |
| Restrictions on usage | None |
| Author/Change controller | ETSI |

## E.2.4	Example HTTP-based CMMF configuration information manifests

An example of the CMMF configuration information manifest is provided in this clause. The example assumes the following:

The application resource is the MPEG-DASH manifest shown in table E.4 and available at https://www.example.com/manifest.mpd.

Video and audio segments are intended to be obtained from CMMF Transport Objects located at the identified service locations. However, the CMMF Receiver should obtain video and audio initialization segment source objects from the identified service locations instead of their corresponding CMMF Transport Objects.

CMMF Transport Objects and source objects are hosted at service locations with base URLs https://www.cdn-a.example.com and https://www.cdn-b.example.com.

Table E.4: Example HTTP-based CMMF configuration information manifest

<?xml version="1.0" encoding="UTF-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 DASH-MPD.xsd" type="static" minBufferTime="PT4S" profiles="urn:mpeg:dash:profile:isoff-live:2011" <Period id="1"> <!-- Video --> <AdaptationSet mimeType="video/mp4" codecs="avc1.4D401F" frameRate="30000/1001" segmentAlignment="true" startWithSAP="1"> <SegmentTemplate timescale="90000" initialization="video/$Bandwidth%/init.mp4v" media="video/$Bandwidth%/$Number$.mp4v" startNumber="1"> <SegmentTimeline> <S t="0" d="180180" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="v0" width="320" height="240" bandwidth="250000"/> <Representation id="v1" width="640" height="480" bandwidth="500000"/> <Representation id="v2" width="960" height="720" bandwidth="1000000"/> </AdaptationSet> <!-- Audio --> <AdaptationSet mimeType="audio/mp4" codecs="mp4a.40" lang="en" segmentAlignment="0" startWithSAP="1"> <SegmentTemplate timescale="48000" initialization="audio/en/init.mp4a" media="audio/en/$Number$.mp4a" startNumber="1"> <SegmentTimeline> <S t="0" d="96000" r="432"/> </SegmentTimeline> </SegmentTemplate> <Representation id="a0" bandwidth="64000"/> </AdaptationSet> </Period> </MPD>

The CMMF configuration information manifest corresponding to the MPEG-DASH manifest is shown in table E.4 is provided in table E.5. Its construction is based on the MPEG-DASH MPD shown in table E.5 and indicated within the applicationResourceLocators parameter. It defines two applicationResourceConfigurations. The first applies to all video and audio segments listed in the MPD and identified by the URL sub-paths video/ and audio/ except the initialization segments identified by the URL sub-paths init.mp4v and init.mp4a. The second applies to just the initialization segments.

Upon receipt of a request to obtain a media resource listed within the MPD belonging to the first application resource configuration, the CMMF Receiver creates two requests (since two service locations are provided) for the CMMF Transport Objects corresponding to the media resource. It replaces the scheme and host name of the requested media resource with the baseURL and inserts a CMMF bitstream variant tag (e.g., cmmf-a or cmmf-b) within the URL. As an example:

A request for the video segment with URL https://example.com/video/250000/1.mp4v is made by the CMMF-Aware Client.

Since cmmfCodingConfiguration is defined and assuming the CMMF receiver supports both cmmfCodeType 0 and cmmfProfile org.etsi.cmmf.a, the CMMF Receiver creates two requests for the CMMF Transport Objects associated with the video segment. These include: https://cdn-a.example.com/video/250000/cmmf-a/1.mp4v and https://cdn-b.example.com/video/250000/cmmf-b/1.mp4v.

The CMMF Receiver requests the two CMMF Transport Objects simultaneously using the above URLs and recovers the video segment once it can fully decode the received CMMF Transport Object information.

Upon receipt of a request to obtain a media resource listed within the MPD belonging to the second application resource configuration, the CMMF Receiver creates a single request to obtain the source object since cmmfCodingConfiguration is not defined. The CMMF Receiver selects a service location to obtain the source object and retrieves it. For example, the initialization segment with URL https://example.com/video/250000/init.mp4v is downloaded using the URL https://cdn-a.example.com/video/250000/1.mp4v.

In both cases, the CMMF Receiver provides the requested media resource to the CMMF-Aware Application once the source object has been recovered.

Table E.5: Example HTTP-based CMMF configuration information manifest

{ "version": 0, "ttl": 600, "applicationResourceLocators": [ { "locator": "https://www.example.com/manifest.mpd", "contentType": "application/dash+xml" "isManifest": true, }, ], "applicationResourceConfigurations": [ { "admissionRules": { "applicationResourcePathPattern": ["video/", "audio/"], }, "ExclusionRules": { "applicationResourcePathPattern": ["init.mp4v", "init.mp4a"], }, "serviceLocations": [ { "baseUrl": "https://cdn-a.example.com", "requestPathMaps": [ { "requestPathPattern": "\/(?=[^\/]*$)", "mappedPath": "/cmmf-a/" } ] }, { "baseUrl": "https://cdn-b.example.com", "requestPathMaps": [ { "requestPathPattern": "\/(?=[^\/]*$)", "mappedPath": "/cmmf-b/" } ] } ], "cmmfCodingConfiguration": { "cmmfVersion": 0, "cmmfCodeType": 0, "cmmfProfile": "org.esti.cmmf.a", } }, { "serviceLocations": [ { "baseUrl": "https://cdn-a.example.com", }, { "baseUrl": "https://cdn-b.example.com", } ] } ] }

Annex F (normative):
CMMF bitstream profiles and content preparation templates

# F.0	Introduction

This annex defines CMMF bitstream profiles and content preparation templates for use when constructing CMMF bitstreams.

# F.1	CMMF bitstream profile A

## F.1.1	Introduction

While multiple valid CMMF bitstream realizations are possible which conform to clauses 4 and 5, a CMMF bitstream construction profile is specified in this clause that provides a minimum set of capabilities and functionalities required for media delivery using CMMF over HTTP/TCP or other reliable transport protocols. This profile is identified by the profile identifier org.etsi.cmmf.a.

## F.1.2	CMMF bitstream layout

A single media resource (e.g., a CMAF segment, a media track, or a media file) is first prepared for encoding and encapsulation into a CMMF bitstream/Transport Object. Each media resource shall be treated as a single block for the purposes of encoding it using a supported CMMF code type. The media resource may first be partitioned into block_num_symbols equal-sized source symbols, where padding is added if necessary. These source symbols are encoded using a supported CMMF code type where at least block_num_symbols encoded symbols are generated. Each encoded symbol is packaged within an individual CMMF packet() subatom belonging to one CMMF bitstream/Transport Object (i.e., no two CMMF bitstreams/Transport Objects should contain an identical CMMF packet() subatom).

CMMF bitstreams/Transport Objects shall be constructed using the CMMF subatoms and structures as shown in figure F.1 below. Each CMMF bitstream/Transport Object shall begin with the CMMF synchronization (sync()) structure, followed by one bitstream header subatom containing the bitstream_header() structure, one block header subatom containing the block_header() structure, and one or more packet subatoms each containing the packet() structure.

Figure F.1: CMMF bitstream layout for CMMF bitstream profile A

## F.1.3	Required fields and their values

### F.1.3.1	Introduction

Several realizations of a CMMF bitstream/Transport Object conforming CMMF bitstream profile A are possible depending on the code type used, how coefficient information is communicated within the CMMF bitstream/Transport Object, etc. This clause provides the required values for the fields of each CMMF structure contained within a CMMF bitstream/Transport Object for each of these realizations.

### F.1.3.2	Required Fields and values applying to all CMMF bitstreams

Requirements for the construction of the various structures shown in figure F.1 are provided within this clause. These requirements shall apply to all CMMF bitstreams conforming to profile A regardless of the CMMF code type in use. Fields not specified in table F.1 below are either determined during CMMF bitstream preparation, based on the settings of other fields, dependent on the CMMF code type in use, or optional.

Table F.1: Required CMMF bitstream fields and values

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| sync() | b_content_encode_uuid | TRUE | b(1) |
|  | content_encode_uuid | See clause 6.1.3.4 | v(128) |
| subatom() | b_bitstream_id_present | TRUE | b(1) |
|  | bitstream_id | See clause 6.1.2.4 | v(16) |
| bitstream_header() | b_content_source_split | FALSE | b(1) |
|  | b_rfc5052 | FALSE | b(1) |
|  | block_count_minus1 | 0 | u(8) |
|  | b_content_block_separate_sources | FALSE | b(1) |
|  | b_profile_information_present | TRUE | b(1) |
|  | profile_type_size | 15 | u(4) |
|  | profile_type | org.etsi.cmmf.a | v(120) |
|  | profile_description | 0 | v(32) |
| block_header() | block_index | 0 | u(8) |
|  | block_size | content_source_size | u(32) |
|  | b_block_max_symbol_index_present | FALSE | b(1) |
|  | b_block_content_source_index_present | FALSE | b(1) |
|  | b_block_composite_sources | FALSE | b(1) |
|  | b_addl_block_coding_info_present | FALSE | b(1) |
|  | block_mask: Bit 0 | 1 | v(1) |
|  | block_mask: Bit 2 | 0 | v(1) |
| packet_header() | packet_mask: Bit 2 | 0 | v(1) |
|  | packet_mask: Bit 6 | 0 | v(1) |
| packet() | coded_symbol | See clause 6.1.6.2 |  |

The bitstream_id in the subatom() structure may either be randomly generated at the time the CMMF bitstream/Transport Object is created or deterministically chosen prior to creating the CMMF bitstream/Transport Object.

The block_size field in the block_header() structure shall be equal to the content_source_size field contained within CMMF bitstream’s bitstream_header() structure.

### F.1.3.3	code_type 0 (xCD-1) required fields and values

#### F.1.3.3.1	Introduction

When code_type 0 (xCD-1) is used to encode content, the code_type field in the bitstream_header() structure shall be set to 0.

The required fields to support communication of the necessary coefficient information must also be set. These fields depend on the method in which the coefficient information is communicated within the bitstream. A compliant bitstream shall support only one of the following methods:

Block PRNG

Packet PRNG

Coefficient vector

The required fields and their values for each method used to communicate coefficient information is provided within this clause. Additionally, the required fields and their values when the encoded symbol is systematic and/or coefficient information encryption is used are also provided.

#### F.1.3.3.2	Block PRNG coefficient information

The required fields and their settings for a bitstream using a block PRNG to communicate code_type 0 coefficient information is provided in table F.2.

Table F.2: Required CMMF bitstream fields and values for code_type 0 using a block PRNG

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
| block_header() | block_mask: Bit 4 | 1 | v(1) |
|  | prng_parameters() | See table F.5 |  |
| packet_header() | b_systematic_symbol | FALSE | b(1) |
|  | packet_mask: Bit 0 | 1 | v(1) |
|  | packet_mask: Bit 3 | 0 | v(1) |
|  | packet_mask: Bit 4 | 0 | v(1) |
|  | packet_symbol_index | See clause 6.1.7.4 |  |

#### F.1.3.3.3	Packet PRNG coefficient information

The required fields and their settings for a bitstream using a packet PRNG to communicate code_type 0 coefficient information is provided in table F.3. The fields for the packet_header() structure show the required values when coded_symbol is not systematic (see clause F.1.3.3.5 for the required fields and their settings when coded_symbol is systematic).

Table F.3: Required CMMF bitstream fields and values for code_type 0 using a packet PRNG

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
| block_header() | block_mask: Bit 4 | 0 | v(1) |
| packet_header() | b_systematic_symbol | FALSE | b(1) |
|  | packet_mask: Bit 0 | 0 | v(1) |
|  | packet_mask: Bit 3 | 1 | v(1) |
|  | packet_mask: Bit 4 | 0 | v(1) |
|  | prng_parameters() | See table F.5 |  |

#### F.1.3.3.3	Coefficient vectors coefficient information

The required fields and their settings for a bitstream using a coefficient vector to communicate code_type 0 coefficient information is provided in table F.4. The fields for the packet_header() structure show the required values when coded_symbol is not systematic or it is not signalled as systematic using the b_systematic_symbol field (i.e., the coefficient vector is used to signal coded_symbol is systematic). See clause F.1.3.3.5 for the required fields and their settings when coded_symbol is systematic and the b_systematic_symbol field is set.

Table F.4: Required CMMF bitstream fields and values for code_type 0 using coefficient vectors

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
| block_header() | block_mask: Bit 4 | 0 | v(1) |
| packet_header() | b_systematic_symbol | FALSE | b(1) |
|  | packet_mask: Bit 0 | 0 | v(1) |
|  | packet_mask: Bit 3 | 0 | v(1) |
|  | packet_mask: Bit 4 | 1 | v(1) |
|  | coefficient_vector() | See clause 6.1.15 |  |

#### F.1.3.3.4	PRNG parameters

A CMMF bitstream using either a block PRNG or packet PRNG to communicate coefficient information shall contain the prng_parameters() structure in either the block_header() or packet_header() structures respectively. The required fields and their values when this structure is present are provided in figure F.5.

Table F.5: Required CMMF bitstream fields and values when prng_parameters() is present

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| prng_parameters() | prng_type | 000b | v(3) |
|  | prng_seed_bits_code | See table 50 | v(2) |
|  | prng_seed | See clause 6.1.5.25.3 |  |
|  | prng_density_percentage | See clause 6.1.5.25.4 | v(7) |

#### F.1.3.3.5	Systematic coded_symbol’s

A CMMF bitstream may only indicate a coded_symbol is systematic using the b_systematic_symbol field within the packet_header() structure when coefficient information is communicated using a packet PRNG or coefficient vector. In such cases and where coded_symbol is systematic, the fields and corresponding values provided in table F.6 shall be present in the CMMF bitstream.

Table F.6: Required CMMF bitstream fields when coded_symbol is systematic

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
| block_header() | block_mask: Bit 4 | 0 | v(1) |
| packet_header() | b_systematic_symbol | TRUE | b(1) |
|  | packet_mask: Bit 0 | 1 | v(1) |
|  | packet_mask: Bit 3 | 0 | v(1) |
|  | packet_mask: Bit 4 | 0 | v(1) |
|  | packet_symbol_index | See clause 6.1.7.4 |  |

#### F.1.3.3.6	Encrypted coefficient information

Coefficient information may be encrypted within the CMMF bitstream when code_type 0 (xCD-1) is used regardless of the method used to communicate the coefficient information.

When coefficient information is not encrypted, the fields and corresponding values in the CMMF bitstream shall be set according to table F.7.

Table F.7: Field values when encrypted coefficient information is not used

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
|  | b_block_cc_encrypted | FALSE | b(1) |
| block_header() | block_mask: Bit 3 | 0 | v(1) |
| packet_header() | packet_mask: Bit 1 | 0 | v(1) |

When coefficient information is encrypted, the fields and values present in the CMMF bitstream are provided below.

##### F.1.3.3.6.1	Encrypted block PRNG coefficient information

Table F.8 provides the required fields and values when code_type is 0 (xCD-1), coefficient information is communicated using a block PRNG, and the coefficient information is encrypted.

Table F.8: Field values when block PRNG coefficient information is encrypted

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
|  | b_block_cc_encrypted | TRUE | b(1) |
|  | bitstream_encryption_key_id | See clause 6.1.4.15 |  |
| block_header() | block_mask: Bit 3 | 1 | v(1) |
|  | block_mask: Bit 4 | 1 | v(1) |
|  | block_cc_encryption_algorithm | 000b | v(3) |
|  | block_cc_encryption_mode | See clause 6.1.5.24.5 |  |
|  | block_cce_key_size_exp | See clause 6.1.5.24.6 |  |
|  | block_cce_key | See clause 6.1.5.24.6 |  |
|  | b_addl_block_cce_params_present | See clause 6.1.5.24.7 |  |
|  | addl_cce_parameters() | See clause 6.1.5.24.8 |  |
|  | prng_parameters() | See table F.5 |  |
| packet_header() | b_systematic_symbol | FALSE | b(1) |
|  | packet_mask: Bit 0 | 1 | v(1) |
|  | packet_mask: Bit 1 | 0 | v(1) |
|  | packet_mask: Bit 3 | 0 | v(1) |
|  | packet_mask: Bit 4 | 0 | v(1) |
|  | packet_symbol_index | See clause 6.1.7.4 |  |

NOTE: 	All fields contained between /* block encryption info start */ and /* block encryption info end */ within the block_header() structure are encrypted using the bitstream encryption key with ID bitstream_encryption_key_id and the prng_parameters() structure contained within the block_header() structure is encrypted using the block_cce_key and addl_cce_parameters().

##### F.1.3.3.6.2	Encrypted packet PRNG coefficient information

Table F.9 provides the required fields and values when code_type is 0 (xCD-1), coefficient information is communicated using a packet PRNG, the coded_symbol is not systematic, and the coefficient information is encrypted.

Table F.9: Field values when packet PRNG coefficient information is encrypted

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
|  | b_block_cc_encrypted | TRUE | b(1) |
|  | bitstream_encryption_key_id | See clause 6.1.4.15 |  |
| block_header() | block_mask: Bit 3 | 1 | v(1) |
|  | block_mask: Bit 4 | 0 | v(1) |
|  | block_cc_encryption_algorithm | 000b | v(3) |
|  | block_cc_encryption_mode | See clause 6.1.5.24.5 |  |
|  | block_cce_key_size_exp | See clause 6.1.5.24.6 |  |
|  | block_cce_key | See clause 6.1.5.24.6 |  |
|  | b_addl_block_cce_params_present | See clause 6.1.5.24.7 |  |
|  | addl_cce_parameters() | See clause 6.1.5.24.8 |  |
| packet_header() | b_systematic_symbol | FALSE | b(1) |
|  | packet_mask: Bit 1 | 1 | v(1) |
|  | packet_mask: Bit 3 | 1 | v(1) |
|  | packet_mask: Bit 4 | 0 | v(1) |
|  | b_addl_packet_cce_params_present | See clause 6.1.7.5.2 |  |
|  | addl_cce_parameters() | See clause 6.1.5.24.8 |  |
|  | prng_parameters() | See table F.5 |  |

NOTE: 	All fields contained between /* block encryption info start */ and /* block encryption info end */ within the block_header() structure are encrypted using the bitstream encryption key with ID bitstream_encryption_key_id. The prng_parameters() structure is encrypted using the block_cce_key and addl_cce_parameters()contained within the packet_header() structure.

##### F.1.3.3.6.3	Encrypted coefficient vector information

Table F.10 provides the required fields and values when code_type is 0 (xCD-1), coefficient information is communicated using a coefficient vector, the coded_symbol is not systematic, and the coefficient information is encrypted.

Table F.10: Field values when packet PRNG coefficient information is encrypted

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
|  | b_block_cc_encrypted | TRUE | b(1) |
|  | bitstream_encryption_key_id | See clause 6.1.4.15 |  |
| block_header() | block_mask: Bit 3 | 1 | v(1) |
|  | block_mask: Bit 4 | 0 | v(1) |
|  | block_cc_encryption_algorithm | 000b | v(3) |
|  | block_cc_encryption_mode | See clause 6.1.5.24.5 |  |
|  | block_cce_key_size_exp | See clause 6.1.5.24.6 |  |
|  | block_cce_key | See clause 6.1.5.24.6 |  |
|  | b_addl_block_cce_params_present | See clause 6.1.5.24.7 |  |
|  | addl_cce_parameters() | See clause 6.1.5.24.8 |  |
| packet_header() | b_systematic_symbol | FALSE | b(1) |
|  | packet_mask: Bit 1 | 1 | v(1) |
|  | packet_mask: Bit 3 | 0 | v(1) |
|  | packet_mask: Bit 4 | 1 | v(1) |
|  | b_addl_packet_cce_params_present | See clause 6.1.7.5.2 |  |
|  | addl_cce_parameters() | See clause 6.1.5.24.8 |  |
|  | coefficient_vector() | See clause 6.1.15 |  |

NOTE: 	All fields contained between /* block encryption info start */ and /* block encryption info end */ within the block_header() structure are encrypted using the bitstream encryption key with ID bitstream_encryption_key_id. The coefficient_vector() structure is encrypted using the block_cce_key and addl_cce_parameters() contained within the packet_header() structure.

##### F.1.3.3.6.4	Encrypted systematic coded_symbol

Table F.11 provides the required fields and values when code_type is 0 (xCD-1), the coded_symbol is systematic, coefficient information is communicated within the CMMF bitstream/Transport Object using either a packet PRNG or coefficient vector, and coefficient information is encrypted.

Table F.11: Field values when packet PRNG coefficient information is encrypted

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 0 | u(4) |
|  | b_block_cc_encrypted | TRUE | b(1) |
|  | bitstream_encryption_key_id | See clause 6.1.4.15 |  |
| block_header() | block_mask: Bit 3 | 1 | v(1) |
|  | block_mask: Bit 4 | 0 | v(1) |
|  | block_cc_encryption_algorithm | 000b | v(3) |
|  | block_cc_encryption_mode | See clause 6.1.5.24.5 |  |
|  | block_cce_key_size_exp | See clause 6.1.5.24.6 |  |
|  | block_cce_key | See clause 6.1.5.24.6 |  |
|  | b_addl_block_cce_params_present | See clause 6.1.5.24.7 |  |
|  | addl_cce_parameters() | See clause 6.1.5.24.8 |  |
| packet_header() | b_systematic_symbol | TRUE | b(1) |
|  | packet_mask: Bit 0 | 1 | v(1) |
|  | packet_mask: Bit 1 | 1 | v(1) |
|  | packet_mask: Bit 3 | 0 | v(1) |
|  | packet_mask: Bit 4 | 0 | v(1) |
|  | b_addl_packet_cce_params_present | See clause 6.1.7.5.2 |  |
|  | addl_cce_parameters() | See clause 6.1.5.24.8 |  |

NOTE: 	All fields contained between /* block encryption info start */ and /* block encryption info end */ within the block_header() structure are encrypted using the bitstream encryption key with ID bitstream_encryption_key_id. The systematic coded_symbol within the packet() structure is encrypted using the block_cce_key and addl_cce_parameters() contained within the packet_header() structure.

### F.1.3.4	code_type 1 (Raptor), 5 (Reed-Solomon), and 6 (RaptorQ) required fields and values

When code_type 1 (Raptor), 5 (Reed-Solomon), or 6 (RaptorQ) is used to encode content, the code_type field in the bitstream_header() structure shall be set to the appropriate code type as specified in clause 6.1.4.5.

Fields used to support communication of coefficient information shall not be set. This includes bit 4 in the block_mask as specified in clause 5.1.5.19, and bits 3 and 4 in the packet_mask as specified in clause 6.1.7.2. Instead, the encoding symbol ID (ESI) produced by the relevant encoder shall be communicated using the packet_symbol_index field within the packet_header() structure as specified in clause 6.1.7.4. Furthermore, fields used to signal a coded_symbol is systematic (i.e., b_systematic_symbol as specified in clause 6.1.7.1) and coefficient information is encrypted (i.e., b_block_cc_encrypted as specified in clause 6.1.4.13, bit 3 in the block_mask as specified in clause 6.1.5.19, and bit 1 in the packet_mask as specified in clause 6.1.7.2) shall not be set.

NOTE:	Signalling of systematic symbols by each of these code types is accomplished via the ESI (i.e., coded_symbol with packet_symbol_index 0 to num_block_symbols – 1 are systematic).

Table F.12 provides the required fields and values when code_type is 1 (Raptor), 5 (Reed-Solomon), or 6 (RaptorQ).

Table F.12: Required CMMF bitstream fields and values for code_type 1, 5, and 6

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | code_type | 1, 5, or 6 | u(4) |
|  | b_block_cc_encrypted | FALSE | b(1) |
| block_header() | block_mask: Bit 1 | 0 | v(1) |
|  | block_mask: Bit 3 | 0 | v(1) |
|  | block_mask: Bit 4 | 0 | v(1) |
| packet_header() | b_systematic_symbol | FALSE | b(1) |
|  | packet_mask: Bit 0 | 1 | v(1) |
|  | packet_mask: Bit 1 | 0 | v(1) |
|  | packet_mask: Bit 3 | 0 | v(1) |
|  | packet_mask: Bit 4 | 0 | v(1) |
|  | packet_symbol_index | See clause 6.1.7.4 |  |

### F.1.3.5	Block integrity required fields and values

In cases where verification that the received and decoded data in the block matches that of the original source is needed, block integrity information may be added to the CMMF bitstream. The presence of this information is independent of the code type in use or whether coefficient information is encrypted. Table F.13 provides the required fields and values when block integrity information is included within the CMMF bitstream.

Table F.13: Required CMMF bitstream fields and values for block integrity verification

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| block_header() | block_mask: Bit 5 | 1 | v(1) |
|  | fb_integrity() |  |  |
| fb_integrity() | fb_hash_type | See table 66 | v(2) |
|  | fb_hash_algorithm | See table 67 | v(3) |
|  | fb_hash_size | See table 69 | v(4) |
|  | b_fb_integrity_ext | FALSE | b(1) |
|  | fb_hash | See clause 6.1.13.5 |  |

The hash fb_hash shall be a hash of the data comprising the original source object.

### F.1.3.6	Packet integrity required fields and values

In cases where verification that the received data in a packet matches the data contained in the CMMF bitstream at the time it was created is needed, packet integrity information may be added to the CMMF bitstream. This presence of this information is independent of the code type in use or whether coefficient information is encrypted. Table F.14 provides the required fields and values when packet integrity information is included within the CMMF bitstream.

Table F.14: Required CMMF bitstream fields and values for packet integrity verification

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| packet_header() | packet_mask: Bit 5 | 1 | v(1) |
|  | packet_integrity() |  |  |
| packet_integrity() | packet_hash_algorithm | See table 70 | v(3) |
|  | packet_hash_size | See table 72 | v(3) |
|  | b_packet_integrity_ext | FALSE | b(1) |
|  | packet_hash | See clause 6.1.14.4 |  |

The hash packet_hash shall be a hash of the data comprising the coded_symbol within the packet() structure at the time the CMMF bitstream was created.

## F.1.4	Object based live or streaming sources

A CMMF Encoder may be configured to encode content from a live or streaming source where the full source object may not be available to the CMMF Encoder before it starts creating a CMMF bitstream/Transport Object. In such cases, the required fields and values within the CMMF bitstream to indicate that this is the case are provided in table F.15.

Table F.15: Required CMMF bitstream fields and values for live or streaming sources

| CMMF structure | CMMF field name | Value | Bit field encoding |
| --- | --- | --- | --- |
| bitstream_header() | content_source_size | 0 | u(64) |
|  | content_source_type | 011b | v(3) |
| block_header() | block_size | 0 | u(32) |
|  | block_mask: Bit 0 | 1 | v(1) |
|  | b_sufficient_symbols_present | TRUE | b(1) |

Ingested source data is immediately partitioned into source symbols with a predetermined block_symbol_size. These source symbols may be immediately encoded within a CMMF bitstream/Transport Object as systematic coded_symbols when code_type is 0 (xCD-1) or encoded symbols ESI 0 through num_block_symbols – 1 when code_type is 1 (Raptor), 5 (Reed-Solomon), or 6 (RaptorQ). Furthermore, the CMMF Encoder may add packet() subatoms to a CMMF bitstream/Transport Object with a packet_symbol_index that follows a predetermined pattern. For example, a CMMF bitstream/Transport Object may be constructed where the source symbol indexes associated with the packet() subatoms added to the CMMF bitstream/Transport Object are 0, 2, 4, etc.

Once the CMMF Encoder has obtained the complete object from the source, it may then create and append either non-systematic packet() subatoms when code_type is 0 (xCD-1) or encoded symbols with ESI greater than num_block_symbols – 1 when code_type is 1 (Raptor), 5 (Reed-Solomon), or 6 (RaptorQ). Finally, the total number of packet() subatoms contained within the CMMF bitstream/Transport Object shall be at least block_num_symbols.

## F.1.5	CMMF bitstream/Transport Object deployment

A one-to-many mapping between a media resource/source object (e.g., CMAF segment) and the CMMF bitstreams/Transport Objects created to deliver that resource should exist where a CMMF Receiver or an application provider may fully extract the media resource from:

-	Any individual CMMF bitstream/Transport Object when it is obtained in its entirety from one service location if b_sufficient_symbols_present within the CMMF bitstreams block_header() subatom is set, or

-	Some combination of CMMF bitstreams/Transport Objects when partial bitstreams/Transport Objects are obtained from multiple service locations.

Each individual CMMF bitstream/Transport Object constructed to deliver a media resource/source object should consist of a unique encoding (i.e., representation or variation) of the media resource/source object such that a CMMF Receiver or application provider may jointly decode multiple partially received CMMF bitstreams/Transport Objects allowing for the recovery of the media resource/source object in its entirety.

A one-to-one mapping should exist between each service location and the CMMF bitstream representation hosted at or contributed to that service location.

Configuration Information required by a CMMF Receiver as specified in clause E.2 to configure and stream media using CMMF should be communicated to the CMMF Receiver/Encoder prior to the start of a delivery session.

# F.2	Content preparation template for CMMF bitstream profile A encoding

## F.2.1	Introduction

This clause defines a content preparation template for use when configuring a CMMF Encoder that generates CMMF bitstreams/Transport Objects conforming to the profile specified in clause F.1 with profile identifier org.etsi.cmmf.a.

The content preparation template specified within this clause is intended to be a used to configure only the process for encoding CMMF bitstreams and/or creating CMMF Transport Objects. Therefore, the following apply:

A process external to the CMMF Encoder receives and processes requests (e.g., from a CMMF Receiver) for a CMMF bitstream/Transport Object.

A process external to the CMMF Encoder identifies and obtains the content required by the CMMF Encoder to create the requested CMMF bitstream/Transport Object. This content may consist of the original media resource/source object, a pre-existing CMMF bitstream/Transport Object, multiple pre-existing CMMF bitstreams/Transport Objects, etc.

Upon reception of a request to create a CMMF bitstream/Transport Object and the content to be encoded or re-encoded, the CMMF Encoder will create a new CMMF bitstream/Transport Object and make it available to a process external to the CMMF Encoder for further processing (e.g., delivery, caching, etc.).

It is further assumed that the following information is made available to the CMMF Encoder when it is triggered to produce a new CMMF bitstream/Transport Object:

The CMMF bitstream content preparation template as specified in clause F.2.2.

In the case where the request URL is used to determine the construction of the CMMF bitstream/Transport Object (e.g., see clause E.2.2), the request URL containing the required representation/variant information.

The media resource (e.g., source object, pre-existing CMMF bitstream/Transport Object, etc.) to be used when creating the new CMMF bitstream/Transport Object.

## F.2.2	Semantics of the CMMF bitstream profile A content preparation template

The semantics of the CMMF bitstream profile A content preparation template are provided in table F.16.

Table F.16: Semantics of the CMMF bitstream profile A content preparation template

| Property Name |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- |
| cmmfConfigurationInformationLocator |  |  |  | O | A URL to the CMMF configuration information as specified in clause E.2.3. This URL may be used by the CMMF Encoder to obtain the CMMF configuration information which can then be used to determine which CMMF bitstream variant to create when a request URL is received for a CMMF Transport Object. |
| bitstreamVersion |  |  |  | M | The version of CMMF bitstream syntax as specified in clause 6.1.3.2. The present document specifies the CMMF bitstream syntax version 0. |
| bitstreamId |  |  |  | O | A bitstream identifier as specified in clause 6.1.2.4. If present, all CMMF bitstreams shall have a bitstream_id of bitstreamId. If absent, the bitstream_id value is randomly generated. |
| contentSourceType |  |  |  | OD Default "000b" | A content source type as specified in clause 6.1.4.2. If absent, the default value shall be 000b (Not indicated) |
| codeType |  |  |  | M | A CMMF code type as specified in clause 6.1.4.5. |
| profile |  |  |  | M | A CMMF profile as specified in clause 6.1.4.11. The value shall be org.etsi.cmmf.a. |
| profileDescription |  |  |  | OD Default 0 | A profile description as specified in clause 6.1.4.12. If absent, the profile description is 0. |
| blockSizeToBlockNumSymbolsMaps |  |  |  | O | An array of block size map objects that provides a mapping between the source object size and block_num_symbols as specified in clause 6.1.5.5. If present, the map is used to determine block_num_symbols. If absent, the CMMF Encoder shall determine block_num_symbols. |
|  | blockSizeMin |  |  | M | The minimum source object size in bytes for which the block size map object applies. |
|  | blockSizeMax |  |  | M | The maximum source object size in bytes for which the block size map object applies. |
|  | blockNumSymbols |  |  | M | The block_num_symbols used if the source object size is within the range [blockSizeMin, blockSizeMax]. |
| symbolsPresentTarget |  |  |  | O | The percentage of packet() subatoms present within the bitstream in the range (0, 100]. If present, the number of packet() subatoms in the bitstream shall be ceil(block_num_symbols x symbolsPresentTarget / 100). If absent, the number of packet() subatoms in the bitstream shall be equal to block_num_symbols. |
| liveStreamingBlockSymbolSize |  |  |  | OD Default 10 000 | The block_symbol_size in bytes to use if contentSourceType is 011b and the source object size is unknown (i.e., content_source_size as specified in clause 6.1.4.1 is 0). This parameter is ignored otherwise. If absent, contentSourceType is 011b, and the source object size is unknown, the default block_symbol_size is 10 000 bytes. |
| symbolArrangementMap |  |  |  | O | An array of symbol arrangement map objects that provides the order of packet() subatoms within the bitstream based on the packet_symbol_index as specified in clause 6.1.7.4. This parameter is only valid when: - codeType is 0, xcdInformation.coefficientType is packet_prng, and signalSystematicSymbols is True; or - codeType is 1 (Raptor), 5 (Reed-Solomon), or 6 (RaptorQ). If present, the cmmfConfigurationInformationLocator shall also be present. If absent, the order of packet() subatoms is determined by the CMMF Encoder. |
|  | variantId |  |  | M | The CMMF bitstream variant for which the symbol order applies. The variantId may be provided to the CMMF Encoder within the received request URL (e.g., as a subpath) or via other means. |
|  | symbolOrder |  |  | M | An array of symbol indexes (i.e., packet_symbol_index) providing the order in which packet() subatoms shall appear within the CMMF bitstream. When codeType is 0, xcdInformation.coefficientType is packet_prng, and signalSystematicSymbols is True, the symbol indexes refer to the source symbol indexes of the systematically encoded symbols. When codeType is 1 (Raptor), 5 (Reed-Solomon), or 6 (RaptorQ), the symbol indexes refer to the encoding symbol ID (ESI). |
| xcdInformation |  |  |  | O | An object that provides additional configuration information. These parameters are only valid when codeType is 0 and shall be ignored otherwise. |
|  | blockFieldSizeExponent |  |  | OD Default 2 | The field size exponent defining the Galois field used during encoding as specified in clause 6.1.5.23. If absent, the field size exponent shall be 2. |
|  | coefficientType |  |  | OD Default vector | The coding coefficient type to use. Possible values include block_prng (see clause F.1.3.3.2), packet_prng (see clause F.1.3.3.3), or vector (see clause F.1.3.3.4). If absent, coefficient information will be communicated within a coefficient vector. |
|  | signalSystematicSymbols |  |  | OD Default False | Determines if systematically encoded symbols are signaled within the bitstream. This parameter is only valid when coefficientType is packet_prng or vector, and shall be ignored otherwise. If absent, systematic encoded symbols are not signaled within the bitstream (i.e., b_systematic_symbol = False) |
|  | coefficientInformation |  |  | O | An object providing coding coefficient information for the generation/population of coefficient information within the CMMF bitstream. If absent, the default values for each property specified within this object are used. |
|  |  | prngParameters |  | O | An object providing the parameters used to produce a pseudo-random number describing the coding coefficients. These parameters only apply when coefficientType is either block_prng or packet_prng, and shall be ignored otherwise. If present, the values defined within this object are used. If absent, the default values are used. |
|  |  |  | blockPrngSeed | O | A seed value used to initialize the PRNG for generating coding coefficients for the block as specified in clause 6.1.5.25.3. This parameter is only valid when coefficientType is block_prng and shall be ignored otherwise. If absent, a random seed value is used. |
|  |  |  | packetPrngSeeds | O | An array of seed values used to initialize the PRNG for generating coding coefficients for each packet() as specified in clause 6.1.5.25.3. The array must be greater than block_num_symbols long. This parameter is only valid when coefficientType is packet_prng and shall be ignored otherwise. If absent, random seed values are used. |
|  |  |  | prngDensityPercentage | O | The PRNG density as specified in clause 6.1.5.25.4. If absent, the density is maximized for the defined blockFieldSizeExponent. |
|  |  | coefficientVectorMaps |  | O | An array of coefficient vector maps providing pre-determined coding coefficient vectors for the CMMF bitstream variant indicated. The parameter is only valid when coefficientType is vector and ignored otherwise. If coefficientType is vector and the absent or a coefficient vector map is not defined for a particular variantId, numSymbols, and fieldSizeExp, the CMMF Encoder will randomly generate all necessary coding coefficient vectors. |
|  |  |  | variantId | M | The CMMF bitstream variant the symbol order applies. The variantId may be provided to the CMMF Encoder within the received request URL (e.g., as a subpath) or via other means. |
|  |  |  | numSymbols | M | A number used to identify the coding coefficient vectors to use (e.g., the coefficient vector map is selected if numSymbols is equal to block_num_symbols). |
|  |  |  | fieldSizeExp | M | The field size exponent defining the Galois field for the provided coding coefficient vectors as specified in clause 6.1.5.23. |
|  |  |  | coefficientVectors | M | An array of coding coefficient vectors. Each element is a bit string of length numSymbols x fieldSizeExp as specified in clause 6.1.15.1. |
|  | encryptionParameters |  |  | O | An object providing CMMF bitstream encryption parameters used to encrypt coding coefficient information. If present, coding coefficient information is encrypted. If absent, coding coefficient information is not encrypted. |
|  |  | encryptionServiceEndpoint |  | M | A URL to the endpoint where a CMMF Encoder and Decoder can obtain a bitstream encryption key. |
|  |  | bitstreamEncryptionKeyId |  | O | A bitstream encryption key identifier as specified in clause 6.1.4.15. If absent, bitstream_encryption_key_id shall be obtained from encryptionServiceEndpoint. |
|  |  | blockEncryptionMode |  | OD Default 0100b | The block encryption mode as specified in clause 6.1.5.24.5. If absent, block_cc_encryption_mode is 0100b (Counter). |
|  |  | blockKeySizeExp |  | OD Default 8 | The key size exponent used to encrypt coding coefficient information as specified in clause 6.1.5.24.6. If absent, block_cce_key_size_exp is 8. |
| blockIntegrity |  |  |  | O | An object that provides parameters when block integrity checks are to be used as specified in clause 6.1.13. If present, block integrity is used within the bitstream. If absent, block integrity is not used. |
|  | hashType |  |  | OD Default 00b | The hash type used on the source object as specified in clause 6.1.13.1. If absent, hash type shall be 00b (i.e., the hash is calculated on the full source object or block of source data) |
|  | hashAlgorithm |  |  | OD Default 000b | The algorithm used to calculate the hash as specified in clause 6.1.13.2. If absent, the hash algorithm shall be 000b (i.e., SHA-1 [9]). |
| packetIntegrityHashAlgorithm |  |  |  | O | The algorithm used to verify the integrity of data in packet() subatoms as specified in clause 6.1.14. If present, packet integrity is used. If absent, packet integrity is not used. |
| Key M=mandatory, O=optional, OD=optional with default value, CM=conditionally mandatory |  |  |  |  |  |

## F.2.3	Schema of the CMMF bitstream profile A content preparation template

Table F.17 specifies the JSON schema for the CMMF bitstream profile A content preparation template.

Table F.17: JSON schema of the CMMF bitstream profile A content preparation template

{ "type": "object", "properties": { "cmmfConfigurationInformationLocator": { "description": [ "A URL to the CMMF configuration information associated with the CMMF Transport Objects" "the CMMF Encoder will generate." ], "type": "string" }, "bitstreamVersion": { "description": "The version of CMMF bitstream syntax to use. The value shall be 0", "type": "integer", "const": 0 }, "bitstreamId": { "description": "A bitstream identifier that will be added to each CMMF bitstream.", "type": "string" }, "contentSourceType": { "description": "A content source type as specified in clause 6.1.4.2.", "type": "string", "enum": ["000b", "001b", "011b"] }, "codeType": { "description": "A CMMF code type as specified in clause 6.1.4.5.", "type": "integer", "enum": [0, 1, 5, 6] }, "profile": { "description": [ "A CMMF profile as specified in clause 6.1.4.11. The value shall be org.etsi.cmmf.a." ], "type": "string" }, "profileDescription": { "description": [ "A CMMF profile description as in clause 6.1.4.12." ], "type": "string" }, "blockSizeToBlockNumSymbolsMaps": { "description": "Array of block size map objects.", "type": "array", "items": { "type": "object", "properties": { "blockSizeMin": { "description": "The minimum source object size in bytes.", "type": "integer" }, "blockSizeMax": { "description": "The maximum source object size in bytes.", "type": "integer" }, "blockNumSymbols": { "description": [ "The block_num_symbols used if the source object size is within", "the range [blockSizeMin, blockSizeMax]." ], "type": "integer" } }, "required": ["blockSizeMin", "blockSizeMax", "blockNumSymbols"] } }, "symbolsPresentTarget": { "description": "The percentage of packet() subatoms present within the bitstream.", "type": "integer", "minimum": 1, "maximum": 100 }, "liveStreamingBlockSymbolSize": { "description": [ "The block_symbol_size in bytes if contentSourceType is 011b and the source object", "size is unknown." ], "type": "integer" }, "symbolArrangementMap": { "description": "Array of symbol arrangement objects.", "type": "array", "items": { "type": "object", "properties": { "variantId": { "description": "The CMMF bitstream variant for which the symbol order applies.", "type": "string" }, "symbolOrder": { "description": [ "An array of symbol indexes describing the order in which packet() subatoms", "shall appear within the CMMF bitstream." ], "type": "array", "items": { "type": "integer" } } }, "required": ["variantId", "symbolOrder"] } }, "xcdInformation": { "description": [ "An object providing additional configuration information when codeType is 0." ], "type": "object", "properties": { "blockFieldSizeExponent": { "description": "The field size exponent defining the Galois field used.", "type": "integer", "enum": [2, 4, 8, 16] }, "coefficientType": { "description": "The coding coefficient type to use.", "type": "string", "enum": ["block_prng", "packet_prng", "vector"] }, "signalSystematicSymbols": { "description": [ "Systematically encoded symbols are signalled within the bitstream." ], "type": "boolean" }, "coefficientInformation": { "description": [ "An object providing coding coefficient information." ], "type": "object", "properties": { "prngParameters": { "description": [ "An object providing the parameters used to produce a pseudo-random number", "describing the coding coefficients." ], "type": "object", "properties": { "blockPrngSeed": { "description": [ "A seed value to initialize the PRNG for generating coding coefficients", "when coefficientType is block_prng." ], "type": "integer" }, "packetPrngSeeds": { "description": [ "An array of seed values used to initialize the PRNG for generating coding", "coefficients for each packet() when coefficientType is packet_prng." ], "type": "array", "items": { "type": "integer" } }, "prngDensityPercentage": { "description": "The PRNG density.", "type": "number", "minimum": 0.0, "maximum": 1.0, } } }, "coefficientVectorMaps": { "description": [ "An array of coefficient vector maps providing pre-determined coding", "coefficient vectors." ], "type": "array", "items": { "type": "object", "properties": { "variantId": { "description": [ "The CMMF bitstream variant for which the symbol order applies." ], "type": "string" }, "numSymbols": { "description": [ "The number of symbols for which the coding coefficient vectors are", "defined." ], "type": "integer", }, "fieldSizeExp": { "description": [ "The field size exponent defining the Galois field for which the", "coding coefficient is defined." ], "type": "integer", "enum": [2, 4, 8, 16] }, "coefficientVectors": { "description": [ "An array of coding coefficient vectors." ], "type": "array", "items": { "type": "string", } } }, "required": ["variantId", "numSymbols", "fieldSizeExp", "coefficientVectors"] } } } }, "encryptionParameters": { "description": [ "An object providing CMMF bitstream encryption parameters used to encrypt" "systematically encoded symbols and coding coefficient information." ], "type": "object", "properties": { "encryptionServiceEndpoint": { "description": [ "A URL to the endpoint where a CMMF Encoder or Decoder can obtain a", "bitstream encryption key." ], "type": "string" }, "bitstreamEncryptionKeyId": { "description": [ "A bitstream encryption key identifier.", ], "type": "string" }, "blockEncryptionMode": { "description": [ "The block encryption mode.", ], "type": "string", "enum": ["0000b", "0001b", "0010b", "0011b", "0100b", "0101b"] }, "blockKeySizeExp": { "description": [ "The key size exponent used.", ], "type": "integer" } }, "required": ["encryptionServiceEndpoint"] } } }, "blockIntegrity": { "description": [ "An object providing parameters when block integrity checks are to be used." ], "type": "object", "properties": { "hashType": { "description": "The hash type used on the source object.", "type": "string", "enum": ["00b", "01b"] }, "hashAlgorithm": { "description": "The algorithm used to calculate the hash.", "type": "string", "enum": ["000b", "001b", "010b"] } } }, "packetIntegrityHashAlgorithm": { "description": [ "The algorithm used to verify the integrity of data provided in packet() subatoms." ], "type": "string", "enum": ["000b", "001b", "010b"] } }, "required": ["bitstreamVersion", "codeType", "profile"] }

## F.2.4	IANA registration of CMMF bitstream profile A content preparation template

This clause provides the formal MIME type registration for the CMMF bitstream profile A content preparation template compliant with the JSON schema specified in clause F.2.3. It is referenced from the registry at http://www.iana.org/assignments/media-types.

The MIME Type and Subtype are defined in table F.18.

Table F.18: MIME media type registration of CMMF bitstream profile A content preparation template

| Parameter | Value |
| --- | --- |
| MIME media type name | application |
| MIME subtype name | cmmf-cpt-a+json |
| Required parameters | None |
| Optional parameters | None |
| Encoding considerations | This is a JSON document, and the encoding considerations are the same as for media type application/json defined in IETF RFC 8259. |
| Security considerations | This media format is a JSON document and inherits the security considerations provided in RFC 8259. The format does not incorporate any active or executable content. However, other forms of material from outside resources can be referenced by the format, and this material can contain active or executable content. Such material is expected to be identified by its own MIME type, and the security considerations of that format should be taken into account. This format is highly susceptible to manipulation or spoofing for attacks designed to inject fake information. Both integrity protection and source authentication are recommended to prevent misleading of the recipient. |
| Interoperability considerations | The present document defines a platform-independent expression of a document, and it is intended that wide interoperability can be achieved. |
| Published specification | ETSI TS 103 973 (the present document) |
| Applications which use this media type | Various |
| Additional information | File extension(s): json Intended usage: COMMON |
| Other information/general comment | None |
| Person & email address to contact for further information | Jason Cloud (jason.cloud@dolby.com) |
| Restrictions on usage | None |
| Author/Change controller | ETSI |

## F.2.5	Example use of CMMF bitstream profile A content preparation template

Table F.19 provides an example of the CMMF bitstream profile A content preparation template. It can be used to create the CMMF bitstream subatoms provided in clause C.1 that are relevant to CMMF bitstream profile A. The example makes the following assumptions:

A difference between the subatom constructions in clause C.1 and the example is the inclusion of the profile information within the bitstream_header() subatom.

The CMMF configuration information manifest (see clause E.2.4) describing the CMMF bitstream variants is available at https://www.example.com/cmmf_ci_manifest.mpd.

Table F.19: Example CMMF bitstream profile A content preparation template

{ "cmmfConfigurationInformationLocator": "https://www.example.com/cmmf_ci_manifest.mpd", "bitstreamVersion": 0, "contentSourceType": "001b", "codeType": 0, "profile": "org.etsi.cmmf.a." "blockSizeToBlockNumSymbolsMaps": [ { "blockSizeMin": 0, "blockSizeMax": 128, "blockNumSymbols": 1 }, { "blockSizeMin": 129, "blockSizeMax": 1048576, "blockNumSymbols": 4 }, { "blockSizeMin": 1048577, "blockSizeMax": 2097152, "blockNumSymbols": 8 }, { "blockSizeMin": 2097153, "blockSizeMax": 4194304, "blockNumSymbols": 16 } ], "xcdInformation": { "signalSystematicSymbols": "true", "coefficientInformation": { "coefficientVectorMaps": [ { "variantId": "/cmmf-a/", "numSymbols": 4, "fieldSizeExp": 2, "coefficientVectors": ["1000b", "0010b", "0101b", "1111b"] }, { "variantId": "/cmmf-b/", "numSymbols": 4, "fieldSizeExp": 2, "coefficientVectors": ["0100b", "0001b", "1010b", "1111b"] }, { "variantId": "/cmmf-a/", "numSymbols": 8, "fieldSizeExp": 2, "coefficientVectors": [ "10000000b", "00100000b", "00001000b", "00000010b", "00000001b", "00000100b", "00010000b", "11111111b" ] }, { "variantId": "/cmmf-b/", "numSymbols": 8, "fieldSizeExp": 2, "coefficientVectors": [ "01000000b", "00010000b", "00000100b", "00000001b", "00000010b", "00001000b", "00100000b", "11111111b" ] }, { "variantId": "/cmmf-a/", "numSymbols": 16, "fieldSizeExp": 2, "coefficientVectors": […] }, { "variantId": "/cmmf-b/", "numSymbols": 16, "fieldSizeExp": 2, "coefficientVectors": […] } ] } } }