3GPP SA4-MBS SWG AH	S4aI250154

# Paris, FR, 03rd–05th September 2025	is revision of S4aI250110

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0002 | rev | 0 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Update of requirements of existing Key Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Link Rel-20 SA1 requirements with existing key issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | TR 22.883 requirements related to existing key issues added in their potential requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-20 SA1 requirements not addressed. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | 3.1, 6.1, 6.2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AS Energy Information: energy-related information collected and exposed by Application Servers.

carbon emission: quantity of equivalent carbon dioxide emitted (e.g. kg of CO2 equivalent).

carbon intensity: Global greenhouse gases emitted per unit of generated electricity, measured in grams of CO₂ equivalent per watt–hour intended for conversion to carbon emissions as defined in TS 22.261 [5] and TS 23.700-66 [20].

energy availability: the remaining amount of energy (e.g. in kWh) locally available for consumption. For devices, network elements and functions, energy availability may be limited and/or intermittent, in particular when relying on batteries and/or renewable energy sources (e.g. off-grid base stations, satellites etc) or during power grid heavy load or disruptions.

energy capacity: the maximum amount of energy (e.g. in kWh) that can be locally available for consumption (either locally produced and/or stored) by a device or a network element or function.

energy consumption: integral of power consumption over time, as defined in TS 28.310 [2].

energy credit: a quantity associated with the subscriber that can be used for credit control by the 5G System per TS 22.261 [5].

energy efficiency (EE): ratio between performance and energy consumption.

NOTE 1:	The performance may be measured based on, e.g., data volume, latency, number of active users, etc. as defined in TS 28.310 [2].

energy-related characteristics: information which characterize the energy to power the operator’s network in terms of energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions.

NOTE 2:	Which energy-related characteristics are relevant depends on the scenario.

energy supply mix: the combination of the various energy sources (i.e. renewable and not) used to meet energy needs of a device or a network element or function.

maximum energy consumption: a policy establishing an upper bound on the quantity of energy consumption [2] by the 5G system in a specific period of time, or space, e.g. energy consumption inside a given service area as defined in ETSI ES 202 706-1 [4].

maximum energy credit limit: a policy establishing an upper bound on the aggregate quantity of energy consumption by the 5G System to provide services to a specific subscriber, e.g. in kilowatt hours [56].

Network Energy Information: NF Energy Information and/or AS Energy Information.

NF Energy Information: energy-related information collected from Network Functions and exposed by the EIF.

renewable energy: energy from renewable sources, defined as energy from renewable non-fossil sources, namely wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases [3].

UE Energy Information: energy-related information collected by the UE.

2nd Change

## 6.1	Key Issue #1: Energy-related Information exposure

### 6.1.1 	Description

As described in the use cases summarised in clause 5.1, energy-related information needs to be collected, estimated, and exposed by the 5G System. This information is not only necessary for internal network optimisation, but it will also benefit service adaptation by third-party Application Service Providers, empower mobile subscribers and enable energy consumption (and thus carbon emission) attribution. With the consent of Mobile Network Operators, it is relevant to expose energy-related information (e.g., energy consumption information, energy efficiency information, renewable energy and carbon emission information) to authorised consumers. TR 23.700-66 [20] study and identify potential enhancements to the 5G System (e.g. including network energy related information exposure, enhancement for subscription and policy control to enable energy efficiency as service criteria) to improve energy efficiency and to support energy saving in the network, taking the Energy requirements in TS 22.261 [5] into consideration. The purpose of this Key Issue is to extend this work, to study and identify potential enhancements to the energy-related information exposure for media architectures, applications and services (e.g., taking into consideration the 5G Media Streaming System according to TS 26.501 [23], 5G Multicast–Broadcast User Services according to TS 26.502 [58], the Real-time Media Communication according to TS 26.506 [59], Split rendering Media session Enabler according to TS 26.565 [65], etc.) including UE-related energy information exposure. As explained in clause 4.2.2, a UE data collection and reporting architecture already exist in the 5G System. But energy-related information is missing.

In this context, the subsequent analysis by this Key Issue should consider:

1.	How should UE energy-related information be reported by a UE to the 5G System?

2.	Which reference points should be used to report UE energy-related information to the Data Collection AF?

3.	Would it be useful to expose energy-related information of the network to the Media Session Handler to help it optimize its media session in an energy-efficient way?

4. 	How to allow a UE to report its energy-related information without exposing its energy consumption rate?

This analysis will need to take into consideration existing work done in 3GPP but also other market trends. Application Service Providers are often reluctant to deploy solutions specific to mobile networks. Proposing technologies already supported in their services or technologies that are agnostic to the network, or which operate passively without the active involvement or knowledge of applications, are more likely adopted.

### 6.1.2	Potential requirements

Clause 6.4 in TR 22.882 [56] and clause 6.1.1 in TR 22.883 [85] contain the consolidated requirements extracted from use cases, related to information exposure related with this Key Issue:

[22.882-CPR 6.1-7] Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy consumption information associated with a set of application servers. [22.882-CPR 6.3-2] Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party, independently from NG-RAN deployment scenarios. [22.882-CPR 6.3-4] Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, through same update rate e.g. hourly or daily. [22.882-CPR 6.4-1] Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption for serving this 3rd party. [22.882-CPR 6.4-2] Subject to operator’s policy, the 5G system shall support a means to expose energy consumption to authorized third parties for services, including energy consumption information related to the condition of energy credit limit (e.g. when the energy consumption is reaching the energy credit limit). [22.882-CPR 6.4-3] Subject to operator policy, the 5G system shall provide means for the trusted 3rd party, to configure which network performance statistic information (e.g. the data rate, packet delay and packet loss) for the communication service provided to the 3rd party, needs to be exposed along with the information on energy consumption for serving this 3rd party. [22.882-CPR 6.4-4] Based on operator policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information and prediction on energy consumption of the 5G network per application service to the 3rd party. [22.882-CPR 6.4-5] Subject to operator’s policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time. [22.883-CPR 6.1.1-1] Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall be able to collect, and expose to authorized 3rd parties, the carbon equivalent emissions resulting from the use of the communication service, related to one or more specific UEs of home network subscribers (e.g. fleet of vehicles, IoT devices, company phones etc), over a specific time period (e.g. month etc). [22.883-CPR 6.1.1-2] Subject to operator’s policy and user consent, the 5G network shall be able to expose to an authorized 3rd party information about network energy consumption and carbon equivalent emissions for a specific service data flow. [22.883-CPR 6.1.1-3] Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to expose to an authorized 3rd party the current and expected energy related characteristics of the network resources used in serving a UE. [22.883-CPR 6.1.1-4] Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to provide UEs with information related to the current energy-related characteristics of their serving network.

Additional potential requirements identified from related work:

[PR 1-1] Where possible, it is required to reuse existing mechanisms (e.g., UE data collection and reporting architecture as in TS 26.531 [21]) and information for exposure of energy-related information. [PR 1-2] It is required to reuse commonly supported client data reporting formats for energy-related information exposure when possible. [PR 1-3] It is required that the 5GMS Client is able to obtain energy-related information from the UE, allowing it to optimise the media delivery sessions it is handling in an energy-efficient manner.

3rd Change

## 6.2	Key Issue #2: Energy-related monitoring and measurement

### 6.2.1 	Description

The measurement of energy consumption by media services is an essential pre-requisite to facilitate efficient energy use and energy saving. The environmental impact of the consumption of media services depends various system actors: equipment manufacturers, networks, datacentre operators, service providers but also users through their usage behaviour.

Requirements of monitoring and measurement relating to energy consumption and efficiency have already been defined by 3GPP in TR 22.882 [56] but those requirements are only related to energy consumption monitoring of the 5G network or 5G System. TR 23.700-66 [20] studies and identifies potential enhancements to the 5G System (e.g. including network energy related information exposure, and enhancement of subscription and policy control to enable energy efficiency as a service criterion) with the dual aims of (i) improving energy efficiency and (ii) supporting energy saving in the network, taking the EnergyServ requirements in TS 22.261 [5] into consideration.

In addition to energy consumption, TR 22.883 [85] identifies requirements concerning the energy-related characteristics needed to reduce the environmental impact of media services, of which the energy supply mix, the carbon footprint, the energy capacity and the availability conditions are relevant.

The purpose of this Key Issue is to extend the above work, by studying and identifying potential enhancements to energy-related monitoring and measurement aspects (including UE-related energy information measurement and monitoring) in the context of the following media delivery architectures, applications and services namely the 5G Media Streaming System according to TS 26.501 [23], 5G Multicast–Broadcast User Services according to TS 26.502 [58], the Real-time Media Communication according to TS 26.506 [59], Split rendering Media session Enabler according to TS 26.565 [65], etc.

The most efficient way to address energy saving in this context is to consider the complete end-to-end media delivery chain, seeking to understand how decisions made at one end of the chain impact energy-related characteristics at the other end of the workflow. As explained in clause 4.2.2, UE data collection has been defined to monitor QoE in the 5G system. But energy-related information is missing.

NOTE 1:	It is presumed that the user has granted consent for its UE data to be collected, reported and subsequently exposed by means outside the scope of the study or following normative work.

NOTE 2:	The collection, reporting and exposure of location-based UE data is expected to comply with regional regulatory requirements and may be further limited by MNO policy.

In this context, the subsequent analysis by this Key Issue will consider the following questions:

1.	Which UE energy-related information will be collected to measure, correlate, and optimize energy usage across the entire streaming distribution chain?

2.	Can existing methods be leveraged to measure/monitor the identified UE energy-related information?

3.	Which UE entity is appropriate to measure this UE energy-related information?

This issue is even more important for advanced media services such as XR services, Split Rendered media services, etc. which are expected to incur substantial energy consumption both at the device and network levels, presenting significant challenges for operators and service providers.

For instance, when a Mobile Network Operator (MNO) deploys a communication service to fulfil application service requirements, such as those of a gaming application, it is crucial for the customer – whether an Application Service Provider (ASP) or an industry vertical – to ensure that the application service reduces energy consumption for both end users and the data network. Failure to do so could necessitate short-notice application layer adaptations within the Application Service Provider's domain. This may result in adjustments being made to service levels in response to anticipated high energy consumption in specific service areas or during peak hours that adversely affect the Quality of Experience for service users.

In a practical scenario, an ASP intends to deploy a gaming service within a designated service area, served by an MNO's 5G network. Various service levels may exist, each associated with specific Key Performance Indicators (KPIs), such as automation levels or video quality targets. To ensure energy efficiency the ASP monitors the energy efficiency of its application service across specified service levels. Monitoring of application energy consumption may occur periodically or may be event-triggered, depending on the ASP's requirements, which are typically outlined in the Service Level Agreement (SLA). The MNO and ASP may agree on a certain energy efficiency target for the application service and optionally for given service levels.

This analysis will need to take into consideration existing work done in 3GPP but also other market trends. Application Service Providers are often reluctant to deploy solutions specific to mobile networks. Proposing technologies already supported in their services or technologies that are agnostic to the network, or which operate passively without the active involvement or knowledge of applications, are more likely adopted.

### 6.2.2	Potential requirements

Clause 6.3 in TR 22.882 [56] and clause 6.1.4 in TR 22.883 [85] contain the consolidated requirements extracted from use cases, related to monitoring and measurement related with this Key Issue:

[22.882-CPR 6.3-1] Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity. NOTE 1: Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. [22.882-CPR 6.3-2] Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party, independently from NG-RAN deployment scenarios. NOTE 2: The granularity of energy consumption measurement could vary according to different situations, for example, when several services share a same network slice, etc. NOTE 3: The energy consumption information can be related to the network resources of network slice, NPNs, etc. [22.882-CPR 6.3-3] Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, through same update rate e.g. hourly or daily. NOTE 4: The network performance statistic information could be the data rate, packet delay and packet loss, etc. [22.883-CPR 6.1.4-1] Subject to user consent, operator policy and regulatory requirements, the 5G network shall be able to assist an authorized 3rd party to identify a set of target UEs for whom to adjust the provided application service, considering criteria such as the current and future (e.g. predicted) energy-related characteristics of their serving network.

Based on TS 28.310 [2] and other related work in 3GPP, the following potential requirements need to be considered in this Key Issue:

[PR 2-1] Where possible, it is required to reuse existing mechanisms (e.g., UE data collection and reporting architecture as in TS 26.531 [21]) and information for measurement and monitoring of energy-related information. [PR 2-2] It is required to reuse commonly supported client data metrics for energy-related information measurement and monitoring when possible. [PR 2-3] Based on the collected and/or predicted energy efficiency information exposed from the 5G System, the Application Service Provider is able to adapt the application service parameters based on the 5GS feedback. Monitoring of application energy consumption may occur periodically or may be event-triggered, depending on the ASP's requirements, which are typically outlined in the Service Level Agreement (SLA). In order to perform such operations, the Application Service Provider requires relevant APIs to be exposed by participating entities related to the usage of media applications in order to expose Energy efficiency related information. For example, in the case of the 5G Media Streaming collaboration described in clause A.2 of TS 26.501 [23], this depends on the relationship between 5GMS Application Provider, 5GMSd-Aware Application, 5GMSd Client, 5GMSd AF, and 5GMSd AS.

End of changes