3GPP SA4-MBS SWG AH	S4aI250157

# Paris, FR, 03rd–05th September 2025	is revision of S4aI250113

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0005 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] New KI on Client Energy management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a Key issue on Client Energy management identified during phase 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 6.6 adding a new Key issue on Client energy management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Objectives of the Study Item not fulfilled. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change
(All new text)

## 6.6		Key Issue #6: Client-driven management of media delivery service energy optimisation

### 6.6.1	Description

Sharing energy-related characteristics from the network with the 5GMS Client or RTC Client enables not only the Application Service Provider but also its customers to leverage this information to optimize energy consumption. The use cases summarized in clause 5.1, particularly use case 5.14 of TR 22.882 [56] which focuses on reducing the greenhouse gas footprint of application services, and use case 5.9 of TR 22.883 [85], concerning notifications to UEs about network energy-related characteristics, underscore the critical role of the client’s actions based on this shared information. These use cases highlight how energy-aware decisions can significantly improve the efficiency of media content consumption, contributing to sustainability goals by minimizing the environmental impact of network operations. This transparency empowers the client to make informed adjustments to media streaming quality, timing, or other parameters in response to the network’s energy conditions. Overall, sharing this information fosters a more collaborative and energy-conscious ecosystem, benefiting the Mobile Network Operator, the Application Service Provider and end users by ensuring service delivery that is efficient, reliable, and environmentally responsible.

In this context, the subsequent analysis by this Key Issue will consider the following question:

1. 	How might the 5GMS Client or RTC Client modify media delivery sessions in response to energy-related characteristics shared by the network via the Energy Information AF instantiated in the 5GMS AF or RTC AF (as applicable) and/or the Energy Information Collector?

### 6.6.2	Potential requirements

Clause 6.1 in TR 22.882 [56] and clause 6.1.4 in TR 22.883 [85] contain the consolidated requirements extracted from use cases concerning energy-related characteristics as service criteria pertaining to this Key Issue:

[22.882-CPR 6.1-7] Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy consumption information associated with a set of application servers. [22.883-CPR 6.1.4-1] Subject to user consent, operator policy and regulatory requirements, the 5G network shall be able to assist an authorized 3rd party to identify a set of target UEs for whom to adjust the provided application service, considering criteria such as the current and future (e.g. predicted) energy-related characteristics of their serving network.

End of changes