3GPP SA4-MBS SWG post 133-e AH	S4aI250180

# Online, 23rd October 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0007 | rev | 0 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI5 Media Application Server Energy management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.12 adding a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KI. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 7.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[87]	Accubattery: https://play.google.com/store/apps/details?id=com.digibites.accubattery

[88]	ETSI TS 103 998: DASH-IF: Content Steering for DASH v1.1.1

2nd Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #10 |  |  |  |  |  | X |
| #11 |  |  |  |  | X |  |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

3rd change
(All new text)

## 7.12	Solution #11: Steering server driven selection of delivery paths based on energy characteristics

### 7.12.1	Key Issue mapping

This solution candidate addresses Key Issue #5 (Media Application Server Energy management) described in clause 6.5.

### 7.12.2	Functional description

#### 7.12.2.1	Introduction

The main streaming video formats such as MPEG-DASH and HLS have been updated to include additional information in their manifests to specify a server (called a "steering server") capable of dynamically selecting the delivery path to use. Currently, this selection is primarily based on Quality of Service (QoS) criteria.

In parallel, networks have collectors of energy-related characteristics, including information that characterizes the network's power supply in terms of energy consumption, energy source, carbon footprint, capacity, and availability. These collectors provide insights into environmental impact at different granularities (per user, per service, etc.). This is notably the case with the Energy Information Function (EIF) defined in the 5G architecture.

However, there is currently no method for using these network energy characteristics to select the video streaming delivery path with the lowest environmental impact.

The proposed solution consists of defining a process that allows steering servers to make this choice based on environmental impact, through the sharing of network energy-related characteristics.

This solution leverages existing interfaces enabling communication between application providers and 5G networks, particularly for exchanging Quality of Experience data. These interfaces are enhanced with network energy-related characteristics, allowing the application provider to use this information on its steering servers.

### 7.12.2	Reference architecture for steering server driven selection of delivery paths based on energy characteristics

Figure 7.12.2.2-11 depicts a reference architecture that realises this candidate solution in the generalised Media Delivery architecture defined in TS 26.501 [23] and TS 26.506 [59].

Figure 7.12.2.2-1: Reference architecture for steering server driven selection of delivery paths based on energy characteristics

The core principle of this architecture revolves around a component called the Energy Information AF, which is instantiated within the Media AF. Its responsibilities vary depending on its current provisioning state, as determined by the 5GMS AF (which includes the Energy Information Exposure Specification provisioned by 5GMS Application Providers - see clause 7.8). These responsibilities may include:

-	Subscribes to and consumes NF Energy Information from the Energy Information Function as defined in TS 23.501 [72]) with required granularities (UE, PDU session and/or QoS flow) in accordance with the parameters of the Energy Information Exposure Specifications.

-	Subscribes to and consumes AS Energy Information from the Application Server in accordance with the parameters of the Energy Information Exposure Specification.

Unlike other solutions described in this document that utilize the Energy Information AF (e.g. Solution #5 or #10), this approach does not require an Energy Information Collector within the device. The primary goal is to enable selection of the delivery path based on energy characteristics that align with user preferences or service provider criteria. Regardless of the chosen delivery path, the energy characteristics are assumed to remain unchanged on the UE. Therefore, the path selection relies solely on the energy information provided by the network through the Energy Information AF.

### 7.12.3	Procedures

Figure 7.12.3-1 below details the different steps for steering server driven selection of delivery paths based on energy characteristics outlined in clause 7.12.2.

Figure 7.12.3-1: Procedures for steering server driven selection of delivery paths based on energy characteristics

The procedure consists in mixing the typical content steering procedure, for example the one described for DASH in [88], and the provision of the Energy Information AF allowing the Media AF to play the role of steering server enable to decide the delivery path based on energy characteristics of each path.

A first step is required to provision Energy Information Collection:

1.	The Application Service Provider provisions the Media AF via reference point M1, including an Energy Information exposure configuration intended for the Energy Information AF instantiated in the Media AF.

2.	The Energy Information AF subscribes to receive NF Energy Information reporting from the Energy Information Function via reference point E12, if relevant.

Then, occurs the provision of steering information:

3.	The Application Service Provider communicates to the Media AF via reference point M1, the Steering Information including the different ways to retrieve the media.

Followed by typical media playback:

4.	The Media-Aware Application triggers the Media Client to start media playback. The Media Player Entry is provided to the Media Client.

5. and 6. 	When the Media-Aware Application has received only a reference to the Service Access Information, the Media Session Handler interacts with the Media AF to acquire the whole Service Access Information.

7.	In parallel, the Media Player is invoked to start media access and playback.

After this initialisation phase, reporting and selection of the delivery path can be done:

8.	The Energy Information Function may submit an NF Energy Information report to the Energy Information AF via reference point E12. The NF Energy Information report may include energy consumption information of different granularities, e.g. UE, PDU Session and/or QoS Flow, as described in clause 5.51.2.3 of TS 23.501 [72].

9.	The Application Service Provider may also submit steering information updated to the Media AF

10.	The Energy Information AF process energy characteristics received for each delivery path and select the path having the characteristics wanted.

11.	The Media Session Handler request steering information to the Media AF.

12.	The Media AF deliver the steering information to the Media Session Handler.

And the content is requested on the selected delivery path: steps 13 to 15 if Media AS1 is selected or steps 16 or 18 if Media AS2:

13. or 15.	The Media Player establishes the transport session for the Media Player Entry with the Media AS.

14. or 17. 	The Media Player establishes the necessary transport sessions for the content with the Media AS.

15. or 18.	The Media Player receives the content from the Media AS

### 7.11.4	Summary

This candidate solution proposes a mechanism allowing application providers to select the video streaming delivery path with the lowest environmental impact, based on:

1.	The collection of network energy-related characteristics by the mobile network operator. This information is provided by the Energy Information Function to a new component called Energy Information AF, included in the Media AF.

2.	Standardised interfaces between the Media AF and the Application Service Provider allowing the Application Service Provider to provision steering information.

3.	The use of this information by the Media AF acting as steering server to dynamically direct users toward the delivery paths with the desired energy characteristics.

This approach optimizes the energy efficiency of multimedia content delivery while maintaining acceptable service quality, thus contributing to reducing the environmental footprint of video streaming services.

The solution applies to any streaming service provider delivering videos via mobile networks and integrates naturally into existing adaptive streaming architectures such as MPEG-DASH and HLS.

The solution has no impact on the UE as steering mechanisms are part of adaptive streaming protocols such as MPEG-DASH and HLS.

End of changes