3GPP TSG-SA4 Meeting #-SA4-e (AH) MBS SWG post 133-e	S4aI250189

# Online, , 23rd Oct 2025 - 23rd Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0028 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD_Ph2] WT#5: 5G System-independent media streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The study item identifies the following work topic - WT#5: 5G System-independent media streaming: 5G Media Streaming features are to a large extent independent of the access network and primarily of the 5G System functionalities; only a subset of the features relies on 5G System functionalities. However, many features of 5G Media Streaming are useful outside of the 5G system and may hence benefit service providers and network operators. A study of opportunities and technical aspects is warranted. The study item identifies the following objectives 1. Document the following additional Key Issues in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture: 2. Study collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. 3. Based on existing architectures, develop one or more deployment architectures that address the key topics and the collaboration models. 4. Map the key topics to basic functions and develop high-level call flows. 5. Identify the issues that need to be resolved. 6. Provide candidate solutions including call flows, protocols and APIs for each of the identified issues. 7. Coordinate work with other 3GPP groups e.g. SA2, SA3, SA5, SA6 and others as needed. 8. Coordinate work with external organizations such as SVTA, CTA WAVE, ISO/IEC JTC29 WG3 (MPEG Systems), 5G-MAG, DVB or IETF, as needed. 9. Identify gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The issues are addressed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The document is submitted for endorsement as basis for future work. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Akamai Blog, “A QUICk Introduction to HTTP/3”, April 2020, https://www.akamai.com/blog/developers/a-quick-introduction-http3

[3]	IETF RFC 9112: "HTTP/1.1", June 2022.

[4]	IETF RFC 9113: "HTTP/2", June 2022.

[5]	IETF RFC 9114: "HTTP/3", June 2022.

[6]	D. Bhat, A. Rizk, and M. Zink, "Not so QUIC: A Performance Study of DASH over QUIC," NOSSDAV'17: Proceedings of the 27th Workshop on Network and Operating Systems Support for Digital Audio and VideoJune 2017 Pages 13–18 https://doi.org/10.1145/3083165.3083175

[7]	AWS: "Achieving Great Video Quality Without Breaking the Bank", Streaming Media June 2019, https://pages.awscloud.com/rs/112-TZM-766/images/GEN elemental-wp-achieving-great-video-quality-without-breaking-the-bank.pdf

[8]	Netflix, "Optimized shot-based encodes: Now Streaming!", Netflix Blog, May 2018, https://netflixtechblog.com/optimized-shot-based-encodes-now-streaming-4b9464204830

[9]	DASH-IF/DVB: "Report on Low-Latency Live Service with DASH", July 2017, available here: https://dash-industry-forum.github.io/docs/Report%20on%20Low%20Latency%20DASH.pdf

[10]	DASH-IF: "IOP Guidelines v5, Low-latency Modes for DASH", available here: https://dash-industry-forum.github.io/docs/CR-Low-Latency-Live-r8.pdf

[11]	ISO/IEC 23009-1: "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

[12]	IETF RFC 8673: "HTTP Random Access and Live Content".

[13]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[14]	3GPP TS 26.238: "Uplink Streaming".

[15]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[16]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[17]		ISO/IEC 13818-1:2019: "Information technology — Generic coding of moving pictures and associated audio information — Part 1: Systems".

[18]	SCTE 35 2020: "Digital Program Insertion Cueing Message", https://www.scte.org/pdf-redirect/?url=https://scte-cms-resource-storage.s3.amazonaws.com/SCTE-35-2020_notice-1609861286512.pdf

[19]	ISO/IEC 23000-19: "Information technology — Multimedia application format (MPEG-A) —Part 19: Common media application format (CMAF) for segmented media".

[20]	ISO/IEC 23009-1:2019/DAMD1: "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats — Amendment 1: CMAF support, events processing model and other extensions".

[21]	VSF TR-06-01:2020, "RIST Simple Profile", https://vsf.tv/download/technical_recommendations/VSF_TR-06-1_2020_06_25.pdf

[22]	VSF TR-06-02: "RIST Main Profile", https://www.videoservicesforum.org/download/technical_recommendations/VSF_TR-06-2_2020_03_24.pdf

[23]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[24]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[25]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[26]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes; Stage 3".

[27]	IETF RFC 6733: "Diameter Base Protocol", October 2012.

[28]	3GPP TS 29.514: "5G System; Policy and Charging Control over Rx reference point; Stage 3".

[29]		IETF RFC 7657: "Differentiated Services (Diffserv) and Real-Time Communication", November 1995.

[30]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP", September 2001.

[31]	IETF RFC 9204: "QPACK: Field Compression for HTTP/3", June 2022.

[32]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport", May 2021.

[33]	IETF RFC 9001: "Using TLS to Secure QUIC", May 2021.

[34]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control", May 2021.

[35]	IETF RFC 5681: "TCP Congestion Control", September 2009.

[36]	M. Kuehlewind and B. Trammell, draft-ietf-quic-manageability-11, "Manageability of the QUIC Transport Protocol", Work in Progress, Internet-Draft, 30 June 2021.

[37]	N. Cardwell et. al. "BBR Updates: Internal Deployment, Code, Draft Plans", 9 March 2021, https://datatracker.ietf.org/meeting/110/materials/slides-110-iccrg-bbr-updates-00.pdf

[38] 	ETSI TS 103 799: "Publicly Available Specification (PAS); DASH-IF Content Protection Information Exchange Format".

[39]	ISO/IEC JTC1/SC29/WG11/N19062 23090-8 FDIS: "MPEG-I: Network-based Media Processing — Network-Based Media Processing Specification".

[40]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".[41]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[42]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[43]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[44]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[45]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[46]	3GPP TS 26.803: "5G Media Streaming (5GMS); Architecture extensions".

[47]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[48]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[49]	Tdoc S4-210723: "Generic architecture for data collection and reporting", submission from BBC, Dolby Laboratories Inc., LM Ericsson and Qualcomm Incorporated to SA4#114-e, May 19-28, 2021.

[50]	Tdoc S2-2103267: "Extension of Naf_EventExposure for observed service experience data collection from UEs", CR from InterDigital to SA2#144e, Apr 12-16, 2021.

[51]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[52]	Tdoc S2-2104496: "Extension of Naf_EventExposure for observed service experience data collection from UEs", CR from Qualcomm Incorporated to SA2#145e, May 17-28, 2021.

[53]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[54]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[55]	3GPP TS 29.554: "Background Data Transfer Policy Control Service; Stage 3".

[56]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[57]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[58]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[59]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[60]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM); Stage 1".

[61]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[62]	3GPP TS 28.542: "Management and orchestration of networks and network slicing; 5G Core Network (5GC) Network Resource Model (NRM); Stage 1".

[63]	3GPP TS 28.543: "Management and orchestration of networks and network slicing; 5G Core Network (5GC) Network Resource Model (NRM); Stage 2 and stage 3".

[64]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[65]	3GPP TS 28.546: "Management and orchestration of networks and network slicing; Fault Supervision (FS); Stage 2 and stage 3".

[66]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[67]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[68]	3GPP TS 23.434: " Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows ".

[69]	3GPP TS 23.700-99: " Study in Network slice capability exposure for application layer enablement (NSCALE)".

[70]	3GPP TS 29.520: " 5G System; Network Data Analytics Services; Stage 3".

[71]	3GPP TR 23.700-40: "Study on enhancement of network slicing; Phase 2".

[72]	3GPP TS 26.531: “Data Collection and Reporting; General Description and Architecture”.

[73]	3GPP TR 26.802: "Multicast Architecture Enhancement for 5G Media Streaming".

[74] 	IETF RFC 822: "STANDARD FOR THE FORMAT OF ARPA INTERNET TEXT MESSAGES", August 13, 1982.

[75]	IETF RFC 1521: "MIME (Multipurpose Internet Mail Extensions)", September 1993.

[76]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers", December 1998.

[77]	IETF RFC 2475: "An Architecture for Differentiated Services", December 1998.

[78]	IETF RFC 3246: "An Expedited Forwarding PHB (Per-Hop Behavior)", March 2002.

[79]	IETF RFC 2597: "Assured Forwarding PHB Group", June 1999.

[80]	S. Hurst, draft-hurst-quic-rtp-tunnelling: "QRT: QUIC RTP Tunnelling", Internet-Draft, Work in Progress.

[81]	J. Ott and M. Engelbart, draft-engelbart-rtp-over-quic: "RTP over QUIC", Internet-Draft, Work in Progress.

[82]	SRT Alliance, “Secure Reliable Transport (SRT) Protocol”, https://github.com/Haivision/srt

[83]	M.P. Sharabayko and M.A. Sharabayko, draft-sharabayko-srt-over-quic-00 ,“Tunnelling SRT over QUIC”, Internet-Draft, Work in Progress, 28 July 2021.

[84]	Robin Marx, Luca Niccolini, Marten Seemann, Lucas Pardue, draft-ietf-quic-qlog-main-schema-09, "Main logging schema for qlog", Internet-Draft, Work in Progress, 8 July 2024.

[85]	Robin Marx, Luca Niccolini, Marten Seemann, Lucas Pardue, draft-ietf-quic-qlog-h3-events-08, "HTTP/3 and QPACK event definitions for qlog", Internet-Draft, Work in Progress, 8 July 2024.

[86]	Robin Marx, Luca Niccolini, Marten Seemann, Lucas Pardue, draft-ietf-quic-qlog-quic-events-08, "QUIC event definitions for qlog", Internet-Draft, Work in Progress, 8 July 2024.

[87]	Roger Pantos and William May, Jr., "HTTP Live Streaming", RFC 8216, August 2017.

[88]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[89]	3GPP TR 26.917: "Multimedia Broadcast Multicast Services (MBMS) and Packet-switchedStreaming Service (PSS) enhancements to support television services".

[90]		"DASH-IF WebRTC-based Streaming", https://dashif.org/news/webrtc/

[91]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework", October 2012.

[92]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage", October 2012.

[93]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[94]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[95]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[96]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[97]	ETSI TS 103 770: "Digital Video Broadcasting (DVB); Service Discovery and Programme Metadata for DVB-I".

[98]	Android Developer Documentation: "Handling Android App Links",
https://developer.android.com/training/app-links

[99]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL".

[100]	ETSI TS 103 769: "Digital Video Broadcasting (DVB); Adaptive media streaming over IP multicast".

[101]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[102]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[103]	IETF RFC 2045: "Multipurpose Internet Mail Extensions Part One: Format of Internet Message Bodies".

[104]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax", January 2005.

[105]	Consumer Technology Association Specification CTA-5004: "Web Application Video Ecosystem – Common Media Client Data", September 2020.

[106]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[107]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[108]	3GPP TS 26.510: "Media delivery: Interactions and APIs for provisioning and media session handling (Release 18)".

[109]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[110]	3GPP TS 29.591: "Network Exposure Function Southbound Services; Stage 3 (Release 18)".

[111]	ETSI TS 103 998: "DASH-IF: Content steering for DASH".

[112]	Draft Text of ISO/IEC FDIS 23009-9: "Information technology - Dynamic adaptive streaming over HTTP (DASH) – Part 9: Redundant Encoding and Packaging for segmented live media (REaP)", ISO/IEC JTC 1/SC 29/WG 3 NO 1165, Jan. 26, 2024. [Online]. Available: https://www.mpeg.org/standards/MPEG-DASH/9/

[113]	Emma Roth, "A popular open-source content delivery network went down for hours", The Verge, Apr. 12, 2024. [Online]. Available: https://www.theverge.com/2024/4/12/24128276/open-source-unpkg-cdn-down (accessed May 9, 2024).

[114]	Sebastian Moss, "Cloudflare recovers from service outage after power failure at core North American data center", Data Center Dynamics, Nov. 3, 2023. [Online]. Available: https://www.datacenterdynamics.com/en/news/cloudflare-recovers-from-service-outage-after-power-failure-at-core-north-american-data-center/ (accessed May 9, 2024).

[115]	Brian Barrett, "How an Obscure Company Took Down Big Chunks of the Internet", Wired, Jun. 8, 2021. [Online]. Available: https://www.wired.com/story/fastly-cdn-internet-outages-2021/ (accessed May 9, 2024).

[116]	Josh Fomon, "CDN Provider Akamai Takes Down Popular Internet Services During Outage", Ookla, Jul. 22, 2021. [Online]. Available: https://www.ookla.com/articles/akamai-outage-july-22-2021 (accessed May 9, 2024).

[117]	Charlotte Trueman, "Cloudflare outage brings hundreds of sites, services temporarily offline", Computer World, Jun. 21, 2022. [Online]. Available: https://www.computerworld.com/article/1627967/cloudflare-outage-brings-hundreds-of-sites-services-temporarily-offline.html (accessed May 9, 2024).

[118]	Jim Salter, "The Internet broke today: Facebook, Verizon, and more see major outages", Ars Technica, Jul. 3, 2019. [Online]. Available: https://arstechnica.com/information-technology/2019/07/facebook-cloudflare-microsoft-and-twitter-suffer-outages/ (accessed May 9, 2024).

[119]	Marc Hoppner, "A content owner, a CDN and a player walk into a bar", (Jan. 6, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=S9EdoQFOQ9I&list
=PLkyaYNWEKcOf98lZxnCcL6y7ZIVU3oSYO&index=12

[120]	Guillaume du Pantavice, "Improving streaming experience with Bayesian optimization, from AB to AZ test", (Dec. 25, 2021). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/
watch?v=t4nRrLygVwo&list=PLkyaYNWEKcOfD1GYFxFbZXDP03XM-cZPg&index=19

[121]	E. Ghabashneh and S. Rao, "Exploring the interplay between CDN caching and video streaming performance", IEEE INFOCOM 2020 – IEEE Conference on Computer Communications, Toronto, ON, Canada, 2020, pp. 516-525.

[122]	K. Vermeulen, L. Salamatian, S. H. Kim, M. Calder, and E. Katz-Bassett, "The central problem with distributed content: common CDN deployments centralize traffic in a risky way", In Proceedings of the 22nd ACM Workshop on Hot Topics in Networks (HotNets ’23). Association for Computing Machinery, New York, NY, USA, 70-78.

[123]	A. Bentaleb, R. Farahani, F. Tashtarian, C. Timmerer, H. Hellwagner, and R. Zimmermann, "Which CDN to Download From? A Client and Server Strategies", (Jan. 6, 2024). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=xCZmCnWgQRE

[124]	Will Law, "Content steering with MPEG DASH", (May 4, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=o9Pa5y-Usxw

[125]	W. Law and Y. Reznik, "DASH content steering at scale", Media Web Symposium (MWS’23), June 2023.

[126]	ETSI TS 103 973: "Coded multisource media format (CMMF) for content distribution and delivery".

[127]	IETF RFC 6330: "RaptorQ forward error correction scheme for object delivery", August 2011.

[128]	IETF RFC 5110: "Reed-Solomon forward error correction (FEC) schemes", April 2009.

[129]	ETSI TS 103 285: "Digital Video Broadcasting (DVB); MPEG-DASH Profile for Transport of ISO BMFF Based DVB Services over IP Based Networks".

[130]	ISO/IEC 26090-8:2020: "Information technology – Coded representation of immersive media, Part 8: Network based media processing".

[131]	IETF RFC 5052: "Forward Error Correction (FEC) Building Block", August 2007.

[132]	IETF RFC 5053: "Raptor Forward Error Correction Scheme for Object Delivery", October 2007.

[133]	IETF RFC 6726: "FLUTE - File Delivery over Unidirectional Transport," November 2012.

[134]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses", March 2022.

[135]	3GPP TS 26.502: "5G multicast-broadcast services; User service architecture (Release 18)".

[136]	Q. De Coninck, O. Bonaventure, C. Huitema, M. Kuehlewind, draft-ietf-quic-multipath-10, "Multipath Extension for QUIC", July 2024.

[137]	IETF RFC 6897: "Multipath TCP (MPTCP) Application Interface Considerations", March 2013

[138]	IETF Draft: "Multipath Extension for QUIC", draft-ietf-quic-multipath-10, July 2024

[139]	Media Source Extension, W3C Working Draft 01, April 2024,
https://www.w3.org/TR/media-source-2/

[140]	Movie labs: "Specification for Enhanced Content Protection", available at: https://movielabs.com/ngvideo/MovieLabs_ECP_v1.4.pdf.

[141]		W3C: "Encrypted Media Extensions", available at: https://www.w3.org/TR/encrypted-media-2/

[142]	Akamai: "Welcome to Adaptive Media Delivery", in Adaptive Media Delivery Implementation Guide, available at: https://learn.akamai.com/en-us/webhelp/adaptive-media-delivery/adaptive-media-delivery-implementation-guide/GUID-3F89E64C-415D-452D-9541-BB650CD783B9.html

[143]	ETSI TS 103 799: "Content Protection Information Exchange Format (CPIX)".

[144]	DASH-IF: "Interoperability Points; Part6-v5.0.0: Content protection and security".

[145]	ETSI TS 104 002: "DASH-IF Forensic A/B Watermarking".

[146]	DASH Industry Forum: "DASH-IF Live Media Ingest Protocol v1.2", February 2024,
https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

[147]	Consumer Technology Association CTA-5005-A: "Web Application Video Ecosystem – DASH-HLS Interoperability Specification",
https://shop.cta.tech/products/web-application-video-ecosystem-dash-hls-interoperability-specification-cta-5005-a

[148]	ISO/IEC 23001-7: "Information technology — MPEG systems technologies Part 7: Common encryption in ISO base media file format files".

[149]	IETF RFC 9330:"Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[150]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)", January 2023.

[151]	IETF RFC 9332: "Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)", January 2023.

[152]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[153]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[154]		Apple Developer Documentation, nw_ip_metadata_get_ecn_flag(_:) | Apple Developer Documentation

[155]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[156]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[157]	Belson, D. and Pardue L.: "Examining HTTP/3 usage one year on", June 2023
https://blog.cloudflare.com/http3-usage-one-year-on

[158]	IETF RFC 5321: “Simple Mail Transfer Protocol”, October 2008.

[159]	HTTP Archive: State of the Web, https://httparchive.org/reports/state-of-the-web#h3

[160]	W3C Working Draft: "WebTransport", May 2024, https://www.w3.org/TR/webtransport

[161]	A. Frindell, E. Kinnear, V. Vasiliev: "WebTransport over HTTP/3", Work In Progress, Internet Draft, draft-ietf-webtrans-http3, https://datatracker.ietf.org/doc/html/draft-ietf-webtrans-http3/

[162]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol", August 2022.

[163]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC", March 2022.

[164]	IETF RFC 9218: "Extensible Prioritization Scheme for HTTP", June 2022.

[165]		ISO/IEC 23009-6: "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 6: DASH with server push and WebSockets".

[166]	L. Curly, K. Pugin, S. Nandakumar, V. Vasiliev, I. Swett: "Media over QUIC Transport", Work In Progress, Internet Draft, draft-ietf-moq-transport, https://datatracker.ietf.org/doc/draft-ietf-moq-transport/07/

[167]		S. Nandakumar, W. Law, M. Zanaty: "Common Catalog Format for moq-transport", Work In Progress, Internet Draft, draft-ietf-moq-catalogformat, https://datatracker.ietf.org/doc/draft-ietf-moq-catalogformat/01/

[168]	Zhang, X., Jin, S., He, Y., Hassan, A., Mao, Z. M., Qian, F., & Zhang, Z. L., "QUIC is not Quick Enough over Fast Internet", in Proceedings of the ACM on Web Conference 2024 (pp. 2713–2722), May 2024.

[169]	Nguyen, M., Nys, P., Pham, S., Silhavy, D., Arbanowski, S., & Steglich, S., "Toward WebTransport Support in HTTP Adaptive Streaming", in Tenth International Conference on Communications and Electronics 2024 (ICCE) (pp. 96–101), IEEE, July 2024.

[170]	Roberto Ramos-Chavez, Espen Braastad, Jamie Fletcher, and Arjen Wagenaar. 2024. "Standards-based OTT A/B Watermarking at Scale", in Proceedings of the 3rd Mile-High Video Conference (MHV '24). Association for Computing Machinery, New York, NY, USA, 94. https://doi.org/10.1145/3638036.3640251

[171]	3GPP TS 26.506: "5g Real-time Media Communication Architecture (Stage 2)".

[172]	Media Source Extension, W3C Editor's Draft 26, October 2023,
https://jyavenard.github.io/media-source/media-source-respec.html#dfn-endstreaming

[173]	5G-MAG Report: "Uplink media delivery: Architectures & Features", available in https://www.5g-mag.com/post/uplink-media-delivery-architectures-features

[174]	3GPP TS 26.113: "Real-Time Media Communication; Protocols and APIs".

[175]	Chaudhary, Sapna, Mukulika Maity, Sandip Chakraborty and Naval Shukla. "A dataset for analyzing streaming media performance over HTTP/3 browsers" in Advances in Neural Information Processing Systems 36, 2024.

[176]	Chaudhary, Sapna, et al. "Managing Connections by QUIC-TCP Racing: A First Look of Streaming Media Performance Over Popular HTTP/3 Browsers", in IEEE Transactions on Network and Service Management, 2024.

[177]	Kazuho Oku, Lucas Pardue, Robin Marx, Luca Niccolini, Marten Seemann, draft-kazuho-httpbis-http3-on-streams-00, "HTTP/3 on Streams", Expired Internet-Draft, 19 August 2024.

[178]	Chellappa, Sindhu, and Radim Bartos. "Is QUIC Quicker with HTTP/3? An Empirical Analysis of Quality of Experience with DASH Video Streaming", in 2022 IEEE International Conference on Advanced Networks and Telecommunications Systems (ANTS). IEEE, 2022.

[179]	Herbots, Joris, et al. "Cross that boundary: Investigating the feasibility of cross-layer information sharing for enhancing ABR decision logic over QUIC", in Proceedings of the 33rd Workshop on Network and Operating System Support for Digital Audio and Video. 2023.

[180]	CTA-5006: "Web Application Video Ecosystem - Common Media Server Data".

[181]	Secure Communication of Network Properties (sconepro), https://datatracker.ietf.org/group/sconepro/about/.

[182]	Standard Communication with Network Elements (scone), https://datatracker.ietf.org/wg/scone/about/

[183]	M. Ihlar, "ABR Video Shaping", IETF BOF March 2024, available here https://datatracker.ietf.org/meeting/119/materials/slides-119-sconepro-how-networks-shape-traffic-02

[184]	"How YouTube coordinates with some MNOs", IETF BOF March 2024, available here https://datatracker.ietf.org/meeting/119/materials/slides-119-sconepro-youtube-plan-aware-streaming-01.

[185]	"Adaptive Bitrate (ABR) Video Shaping", IETF BOF June 2024, available here: https://datatracker.ietf.org/meeting/120/materials/slides-120-sconepro-problem-statement-00.

[186]	"Charter SCONE-PROTOCOL", https://datatracker.ietf.org/meeting/120/materials/slides-120-sconepro-draft-charter-v1-july-20-00

[187]	Unified Streaming: "Introduction to CPIX", Unified Streaming online blog, https://docs.unified-streaming.com/documentation/drm/cpix_intro.html

[188]	castLabs: "FAQ: Digital Rights Management", https://castlabs.com/faq/drm/

## =====  CHANGE =====

## 5.27	5G System-independent media streaming

### 5.27.1	Description

5G Media Streaming features are to a large extent independent of the access network and primarily of the 5G System functionalities; only a subset of the features relies on 5G System functionalities. However, many features of 5G Media Streaming are useful outside of the 5G system and may hence benefit service providers and network operators. A study of opportunities and technical aspects is warranted.

### 5.27.2	Collaboration scenarios

### 5.27.3	Architecture mappings

### 5.27.4	High-level call flows

### 5.27.5	Gap analysis and requirements

### 5.27.6	Candidate solutions

### 5.27.7	Summary and conclusions

## =====  CHANGE =====

## 6.27	5G System-independent media streaming

## =====  CHANGE =====

## 7.4	Recommendations for future work arising from version 20