3GPP TSG-SA4 Meeting #-SA4-e (AH) MBS SWG post 133-e	S4aI250196

# Online, , 23rd Oct 2025 - 23rd Oct 2025									revision of S4aI250193

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.517 | CR | 0036 | rev | 1 | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD_PRO-MED] OpenAPI YAML updates for in-session repair |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED |  |  |  |  |  | Date: |  |  | 2025-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | OpenAPI for In-session repair was not ccomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add OpenAPI YAML for in-session repair |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The code changes associated with this Change Request are available for review at 3GPP Forge: - https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/14 - https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/14/diffs?commit_id=34f2e29b7d8d49db847d1c2667046aa8a4d0887a |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Code changes

The code changes associated with this Change Request are available for review at the following URL on 3GPP Forge:

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/14

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/14/diffs?commit_id=34f2e29b7d8d49db847d1c2667046aa8a4d0887a

The proposed changes are reproduced below for posterity.

## TS26517_MBSUserServiceAnnouncement.yaml

---a/TS26517_MBSUserServiceAnnouncement.yaml
+++b/TS26517_MBSUserServiceAnnouncement.yaml

@@ -2,14 +2,14 @@ openapi: 3.0.0

2	2

3	3		info:

4	4		  title: 'MBS User Service Announcement'

5		-	  version: 2.2.0

5	+	  version: 2.2.1

6	6		  description: |

7	7		    MBS User Service Announcement Element units.

8	8		    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

9	9		    All rights reserved.

10	10

11	11		externalDocs:

12		-	  description: 3GPP TS 26.517 V19.0.0; 5G Multicast-Broadcast User Services; Protocols and Formats

12	+	  description: 3GPP TS 26.517 V19.1.0; 5G Multicast-Broadcast User Services; Protocols and Formats

13	13		  url: http://www.3gpp.org/ftp/Specs/archive/26_series/26.517/

14	14		paths:

15	15		  /user-service-descriptions:

@@ -170,6 +170,8 @@ components:

170	170		          items:

171	171		            $ref: '#/components/schemas/ApplicationServiceDescription'

172	172		          minItems: 1

173	+	        inSessionObjectRepairParameters:

174	+	          $ref: '#/components/schemas/ObjectRepairParameters'

173	175		        postSessionObjectRepairParameters:

174	176		          $ref: '#/components/schemas/ObjectRepairParameters'

175	177		        availabilityInfos:

@@ -276,6 +278,8 @@ components:

276	278		    ObjectRepairParameters:

277	279		      type: object

278	280		      properties:

281	+	        repairLimitPercentage:

282	+	          $ref: 'TS26510_CommonData.yaml#/components/schemas/Percentage'

279	283		        backOffParameters:

280	284		          $ref: '#/components/schemas/BackOffParameters'

281	285		        objectDistributionBaseLocator:

## =====  CHANGE =====

## A.2.1	MBS User Service Announcement schema

Below is the schema specifying the format of User Service Descriptions instance documents using a JSON-based representation. The schema filename is "TS26517_MBSUserServiceAnnouncement.yaml".

Documents following this schema shall be identified with the MIME media type application/3gpp-mbs-user-service-descriptions+json as registered in clause E.2.1 including the following parameters:

-	The profiles parameter (see clause E.2.2) shall include a fully-qualified term identifier from the controlled vocabulary specified in clause C.2.

-	The version parameter (see clause E.2.3) shall contain the value "Rel19" to indicate conformance with this version of the present document.

openapi: 3.0.0 info: title: 'MBS User Service Announcement' version: 2.2.1 description: ¦ MBS User Service Announcement Element units. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. externalDocs: description: 3GPP TS 26.517 V19.1.0; 5G Multicast-Broadcast User Services; Protocols and Formats url: http://www.3gpp.org/ftp/Specs/archive/26_series/26.517/ paths: /user-service-descriptions: get: operationId: discoverUserServiceDescriptions summary: 'Discover User Service Descriptions' description: 'Discover User Service Descriptions that match the supplied query filter(s). At least one filter query parameter must be included in the request URL.' parameters: - in: query name: service-class schema: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' required: true description: 'Filter for User Service Descriptions tagged with the supplied service class term identifier expressed as a fully-qualified URI string from a controlled vocabulary' responses: '200': # OK description: "Success" content: multipart/related: schema: type: string '204': # No Content (no matching User Service Descriptions) description: "No Matches Found" '500': # Internal Server Error $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': # Service Unavailable $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' /user-service-descriptions/{externalServiceId}: get: operationId: retrieveUserServiceDescription summary: 'Retrieve User Service Description' description: 'Retrieve the User Service Description of a single service by supplying its external service identifier.' parameters: - name: externalServiceId in: path required: true schema: type: string description: 'The external service identifier of a User Service provisioned in the MBSF.' responses: '200': # OK description: "Success" content: multipart/related: schema: type: string '404': # Not Found $ref: 'TS29571_CommonData.yaml#/components/responses/404' '500': # Internal Server Error $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': # Service Unavailable $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' components: schemas: UserServiceDescriptions: description: 'A document announcing one or more MBS User Services.' type: object properties: version: type: integer minimum: 1 userServiceDescriptions: type: array items: $ref: '#/components/schemas/UserServiceDescription' minItems: 1 required: - userServiceDescriptions UserServiceDescription: description: 'A description of a single MBS User Service.' type: object properties: serviceIds: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1 class: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' names: type: array items: type: object properties: name: type: string lang: type: string pattern: '^[a-zA-Z]{3}$' example: 'eng' required: - name - lang minItems: 1 descriptions: type: array items: type: object properties: description: type: string lang: type: string pattern: '^[a-zA-Z]{3}$' example: 'eng' required: - description - lang minItems: 1 serviceLanguage: type: string pattern: '^[a-zA-Z]{3}$' example: 'eng' distributionSessionDescriptions: type: array items: $ref: '#/components/schemas/DistributionSessionDescription' minItems: 1 serviceScheduleDescriptions: type: array items: $ref: '#/components/schemas/ServiceScheduleDescription' minItems: 1 required: - serviceIds - class - distributionSessionDescriptions DistributionSessionDescription: type: object properties: distributionMethod: $ref: '#/components/schemas/DistributionMethod' conformanceProfiles: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1 sessionDescriptionLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' applicationServiceDescriptions: type: array items: $ref: '#/components/schemas/ApplicationServiceDescription' minItems: 1 inSessionObjectRepairParameters: $ref: '#/components/schemas/ObjectRepairParameters' postSessionObjectRepairParameters: $ref: '#/components/schemas/ObjectRepairParameters' availabilityInfos: type: array items: $ref: '#/components/schemas/AvailabilityInformation' minItems: 1 securityDescription: $ref: '#/components/schemas/SecurityDescription' timeSynchronizationParameters: $ref: '#/components/schemas/TimeSynchronizationParameters' required: - distributionMethod - sessionDescriptionLocator DistributionMethod: anyOf: - type: string enum: - OBJECT - PACKET - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. ApplicationServiceDescription: type: object properties: entryPointLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' contentType: type: string pattern: '^[a-zA-Z]+\/[a-zA-Z]+$' example: 'application/dash+xml' required: - entryPointLocator - contentType AvailabilityInformation: type: object properties: mbsServiceAreas: deprecated: true type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea' minItems: 1 targetServiceAreas: type: array items: $ref: '#/components/schemas/TargetServiceArea' minItems: 1 mbsFSAId: $ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId' nrParameters: type: array items: $ref: '#/components/schemas/NrParameterSet' minItems: 1 nrRedCapUEInfo: $ref: 'TS29571_CommonData.yaml#/components/schemas/NrRedCapUeInfo' TargetServiceArea: description: Target Service Area type: object oneOf: - required: [ncgiList] - required: [taiList] - required: [geographicAreaList] properties: ncgiList: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi' minItems: 1 description: List of NR Cell Identifiers taiList: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Tai' minItems: 1 description: List of Tracking Area Identifiers geographicAreaList: type: array items: anyOf: - $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Polygon' - $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/PointUncertaintyCircle' minItems: 1 NrParameterSet: type: object properties: freqBandIndicator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' aRFCNValue: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' required: - freqBandIndicator - aRFCNValue ObjectRepairParameters: type: object properties: repairLimitPercentage: $ref: 'TS26510_CommonData.yaml#/components/schemas/Percentage' backOffParameters: $ref: '#/components/schemas/BackOffParameters' objectDistributionBaseLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' objectRepairBaseLocator: $ref: 'TS26510_CommonData.yaml#/components/schemas/AbsoluteUrl' BackOffParameters: type: object properties: offsetTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' randomTimePeriod: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' anyOf: - required: [offsetTime] - required: [randomTimePeriod] ServiceScheduleDescription: type: object properties: id: type: string version: type: integer minimum: 1 start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' repetitionRule: $ref: '#/components/schemas/RepetitionRule' required: - id - version oneOf: - required: [start, stop] - required: [repetitionRule] RepetitionRule: type: object properties: startTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' duration: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' repetitionInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' required: - startTime - duration - repetitionInterval SecurityDescription: type: object properties: mBSSFAddresses: type: array items: $ref: 'TS26510_CommonData.yaml#/components/schemas/AbsoluteUrl' minItems: 1 mBSServiceKeyInfo: type: object properties: mBSId: type: string mBSDomainId: type: string required: - mBSId - mBSDomainId uICCKeyManagement: type: boolean 2GGBAallowed: type: boolean backOffParameters: $ref: '#/components/schemas/BackOffParameters' required: - mBSSFAddresses - mBSSessionKeyInfo TimeSynchronizationParameters: type: object properties: ranTimeTransmitted: type: boolean timeServiceEndpoints: type: array items: $ref: '#/components/schemas/TimeServiceEndpointParameters' minItems: 1 TimeServiceEndpointParameters: type: object properties: protocolScheme: type: string items: $ref: 'TS26510_CommonData.yaml#/components/schemas/AbsoluteUrl' endpoint: type: string accuracy: type: integer minimum: 1 required: - protocolScheme - endpoint