3GPP TSG-SA4 Meeting #-e (AH) MBS SWG post 134	S4aI250204

# Online, , 18th Dec 2025 - 18th Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0018 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED]: Solution for KI1 and KI4 for collecting and exposing Energy-Related information to authorized 3rd parties instantiating Media Application Service Energy Metrics configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-12-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of solutions to Key Issue 1 and Key Issue 4 on Energy-related Collection and Reporting to the 5G media |  |  |  |  |  |  |  |  |
| Summary of change: |  | - From Dallas meeting, proposal to split the initial contribution SA-252103 as listed hereafter: - In the SA-252103 contribution, keep the solution #10 named “the Media Application Service Energy Metrics Reporting Configuration” - In current contribution, propose the solution #12 named “The Media Application Service Energy metrics collection and reporting using new reference points”. Using new reference points does not make sense now. The candidate solution is based on the architecture of Solution #5 defined in clause 7.6.2.4 - In clause 7.13.2.3, addition of new text for report baseline parameters. - Update the Figure 7.13.2.1-1 to add the reference points M3, M5 and R6. Accordingly, their descriptions have been added. - Consider the proposal changes of Orange and BBC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No collection and reporting mechanisms for Energy-related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

(All new text)

## 7.13	Solution #12: Media Application Service Energy metrics collection, reporting and exposureAF

### 7.13.1	Key Issue mapping

This Candidate Solution addresses Key Issue #1 described in clause 6.2 and Key Issue #4 described in clause 6.4.

### 7.13.2	Functional description

#### 7.13.2.1	Introduction

This Candidate Solution describes the energy-related information and procedures for collecting and reporting Media Application Service Energy Metrics from the Media AS at reference points E3 and from the EIF at reference point E12, and for conveying this to the Media Application Provider via reference M1 or to the UE via reference point E5 with the aim of exposing the information to the Media-aware Application and Media Application Provider or, optionally to the Data Collection.AF to be exposed to event consumers. The architecture mapping and procedures are based on the architecture of Solution #5 defined in clause 7.6.2.4.

#### 7.13.2.2	Architecture mapping

Figure 7.13.2.2-1 illustrates how the approach could be instantiated within the instantiation of the reference architecture for collection and exposure of energy-related information in the Media Delivery System described in clause 7.6.2.4. In addition, the Data Collection AF defined in TS 26.531 [21] is optionally instantiated in the Media AF in a generalisation of the instantiation in clause 4.7 of TS 26.501 [23].

Figure 7.13.2.2-1: Metrics reporting feature with the overall generic reference architecture for collection and exposure of energy-related information (clause 7.6.2.2)

The following functions defined in clause 7.6.2.4 are relevant to this Candidate Solution:

-	The Energy Information AF instantiated in the Media AF has the following responsibilities, depending on its current provisioning state obtained from the Media AF:

-	Collecting energy-related information from the Media AS and the EIF, processing the information and creating energy-related information metrics reports.

-	Publishing energy-related information reports to the Media Application Provider after privacy-preserving post-processing

-	The Data Collection AF instantiated in the Media AF has the following responsibilities, depending on its current provisioning state obtained from the Media AF:

-	Receiving energy-related information metrics reports from the Energy Information AF instantiated in the same Media AF.

-	Publishing energy-related information metrics to Event Consumer AF subscribers in the form of exposed events.

-	The Energy Information Collector instantiated in the Media Session Handler of the Media Client has the following responsibilities:

-	Acquires an Energy Information collection configuration from the Energy Information AF via reference point E5 based on an endpoint address configured by the Media Session Handler that was embedded in the Service Access Information the Media Session Handler has acquired.

-	Subscribes to and consumes via reference point E5 Network Energy Information metrics reports from the Energy Information AF according to the Energy Information collection configuration.

-	Collects UE Energy Information from the Media Access Function at reference point M11 according to the Energy Information collection configuration.

The following reference points are involved in reporting Media Application Service Energy metrics:

M1	Network API used by the Media Application Provider to provision Media Application Service Energy reporting in the Energy Information AF instantiated in the Media AF by means of a new Media Application Service Energy Metrics Reporting Configuration. This determines whether and which energy-related information submitted by the Energy information Collector for the UE, the Media AS and the EIF for the Core Network. The Energy Information AF instantiated in the Media AF collects the Energy-related information and creates Energy related information reports as it is in the Release 19 architecture.

E12	This reference point is used by the EIF to provide energy-related information reports to the Energy Information AF instantiated in the Media AF. The Energy Information AF receives the information and creates the report aligned with the provisioned Media Application Service Energy metrics scheme.

E3	After provisioning of Media Application Service Energy Metrics Reporting in the Energy Information instantiated in the Media AF and the configuration of Content Hosting in the Media AS, the Media Application Service Energy Metrics Reporting configuration is obtained by the Media AS from the Energy Information AF instantiated in the Media AF. Based on this configuration, the Media AS collects Media Application Service Energy metrics, creates metrics reports aligned with the Media Application Service Energy metrics scheme of the configuration and uses this reference point to transmit Media Application Service Energy metrics reports to the Energy Information AF instantiated in the Media AF.

E5	Network API used by the Energy Information Collector instantiated in the Media Session Handler to obtain a Media Application Service Energy Metrics Reporting configuration from the Energy Information AF.

The Energy Information Collector fetches the collected Media Application Service Energy metrics, creates the metrics report aligned with the Media Application Service Energy metrics scheme of the configuration.

This API also allows the UE Energy Information Collector to receive the aggregated reports aligned with the Media Application Service Energy metrics scheme of the configuration from the Energy Information AF and propagate to the Media-aware application (via reference point E6 or via M6 by the Media Session Handler) and optionally, to the Media Application Provider (via reference point M8).

E4	Network API used by the Data Collection in AF instantiated in the Media AF to subscribe to and receive Network Energy Information from the Energy Information AF instantiated in the Media AF. Network Energy Information exposed to the Data Collection AF relates to a specific Application Service Provider. In particular, this API allows the Data Collection AF instantiated in the Media AF to receive the aggregated reports aligned with the Media Application Service Energy metrics scheme of the configuration from the Energy Information AF instantiated in the Media AF, and to expose them to event consumers via reference point R6 (see TS 26.531 [21]).

M3	Network API used by the Media AS to retrieve Service Access Information including the endpoint address of the Energy Information AF to be used for interacting with it at reference point 3.

M5	Network API used by the Media Session Handler to retrieve the Service Access Information including the endpoint address of the Energy Information Collector to be used for interacting with it at reference point E5.

M6	Client API used by the Media-aware Application to subscribe to receive Media Application Service Energy Metrics notifications from the Energy Information Collector instantiated in the Media Session Handler.

M8	Network API used by the Media Application Provider to receive energy-related information from the Media-aware Application. This reference point is beyond the scope of 3GPP standardisation.

R6	Network API used by the Data Collection AF to expose a Media Application Service Energy Metrics event to the Event Consumer AF.

### 7.13.3	Procedures

Figure 7.13.3-1 below details the different steps for Media Application Service Energy Metrics Reporting for the approach in the system outlined in clause 7.13.2.1 (based on figure 5.5.3-1 of TS 26.501 [23] but updated so that it applies to the generalised media delivery architecture defined in clause 4.1.2 of [23]).

Figure 7.13.3-1: Procedures for Media Application Service Energy metrics collection and reporting

The steps of the call flow are as follows. Differences from the baseline procedure requiring normative specification are highlighted in boldface.

1:	The Media AF is provisioned by the Media Application Provider with one or more Media Application Service Energy Metrics Reporting configurations , each pertaining to metrics collection by the Media Access Function and/or the EIF and the Media AS functions.

The configurations are transmitted to the Energy Information AF to collect and generate the metrics reports regarding required post-processing functionality and deliver to the Media Application Provider.

2:	The Energy Information AF subscribes to the EIF network function to receive Energy-related information from the Core Network.

NOTE:	These two steps are based on Solution #10 in clause 7.11.

3:	The Media-Aware Application triggers the Service Announcement and Content Discovery procedure. The Service Announcement includes the whole Service Access Information that contains metric configuration information for Media Application Service Energy metrics scheme.

4:	Time passes until the UE initiates session establishment and media delivery.

5:	The Media-Aware Application informs the Media Access Function of impending media delivery.

6:	The Media Session Handler requests the establishment of a media delivery session with the Media AF which confirms successful establishment of the media delivery session.

7:	The media processing pipeline is set up between the Media Access Function, the Media AS and the Media Application Provider.

8:	Service Access Information is retrieved by the Media AS via reference point M3, including the endpoint address of the Energy Information AF to be used for interacting with it at reference point E3.

9:	Using the endpoint address obtained in the previous step, the Media AS retrieves its Media Application Service Energy Metrics Reporting configuration from the Energy Information AF via reference point E3.

10:	Service Access Information is retrieved by the Media Session Handler via reference point M5, including the endpoint address of the Energy Information AF to be used for interacting with it at reference point E5.

11: The Energy Information Collector retrieves its Media Application Service Energy Metrics Reporting configuration from the Energy Information AF via reference point E5.

12: Using the Metrics reporting procedures specified in clause 4.9.2 of TS 26.512 [26512] at reference point M11, the Energy Information Collector queries the Media Access Function on its capability to perform metrics collection in accordance with Media Application Service Energy metrics scheme as defined by its Metrics Reporting configuration. At this step it is checked if the Media Access Function can support the reporting of Media Application Service Energy metrics.

NOTE:	This step is similar to the step 8 of Figure 5.5.3-1 in TS 26.501 but the gap between the Media Player and the Media access is FFS.

13:	The Media Access Function acknowledges its support for the collection of the required metrics of the configured scheme.

14:	The Media Access Function notifies the Media Session Handler of the start of media delivery, the Media Session Handler provide the Metrics collection endpoint address of the Media Access Function to the Energy Information Collector via an internal interface that is not standardised.

During the course of media playback, steps 15–32 below may be repeated, depending on the duration of the playback and the frequency of metrics reporting as set by the metrics configuration for the Media Application Service Energy metrics scheme.

15:	The Media AS initializes and begins countdown of its metrics reporting interval timer for the Media Application Service Energy metrics scheme.

16:	The Energy Information Collector initializes and begins countdown of its metrics reporting interval timer for the Media Application Service Energy metrics scheme.

17:	The Media Access Function accesses media content at the Media AS.

18:	Upon expiration of the timer (associated with the Media Application Service Energy metrics scheme), the Media AS retrieves the logged metrics according to that scheme.

19:	The Media AS reports the logged metrics to the Energy Information AF.

20:	The Energy Information AF processes the information according to the Media Application Service Energy metrics scheme.

21: The timer associated with the Media Application Service Energy metrics scheme in the Energy Information Collector expires.

22:	The Energy Information Collector retrieves the energy metrics from the Media Access Function according to Media Application Service Energy metrics scheme.

23:	The Energy Information Collector processes the information according to the Media Application Service Energy metrics scheme.

24:	The EIF publishes an NF Energy Information Report to the Energy Information AF.

25:	The Energy Information AF publishes a Media Application Service Energy metrics report to the Media Application Provider.

The following steps may occur to expose Media Application Service Energy metrics to the Media-aware Application:

26:	The Energy Information AF publishes a Media Application Service Energy metrics report to the Energy Information Collector via reference point E5 to allow the Media Session Handler to report to the Media-aware Application.

27:	The Energy Information Collector collates the Media Application Service Energy Metric reports of the Media Client and the ones received from the Energy Information AF.

28:	The Energy Information Collector shares the collated Media Application Service Energy Metrics Report with the Media Player via an internal interface that is not standardised.

29:	The Media  Session Handler publishes the final Media Application Service Energy metrics reports to the Media Aware Application at reference point M6.

NOTE: The use of the reference point E6 of the solution#5 may be possible and is FFS.

30:	The Media Aware Application may publish Media Application Service Energy Metrics reports to the Media Application Provider via reference point M8.

If exposure of Media Application Service Energy events is provisioned in the Media AF:

31:	Optionally, the Energy Information AF shares the Media Application Service Energy metrics with the Data Collection AF instantiated in the same Media AF via an internal interface that is not specified.

32:	Optionally, the Data Collection AF exposes a Media Application Service Energy Metrics event to the Event Consumer AF via reference point R6 as specified in TS 26.531 [26531].

Upon the termination of media delivery (as notified by the Media-Aware Application to the Media Access Function) a final round of metrics collection and reporting is performed.

### 7.13.4	Impacts on existing services, entities and interfaces

#### 7.13.4.1	Media AF

The Media AF would be required to support a Media Application Service Energy Metrics Reporting Configuration resource as part of the Provisioning Session(Table 8.2.3.1-1 in TS 26.510).

#### 7.13.4.2	Energy Information AF

The Energy Information AF is instantiated as a subfunction of the Media AF to:

1.	Collect energy-related information per application, per media session, and per media component, from the Media AS via reference points E3.

2.	Collect partial energy-related information about the 5G Core and the RAN from the EIF at reference point E12 in accordance with the Reporting scope defined for Solution #10 in clause 7.11.

Collected information is processed per scheme “Media Application Service Energy” and is reported to the UE Energy Information Collector instantiated within the Media Session Handler via reference point E5.

Processed collected information   be shared with a Data Collection AF instantiated within the same Media AF prior to exposure to authorized third parties at reference point R6, after privacy-preserving post-processing has been applied.

#### 7.13.4.3	Energy Information Collector

Editor’s Note: TODO

The Energy Information Collector is instantiated within the Media Session Handler to:

Fetch collected Energy metrics from the Media Player and process them per Media Application Service Energy scheme.

Receive the collected and processed energy-related information from the Energy Information AF

Create the aggregated Media Application Service Energy report with Media Player and Energy Information AF energy-related information and propagate to the Media-aware Application at reference point M6 and, optionally, to the Media Application Provider at reference point M8.

#### 7.13.4.4	Reference point M1

At reference point M1, the Media Application Provider creates one or more Media Application Service Energy Metrics Reporting Configuration resources associated with the target Provisioning Session to enable --Media Application Service Energy collection and reporting. The provisioning information provided includes the baseline parameters proposed in clause 7.11.2.2.

#### 7.13.4.5	Reference point E12

Existing procedures are used by the Energy Information AF at reference point E12 to subscribe to receive NF Energy Information Reports, and existing procedures are used to receive the energy-related information with different granularities: Slice, PDU Session and/or Media Component(s).(see TS 23.501)

#### 7.13.4.6	Reference point E3

Editor’s Note: TODO

#### 7.13.4.7	Reference point E5

Editor’s Note: TODO

#### 7.13.4.8	Reference point M6

Editor’s Note: TODO

#### 7.13.4.9	Reference point M8

Editor’s Note: TODO

(All new text)

#### 7.13.2.3	Energy-related information

##### 7.13.2.3.1	Media AS Energy Report

This is the metrics report generated periodically by the Application Server describing its energy consumption over the last sampling period.

Table 7.13.2.3.1-1: Baseline parameters of the Media AS energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Sample timestamp | The start date–time of the sampling period described by this report. |
| Duration | The duration of the sampling period described by this report. |
| Media delivery session identifier | A string that uniquely identifies a media delivery session in the Media Delivery System for the purpose of collating information from different system functions. |
| Energy exposing mode | The type of energy information reported by the Media Application Server (energy consumption or carbon equivalent emission) as provisioned in the Media Application Service Energy Metrics Reporting Configuration. |
| Media delivery session energy | The energy consumption or the carbon equivalent emission (depending on the indicated energy exposing mode ¦ depending on the provisioned metrics scheme) of the Media Application Server that was needed to convey the media data of the media delivery session during the sampling period described by this report. |
| Media component energy | A list of Media Component objects, each one indicating the energy consumption or the carbon equivalent emission (depending on the indicated energy exposing mode ¦ depending on the provisioned metrics scheme) of a media component of the media delivery session during the sampling period described by this report. |
| Renewable ratio | The ratio of renewable to non-renewable energyused by the Media Application Server during the sampling period described by this report. |

##### 7.13.2.3.2	Processed EIF Energy Report

This is the metrics report generated periodically by the Energy Information AF function describing the energy consumption of the 5G Core over the last sampling period. The information obtained from the EIF is cast into the context of a particular media delivery session by the Energy Information AF and is shared with the UE(s) involved in this media delivery session.

Table 7.13.2.3.2-1: Baseline parameters of the Processed EIF energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Sample timestamp | The start date–time of the sampling period described by this report. |
| Duration | The duration of the sampling period described by this report. |
| Media delivery session identifier | A string that uniquely identifies a media delivery session in the Media Delivery System for the purpose of collating information from different system functions. |
| Energy exposing mode | The type of energy information reported by the 5G Core (energy consumption or carbon equivalent emission), as provisioned in the Media Application Service Energy Metrics Reporting Configuration. |
| Media delivery session slice energy | The energy consumption or the carbon equivalent emission of a network slice of the 5G Core (depending on the indicated energy exposing mode ¦ depending on the provisioned metrics scheme) that was needed to convey the media data of the Media Delivery Session. |
| Media delivery session energy | The energy consumption or the carbon equivalent emission of the 5G Core (depending on the indicated energy exposing mode ¦ depending on the provisioned metrics scheme) that was needed to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| Media component energy | A list of Media Component objects, each one indicating the 5G Core energy consumption or the carbon equivalent emission (depending on the indicated energy exposing mode ¦ depending on the provisioned metrics scheme) that was needed to convey each media component of the Media Delivery Session during the sampling period described by this report. |
| Radio Access Network energy | A value that indicates the energy consumption or the carbon equivalent emission of the Radio Access Network to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| Core Network energy (UPF) | A value that indicates the energy consumption or the carbon equivalent emission of the Core Network (depending on the indicated energy exposing mode ¦ depending on the provisioned metrics scheme) that was needed by the UPF to convey the media data of the Media Delivery Session during the sampling period described by this report. |
| Renewable ratio | The ratio of renewable to non-renewable energy used by the 5G Core and RAN during the sampling period described by this report. |

##### 7.13.2.3.3	Processed Reporting entity aggregated metrics report

The aggregated processed metrics report is a set of Media Application Service Energy information aggregated by the UE based on the processed metrics report received from the network (see clause 7.11.2.4 above) and Media Application Service Energy metrics received from the UE itself.

Table 7.13.2.3.3-1: Baseline parameters of the processed metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

Editor’s Note: TODO

##### 7.13.2.3.4	Media Application Service Energy metrics event

The Media Application Service metrics event is a collection of Media Application Service metrics reports exposed by the Data Collection AF to subscribing event consumers. It is reported on a per-UE basis. Similar to the processed metrics report described in clause 7.11.2.4 above, the information conveyed is based on per-UE NF Energy Reports received from the EIF and per-UE aggregated Application Service Energy Reports received from the Application Server.

Table 7.13.2.3.4-1: Baseline parameters of the Media Application Service Energy metrics event

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

Editor’s Note: TODO

End of changes