3GPP TSG-SA4 Meeting #-e (AH) MBS SWG post 134	S4aI250207

# Online, , 18th Dec 2025 - 18th Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0008 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED]: Solution for KI1 for collecting and reporting an Energy of Service Metric to authorized 3rd parties |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-12-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of solutions to Key Issue 1 and Key Issue 4 on Energy-related Collection and Reporting to the 5G media |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No collection and reporting mechanisms for Energy-related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Proposal to split the initial contribution as listed hereafter: - In the current contribution, in addition to the 1st and 2nd changes, keep the solution #10 named “the Media Application Service Energy Metrics Reporting Configuration” - In another contribution, propose the solution #12 named “The Media Application Service Energy metrics collection and reporting using new reference points” In Table 7.11.2.2-1: - remove the power consumption scheme to be aligned with the requirements of the study. - add the two schemes “energy renewable source ratio” and “energy contribution ratio”. - Add a new reporting scope for a slice - Add Energy renewable source flag In clause 7.11.2.1, add the hierarchy of the Media Application Service Energy metrics reporting configuration Update the candidate solution with the BBC change proposals in the revision S4-252103r02_BBC.docx Updated according to the revision S4aI250207r01_BBC_BBC.docx |  |  |  |  |  |  |  |  |

Candidate Soluion MAPPING

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 |
| #1 |  |  | X |  |
| #2 |  |  | X |  |
| #3 |  | X |  |  |
| #4 | X |  |  |  |
| #5 | X |  |  |  |
| #6 | X | X |  |  |
| #7 | X |  |  |  |
| #8 |  | X |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  | X |

(All new text)

## 7.11	Solution #10: Media Application Service Energy Metrics Reporting Configuration

### 7.11.1	Key Issue mapping

This Candidate Solution addresses Key Issue #1 (Energy-related Information exposure) described in clause 6.2and Key Issue #4 (Energy-related configuration by the Application Service Provider for media delivery services) described in clause 6.4.

### 7.11.2	Functional description

#### 7.11.2.1	Introduction

This Candidate Solution addresses the definition of the Energy Information Exposure Specification of the solution #7 in clause 7.8 by proposing a Media Application Service Energy metrics configuration inspired by the existing downlink AF-based QoE Metrics Reporting configuration defined in clause 4.0.9 of TS 26.501 [4].

The Media Application Service Metrics Reporting Configuration(s) provided by the Media Application Provider over M1 determine what energy-related information associated with the downlink or uplink direction is to be collected and reported (to the instances listed) by the UE, Application Server and the Core Network, and how often.

A controlled vocabulary of Media Application Service Energy metrics to be reported is defined. Each metric is provisioned by citing the Media Application Service Energy Metrics scheme and, where applicable, by selecting the metrics to be reported. The following Energy metrics are supported:

– Carbon intensity, 
– Energy consumption,
– Energy renewable source ratio;
– Energy contribution ratio.

Energy metrics that are not selected shall not be collected and reported.

More than one of these may be provisioned at the same time in the Media AF by creating multiple Media Application Service Energy Metrics Reporting Configurations under the same Provisioning Session.

#### 7.11.2.2	Media Application Service Energy Metrics Reporting Configuration--

Up to and including Release 19, the Metrics Reporting Configuration resource defined in TS 26.501 [26501] is limited to configuring the reporting of Quality of Experience (QoE) metrics. This clause specifies extensions enabling Energy-related reporting  in addition to QoE in table 7.5.2.2-1. Consequently, the scope of the Metrics Reporting Configuration is extended to cover either QoE metrics reporting or Media Application Service Energy metrics reporting.

Figure 7.11.2.2-1 illustrates the hierarchy of the Media Application Service Energy Metrics Reporting Configuration included in the Provisioning Session information. This configuration is derived from the Metric Reporting Configuration, sharing some common baseline parameters, and is conveyed to the Media Client via the media streaming Service Access Information  defined for downlink media streaming Service Access Information in table 4.2.3-4 of TS 26.501 [23].

The figure is limited to illustrating only the newly introduced parameters, as the baseline Metric Reporting Configuration is already defined in TS 26.510 for release 19.

Figure 7.11.2.2-1: Media Application Service Energy Metrics reporting hierarchy

Table 7.11.2.2-1 describes the baseline parameters of the Media Application Service Energy Metrics Reporting Configuration corresponding to the model in figure 7.11.2.2-1 above. The parameters common with the metrics configuration, as defined in TS 26.510, are not redescribed in the candidate solution.-

Table 7.11.2.2-1:  Baseline parameters of the
Media Application Service Energy Metrics Reporting Configuration

| Abstract element |  |  | Semantics / constraints (abstract) |
| --- | --- | --- | --- |
| Metrics scheme |  |  | Scheme for four possible Media Application Service Energy metrics are defined, corresponding to the energy exposing modes: - carbon intensity; for which metrics are reported in g CO2-e / Wh. - energy consumption for which metrics are reported in Wh. - energy renewable source ratio. - energy contribution ratioW. The Media Application Provider may provision any number of the above metrics, each with the separate set of Metrics Reporting Configuration parameters. The scheme indicates that the metrics to be collected and reported pertain to energy-related information reporting and determine the format of the Aggregated Energy-Related metrics report published by the Energy Information AF (including energy information collected from the EIF for the Core Network and RAN and by the Media AS for the Application Server energy information) and collected by the Energy Information Collector in the Media Session Handler of the UE Media Client. (See NOTE 1). |
| Delivery session sample |  |  | Object containing the information used to select the media delivery sessions of a provision session for which reports shall be generated and sent. |
|  |  | Sample mode | Indicates the method used to determine the set of media delivery sessions for which reports shall be generated and sent. - Sample mode = 0, reports shall be generated and sent for all media delivery sessions. - Sample mode = 1, reports shall be generated and sent for a proportion of the media delivery sessions, as determined by the Sample percentage element. - Sample mode = 2, reports shall be generated and sent for media delivery sessions including at least one component with a MIME content type listed in the Component content types filter below and in accordance with the sample percentage element. Any undefined value of Sample mode, or if the Sample mode is not present, reports shall be generated and sent for all media delivery sessions. |
|  |  | Sample percentage | See description in TS 26.510. The use of this element shall be conditional upon the value of the Sample mode element. |
| Metrics to be reported |  |  | A non-empty list of Media Application Service Energy metrics, each indicated using one of the fully-qualified term identifiers from Table 7.11.2.1-, which shall be collected and reported. All Media Application Service Energy metrics schemes indicated by Metrics scheme share the same controlled vocabulary of metrics. |
| Delivery session energy information |  |  | Energy information pertaining to media delivery sessions. |
|  | Reporting scope |  | Scope of Media Application Service Energy metrics to be collected and reported: - Per slice, identified by its NSSAI: the sum of the energy consumption metrics values of all the media delivery sessions of the application during the sampling period. - Per media delivery session: the sum of the energy consumption metrics values of all the components of the session during the sampling period. - Aggregated energy consumption metric value across all active media delivery sessions in the sample identified by Sample percentage. - Per media delivery session: The energy consumption metric value of all the components of the session or only the ones of content type Component content types filter., |
|  | Component content types filter |  | MIME content types (e.g., video/mp4) used to filter components included in per‑component reporting (NOTE 2). |
| NOTE 1: See architecture mapping proposals in clause 7.13.3. NOTE 2: The applicability of the Component content types filter to any metrics reporting scheme, not just energy-related ones, is for further study. |  |  |  |

The controlled vocabulary identifier for the four metrics described in table 7.11.2.2-1 are listed in table 7.11.2.2-2.

Table 7.11.2.2-1: Controlled vocabulary of Media Application Service Energy metrics

| Metric name |  |  | Controlled vocabulary identifier (URI) |  |  |  | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carbon intensity |  |  | urn:3gpp:media:application:service:energy:metric:carbonIntensity |  |  |  | g CO₂-e/Wh |  |
| Energy consumption |  |  | urn:3gpp:media:application:service:energy:metric:energyConsumption |  |  |  | Wh |  |
| Energy renewable source ratio | urn:3gpp:media:application:service:energy:metric:renewableSourceRatio | Ratio |
| Energy contribution ratio | urn:3gpp:media:application:service:energy:metric:energyContributionRatio | Ratio |

### 7.11.2	Procedures

Procedures for handling the Media Application Service Energy Metrics Reporting Configuration are proposed in the Candidate Solution for collection, reporting and exposure of energy-related information described in clause 7.13.3.

End of changes