3GPP TSG-SA WG4 MBS SWG post #131-bis-e	S4aI250077

Online, May 7th, 2025

Title:	Reply on Inclusion of NTN intended service area in the Service Announcement

Response to:	-

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	SA4

To:	RAN2, SA2

Cc: 	SA2

# Contact Person:

# Name:	Thorsten Lohmar

E-mail Address:

# Attachments:

# 1.	Overall description

SA4 thanks RAN2 for bringing up the topic of “MBS via NTN”. SA4 had not studied the latest extension of the SA2 architecture for allowing MBS Broadcast transmissions via NR NTN. Based on discussions during SA4#131-bis, SA4 has several questions for RAN2 and SA2.

- SA2 added MBS support for NR NTN to TS 23.247 Release 19. The MBSF and MBSTF are optional network functions and usage of NR NTN can be interpreted as an implementation choice for the MBSF/MBSTF. There seem to be no conditions or constraints, only limiting the use of MBS Broadcast Sessions via NR NTN.

- The Service Announcement (TS 23.247, clause 6.11) may include the MBS Service Area, which can be specified in form of a Cell ID list, TAI list, geographical area information or civic address information. It seems that at most only one of these options may be included into service announcement, not multiple.

- Service Announcements for MBS User Services are currently expressed in terms of Cell Id lists or TAI lists only (TS 26.517).

- NG-RAN uses different Cell Ids (“Uu Cell Ids” on air vs “mapped Cell Ids” in RAN) for NR NTN (TS 38.300, clause 16.14.5). It seems that “Uu Cells Ids” should be used MBSF within Service Announcement for NR NTN, while “mapped Cell Ids” should be used by MBSF at Nmbsmf for MBS Session create.

SA4 is wondering whether to include existing NR Cell Ids into the MBS User Service Announcement or to obtain NR NTN-specific “Uu Cell Ids” from NG-RAN. Alternatively, the use of Cell Id lists could be avoided altogether as the MBS Service Area description within the MBS User Service Announcement when an NR NTN is targeted.

According to TS 23.247, clause 7.1.1.2, step 8, the NEF/MBSF derives the NR NTN Intended Service Area (based on local configuration, i.e. the NEF determines that an MBS session is targeting a NR NTN network) from geographical area information. SA4 is wondering,

- whether an (external) AF is aware of whether NR NTN is used for transmission (i.e. a new NR NTN usage flag in the API), and

- how to resolve the NR Intended Service Areas when an internal AF uses a Cell Id list or a TAI Id list for describing the MBS Service Area.

The RAN2 LS (S4-25491) introduces the concept of the Intended Service Area (ISA) and a new identifier for the Intended Service Area (ISA-ID). It seems that a Cell Id list or a TAI list is not needed at the UE side when the ISA or the ISA ID is present in the MBS User Service Announcement. SA4 is wondering whether RAN2 or SA2 sees a technical reason for having both Cell Ids/TAIs and ISA/ISA-ID present in the MBS User Service Announcement.

RAN2 is defining a new SIB to broadcast the mapping between an ISA ID and the geo-shape of the ISA. Thus, it seems preferable to include the ISA ID into MBS User Service Announcement (based on local configuration, when ISA ID is obtainable) to reduce the size of the MBS User Service Announcement.

Note also that the MBS User Service Announcement specified by SA4 in TS 26.517 does not support civic address, since there is no solution for obtaining the geo-shape or a list of Cell Ids from a civic address.

# 2.	Actions

# To SA WG2 and RAN WG2:

# SA4 kindly asks SA2 and RAN2 to clarify the following questions:

- Please clarify which Cell Ids/TAIs to include in the MBS User Service Announcement for NR NTN. SA4 assumes that Cell Ids/TAIs are obtained from local configuration within MBSF/NEF.

- Please clarify whether an AF (internal or external) can select whether to use NR NTN for the MBS Broadcast Session or not.

- Please clarify how to obtain the Intended Service Area, when Cell Ids/TAIs are used by the AF for MBS Service Area description.

- Please clarify whether ISA ID is obtainable from RAN so that the local configuration can also contain the ISA IDs for a given MBS Service Area.

# 3.	Date of next SA4 meetings

TSG SA WG4 Meeting #132		19th May – 23rd May 2025				Fukuoka, JP