3GPP TSG-WG SA4 Post #125 RTC AHG Call	S4aR230102

# Online, 12th October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR |  | rev | 1 | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [iRTCW] Dynamic Policy Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW |  |  |  |  |  | Date: |  |  | 11th October 2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | We propose the definition of a set of APIs to match the procedures defined in TS26.506. This is still work in progress and we propose to use it as baseline pCR to collect all API definitions. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 4.1	General

### 4.1.1	HTTP resource URIs and paths

The resource URI used in each HTTP request to the API provider shall have the structure defined in subclause 4.4.1 of TS 29.501 [22], i.e.:

{apiRoot}/{apiName}/{apiVersion}/{apiSpecificResourceUriPart}

with the following components:

-	{apiRoot} shall be set as described in TS 29.501 [22].

-	{apiName} shall be set as defined by the following clauses.

-	{apiVersion} shall be set to "v2" in this release of the specification.

-	{apiSpecificResourceUriPart} shall be set as described in the following clauses.

### 4.1.2	Usage of HTTP

#### 4.1.2.1	HTTP protocol version

##### 4.1.2.1.1	RTC AF

Implementations of the RTC AF shall expose both HTTP/1.1 [24] and HTTP/2 [31] endpoints at interfaces RTC-1 RTC-3 and RTC-5, including support for the HTTP/2 starting mechanisms specified in section 3 of RFC 7540 [31]. In both protocol versions, TLS [29] shall be supported and HTTPS interactions should be used on these interfaces in preference to cleartext HTTP.

The Application Service Provider may use any supported HTTP protocol version at interface RTC-1.

The Media Session Handler and the RTC AS may use any supported HTTP protocol version at interface RTC-5 and RTC-3.

All responses from the RTC AF that carry a message body shall include a strong entity tag in the form of an ETag response header and a modification timestamp in the form of a Last-Modified response header.

All endpoints shall support the conditional HTTP requests If-none-Match and If-Modified-Since.

#### 4.1.2.2	HTTP message bodies for API resources

The OpenAPI [23] specification of HTTP messages and their content bodies is contained in Annex X.

#### 4.1.2.3	Usage of HTTP headers

##### 4.1.2.3.1	General

Standard HTTP headers shall be used in accordance with clause 5.2.2 of TS 29.500 [21] for both HTTP/1.1 and HTTP/2 messages.

##### 4.1.2.3.2	Media Session Handler identification

The Media Session Handler in the RTC Client shall identify itself to the RTC AF at interface RTC-5 using a User-Agent request header (see section 5.3.3 of RFC 7231 [25]) in which the first element shall be a product identified by the token RTCMediaSessionHandler and optionally suffixed with a product-version.

The Media Session Handler may additionally supply a comment element in the User-Agent request header containing a vendor-specific identification string.

##### 4.1.2.3.3	RTC AF identification

The RTC AF shall identify itself using a Server response header (see section 7.4.2 of RFC 7231 [25]) of the following form:

RTCAF-{FQDN}/{implementationSpecificSuffix}

where {FQDN} shall be the Fully-Qualified Domain Name of the RTC AF exposed to the requesting client, and {implementationSpecificSuffix} shall be determined by the implementation.

##### 4.1.2.3.4	Support for conditional HTTP GET requests

All responses from the RTC AF that carry a resource message body shall include:

-	a strong entity tag for the resource, conveyed in an ETag response header,

-	a resource modification timestamp, conveyed in a Last-Modified response header, and

-	a predicted time-to-live period for the resource, conveyed in a Cache-Control: max-age response header.

All API endpoints on the RTC AF that expose the HTTP GET method shall support conditional requests using the If-None-Match and If-Modified-Since request headers. API clients should not attempt to revalidate their cached copy of a resource using a conditional GET request before the indicated time-to-live period has elapsed.

##### 4.1.2.3.5	Support for conditional HTTP POST, PUT, PATCH and DELETE requests

All API endpoints on the RTC AF that expose the HTTP POST, PUT, PATCH or DELETE methods shall support conditional requests using the If-Match request header. The API client should supply a strong entity tag in an ETag request header when invoking any of these HTTP methods.

### 4.1.3	HTTP response codes

Guidelines for error responses to the invocation of APIs of NF services are specified in clause 4.8 of TS 29.501 [22]. API-specific error responses are specified in the respective technical specifications.

Second Change

## 5.1	Provisioning (RTC-1) API

### 5.1.1	Overview

This clause defines the provisioning API used by the Application Provider to provision resources for their real-time communication sessions. The Provisioning API is an extension of the Provisioning API as defined in TS26.512 clause 7.

The following M1 procedures are relevant for RTC sessions:

| Procedure | Inherited | Extended | Not Relevant | New |
| --- | --- | --- | --- | --- |
| Provisioning Sessions API | X |  |  |  |
| Server Certificates Provisioning |  |  | X (but can be reused) |  |
| Content Preparation Templates |  |  | X |  |
| Content Protocols Discovery |  |  | X |  |
| Content Hosting Provisioning |  |  | X |  |
| Consumption Reporting Provisioning | X |  |  |  |
| Metrics Reporting Provisioning |  | X |  |  |
| Policy Templates Provisioning |  | X (RTCQoSSpecification) |  |  |
| Edge Resources Provisioning | X |  |  |  |
| Event Data Processing Provisioning |  |  | X (but can be reused) |  |
| Configuration Provisioning |  |  |  | X |

## 5.2	Configuration Provisioning API

### 5.2.1	Overview

The Configuration Provisioning API is used by the Application Provider to provision configuration that will be relayed to the MSH for usage with RTC sessions of that Application Provider.

### 5.2.2	Resource Structure

The Configuration Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/{apiVersion}/provisioning-sessions/{provisioningSessionId}/

Table 5.2.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column of the table shall be appended to the URL base path.

Table 5.2.2-1: Operations supported by the Configuration Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Retrieve Configuration Provisioning | configuration-provisioning | POST | Used to create a Configuration resource. |
| Create Configuration Provisioning |  | GET | Used to retrieve an existing RTC1Configuration resource. |
| Update Configuration Provisioning |  | PUT, PATCH | Used to modify an existing RTC1Configuration resource. |
| Destroy Configuration Provisioning |  | DELETE | Used to delete an existing RTC1Configuration resource. |

### 5.2.3	Data Model

#### 5.2.3.1	RTC1Configuration resource

The data model for the RTC1Configuration resource is specified in Table 5.2.3.1-1 below:

Table 5.2.3.1-1: Definition of RTC1Configuration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| offerTrustedStunServers | boolean | 0..1 | Indicates if the AF should provide a list of trusted STUN servers to the UE for usage with RTC sessions of this application provider. |
| stunServers | Array(URL) | 0..1 | An array of trusted STUN servers that the application can use as ICE candidates. |
| offerTrustedTurnServers | boolean | 0..1 | Indicates if the RTC AF should provide a list of trusted TURN servers to the UE for usage with RTC sessions of this application provider. |
| turnServers | Array(URL) | 0..1 | An array of trusted TURN servers that the application can use as ICE candidates. |
| offerTrustedSwapServers | boolean | 0..1 | Indicates if the AF should provide a list of trusted SWAP servers to the UE for usage with RTC sessions of this application provider. |
| swapServers | Array(URL) | 0..1 | An array of trusted WebRTC signaling servers that support the SWAP protocol. If provided, the application shall use one of the listed servers for RTC sessions of this application provider. |

### 5.2.4	Operations

The RTC AF shall relay the configuration information to the MSH using the Configuration procedure, if requested by the Provisioning information.

3rd Change

## 7.1	Dynamic Policy API

### 7.1.1	Overview

A RTC AF shall support the Dynamic Policy API over the RTC-3 and RTC-5 interfaces, as defined in the present sub-clause. The Dynamic Policy API allows both the MSH and the trusted ICE or WebRTC Signaling Server AS to request a specific QoS and charging policy to be applied to the data flows of an RTC session. The API defines a set of data models, resources, and the related procedures for the creation and management of the dynamic policy request.

The RTC AF uses either the Npcf_PolicyAuthorization API over N5, or the Nnef_AFSessionWithQoS over N33 interface to request the allocation of the dynamic policy with the PCF.

The security procedures as described in clause 13.4 of TS33.501 shall apply for the Dynamic Policy API over both RTC-3 and RTC-5 interfaces.

### 7.1.2	Resource structure

The Dynamic Policies API is accessible through the following URL base path:

{apiRoot}/3gpp-rtc35/{apiVersion}/dynamic-policies/

The table below specifies the operations and the corresponding HTTP methods that are supported by this API. The sub-resource path specified in the second column shall be appended to the URL base path.

Table 6.3-1: Operations supported by the Dynamic Policies API

| Resource name | Sub-resource path | Allowed HTTP methods | Description |
| --- | --- | --- | --- |
| Dynamic Policies |  | POST | Create a new Dynamic Policy resource. If the operation succeeds, the URL of the created Dynamic Policy Instance resource shall be returned in the Location header of the response. |
| Dynamic Policy | {dynamicPolicyId} | GET | Read a Dynamic Policy resource. |
|  |  | PUT | Replace an existing Dynamic Policy resource. |
|  |  | PATCH | Modify an existing Dynamic Policy resource. |
|  |  | DELETE | Delete an existing Dynamic Policy resource. |

### 7.1.3	Data model

#### 7.1.3.1	DynamicPolicy resource

The DynamicPolicy resource for RTC is define in the following table.

Table 6.3-2: Definition of Dynamic Policy resource

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| dynamicPolicyId | ResourceId | 1..1 | RO | Unique identifier for this Dynamic Policy. |
| policyTemplateId | ResourceId | 1..1 | C: RW R: RO U: RW | Identifies the Policy Template which should be applied to the application flow(s). |
| provisioningSessionId | ResourceId | 1..1 | C: RW R: RO U: RW | Uniquely identifies Provisioning Session, which is linked to the Application Service Provider. |
| qosSpecification | RTCQoSSpecification | 0..1 | C: RW R: RO U: RW | Describes the network Quality of Service properties of this Dynamic Policy. |

The RTCQoSSpecification object is defined in the following table.

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| mediaIdentifier | String | 1..1 |  | Provides an identifier for the media stream to associate with the corresponding service component in the QoS Policy. |
| marBwDlBitRate | BitRate | 1..1 |  | Maximum requested bit rate for the Downlink. |
| marBwUlBitRate | BitRate | 1..1 |  | Maximum requested bit rate for the Uplink. |
| minDesBwDlBitRate | BitRate | 0..1 |  | Minimum desired bit rate for the Downlink. |
| minDesBwUlBitRate | BitRate | 0..1 |  | Minimum desired bit rate for the Uplink. |
| mirBwDlBitRate | BitRate | 1..1 |  | Minimum requested bit rate for the Downlink. |
| mirBwUlBitRate | BitRate | 1..1 |  | Minimum requested bandwidth for the Uplink. |
| desLatency | Integer | 0..1 |  | Desire Latency. |
| desLoss | Integer | 0..1 |  | Desired Loss Rate. |
| pduSetMarking | PDUSetMarking | 0..1 |  | A description of the PDU Set and End of Burst marking configuration for the session. In release-18, this shall only be present in the case of an RTP stream. |

The PDUSetMarking object is defined in the following table:

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| serviceDataFlowDescription | ServiceDataFlowDescription | 1..1 |  | The 5-Tuple that identifies the service data flow for which the QoS dynamic policy is requested. |
| headerExtensionVersion | Integer | 1..1 |  | The RTP header extension version. |
| localIdentifier | Integer | 1..1 |  | A unique identifier of the RTP header extension in the scope of the media session. |
| format | Boolean | 0..1 |  | Indicates if a short or a long header extension format is used. When set to false, a short 1-byte header extension format is being used. |
| pduSetSizeActive | Boolean | 0..1 |  | A flag to indicate if the PDU Set size in bytes is present in the RTP header extension. |
| eobMarkingActive | Boolean | 0..1 |  | A flag to indicate if the End of Burst signaling is activated for this RTP stream. |

### 7.1.4	Operations

The policyTemplateId uniquely identifies the Policy Template, to which the Dynamic Policy Instance is associated. An  RTC session is composed of multiple components that correspond to RTP streams or data channels. Multiple RTP media sessions may be multiplexed into a single RTP stream, thus producing a single QoS flow.

The Dynamic Policy resource contains a serviceDataFlowDescription property which contains the service data flow template according to TS 23.503. The ServiceDataFlowDescription shall contain a flowDescription object (including 5-Tuples, Type of Service, Security Parameter Index, etc.).

The qosSpecifcation object shall be present and it shall contain the following properties:

-	marBwDlBitRate or marBwUlBitRate, indicating the maximum requested bit rate by the Media Session Handler.

-	mirBwDlBitRate or mirBwUlBitRate, indicating the minimum requested bit rate by the Media Session Handler.

-	minDesBwDlBitRate or minDesBwUlBitrate, indicating the minimum bit rate desired by the Media Session Handler.

The requested QoS parameters shall be within the limits of the provisioning QoS Policy Tempalte.