Agenda item: 	4.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] On the definition of Data Burst

Document for	Discussion and Agreement

# Introduction

The data burst was initially defined by SA2 in TS 23.501 [1]:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.

However, it is not clear how short is “short”. SA4 then gave a second definition to resolve this issue. Furthermore, the new definition mandates that there is an idle period between two data bursts. The SA4 definition was given in TS 26.522 [2]:

Data Burst: A data burst is a set of multiple PDUs generated and sent by the application such that there is an idle period between two data bursts. A Data Burst can be composed of one or multiple PDU Sets.

Although the SA4 definition resolves the issue with “short” in the SA2 definition, it has its own issue with the “idle period”. This is because, depending on the implementation of the congestion control algorithm, a time gap or idle period may always exist between any two adjacent RTP packets. For example, for Self-Clocked Rate Adaptation for Multimedia (SCReAM2) [3], the associated packet pacing algorithm described in clause 4.2.5 of RFC 8298 [3] tries to evenly distribute packets in time. This means that any arbitrary grouping of packets into data bursts will satisfy the definition of the data burst, but obviously arbitrary grouping is not helpful for the network.

Even if the congestion control algorithm sends the packets back-to-back, the underlying network protocol stack may introduce an idle period between adjacent packets. Take Ethernet as an example, there is always a time gap between any two Ethernet frames due to the Ethernet design. Specifically, as pointed out in [4],

“all adaptors wait 9.6 µs after the end of one frame before beginning to transmit the next frame”

Therefore, an idle period may always exist between two adjacent packets, and the wording “that there is an idle period between two data bursts” in the definition in 26.522 essentially says nothing.

# Candidate revisions of the data burst definition

Given that an idle period may always exist between two adjacent packets, how do we modify the SA4 definition to make it meaningful? A natural attempt is to add a qualification on the duration of the idle period, for example as follows (where the track change shows the potential revision):

Definition 1:

Data Burst: A data burst is a set of multiple PDUs generated and sent by the application such that there is an idle period between two data bursts that is greater than a threshold. A Data Burst can be composed of one or multiple PDU Sets.

If we adopt a threshold, then a question is: how is the threshold selected? The simplest solution is to leave it to the application of the sender, but the decision by the sender may not align with how the data burst information is used by the network. A more sophisticated solution is to let the network tell the threshold to the sender, but this involves standard specification of the signaling between the network and the application.

The definition of the data burst should be conducive to the SA4 specification of data burst characteristic signaling and the implementation. Considering this, the adoption of a threshold has a problem, because it could make the signaling of the burst size impossible without excessive delay. Specifically, there are scenarios where even if there is no idle period between two groups of packets, it makes sense to map the two groups to two data bursts. One example scenario is shown in Figure 1. The size of a video frame is typically not known before video encoding of that video frame is completed. So at the departure time of the first audio packet (the first green line), the sender does not know the size of the video frame (blue) yet. If the audio packets and the video packets are grouped as a single data burst and the burst size is carried by the first packet of the data burst (which is the first audio packet), then the sender does not know the video frame size and hence the burst size precisely. Therefore, it would be better to have two time-overlapping data bursts, one for audio packets and the other for video packets, and there is no idle period between the two data bursts. Note that, however, if the video packets are sent before the audio packets, as shown in Figure 2, there is no problem, because the size of an audio frame and the periodicity are typically fixed and are known to the sender in advance.

Figure 1 The size of the video frame (comprising of the left video packets) is not known to the sender at the departure time of the first audio packet.

Figure 2 The size of the audio frame (comprising of the audio packets) is known to the sender at the departure time of the first video packet.

Given the above reasons, it is beneficial to let the application at the sender to decide how to group the packets into data bursts based on the information available which may be implementation specific. Therefore, we proposed the following definition:

Definition 2:

Data Burst: A data burst is a set of multiple PDUs generated and sent by the application. A Data Burst can be composed of one or multiple PDU Sets.

NOTE: the application at the sender based on implementation decides how to group PDUs into data bursts.

But with this, there will be two definitions, one in SA2 and the other in SA4, which should be avoided if possible.

To strive to have a single definition across SA2 and SA4, SA4 could suggest SA2 modifying the SA2 definition to address SA4’s concern on the wording “short”. To do this, for the SA2 definition in TS 23.501 [1], a note can be added to address the issue of ‘short’, as follows:

Definition 3:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.

NOTE 2: the application at the sender based on the implementation decides on the short period of time.

A fourth definition was proposed in [5]:

Definition 4:

Data Burst: A data burst is a set of consecutive PDUs with shorter inter-PDU gaps than the PDUs sent before or after the data burst.

NOTE 1: A data burst can be composed of one or multiple PDU Sets.

NOTE 2: Meaning of a short inter-PDU gap needs to be determined by the application and can vary depending on different factors including the available bandwidth and link utilization.

While Definition 4 eliminates the need for an absolute threshold in Definition 1, it relies on a comparison of the time gaps within a data burst and the gaps to packets outside the data burst. But this will run into a problem when indicating the data burst size as shown in Figure 1, because it will group the green packets and the blue packets into a single data burst and the burst size is not known at the departure time of the first packet (the first green packet) of the data burst unless the green packets are delayed on purpose.

# Proposals

Proposal 1: Adopt the below definition of the data burst in TS 26.522.

Proposal 2: SA4 sends an LS to SA2 to improve the SA2 definition in TS 23.501 [1] by considering adding a note to address the issue of ‘short’ as follows:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.

NOTE 2: The sender application determines “a short period of time” based on its implementation.

References

[1]		TS 23.501, "System architecture for the 5G System (5GS)", Rel-19, V19.1.0, Sep 2024.

[2]		TS26.522, 5G Real-time Media Transport Protocol Configurations, Rel-18.0.0, March 2024.

[3]		RFC 8298, Self-Clocked Rate Adaptation for Multimedia, draft-johansson-ccwg-rfc8298bis-screamv2-00, 2024.

[4] 	L. L. Peterson, B. S. Davie, “Computer Networks – A systems Approach”, 6th Ed, 2022.

[5]		Nokia, S4aR250021, "On the definition of data burst", SA4-e (AH) RTC SWG post 130, 15 January 2025, online.