SA4-e (AH) RTC SWG post 131	S4aR250075

# 19 March 2025, Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0012 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP_Ph2] SDP signaling for N6-unmarked PDUs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PDU Set and End of Data Burst marking only applies to RTP PDUs since marking is done via an RTP header extension. Hence, PDUs belonging to protocols such as RTCP, STUN, etc. cannot be marked i.e., they do not carry the PDU Set Information. SA4 concluded in TR 26.822 that it would be beneficial for senders to signal application-defined PDU Set Importance (PSI) values to the 5GC for N6-unmarked PDUs. Based on the RTC procedures defined in TS 26.113, this information can be provided in the Application Flow Description populated by the Media Session Handler (MSH) in the Media Client when the MSH creates a Dynamic Policy Instance, and can be subsequently passed to the 5GC by the Media AF. For signaling to the AF, the Media Session Handler in the Media Client needs to acquire the N6-unmarked PDU information from the Media AS during the WebRTC signalling phase of the RTC session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new SDP attribute is defined that can be used to indicate PSI for N6-unmarked PDUs from the Media AS to the Media Client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to indicate sender-defined PSI values to the 5GC for N6-unmarked PDUs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6, 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A summary of the related solution in TR 26.822 is provided in S4-250336. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (All new) * * * *

# 6	SDP signaling

## 6.1	SDP signaling for N6-unmarked PDUs

A new session-level SDP attribute called "unmarked-pdu-info" describes a mapping between protocols of PDUs that cannot be marked using the RTP HE for PDU Set marking defined in clause 4.2 (i.e. N6-unmarked PDUs) and sender-defined PDU Set Importance (PSI) values associated to such protocols.

The “unmarked-pdu-info” attribute shall conform to the following ABNF syntax:

unmarked-pdu-info = "a= unmarked-pdu-info"  1*(SP "[" protocol "=" char-val [SP packet-type "=" num-val] SP psi "=" psi-val "]")

protocol = "protocol"

packet-type = "packet-type"

psi = "psi"

zerotofive = "0" /"1" / "2" / "3" / "4" / "5"

psi-val = DIGIT / (“1” zerotofive) ; numeric values 0-15

The values have the following semantics:

-	protocol: Application protocol name of N6-unmarked PDUs.

-	packet-type: Packet type specific to the application protocol. This parameter is optional. It is currently only used for RTCP and shall be set to one of the IANA registered RTCP Control Packet Types.

-	psi: PDU Set Importance value in the range 0 to 15 (inclusive).

An example usage is provided below:

a=unmarked-pdu-info [protocol=RTCP packet-type=203 psi=5] [protocol=STUN psi=3]

The “unmarked-pdu-info” attribute shall only be used as a session-level attribute and not as a media-level attribute.

An RTP sender that uses the RTP HE for PDU Set marking for its media shall use the “unmarked-pdu-info” attribute, if it intends to assign a PSI value to its outgoing N6-unmarked PDUs (e.g., STUN and RTCP packets).

An RTP sender shall include the “unmarked-pdu-info” attribute only together with the RTP HE for PDU Set marking in an SDP message.

The “unmarked-pdu-info” attribute can be included in the SDP offer or SDP answer. However, it only applies to outgoing packets of an RTP endpoint. Therefore, an RTP endpoint should exclude this attribute from the SDP answer if it is present in the SDP offer, unless the endpoint is an RTP sender using PDU Set marking and needs to indicate the appropriate PSI values for its N6-unmarked outgoing packets.

NOTE: The above SDP signalling is not sufficient on its own for appropriate PDU Set handling of N6-unmarked PDUs. The attribute can e.g. be used by an AS to indicate to a UE the appropriate PSI values for its N6-unmarked PDUs. The UE can then use control plane signalling via the AF to inform the 5GC of the PSI values that needs to be used for the downlink traffic carrying N6-unmarked PDUs. However, this signalling is not in the scope of this specification. For the RTC architecture, the related signaling using the RTC Dynamic Policy API is defined in TS 26.113 [X] clause 10.3.

# * * * * Second change * * * *

# 2	References

….

[X] 3GPP TS 26.113: "Real-Time Media Communication; Protocols and APIs.”