3GPP TSG-SA4 RTC SWG Ad hoc Meeting post#131	S4aR250084

Online, 26 March 2025

Source:	InterDigital Communications

Title:	Pseudo-CR on Delay adaptation in UE

Spec:	3GPP TS 26.567 v0.5.1

Agenda item:	4.5

Document for:	Discussion and agreement

1. Introduction

Draft TS 26.567 specifies Processing Delay adaptation in clause 7.3.2 and defined message format in Annex A. Processing Delay adaptation may be used with the Split Adaptation defined in clause 7.3.1. In this case the SR-DCMTSI client in terminal renders some objects of the scene locally, and the Processing Delay Adaptation may be used on the locally rendered objects.

The MF may need to change the Level-of-Details (LoD) of objects of the scene rendered by the SR-DCMTSI client in terminal. However, there is no mean to synchronize the state of the objects of the scene rendered by the SR-DCMTSI client in terminal with the MF.

Processing Delay adaptation is essential to maintain the quality of the user experience at an acceptable level when the network conditions are changing. This delay adaptation may rely on different scene representations to modify the delay of various processing tasks involved in the rendering loop such as:

-	the update of the XR scene graph representation by considering animation, user interactivity, physics simulation,

-	the culling techniques (e.g., camera frustrum, back-face) process with respect to the user current point of view to exclude objects or portions of the XR scene that are not visible to the user,

-	the rendering of the XR scene with the proper lighting conditions.

2. Reason for Change

When Split Adaptation procedure is used, the SR-DCMTSI client in terminal renders some objects of the scene locally. The Processing Delay Adaptation may be used on the locally rendered objects:

-	The SR-DCMTSI client in terminal may determine and change the LoD of the locally rendered objects to adjust the processing delay. The SR-DCMTSI client in terminal shall use the asset request message defined in clause A.2.7 to request the asset with the corresponding LoD from the MF.

-	The MF may determine and change the LoD of the objects rendered by the SR-DCMTSI client in terminal. The MF shall use the adaptation split message defined in clause A.2.3 to notify the SR-DCMTSI client in terminal to change the LoD of the objects rendered by the SR-DCMTSI client in terminal.

We proposed to extend:

-	the asset request message defined in clause A.2.7 to request the asset with one or more corresponding LoDs from the MF.

-	the adaptation split message defined in clause A.2.3 to notify the SR-DCMTSI client in terminal to change the LoD of the objects rendered by the SR-DCMTSI client in terminal.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 26.567 v0.5.1.

* * * First Change * * * *

### 7.3.2	Processing Delay adaptation based on QoE metrics

An SR-DCMTSI client or an MF may trigger further procedures during a split rendering. An additional procedure may include adjusting various round-trip delays between the SR-DCMTSI client and the MF during a split rendering session. The delay adaptation information may be sent from the SR-DCMTSI client to the MF periodically when the measured QoE metrics (e.g., poseToRenderToPhoton, roundtripInteractionDelay) goes out of the target delay range. Upon receiving the notification, an MF may adjust the delay involved in some of the processing tasks. For example, an MF may change the Level of Detail (LoD) of the objects in an XR scene which impacts the processing complexity of the XR scene. Delay adaptation procedure may include data exchange, for example, exchange of information messages for delay adaptation. The following generic procedure may apply, while the exact details may depend on the DC-application being rendered.

Figure 7.3.2-1: General procedures for processing delay adaptation based on QoE metrics information

The steps are as follows:

1 to 3:	As described in clause 7.3.1.

The steps for processing delay adaptation based on QoE metrics information during the rendering loop are as follows:

The SR-DCMTSI client measures and collects the QoE metrics negotiated in the delay adaptation configuration message. The QoE metrics considered for delay adaptation may contain all or a subset of the QoE latency metrics (e.g., poseToRenderToPhoton, roundtripInteractionDelay) negotiated in the metrics configuration message in clause 6.3.1.

4: 	The SR-DCMTSI client in terminal sends metadata required for rendering to the MF. In addition to the pose, pose predictions and user input metadata, the metadata may include a delay adaptation information message to the MF. The trigger to send the delay adaptation information message is based on the configured periodicity to inform the MF that the measured delay of a QoE latency metric is out of the target delay range. For example, the measured poseToRenderToPhoton QoE latency metric goes out of the target delay range due to new network conditions.

5a:	The MF may adjust the processing delay based on the delay adaptation information message.

NOTE: 	For example, the MF may change the LoD of the objects that are part of the scene for the delay adaptation.

6 and 5b:	The SR-DCMTSI client in terminal and the MF render the frame.

7 and 8:	As described in clause 7.3.1.

When processing delay adaptation procedure is used with Split Adaptation procedure, the SR-DCMTSI client in terminal renders some objects of the scene locally. Two cases may happen:

-	The SR-DCMTSI client in terminal may determine and change the LoD of the locally rendered objects to adjust the processing delay. The SR-DCMTSI client in terminal shall use the asset request message defined in clause A.2.7 to request the asset with one or more corresponding LoDs from the MF.

-	The MF may determine and change the LoD of the objects rendered by the SR-DCMTSI client in terminal. The MF shall use the adaptation split message defined in clause A.2.3 to notify the SR-DCMTSI client in terminal to change the LoD of the objects rendered by the SR-DCMTSI client in terminal.

* * * 2nd Change * * * *

## A.2.3	Adaptation Split

An SR-DCMTSI client that supports adaptive split rendering shall support the split adaptation message as defined in table A.2.3-1 below based on the split adaptation message defined clause C.2.3.2 of TS 26.565.

Table A.2.3-1 Message format for split adaptation messages

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:asrp:sr-split |
| message | Object | 1..1 | Message content |
| subtype | string | 1..1 | An identifier of the subtype of the message, it may be a request (REQ) for new split or acknowledgement (ACK), acceptance (OK) or rejection of a request (NOK). |
| renderingSplitId | string | 1..1 | An identifier of the rendering split unique within the scope of the SR session |
| renderingSplit | Object | 0..1 | An object identifying objects or the object Level-of-Details (LoD) to be rendered and where they are to be rendered (MF or UE). The message shall be a dictionary object. with keys “MF” and “UE”, and values corresponding to a key shall be a list of named nodes from the scene description being rendered in the SR session. The keys shall indicate where the objects named in the corresponding value list are rendered. |

* * * 3rd Change * * * *

## A.2.4	Seamless Adaptive Split

An SR-DCMTSI client that supports the adaptive split rendering with seamless adaptation shall support the seamless adaptive split message format defined below.

Table A.2.4-1 Message format for seamless adaptive split messages

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| Id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp: split-rendering: v1: asrp:sr-split-seamless |
| message | Object | 1..1 | Message content |
| subtype | string | 1..1 | An identifier of the subtype of the message, it may be a request (REQ) for new split or acknowledgement (ACK), acceptance (OK) or rejection of a request (NOK). |
| renderingSplitId | string | 1..1 | An identifier of the rendering split unique within the scope of the SR session |
| renderingSplit | Object | 0..1 | An object identifying objects or the object LoD to be rendered and where they are to be rendered (MF or UE). The message shall be a dictionary object. with keys “MF” and “UE”, and values corresponding to a key shall be a list of named nodes from the scene description being rendered in the SR session. The key ‘UE is used for objects that are to be rendered by the UE and key ‘MF is used for objects that are to be rendered by the MF, when seamlessSplit conditions are not met. |
| seamlessSplit | object | 0..1 | An object that if present indicates a seamless adaptation of the rendering process when possible. |
| radius | number | 1..1 | A distance in meters that defines a sphere centered at the UE, such that preferential rendering is used for objects that lie within this sphere. An object lies within the preferential rendering sphere if for all or some points defining the collider associated with the object the distance from the UE is less than the radius. |
| type | string | 1..1 | A string that indicates the type of preferential rendering to be used for the objects defined by the key ‘UE’ in renderingSplit. The following values are supported: “Local”: The objects are rendered at the UE when they are within the sphere define by R and rendered by the MF when they are outside of it. “LOD”: The MF renders the objects when they are within the radius R at a high fidelity. For example, 3D models with a higher Level-of-detail may be rendered. |

NOTE: 	Deterministic calculation of objects to be rendered by the UE, by the MF, and by the DC AS needs to be ensured. The floating point representation used by the UE, MF and the DC AS for physics calculation and latency between the UE, MF and the DC AS may impact determinism of such calculations.

* * * 4th Change * * * *

## A.2.7	Asset Request

An SR-DCMTSI client that supports the split rendering shall support the asset request messages below.

Table A.2.7-1 Message format for asset requests

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:asrp:sr-asset |
| message | Object | 1..1 | Message content |
| request | string | 1..1 | A request for assets, identifying the assets requested, for example as a list of nodes of a scene graph or a list of nodes of a scene graph corresponding to the Level-of-Details of the assets (objects), or a list of URIs which reference assets for nodes in the scene graph. |

* * * End of Changes * * * *