3GPP TSG-SA4 RTC SWG Ad hoc Meeting post#131	S4aR250085

Online, 26 March 2025

Source:	InterDigital Communications

Title:	Pseudo-CR on Delay adaptation configuration

Spec:	3GPP TS 26.567 v0.5.1

Agenda item:	4.5

Document for:	Discussion and agreement

1. Introduction

Draft TS 26.567 specifies Processing Delay adaptation in clause 7.3.2 and defined message format in Annex A. Processing Delay adaptation may be used with the Split Adaptation defined in clause 7.3.1. In this case the SR-DCMTSI client in terminal renders some objects of the scene locally, and the Processing Delay Adaptation may be used on the locally rendered objects.

However, to enable Processing Delay Adaptation in the SR-DCMTSI client in terminal, it needs to know the different LoDs available for locally rendered objects.

Processing Delay adaptation is essential to maintain the quality of the user experience at an acceptable level when the network conditions are changing. This delay adaptation relies on different scene representations to modify the delay of various processing tasks involved in the rendering loop such as:

-	the update of the XR scene graph representation by considering animation, user interactivity, physics simulation,

-	the culling techniques (e.g., camera frustrum, back-face) process with respect to the user current point of view to exclude objects or portions of the XR scene that are not visible to the user,

-	the rendering of the XR scene with the proper lighting conditions.

An object may have several representations (i.e., several variants of its geometry, texture, media). These object representations are often called object Levels-of-Details (LoDs) to be selected by the XR application for optimizing the trade-off between rendering quality and processing delay during the runtime. This trade-off between object rendering quality and processing delay is mentioned in the introduction section of the Level of Detail for 3D Graphics book [1] p.30:

“when rendering, use a less detailed representation for small, distant, or unimportant portions of the scene. This less detailed representation typically consists of a selection of several versions of objects in the scene, each version less detailed and faster to render than the one before.”

The criteria to select one object representation among the others may vary and is application dependent.

For example, a distance criteria from the current user viewpoint can be used to represent far objects with lower geometry and/or texture details with respect to close objects as shown in Figure 1.

Figure 1: LOD based on the number of mesh triangles with a distance criteria from Level of Detail for 3D Graphics book [1]

LoD generation of 3D asset is widely supported by the Digital Content Creation (DCC) tools such as Autodesk Maya, Unreal Engine, Houdini, Unity 3D.

A content creator may provide different objects LoDs in a scene description file for portability purpose between game engines. For example, the LoD functionality is supported the Khronos glTF 2.0 with the Microsoft MSFT_lod [2] extension.

Figure 2 provides an example of creating three LOD levels for an object based on a screen coverage criteria using the glTF MSFT_lod extension:

- The “High_LOD” object representation is rendered for a screen coverage from 0.5 to 1.0,

- The “Medium_LOD” object representation is rendered for a screen coverage from 0.2 to 0.5,

- The “Low_LOD” object representation is rendered for a screen coverage from 0.01 to 0.2,

- The object is not rendered for a screen coverage below 0.01.

"nodes": [

{

"name": "High_LOD",

"mesh": 0,

"extensions": {

"MSFT_lod": {

"ids": [

1,

2

]

}

},

"extras": {

"MSFT_screencoverage": [

0.5,

0.2,

0.01

]

}

},

{

"name": "Medium_LOD",

"mesh": 1

},

{

"name": "Low_LOD",

"mesh": 2

}

]

Figure 2: Example of glTF MSFT_lod [2] extension defining three level of details

To point out that this glTF MSFT_lod extension is generic in the sense that the selection criteria and the corresponding thresholds are provided in a MSFT_screencoverage metadata specified in extras parameter (i.e., non-normative). Other criteria (e.g., viewing distance, field of view) may be defined to select these three object representations.

As described, the LoD mechanism handling multiple representations of an object is a generic method to introduce processing delay flexibility by modifying the object rendering quality in a controllable manner.

An XR scene may contain many objects having multiple LoDs, leading to offer to the XR application a wide range of processing delay capabilities.

2. Reason for Change

When Split Adaptation procedure is used, the SR-DCMTSI client in terminal renders some objects of the scene locally and the Processing Delay Adaptation may be used on the locally rendered objects.

The SR-DCMTSI client may have to determine the LoD of the locally rendered objects to adjust the processing delay. To do so, it must know the different LoDs available for the involved objects.

We proposed two options to provide to the SR-DCMTSI client in terminal the different LoDs available for locally rendered objects:

-	Option 1: extend the Processing Delay Adaptation Configuration format to include the reference of the different LoDs available for each object rendered by the SR-DCMTSI client in terminal.

-	Option 2: use the glTF MSFT_lod [2] vendor extension with the SD-Rendering-glTF-Core defined in clause 9.2 of TS 26.119 [6] to provide the scene description file with different LoDs available for each object rendered by the SR-DCMTSI client in terminal.

3. Proposal

It is proposed to agree on one of the two following changes in option 1 (1st change) or option 2 (2nd and 3rd changes) to 3GPP TS 26.567 v0.5.1.

4. Reference

[1]	David Luebke, Martin Reddy, Jonathan D. Cohen, Amitabh Varshney, Benjamin Watson, Robert Huebner (2003). “Level of Detail for 3D Graphics”, https://www.sciencedirect.com/book/9781558608382/level-of-detail-for-3d-graphics.

[2]	Microsoft MSFT_lod extension of Khronos glTF 2.0: https://github.com/KhronosGroup/glTF/tree/main/extensions/2.0/Vendor/MSFT_lod.

* * * 1st Change * * * *

Option1:

## A.2.5	Processing Delay Adaptation based on QoE metrics

### A.2.5.1	Configuration format

The MF shall share the configuration information of the delay adaptation procedure with the SR-DCMTSI client in terminal during the split rendering session negotiation and establishment processes. The configuration may be updated during the rendering loop. The configuration information of the delay adaptation procedure shall be in JSON format according to the Metadata Data Channel Message Format defined in clause 5.4.3. The message type shall be “urn:3gpp:split-rendering:v1:daqoe:configuration”.

Table A.2.5.1-1 – Configuration message format for Processing Delay adaptation based on QoE metrics

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:daqoe:configuration |
| message | Object | 1..1 | Message content |
| qoeMetrics | array | 1..1 | An array of the QoE metrics for which delay adaptation is considered. This qoeMetrics array may contain all or a subset of the QoE latency metrics negotiated in the metrics configuration message in clause 6.3.1. |
| qoeMetricId | string | 1..1 | A unique identifier of the QoE metric within the scope of the split rendering session. The name of that QoE metric is chosen as unique ID, this name should be consistent with the name provided in the metrics reporting configuration defined in clause 6.3.1. |
| periodicity | string | 1..1 | The periodicity of the delay adaptation information for that QoE metric. It may be expressed as a multiple of the "Measure-Resolution" defined in clause 16.3.2 of TS 26.114 [7]. Whenever a delay adaptation information message is sent, the SR-DCMTSI client in terminal shall reset its timer to the value of the periodicity property and it shall begin countdown of the timer again. |
| flexibleObjects | array | 0..1 | An array of objects for which several LoDs are available for the delay adaptation. |
| flexibleObjectId | string | 1..1 | A unique identifier of an object within the scope of the split rendering session. For example, the index of the node of the object in the scene description. |
| levelIds | Array(string) | 1..1 | An array of node identifiers in a scene description corresponding to the LoDs of the object identified by the flexibleObjectId. |
| criteria | string | 0..1 | An information to guide the application for determining the object and corresponding LoD available in the XR scene. This information can be either: - “VIEWING_DISTANCE”: for increasing the LoD (i.e., by increasing the processing delay) for close object(s) if the measured delay is below the target delay and by decreasing the LoD (i.e., by decreasing the processing delay) for far object(s) if the measured delay is above the target delay, - “FIELD_OF_VIEW”: for increasing the LoD (i.e., by increasing the processing delay) for object(s) in the center of the FoV if the measured delay is below the target delay and by decreasing the LoD (i.e., by decreasing the processing delay) for object(s) located at the borders of the FoV if the measured delay is above the target delay, - “SCREEN_COVERAGE”: for increasing the LoD (i.e., by increasing the processing delay) for object(s) having larger screen coverage if the measured delay is below the target delay and by decreasing the LoD (i.e., by decreasing the processing delay) for object(s) having smaller screen coverage if the measured delay is above the target delay. |

NOTE: 	The target delay range for a QoE metric is delimited by a minimum threshold and a maximum threshold delay value. The thresholds and the target delay, for each QoE metric in the qoeMetrics array, can be provided by the DC Application Server to the SR-DCMTSI client in terminal.

### A.2.5.2	Metadata format

During a IMS-based split rendering session, the operating environment of the serving MF, the resources of SR-DCMTSI client or the network conditions may change. Consequently, the roundtrip delay may need to be adjusted to deliver a consistent QoE.

When delay adaptation procedure is enabled, the SR-DCMTSI client in terminal checks for the QoE metrics being monitored whether the measured delays are within the target delay range or not. When a measured delay is outside the target delay range for a QoE metric, the SR-DCMTSI client in terminal may report the measured delay based on the configured periodicity.

The delay adaptation information message format that is used for IMS-based split rendering shall comply with the format defined in Table A.2.5.2-1. The delay adaptation information message shall be carried as part of the data channel messaging mechanism. The metadata data channel message shall be in JSON format according to the Metadata Data Channel Message Format defined in clause 5.4.3. The message type shall be “urn:3gpp:split-rendering:v1:daqoe:information”.

Table A.2.5.2-1 Metadata format for Processing Delay Adaptation information message based on QoE metrics

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the IMS-based split rendering session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:daqoe:information |
| message | object | 1..1 | Message content |
| qoeMetrics | array | 1..1 | An array of the QoE metrics for which delay adaptation is needed. This qoeMetrics array may contain all or a subset of the QoE metrics negotiated in the configuration message in clause A.1.5.1. |
| qoeMetricId | string | 1..1 | A unique identifier of the QoE metric within the scope of the split rendering session. |
| delayValue | number | 1..1 | The measured delay value of that QoE metric. |

* * * 2nd Change * * * *

Option 2:

Annex B (normative):
Scene processing capabilities

# B.1	General

This annex defines scene processing capabilities supported by this specification in addition to the capabilities recommended for the respective device type in clause 5.2.

#### B.2.1	Scene Processing for Delay adaptation based on QoE metrics

The glTF MSFT_lod extension is a vendor extension to the SD-Rendering-glTF-Core defined in clause 9.2 of TS 26.119 [6].

When Delay adaptation based on QoE metrics is used, the SR-DCMTSI client in terminal and the MF shall support the MSFT_lod extension of glTF 2.0.

The schema of the glTF MSFT_lod extension is available at [15] https://github.com/KhronosGroup/glTF/tree/main/extensions/2.0/Vendor/MSFT_lod.

The glTF MSFT_lod extension provides the identification of which parts/entities (e.g., nodes of the graph representing objects) of the XR scene are flexible, i.e., which parts/entities provide several representations corresponding to several level-of-Details (LoD) to be used for the delay adaptation.

The 3GPP_adjustmentCriteria metadata in extras parameter allows to guide the application for determining the object and corresponding LoD available in the XR scene. The criteria shall be same for all objects that are flexible in the scene. The 3GPP_adjustmentCriteria metadata shall take one of following values:

-	VIEWING_DISTANCE: for increasing the LoD (i.e., by increasing the processing delay) for close object(s) if the measured delay is below the target delay and by decreasing the LoD (i.e., by decreasing the processing delay) for far object(s) if the measured delay is above the target delay.

-	FIELD_OF_VIEW: for increasing the LoD (i.e., by increasing the processing delay) for object(s) in the center of the FoV, if the measured delay is below the target delay and by decreasing the LoD (i.e., by decreasing the processing delay) for object(s) located at the borders of the FoV, if the measured delay is above the target delay.

-	SCREEN_COVERAGE: for increasing the LoD (i.e., by increasing the processing delay) for object(s) having larger screen coverage if the measured delay is below the target delay and by decreasing the LoD (i.e., by decreasing the processing delay) for object(s) having smaller screen coverage if the measured delay is above the target delay.

An example of creating three LoD levels for an object based on a screen coverage criteria using the glTF MSFT_lod extension is provided in Table B.2.1-1.

Table B.2.1-1: Example of glTF MSFT_lod extension defining three level of details

"nodes": [

{

"name": "High_LOD",

"mesh": 0,

"extensions": {

"MSFT_lod": {

"ids": [

1,

2

]

}

},

"extras": {

"3GPP_adjustmentCriteria": "SCREEN_COVERAGE"

}

},

{

"name": "Medium_LOD",

"mesh": 1

},

{

"name": "Low_LOD",

"mesh": 2

}

]

* * * 3rd Change * * * *

# 2	References

[15]	Microsoft MSFT_lod extension of Khronos glTF 2.0: https://github.com/KhronosGroup/glTF/tree/main/extensions/2.0/Vendor/MSFT_lod.

* * * End of Changes * * * *