Agenda item: 	4.5

Source: 	Qualcomm Inc.

Title: 	[SR_IMS] MF Capability Registration and Discovery

Document for	Discussion and Agreement

# Introduction

The MF is increasingly involved in more advanced media processing during IMS sessions. This includes support for network and split rendering as specified in TS 26.567. As such, the need for more advanced MF registration and discovery arises. In a way, the MF becomes more and more similar to an Edge Application Server (EAS) in the edge architecture.

In this contribution, we list EAS capabilities as used during the EAS registration and discovery. We propose to introduce similar capabilities for MF registration and discovery.

# EAS Capability Registration and Discovery

The EAS performs capability registration using the Eees_EASRegistration_Request operation. The operation allows the EAS to register detailed service capability information with the EES. The EASRegistration_Request includes structured information such as supported service KPIs, available resources, and network connectivity characteristics.

The EASServiceKPI structured data type used in registration is detailed below:

Table 8.1.5.2.4-1: Definition of type EASServiceKPI

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maxReqRate | Uinteger | O | 0..1 | Maximum request rate from the Application Client supported by the EAS. The minimum and maximum value shall be 0 and 100 respectively. |  |
| maxRespTime | Uinteger | O | 0..1 | The maximum response time, in the units of milliseconds, advertised for the AC's service requests. This includes the round trip time of the request and response packet for communication between the AC and EAS, the processing time at the EAS and time required by EAS to consume any 3GPP core network capabilities. |  |
| avail | Uinteger | O | 0..1 | Advertised percentage of time the server is available for the AC's use. The minimum and maximum value shall be 0 and 100 respectively. |  |
| avlComp | Uinteger | O | 0..1 | The maximum compute resource available for the AC. |  |
| avlGraComp | Uinteger | O | 0..1 | The maximum graphical compute resource available for the AC. |  |
| avlMem | Uinteger | O | 0..1 | The maximum memory resource available for the AC. |  |
| avlStrg | Uinteger | O | 0..1 | The maximum storage resource available for the AC. |  |
| connBand | BitRate | O | 0..1 | The connection bandwidth in Kbit/s advertised for the AC's use. |  |

In addition to the KPIs, the registration procedure allows for signaling a list of EAS features as defined by the following attribute:

| easFeats | array(string) | O | 1..N | Service specific features supported by the EAS (e.g. single vs multi-player gaming service). |  |
| --- | --- | --- | --- | --- | --- |

Upon receiving the EASRegistration_Request, the EES validates and stores the capability information, updating any prior registration details.

The discovery mechanism involves the Eees_EASDiscovery_Request operation initiated by the AC or other network entities. This request specifies desired capabilities for optimal service delivery. Upon receipt of the request, the EES processes the query by matching it with the registered EAS capability profiles stored in its registry.

The EES then returns an EASDiscovery response containing detailed information about the matched EAS instances, including available resources, performance indicators, and service KPIs. The AC or requesting entity subsequently selects the most suitable EAS instance based on this detailed capability data.

These registration and discovery procedures ensure optimized selection and use of EAS resources, facilitate dynamic resource allocation, and enhance overall efficiency and flexibility in edge application delivery.

# Enhanced MF Capability Registration and Discovery

As the MF’s role evolves and becomes more complex, the MF should support similar capability registration and discovery as the EAS. The MF shall be able to register a profile that includes KPIs similar to the EAS KPIs as well as application specific features, including the application id.

A basic use case of SR_IMS is the network rendering of a 3D game. In this scenario, the UE establishes a session to the IMS AS requesting a specific application. The IMS AS discovers an MF that has the capabilities required to run that specific 3D game, e.g. pre-installed game, graphics and compute capabilities, proximity to the UE, etc. The call is then routed to that selected MF, which terminates the IMS call negotiation to offer the downlink media of the rendered content and receive user’s actions.

# Proposal

We propose to reflect these finding in the LS to SA2 and ask them to define ways to register and discover MF capabilities.

# References

[1]	 	TS 29.558, Enabling Edge Applications; Application Programming Interface (API) Specification; Stage 3

[2]	 	TS 24.229, IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3