3GPP SA4-e RTC SWG post #131-bis-e	S4aR250091

e-Meeting, 30 April 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.830 | CR |  | rev |  | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_iRTCW_Ph2] Addition of reference point RTC-12 on Sol#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_iRTCW_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Hi Yoshihiro and Saba, For the below statement can we also add RTC-12 reference point as below? “The above metadata may be carried over WebRTC data channel at reference point RTC-4m or RTC-12. The data channel message format for split rendering is specified in 8.3.3 of TS 26.565 [13].” Sorry for my late comment. Thanks and Regards, Srinivas G |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference point of RTC-12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.3	Solution#2: RTC Signalling and Metadata

### 6.3.1	Solution description

This solution addresses Key Issue #2.

This solution provides the relevant work outcomes which can be incorporated into RTC-based services.

### 6.3.2	Candidate Metadata for RTC

3GPP TS 26.506 [9] specified the relevant reference points and interfaces to enable real-time media communication including the immersive media such as AR and XR. It implies RTC application supports the exchange of real-time metadata information for about the XR session as defined in 3GPP TS 26.119 [12]. Clause 12.2 of 3GPP TS 26.119 [12] provides the list and the format of timed metadata of an XR session and they are required to be complied to the relevant technical specifications of 3GPP TS 26.565 [13] and 3GPP TS 26.264 [14].

-	Pose format: It is periodically exchanged between the RTC endpoint in the UE and the other RTC endpoint in RTC AS when split rendering is required. The message format of pose is specified in Table 12.2-1 of 3GPP TS 26.119 [12] and the message type may be "urn:3gpp:split-rendering:v1:pose" if applied 3GPP TS 26.565 [13].

-	Action format: It is exchanged per request between the RTC endpoint in the UE and the other RTC endpoint in RTC AS when split rendering is required. The message format of action is specified in Table 12.3-1 of 3GPP TS 26.119 [12] and the message type may be "urn:3gpp:split-rendering:v1:action" if applied 3GPP TS 26.565 [13].

-	Available visualization space format: It may be exchanged to the remote RTC endpoint in RTC AS when the virtual scene is rendered in the RTC AS (such as split rendering). The message format of availableVisualizationSpace is specified in Table 12.4-1 of 3GPP TS 26.119 [12]. If it is required to be supported by RTC application, the message type for split rendering needs to be specified.

The above metadata may be carried over WebRTC data channel at reference point RTC-4m or RTC-12. The data channel message format for split rendering is specified in clause 8.3.3 of 3GPP TS 26.565 [13].

NOTE:	Device type identifier as specified in clause 12.5 of 3GPP TS 26.119 [12] is used for informing the device type and capability of the XR device in metadata. RTC applications may use the identifier to indicate the device capabilities for XR session, however the device type for RTC application is not limited to XR. Thus, the use of this identifier is for further study.

### 6.3.3	Candidate signalling for RTC

3GPP TS 26.506 [12] specified the reference point RTC-4s to exchange the signalling information for RTC session between the RTC endpoint in the UE and the WebRTC Signalling Function in RTC AS for the purpose of establishing the RTC sessions and managing the media flows. It also includes the information about the available transport protocol, NAT traversal route, network addresses and is carried over WebSocket connection. It may comply with WebRTC APIs such as RTCPeerConnection.