Agenda item: 	7

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED] Discussion on Profile Restrictions

Document for	Discussion and Agreement

# Introduction

This contribution provides the technical justification for the profile restrictions introduced for 3D (clause 5.6.2.2) and 2D (clause 5.6.3) avatars in the Draft base CR for AvCall-MED (Rel-19) so they can be finalized in TS 26.264. The justifications reference existing industry practices, device/runtime performance constraints, and interoperability considerations across glTF 2.0, OpenXR/face-tracking APIs, and common game/XR engines. The scope and messaging framework remain those of IBACS (TS 26.264), onto which the base CR layers Avatar communication.

# Profile Restrictions and Justification

## 2.1 Background

IBACS (TS 26.264 Rel-18) defines JSON metadata over a reliable, in-order application data channel (3gpp-ar-metadata) and reuses MPEG-I scene/SD where needed. Avatar communication in the base CR retains this approach for animation streams and adds ARF-based avatar containers for base avatars.

The Avatar functionality in IBACS has the following goals:

- Interoperability with de-facto authoring pipelines (glTF 2.0, PBR, blendshapes/joints), runtime APIs (OpenXR, ARKit-style coefficients), and common engines (Unity/Unreal).

- Predictable performance on mobile XR/handset GPUs under real-time audio-video constraints (triangle and vertex-shader cost, bandwidth of animation streams, predictable memory footprints).

- Extensibility with forward compatibility for features like compression, animation techniques, and new representation formats, such as gaussian splats.

## 3D avatar simple profile

### 2.2.1 Container and core assets

Restriction: Base avatar packaged in ARF (Zip), must include Skeleton with IBMs, Skin referencing meshes, and GLB (glTF 2.0) meshes with positions/normals/UVs and triangle topology; multiple LoDs supported.

Justification:

- glTF 2.0 is the industry’s common runtime-ready payload with PBR metallic-roughness materials, widely supported across engines/tools. Triangle topology with positions/normals/UVs is the minimal interoperable set for physically-based shading in glTF. This ensures model portability and predictable rendering.

- Linear Blend Skinning (LBS) maps directly to engine pipelines and GPU skinning; ARF’s structure ensures a typical authoring/runtime path that is widely adopted by industry.

### 2.2.2 Skinning weights

Restriction: Dense tensor of weights; max 4 influences/vertex; weights per vertex sum to 1.0.

Justification:

- Hardware/engine norm: Unity and Unreal optimize for 4 bone influences; more than 4 influences would increase vertex attribute bandwidth, with diminishing perceptual return. 4-influence layouts are “fixed-width” and map cleanly to vertex streams on low-end/mobile hardware.

- QoE: 4 influences maintain deformation quality for conversational use as facial animation relies more on blendshapes than deep multi-influence body weighting. The additional complexity of supporting more influences by vertex would not pay off in terms of quality of experience.

### 2.2.3 Facial animation

Restriction: 3D avatars shall include a BlendshapeSet for the face; more than 50 blendshapes, and either use or map to a recognized set (e.g., ARKit-style coefficients or platform-specific OpenXR extension names).

Justification:

- Ecosystem parity: ARKit [3] exposes a named set of 52 blendshape coefficients (e.g., mouthSmileLeft, browInnerUp, jawOpen) routinely targeted by avatar pipelines and commercial SDKs; Meta’s OpenXR XR_FB_face_tracking2 similarly exposes blendshape weights for facial regions. Aligning to these families minimizes per-platform retargeting.

- Coverage vs. bitrate: 50+ coefficients deliver natural lip/jaw/eye/cheek motion with compact streams. With 16-bit quantization at 60 fps, 52 coefficients ≈ 49.9 kbps; with 32-bit floats ≈ 99.8 kbps, both reasonable bitrates well below typical video bitrates. Applying compression schemes would bring these values even more down.

### 2.2.4 PBR textures and material set

Restriction: Albedo (base color) required; normal, occlusion, and metallic-roughness textures optional.

Justification:

- Interoperability: glTF 2.0’s metallic-roughness PBR is the ubiquitous baseline; keeping optional maps avoids forcing pipelines while preserving realism when devices/bandwidth allow.

### 2.2.5 Polygon budget guidance

Restriction: Highest LoD should (guidance) target ≈ 200 k triangles on mobile; lower LoDs preserve animation and reduce complexity.

Justification:

- Field evidence: ARM public samples [1] indicate frames rendering ≈ 210 k triangles at ~30 fps on representative mobile GPUs; budgeting ~200 k for a single avatar at close-up is a pragmatic limitation.

## 2D avatar profile

### 2.3.1 Geometry and textures

Restriction: Single planar mesh/quad in GLB (glTF 2.0), textured with a still image (formats per TS 26.264 clause 5.5).

Justification:

- Ecosystem practice: 2D avatar systems (e.g., Live2D [2]) use a textured mesh plus deformations; a single quad/planar mesh maximizes compatibility and minimizes vertex processing, ideal for low-end phones and 2D-only UIs.

- Interoperability: GLB (glTF 2.0) container keeps a unified asset path with 3D avatars; TS 26.264 already defines still-image formats, avoiding new media types.

### 2.3.2 Landmark-based animation

Restriction: Animation uses 2D landmark coordinates with ala_is_3d_flag=false per ARF; sparse tensors identify affected vertices.

Justification:

- Industry-proven approach: Real-time face-landmark pipelines (e.g., MediaPipe Face Mesh) drive 2D deformations from landmarks efficiently, scaling from minimal sets to high fidelity (e.g., 68→468 points). Using 2D landmarks keeps compute minimal for basic devices.

- Bandwidth efficiency: Example at 60 fps, 68 landmarks with 16-bit (x,y) ≈ 130.6 kbps; 106 landmarks ≈ 203.5 kbps. High-fidelity 468×3D landmarks would be ≈ 1.35 Mbps, which is unnecessary for a 2D mesh; the profile’s 2D choice is therefore appropriate for budget and latency.

### 2.3.3 Asset structure and LoD

Restriction: Single-LoD asset (mesh + texture); LandmarkSet references 2D mesh vertices for key facial features.

Justification:

- Simplicity and robustness: One LoD avoids resampling landmarks across meshes and cuts authoring/testing cost; quality scaling is achieved by texture resolution rather than geometry changes in 2D.

### 2.3.4 Optional voice-driven animation (DNN) inside ARF

Restriction: Optional voice-driven 2D animation via a proprietary framework stored in ARF; not mandated.

Justification:

- Interoperable extensibility: Keeps baseline 2D compatible while allowing vendors to ship voice-to-animation models as assets. This capability is discoverable through supportedAnimations in ARF without fragmenting the profile.

# Proposal

We propose to endorse the current profile definitions and to continue work on defining more profiles as needed in the future.

# References

[1]	 	Real-time 3D Art Best Practices - Geometry Guide, https://developer.arm.com/documentation/102448/0200/Triangle-and-polygon-usage

[2]		Live2D, https://docs.live2d.com/en/cubism-editor-manual/deformer/

[3]		Apple Developer, Blendshapes, https://developer.apple.com/documentation/arkit/arfaceanchor/blendshapes

[4]		Unity QualitySettings.skinWeights, https://docs.unity3d.com/6000.1/Documentation/ScriptReference/QualitySettings-skinWeights.html

[5]		Unreal skeletal mesh rendering, https://dev.epicgames.com/documentation/en-us/unreal-engine/skeletal-mesh-rendering-paths-in-unreal-engine