Agenda item: 	4.5

Source: 	Qualcomm Inc.

Title: 	[AIML_IMS-MED] Work prioritization

Document for	Discussion and Agreement

# Introduction

In this contribution, we argue for creating 2 different tracks for this work item:

- Remote inference: inference is performed completely in the network.

- Split inference: inference can be split between network and UE.

# Work Prioritization

The AIML_IMS-MED work item explicitly targets enabling AI/ML media processing in IMS services by extending TS 26.114 and TS 26.264, with “no changes to the IMS DC architecture expected”. SA4 committed to informing SA2 about any stage-2 implications. The target completion is set to SA#111 (March 2026). This frames the bar for Rel-20: deliver via existing IMS DC constructs.

Remote inference is implementable now with zero architecture changes because it reuses the existing IMS Data Channel pattern: bootstrap DC for discovery and an application DC for control, so there are no new network functions or reference points to add. That directly aligns with the AIML_IMS‑MED WID, which expects no changes to the IMS DC architecture while SA4 proceeds to extend TS 26.114/TS 26.264, and it leverages the already documented call flows in the TR.

MF selection is already covered as MF profiles may be defined to carry the capabilities needed to choose which MF runs which DNN. Because the mechanism is completely within SA4 scope, we can go straight to stage-3 CRs on TS 26.114/TS 26.264 and still meet the WID’s Rel-20 timeline (target SA#111). In short: no architecture work, mature procedure, clear spec path for remote inference is the pragmatic Rel-20 choice.

Split inference changes the key architecture. Executing a model across UE and network implies moving and potentially storing split models and intermediate representations close to the MF. That results in significant functionality enhancements than media/control signaling. The WID already anticipates that split inferencing brings stage-2 implications and stresses minimal impact on existing architectures. Honoring that commitment, requires SA2 to define the proper enhancements and procedures before SA4 can safely standardize the stage 3 aspects.

Concretely, split inference needs a repository function, a BAR-like, network-resident store for AI split models, with publish/lookup/fetch interfaces, versioning, integrity, and policy aspects, plus enhanced reference points to MF/MRF and DC-related functions. Those are new architectural enablers, not mere DC stage-3 tweaks, which pushes split inference outside a realistic Rel-20 timebox.

Thus, the right path is to prioritize remote inference in Rel-20 and open a dialogue with SA2 to define the repository and related stage-2 procedures, setting split inference up for the following release.

# Proposal

Due to the strong dependency of the split inference approach on IMS architecture modifications by SA2, this track should be considered high risk and unlikely to happen. The focus and priority should be on the remote inference track.

Hence, we propose to prioritize the remote inference track and work to complete it first. At the same time the group should trigger discussions with SA2 to plan for the realization of the split inference scenario.