3GPP TSG-SA4 Meeting #135	S4aR250220

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR |  | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on AIML processing in IMS calls |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_IMS-MED |  |  |  |  |  | Date: |  |  | YYYY-MM-DD |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AI/ML features require a standardized model representation and a consistent transport method aligned with the existing DCMTSI data channel framework. Specifying normative requirements reduces fragmentation across vendor specific runtimes and payload conventions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify ONNX as the normative AI/ML model format, including a fixed ONNX version and minimum opset, and specify that AI/ML application data is exchanged using DCMTSI data channel procedures with UTF 8 JSON as the default message encoding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementations are likely to diverge across incompatible model formats, opset levels, and data channel payload conventions, harming interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# Annex AD (normative): AI/ML assisted media processing for MTSI

## AD.1 General

This annex specifies procedures for AI/ML assisted media processing for MTSI and DCMTSI clients.

AI/ML processing is executed within the UE by an AI/ML application and an associated inference engine. The purpose of the mechanisms in this annex is to:
1) discover and select AI/ ML processing tasks and associated models,
2) configure and activate on-device inference during a call,
3) bind AI/ML processing to one or more RTP based media streams in the session using existing SDP identifiers,
4) exchange AI/ML related control and metadata with a DC AS using application data channels.

### AD.1.1 Terms and definitions

AI/ML application: A data channel application that provides AI/ML assisted media processing functionality during an IMS session.

AI/ML processing task: A well-defined AI/ML function applied to one or more media streams, for example speech to text captioning, translation, noise suppression, or scene description.

AI/ML model: A set of parameters and associated metadata required by an AI/ML inference engine to execute an AI/ML processing task.

AI/ML inference engine: A local execution environment in the UE that performs inference for an AI/ML model, for example a runtime exposing APIs similar to frameworks such as WebNN.

AI/ML metadata: Data derived from media streams by an AI/ML processing task, including timing and binding information that associates the metadata with media streams and media time.

## AD.2 Terminal architecture

An AI/ML capable DCMTSI client supporting this annex shall support the following logical functions:

- Media engine functions for RTP based audio and video media as specified in this specification.

- A data channel client supporting bootstrap data channels and application data channels according to clause 6.2.10 and clause 6.2.13.

- An AI/ML application execution environment, for example a web runtime used by data channel applications.

- An AI/ML inference engine that can execute at least one AI/ML model locally in the UE.

- A capability discovery function that can expose inference related capabilities to the AI/ML application, for example by enumerating available execution devices and supported operators, data types and resource limits in a manner aligned with local inference frameworks such as WebNN.

- A binding function that enables association of AI/ML processing tasks and derived metadata to specific RTP streams and media time.

The exact internal interfaces between the AI/ML application, the media engine and the inference engine are implementation specific and are not standardized by this specification.

## AD.3 End-to-End Reference Architecture

The DC reference architecture applies with the following clarifications:

- The DC AS and related repositories provide provisioning of AI/ML applications and models, including discovery information and update mechanisms.

- The UE retrieves AI/ML applications and models using HTTP over a bootstrap data channel.

- Application data channels may be established towards a DC AS for AI/ML task control, policy exchange or delivery of derived AI/ML metadata.

- The IMS Media Function does not perform inference and does not receive or process the RTP media streams for AI/ML purposes as part of the procedures in this annex.

## AD.4 AI/ML call flows

### AD.4.1 AI/ML model delivery

AI/ML applications and models intended for use during a DCMTSI session may be delivered using the bootstrap data channel framework.

The procedure is as follows:

- A DCMTSI client shall establish a bootstrap data channel using the "http" subprotocol as specified in clause 6.2.10.

- The client may retrieve an AI/ML application package, including associated resources, from a data channel application repository (DCAR) using HTTP over the bootstrap data channel.

- An AI/ML model may be delivered as part of the application package or as a separate resource retrievable via HTTP over the bootstrap data channel.

- The repository response describing available applications may include metadata that indicates that an application is an AI/ML application and the set of supported AI/ML processing tasks.

- The application and model resources may be updated using normal HTTP procedures over the bootstrap data channel.

### AD.4.2 On-device inferencing operation

This clause describes a generic procedure to activate an AI/ML processing task using on-device inference.

During an ongoing session, the user or the application logic selects an AI/ML processing task. The AI/ML application performs local capability discovery, for example using inference APIs similar to WebNN, and selects a compatible model or model variant. If the AI/ML application requires coordination with a DC AS, the UE establishes an application data channel as specified in clause 6.2.13, and associates it with the AI/ML application using the a=3gpp-req-app SDP attribute. The AI/ML application configures the inference engine and binds the selected task to one or more media streams using the binding rules in clause AD.7. The AI/ML application processes media locally in the UE and produces AI/ML metadata for local rendering and/or for transmission over the application data channel.

The AI/ML application may deactivate or change tasks during the session, which may require updating configuration and, when applicable, updating the set of application data channels.

## AD.5 Capabilities for AI/ML

### AD.5.1 UE capabilities

A DCMTSI client supporting this AI/ML processing shall be able to determine and expose to the AI/ML application a set of local inference capabilities, including at least:

- supported execution devices, for example CPU, GPU, NPU or equivalent accelerators,

- supported operator sets and data types, as applicable for the local inference framework,

- resource limits relevant to model execution, including memory constraints and any limits on concurrent tasks,

- availability of audio and video media access points needed by the AI/ML application, for example access to decoded media frames.

When a web runtime is used, the capability discovery may be aligned with frameworks such as WebNN.

### AD.5.2 Network capabilities

A DC AS supporting AI/ML processing may provide:

- repositories and discovery information for AI/ML applications and models,

- policy information, for example restrictions on tasks or model usage,

- application data channels for coordination with the AI/ML application.

Any capability related to performing inference in the network is outside the scope of Phase 1.

## AD.6 AI/ML formats

An AI/ML model used with DCMTSI procedures shall be represented in ONNX format. The ONNX model representation shall conform to ONNX version 1.16.0. The model shall declare a minimum required opset version of 18. The model shall be encoded using the ONNX Protocol Buffers representation as defined by ONNX.

## AD.7 AI/ML metadata

The model repository metadata shall be encoded as a UTF 8 JSON object. The metadata object shall include a model identifier and model version identifier. The metadata object shall identify the model format as ONNX and shall indicate ONNX version, minimum required opset, and the ONNX IR version used by the model.

The metadata object shall describe the model inputs and outputs with sufficient information to enable deterministic execution, including tensor element type and tensor shape, and any additional conventions required for correct execution such as dynamic axes, layout, and normalization. The metadata object shall include execution constraints required for compliant operation, including required operator support, required data types, quantization convention if applicable, and minimum resource requirements.

## AD.8	Negotiation and Signalling for AI/ML Media Processing

Editor’s note: Negotiation and signalling for AI/ML (model delivery, inferencing etc).

## AD.9 Data channel transport of AI/ML data

### AD.9.1 Bootstrap data channel transport

Bootstrap data channels shall use the HTTP subprotocol as specified in clause 6.2.10. AI/ML application packages and model resources may be retrieved using HTTP GET over the bootstrap data channel.

The DCMTSI client shall not transmit user media over the bootstrap data channel.

### AD.9.2 Application data channel transport

AI/ML control and metadata exchange may use one or more application data channels.

The subprotocol used on the application data channel for AI/ML control and metadata shall be defined as "3gpp-ai". Messages carried using this subprotocol shall be UTF-8 encoded JSON objects.

The following generic message types are defined:

- capability: indicates UE inference capability summary.

- task: indicates selection of an AI/ML processing task and model identifiers.

- configuration: provides task configuration parameters including binding to one or more media stream mid values.

- status: indicates lifecycle state and error reporting.

- metadata: conveys derived AI/ML metadata bound to a media stream and time.

The detailed schema of each message type is specified by the AI/ML application. When interoperability is required between different vendors, the schema should be standardized for the specific task.

==============End of change==============