3GPP TSG SA WG4 RTC SWG Telco	S4aR230108

# Online, 25 October 2023					    										Revision of S4aR230101

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR |  | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] RTP header extension for rendered pose |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 23 October 2023 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the RTC SWG telco on 10 October, the document S4aR230101 was agreed with two changes: All text is in brackets and definition of timestamp uses XR Runtime Clock instead of NTP Timestamp. This pCR proposes implements the agreed changes to timestamp, adds units for the x,y,z fields in the header extension and proposes to remove the brackets for inclusion into TS 26.522 |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 4.4	RTP Header Extensions

### 4.4.1	General

TBA

### 4.4.2	RTP Header Extension for PDU Set Marking

### 4.4.3	RTP Header Extension for Rendered Pose

A server that uses RTP to deliver pre-rendered video streams to a UE should include an RTP header extension for the rendered pose to indicate the pose used for rendering the media. The rendered pose RTP header extension may also be used with audio streams.

An RTP client that supports the RTP header extension for rendered pose shall negotiate the use the of the extension using SDP with the “extmap” attribute as defined in RFC8285 with the following URN: “urn:3gpp:xr-rendered-pose”. The syntax for the extmap attribute shall conform to the following ABNF syntax:

extmap-attr="a=extmap:" 1*5DIGIT ["/" direction] SP "urn:3gpp:xr-rendered-pose" [SP ”media:” 1*(SP token)]

The direction shall be defined as in RFC8285. The extension attribute “media” is followed by a list of tokens for “mid” (as defined in RFC 5888) for media streams that can reuse the rendered pose included in the RTP header extension. Further details on reuse are provided later in the section.

An RTP client that supports the RTP header extension for rendered pose and receives an SDP offer with “a=extmap” attribute with the URN: “urn:3gpp:xr-rendered-pose” shall remove the attribute from the answer for any media that will not use the extension, and retain it for any media that will use it.

The server delivers the rendered frames using one or more video streams, depending on the view and projection configuration that is selected by the UE. The server should use the RTP header extension for rendered pose to associate the selected pose with the rendered frame.

If negotiated successfully, an RTP sender should add the RTP header extension for rendered pose in the RTP stream. The frequency of RTP header extension for rendered pose shall be at least once in a frame. It may be sent more often but not necessarily in every RTP packet.

The 2-byte (RFC8285) RTP header extension format shall be used for signalling the rendered pose header extension as follows:

0                   1                   2                   3
 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|     0x100            | appbits|             length            |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|       ID      |    L=36+2n    |     x                      …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     y                      …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     z                      …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     rx                     …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     ry                     …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     rz                     …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
                                |     rw                     …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                               |     timestamp              …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                     timestamp continued    …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          timestamp continued  |     action_id #1              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      action_id #2                |              ...          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

x (32 bits): x coordinate of the position of the rendered pose in meters.

y (32 bits): y coordinate of the position of the rendered pose in meters.

z (32 bits): z coordinate of the position of the rendered pose in meters.

rx (32 bits): x coordinate of the orientation quaternion of the rendered pose.

ry (32 bits): y coordinate of the orientation quaternion of the rendered pose.

rz (32 bits): z coordinate of the orientation quaternion of the rendered pose.

rw (32 bits): w coordinate of the orientation quaternion of the rendered pose.

NOTE: XR server should be aware of the XR space used by the XR client for the pose fields defined above. Signalling aspects for this XR space is not in the scope of this specification.

timestamp (64 bits): Timestamp that corresponds to the predicted time for the pose. This timestamp uses the XR system clock. There is no requirement to synchronize the timestamps of the RTP stream to the XR system clock. The timestamp is passed to the XR runtime together with the rendered swapchain images (e.g. as part of the xrEndFrame call in OpenXR).

action_id (32 bits): A list of actions that were processed for the rendering of the frame are listed using action identifiers. The number of action identifiers in one RTP header extension for rendered pose shall be no more than 10. Hence, the maximum size of the header extension is 36+2*n, where n is the number of action identifiers in the header extension.

When both video and audio are delivered to the UE, or when either audio or video is delivered using multiple real-time streams (e.g., left eye + right eye), multiple RTP streams may be associated with the same header extension data, e.g., the same pose may have been used for generating multiple streams. This may lead to sending the same header extension data multiple times in different streams.

A sender may reuse the rendered pose RTP header extension of one stream for multiple RTP streams. For example, only the video stream carries the rendered pose RTP header extension, but the rendered pose is applicable also for the audio bitstream. In this case, the sender shall include the extension attribute media followed by a semi-colon separated list of media ID (MID) values in the "a=extmap” attribute. The MID values indicate all media streams for which the rendered pose RTP header extension is applicable to. If the extension attribute media is present, then the media description of all bitstreams that reuse the header extension shall include the attribute “mid” as defined in RFC 5888.

NOTE: In case there is a mismatch between the frame rates of the streams, the UE may use the few most recent samples from the source RTP stream to obtain a synchronized sample in the dependent stream via interpolation. Alternatively, the UE may choose to not perform any interpolation and simply use the last available sample from the source RTP stream for the dependent stream. It is left to the discretion of the UE application to select an appropriate synchronization method.

## Second change

Annex A (informative):
Examples of SDP offers and answers

A.1 SDP example for rendered pose RTP header extension

An example SDP description using the rendered pose RTP header extension is presented below. Using the extension attribute media, the rendered pose RTP header extension with URI urn:3gpp:xr-rendered-pose provided in the video stream with MID m1 is also applicable to another video stream with MID m3.

v=0

o=alice 2890844526 2890844526 IN IP4 host.atlanta.example.com

s=SDP Session

c=IN IP4 host.atlanta.example.com

t=0 0

m=application 1001 UDP/DTLS/SCTP webrtc-datachannel

a=sendonly

m=video 23458 RTP/AVP 96

a=mid:m1

a=recvonly

a=rtpmap:96 H264/90000

a=extmap:1 urn:3gpp:xr-rendered-pose media:m3

m=audio 23468 RTP/AVP 97

a=mid:m2

a=recvonly

a=rtpmap:97 PCMU/8000

m=video 23478 RTP/AVP 97

a=mid:m3

a=recvonly

a=rtpmap:96 H264/90000