# 3GPP TSG-SA4 Meeting (AH) RTC SWG post #131-bis-e							S4aR250099

# Online, 30 April – 8 May 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR | <CR#> | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AvCall-MED] Update on Avatar Formats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Canada |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED |  |  |  |  |  | Date: |  |  | 2025-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | AR Calls have been enhanced with Avatar communication capaibilities. The stage 3 aspects related to formats and signaling are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds the formats, protocols, and signaling to add support for Avatar communication to AR calls. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 will be missing stage 3 support for Avatar communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[3]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[5]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[6]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[7] 	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[8] 	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[9]	ISO/IEC 23090-39, Information technology — Coded representation of immersive media — Part 39: Avatar Representation Format

Next change

## 3.1	Terms

Next change

## 3.3	Abbreviations

Next change

## 4.3	End-to-End Reference Architecture

Next change

## 5.6 	Avatars

An AR-MTSI client in terminal offering 3D avatar communication shall support the Avatar Representation Format (ARF) as specified in [9]. An AR-MTSI client that supports 3D avatars shall support  [either] the ISOBMFF [or the Zip container formats]. and shall have at least one base avatar stored in the Base Avatar Repository (BAR) [in one of the two supported container formats]. The ARF document in the base avatar container shall include the following information:

-	Metadata about the user, including name, age, and gender,

- 	A list of the supported animations, which includes at least one animation type (e.g., face or landmark animation),

- 	At least one asset with at least one level of detail, and

- 	All data of relevant assets shall be contained in the ARF container of the base avatar.

An AR-MTSI client that supports 3D avatars shall support the following data formats for the avatar assets:

- 	3D meshes that conform to the binary glTF (GLB) format,

- 	Texture data components that conform to still image formats as defined in section 5.5,

- 	Sparse and dense tensor data formats as described in the ARF specification [9], and

- 	At least one of blendshape sets, skeleton, and/or landmark sets.

[An AR-MTSI client that supports 2D avatars shall support the following data formats:

- 	Image assets that conform to the formats described in 5.5 on still images, and

- 	2D landmark sets and the associated base meshAn AR-MTSI client that supports 2D avatars may support the following data formats:

- 	Animation ONNX models with opset 11 or higher]

Next change

# 6	AR Metadata

## 6.1	General

Real-time scene creation for an AR conference with two or more participants may be done by the MF to create a symmetric experience for all participants. For an MF to create a scene, it may request the following information from the UEs:

-	spatial description of the space surrounding the UE e.g., the occlusion-free space around the user in which the AR media will be rendered.

-	media properties indicating the AR media that the UE will be sending, and thus have to be incorporated in the scene.

-	receiving media capabilities of the UEs, which may include

-	UE media decoding capabilities

-	UE hardware capabilities (e.g., the display resolution)

-	information based on detecting the location, orientation, and capabilities of physical world devices, eligible for usage in an audio-visual communications session

- 	information on whether each user should be represented by their avatar, and if so, the Avatar ID of the user, and the avatar capabilities of the UEs

Based on this information the MF creates a scene which includes:

-	defining the placement of the user and the AR media in that scene, including e.g., the position, size, depth from the user, anchor type, and recommended resolution (or quality)

-	specific rendering properties for the AR media, e.g., for a 2D object to be rendered with a billboarding effect

The MF can then share the scene with the participant UEs using a supported scene description format. This scene description may be different for different UEs.

NOTE:	The scene as sent by the MF allows the UE to 1) select and request any related media (for example, in a quality and bitrate based on the rendering characteristics or network connection), 2) render the complete scene on a (virtual) display device, and 3) update the rendering and requested media dynamically (e.g., according to the movement and view orientation of the user).

An AR-MTSI terminal may request the MF to generate the animation streams based on the UE’s supplied media streams.

Next change

### 6.6	Avatar Animation

An AR-MTSI client or MF that supports avatars should support the exchange of avatar animation data over the data channel according to the sample formats described in clause 8 of ISO/IEC 23090-39 [9]. When the data channel is used to send animation data,  the metadata data channel message format defined in clause 6.2 should be used and the avatar animation messages shall have the type “urn:3gpp:ar:v2:avatar:animation” and the format shown in Table 6.6-1.

Table 6.6-1: Message format for avatar animation messages

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:ar:v2:avatar:animation |
| message | Object | 1..1 | Message content |
| subtype | number | 1..1 | An identifier of the subtype of the animation message. Value 1 indicates a facial animation, value 2 indicates a joint animation, and value 3 indicates a landmark animation. Other values are reserved for future use. |
| payload | Object | 1..1 | The avatar animation sample format corresponding to the message subtype. |

No compression scheme is defined for the animation samples.

Next change

### 6.4.X	Integration in Scene Description

Next change

### 6.5.4	Network Animation and Rendering

Next change

## 7.3 	SDP Negotiation and Signaling of Avatars

Next change

## 8.4		Data Channel Transport of Avatar Data

Next change

## A.2	Avatar Communication Call Flows

Next change

Annex B:
Base Avatar Management Interface  
(Informative)

End of changes