3GPP TSG-SA4 Meeting #-RTC SWG AH	S4aR250137

# Paris, France, 3rd Sep 2025 - 5th Sep 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | 0005 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solution for handling of N6 unmarked PDU's PDU Set importance in UPF pre-configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2, 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Sol #20 UPF only solution for handling unmarked PDU;s does not provide details on how the UPF can pre-configure the PDU Set importance value taking a acount the relative and application specific nature of PDU Set importance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Detail in a separate solution how the UPF can implement setting of PDU set importance based on pre-configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Guidelines and details for unmarked N6 PDU’s will be missing on how to set the PDU Set importance |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new clause) 7.1, 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

## 6.X	Solution #X: Handling of N6 unmarked PDU's PDU Set importance in a UPF pre-configuration

### 6.X.1	Key Issue mapping

This solution gives an extended solution on the KI#2: QoS handling requirements for lone PDUs extending the solution in clause 6.20.

### 6.X.2	Description

Solution #20 in clause 6.20 detailed how a User Plane Function could handle N6 unmarked PDU’s, i.e. lone PDU’s, PDU’s that are not marked with an RTP header Extension for PDU Set Marking.

The UPF (User Plane Function) is responsible for mapping the RTP packets and encapsulating them in GTP-U packets to send to NG-RAN.

Most corresponding fields can be derived from the RTP Header extension for PDU Set marking as described in Solution #20 or can be derived on straight forward logic at the UPF.

In addition the solution #20 describes the setting of the PDU Set importance by the 5G System based on pre-configuration, or by the AF (Application Function).

The solution in clause 6.15 details a way for the application function (AF) to provide a mapping from protocol type and payload type in an unmarked PDU info list to enable application specific PDU Set marking with PSI (PDU Set importance information).

In that (6.15) solution the protocol description is amended and additional interpretation of this kind of signalling is required from the UPF and also needs to be generated by the AF.

In this scenario, it is important that the UPF can detect the protocol type and payload type of incoming packets. For RTP/RTCP this is easier in case the packets are partitioned based on the PT field in the RTP/RTCP header.

However for non RTP/RTCP cases it becomes less clear how the UPF can achieve this protocol identification.

Observation 1: 	It is not clear beyond RTP/RTCP how UPF can detect the protocol type of an incoming packet

Another issue with mapping PDU Set importance to a list of values based on both protocol types and/or payload types introduces additional complexity looking this information up for each incoming packet in a database or lookup table.

Observation 2: 	Applying PDU set importance based on a lookup step on a long list of protocols and pt values combined with protocol identification introduces a potential performance penalty to complete both steps limiting the potential UPF throughput.

The simple pre-configuration is mentioned in solution #20 as an alternative solution setting the unmarked PDU’s.

Such solution is simple but does not take into account the application specifics, different applications may have different ranges of PDU Set importance which are used in the NG-RAN to discard PDU sets of lower importance only.

So the PDU Set importance values are not absolute and a generic pre-configuration may not be sufficient.

Observation 3: 	The simple pre-configuration does not take the relative and application specific nature of PDU Set importance into account

Further, unmarked PDU’s are handled in different ways.

Video packets that are unmarked may be assigned a PSI value at the UPF based on the bit stream syntax (as described in TS 26.522 clause 4.2.6.2).

Audio packets that are unmarked would typically be less than 10-20 percent of the overall bit-rate but are important for intelligibility.

RTCP packets can be important as they include important feedback information, they typically consume less than 5 percent of the total bit-rate.

STUN packets are used sparsely for determining the public IP address and consume a minimal percentage of the bandwidth, (also to not affect the RTP/RTCP session running on the same port)

OTHER packets should be sparse as well but may be critical for system operation.

Observation 4: Unmarked packets for video can be marked at the UPF following guidelines in place in TS 26.522, other packets are typically sparse but critical for the service or application

Based on this observation, and given the PDU Set importance is used the RAN for PDU Set discard (in this case of a single PDU), given the lower bandwidth compared to the video bandwidth compared the unmarked PDU should be assigned a PSI value lower or equal (better or equal) compared to the lowest PSI /highest importance PDU set in the stream.

Proposal: The UPF can consider this relative aspect and the application behaviour into account by setting the PDU Set importance to a value lower or equal to the lowest PSI value observed in the stream.

This solution takes the relative nature of PDU Set importance into account and can be implemented in quite straightforward ways at the UPF. Further, no significant new signalling or requirements are introduced in this solution.

### 6.X.3	Conclusion

Based on the analysis in the above, it is proposed to make the following conclusions.

-	PDU Set marking at the UPF based on a simple pre-configuration is possible but may not take the application specifics into account and the relative nature of PDU Set importance.

-	Non video Unmarked PDU’s are typical sparse or low bandwidth but critical for the application

-	Video PDUs can usually still be marked given currently available guidelines

-	Unmarked PDU’s are usually sparse and low bandwidth but critical to the service and should not be discarded at the NG-RAN

-	By taking the relative nature of PDU set importance into account, the UPF can set the value of unmarked and sparse N6 unmarked PDU’s to a value equal to or smaller than the smallest PSI found in the stream. This works as most unmarked PDU will be critical packets important for the service and are not desirable to discard at the RAN.

-	Guidelines for handling unmarked PDU’s should take this solution into account.

**CHANGE 2**

## 7.1	Key issues progress

Table 7.1-1: Summary of key issue progress

| Key Issue # | Short Description | Progress | Objectives met | Normative work proposed | Convergence possible? | Dependencies |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | PSSize accuracy | Solutions #4, #7 and #23 | Partially | Yes #4, Yes #7, Yes #23 | #4 and 7 are complementary solutions #23 is not complementary | #7 depends on SA2 #4 is fully contained in SA4 #23 has dependency on RAN feedback |
| 2 | Lone PDU | Solutions #2, #15, #20 ,#21 and #X | Partially | No #2, Yes #15 Yes #20 No #X | #2 provides gap analysis. Complementary solutions in #15 and #20 | SA2 |
| 3 | FEC support | Solutions #5, #17 and #18 | Yes | Yes #5, Yes #17, Yes #18 | Complementary solutions FEC schemes, congestion control and FEC awareness | None |
| 4 | FEC awareness | Solutions #8, #10, #17, #18, #19, #27 | Yes | Yes #8 Yes #10 Yes #17 Yes #18 Yes #19 | Complementary solutions | RAN2, SA2 |
| 5 | RTP transport for XR | Not progressed | - | - | - | - |
| 6 | Encryption | Solution #3 | Yes | Yes (but limited) | Single solution | None |
| 7 | RTCP messages for 5G | Solution #14 | Yes | No | Single solution | None |
| 8 | RTP retransmission | Solutions #9 and #11 | Yes | Yes #9, Yes #11 | Complementary solutions | SA2, RAN2 |
| 9 | RTP Multiplexing | Solutions #12 #22 | Yes | Yes #12 and #22 | Complementary solutions | SA2 |
| 10 | Use cases | Not progressed | - | - | - | - |
| 11 | Enhancements to PDU Set HE | Not progressed | - | - | - | - |
| 12 | Data Burst marking | Solutions #6, #13, #16, #24, #25, #26, #28 | Yes | Yes #6, Yes #16, #13: Reinforces the normative work proposed in #6 and #16, #24: No #25: Yes #26: Yes #28: No | Some solutions are complementary and convergence needed for others, possible way forward proposed in clause 7.13. | SA2, RAN2 |
| 13 | Applicability of PDU Set HE | Solution #1 | Partially | Yes #1 | Single solution | SA2, RAN2 |
| 14 | QoS flow multiplex | Solution #12 | Partially | Yes | Single solution | SA2 |
| 15 | Media and metadata | Not progressed | - | - | - | - |

** END OF CHANGES**