Source:	HEAD acoustics GmbH

Title:	Updated measurement results for HInT

Document for:	Discussion

# Introduction

The 3GPP work item HInT [1] introduces test methods and requirements in 3GPP TS 26.131 [2] and TS 26.132 [3] for the analogue and digital headset interface of UE. Based on the latest drafts of change requests for 3GPP TS 26.131 [4] and 26.132 [5], an initial test series was conducted in order to investigate the applicability of test methods as well as the provisional requirements.

# Devices

Nine recent commercially available mobile phones were used for the test series and are listed in Table 1. Information on supported electrical interfaces (analogue, Bluetooth/BT and USB-C/digital) are provided as well.

Table 1: Devices evaluated in test series

| DUT alias | Comments |
| --- | --- |
| DUT1 | - Not equipped with jack plug, analogue connection only via USB-C jack plug adapter. - Digital connection via USB-C does not accept generic headset interface. |
| DUT2 | - Not equipped with jack plug, analogue connection only via USB-C jack plug adapter. - Digital connection via USB-C does not accept generic headset interface. |
| DUT3 | - Digital connection via USB-C not supported. |
| DUT4 | - Not equipped with jack plug, analogue connection only via USB-C jack plug adapter. |
| DUT5 | - Not equipped with jack plug, analogue connection only via USB-C jack plug adapter. - Digital connection via USB-C does not accept generic headset interface. |
| DUT6 | - Not equipped with jack plug, analogue connection only via included (digital) USB-C-to-analogue adapter. - Analogue USB-C jack plug adapter not supported. - Digital connection via USB-C does not accept generic headset interface. |
| DUT7 | - Digital connection via USB-C does not accept generic headset interface. |
| DUT8 | - Analogue connection possible via jack plug, included (digital) USB-C-to-analogue adapter and USB-C jack plug adapter |
| DUT9 | - Digital connection via USB-C does not accept generic headset interface. |

Since a single mobile phone can be used with multiple codecs/connections and typically provide more than one electrical interface, several individual configurations and usecases of electrical interface are evaluated in the present document. For each combination of device, configuration and electrical interface, a unique identifier (ID) is used:

[device]-[connection]-[bandwidth]-[electrical interface]

With:

[device]				DUT alias according to Table 1.

[connection]			network connection type: 3G or VoLTE. Bitrates according to [3].

[bandwidth]			Abbreviation of bandwidth mode: NB, WB or SWB

[electrical interface]	Abbreviation describing the type of analogue or digital electrical interface:

A: analogue jack plug of the device is used.

AC: analogue adapter to USB-C is used.

ADC: included analogue<->USB-C adapter was used (can be a digital USB-interface!).

BT: Bluetooth is used (digital).

USB: USB-C is connected to reference interface (digital).

Examples:

- DUT1-3G-WB-USB: DUT1 is tested in 3G/circuit-switched mode, speech is encoded/decoded with AMR-WB at 12.65 kbit/s, UE is connected via digital USB-C reference interface to the measurement system.

- DUT3-VoLTE-SWB-A: DUT3 is tested in 4G/packet-switched mode via VoLTE, speech is encoded/decoded with EVS-SWB at 24 kbit/s, UE is connected via analogue jack plug to the measurement system.

# Test setup

The measurement series were setup according to the specification of the latest drafts of change requests for 3GPP TS 26.131 [4] and 26.132 [5].

# Results

## Junction loudness ratings

### Send JLR

Results for JLR measurements in receive are provided in Table 2 to Table 4. For all bandwidth modes, a common provisional requirement of 0 +/- 3 dB applies.

Table 2: Results for JLR in send (NB)

| DUT | JLR-SND [dB] | Requirement pass? |
| --- | --- | --- |
| DUT1-3G-NB-AC | -2.3 | TRUE |
| DUT1-3G-NB-BT | -0.0 | TRUE |
| DUT2-3G-NB-AC | 0.3 | TRUE |
| DUT2-3G-NB-BT | -2.4 | TRUE |
| DUT3-3G-NB-A | 0.2 | TRUE |
| DUT3-3G-NB-BT | -0.8 | TRUE |
| DUT3-3G-NB-USB | -1.6 | TRUE |
| DUT4-3G-NB-AC | -4.2 | FALSE |
| DUT4-3G-NB-BT | 0.6 | TRUE |
| DUT4-3G-NB-USB | -9.1 | FALSE |
| DUT5-3G-NB-AC | 2.5 | TRUE |
| DUT5-3G-NB-BT | 5.6 | FALSE |
| DUT7-3G-NB-A | 8.7 | FALSE |
| DUT7-3G-NB-BT | -3.1 | FALSE |
| DUT8-3G-NB-A | 3.6 | FALSE |
| DUT8-3G-NB-AC | -2.3 | TRUE |
| DUT8-3G-NB-ADC | -2.5 | TRUE |
| DUT8-3G-NB-BT | -2.8 | TRUE |
| DUT8-3G-NB-USB | -5.5 | FALSE |
| DUT9-3G-NB-A | -3.6 | FALSE |
| DUT9-3G-NB-AC | -2.4 | TRUE |
| DUT9-3G-NB-BT | -1.8 | TRUE |

Table 3: Results for JLR in send (WB)

| DUT | JLR-SND [dB] | Requirement pass? |
| --- | --- | --- |
| DUT1-3G-WB-AC | -2.5 | TRUE |
| DUT1-3G-WB-BT | 1.4 | TRUE |
| DUT2-3G-WB-AC | 2.6 | TRUE |
| DUT2-3G-WB-BT | -0.4 | TRUE |
| DUT3-3G-WB-A | 2.0 | TRUE |
| DUT3-3G-WB-USB | -1.6 | TRUE |
| DUT4-3G-WB-AC | -3.9 | FALSE |
| DUT4-3G-WB-BT | 0.5 | TRUE |
| DUT4-3G-WB-USB | -8.7 | FALSE |
| DUT5-3G-WB-AC | 2.0 | TRUE |
| DUT5-3G-WB-BT | -0.4 | TRUE |
| DUT6-3G-WB-ADC | 2.8 | TRUE |
| DUT7-3G-WB-A | 10.7 | FALSE |
| DUT7-3G-WB-BT | -1.1 | TRUE |
| DUT8-3G-WB-A | 4.4 | FALSE |
| DUT8-3G-WB-AC | -2.2 | TRUE |
| DUT8-3G-WB-ADC | -2.5 | TRUE |
| DUT8-3G-WB-BT | -1.2 | TRUE |
| DUT8-3G-WB-USB | -6.7 | FALSE |
| DUT9-3G-WB-A | -2.2 | TRUE |
| DUT9-3G-WB-AC | -3.6 | FALSE |
| DUT9-3G-WB-BT | -0.9 | TRUE |

Table 4: Results for JLR in send (SWB)

| DUT | JLR-SND [dB] | Requirement pass? |
| --- | --- | --- |
| DUT2-VoLTE-SWB-AC | 3.4 | FALSE |
| DUT2-VoLTE-SWB-BT | -1.4 | TRUE |
| DUT3-VoLTE-SWB-A | 5.0 | FALSE |
| DUT3-VoLTE-SWB-BT | 0.4 | TRUE |

### Receive JLR

Results for JLR measurements in receive are provided in Table 5 to Table 7. For all bandwidth modes, the common provisional requirements apply for the corresponding volume settings (if applicable for interface):

- nominal (NOM): -3…+3 dB

- maximum (MAX): > -15 dB

- minimum (MIN): < 16 dB

Table 5: Results for JLR in receive (NB)

| DUT | Volume Control | JLR-RCV [dB] | Requirement pass? |
| --- | --- | --- | --- |
| DUT1-3G-NB-AC | MAX | -0.4 | TRUE |
|  | MIN | 26.6 | FALSE |
|  | NOM | 1.0 | TRUE |
| DUT1-3G-NB-BT | NOM | 0.0 | TRUE |
| DUT2-3G-NB-AC | MAX | -6.3 | TRUE |
|  | MIN | 18.8 | FALSE |
|  | NOM | 0.7 | TRUE |
| DUT2-3G-NB-BT | NOM | -0.1 | TRUE |
| DUT3-3G-NB-A | MAX | -13.1 | TRUE |
|  | MIN | 6.4 | TRUE |
|  | NOM | -0.7 | TRUE |
| DUT3-3G-NB-BT | NOM | 0.1 | TRUE |
| DUT3-3G-NB-USB | NOM | 1.5 | TRUE |
| DUT4-3G-NB-AC | MAX | -7.9 | TRUE |
|  | MIN | 19.2 | FALSE |
|  | NOM | 1.1 | TRUE |
| DUT4-3G-NB-BT | NOM | -0.1 | TRUE |
| DUT4-3G-NB-USB | NOM | -0.4 | TRUE |
| DUT5-3G-NB-AC | MAX | -15.5 | FALSE |
|  | MIN | 8.4 | TRUE |
|  | NOM | 0.3 | TRUE |
| DUT5-3G-NB-BT | NOM | -2.3 | TRUE |
| DUT7-3G-NB-A | MAX | -16.7 | FALSE |
|  | MIN | 8.3 | TRUE |
|  | NOM | 0.3 | TRUE |
| DUT7-3G-NB-BT | NOM | -1.0 | TRUE |
| DUT8-3G-NB-A | MAX | -15.3 | FALSE |
|  | MIN | 8.8 | TRUE |
|  | NOM | 0.7 | TRUE |
| DUT8-3G-NB-AC | MAX | -6.1 | TRUE |
|  | MIN | 18.1 | FALSE |
|  | NOM | -0.1 | TRUE |
| DUT8-3G-NB-ADC | MAX | -14.7 | TRUE |
|  | MIN | 9.5 | TRUE |
|  | NOM | 1.4 | TRUE |
| DUT8-3G-NB-BT | NOM | -0.0 | TRUE |
| DUT8-3G-NB-USB | NOM | -0.2 | TRUE |
| DUT9-3G-NB-A | MAX | -19.3 | FALSE |
|  | MIN | 3.9 | TRUE |
|  | NOM | -0.3 | TRUE |
| DUT9-3G-NB-AC | MAX | -3.1 | TRUE |
|  | MIN | 19.9 | FALSE |
|  | NOM | -0.6 | TRUE |
| DUT9-3G-NB-BT | NOM | -2.4 | TRUE |

Table 6: Results for JLR in receive (WB)

| DUT | Volume Control | JLR-RCV [dB] | Requirement pass? |
| --- | --- | --- | --- |
| DUT1-3G-WB-AC | MAX | -19.9 | FALSE |
|  | MIN | 7.2 | TRUE |
|  | NOM | -0.9 | TRUE |
| DUT1-3G-WB-BT | NOM | 0.8 | TRUE |
| DUT2-3G-WB-AC | MAX | -4.9 | TRUE |
|  | MIN | 20.1 | FALSE |
|  | NOM | 1.1 | TRUE |
| DUT2-3G-WB-BT | NOM | -0.2 | TRUE |
| DUT3-3G-WB-A | MAX | -11.2 | TRUE |
|  | MIN | 8.3 | TRUE |
|  | NOM | 1.3 | TRUE |
| DUT3-3G-WB-USB | NOM | 0.9 | TRUE |
| DUT4-3G-WB-AC | MAX | -5.3 | TRUE |
|  | MIN | 21.7 | FALSE |
|  | NOM | -0.9 | TRUE |
| DUT4-3G-WB-BT | NOM | -0.3 | TRUE |
| DUT4-3G-WB-USB | NOM | 2.0 | TRUE |
| DUT5-3G-WB-AC | MAX | -13.0 | TRUE |
|  | MIN | 12.6 | TRUE |
|  | NOM | -1.1 | TRUE |
| DUT5-3G-WB-BT | NOM | -0.7 | TRUE |
| DUT6-3G-WB-ADC | MAX | -16.3 | FALSE |
|  | MIN | 14.5 | TRUE |
|  | NOM | -2.2 | TRUE |
| DUT7-3G-WB-A | MAX | -12.4 | TRUE |
|  | MIN | 12.7 | TRUE |
|  | NOM | -1.3 | TRUE |
| DUT7-3G-WB-BT | NOM | -0.9 | TRUE |
| DUT8-3G-WB-A | MAX | -13.7 | TRUE |
|  | MIN | 10.5 | TRUE |
|  | NOM | -1.6 | TRUE |
| DUT8-3G-WB-AC | MAX | -10.2 | TRUE |
|  | MIN | 14.1 | TRUE |
|  | NOM | -1.2 | TRUE |
| DUT8-3G-WB-ADC | MAX | -10.3 | TRUE |
|  | MIN | 14.0 | TRUE |
|  | NOM | -1.3 | TRUE |
| DUT8-3G-WB-BT | NOM | -0.1 | TRUE |
| DUT8-3G-WB-USB | NOM | 4.3 | FALSE |
| DUT9-3G-WB-A | MAX | -19.9 | FALSE |
|  | MIN | 3.1 | TRUE |
|  | NOM | -0.8 | TRUE |
| DUT9-3G-WB-AC | MAX | -7.4 | TRUE |
|  | MIN | 15.8 | TRUE |
|  | NOM | 0.1 | TRUE |
| DUT9-3G-WB-BT | NOM | -1.7 | TRUE |

Table 7: Results for JLR in receive (SWB)

| DUT | Volume Control | JLR-RCV [dB] | Requirement pass? |
| --- | --- | --- | --- |
| DUT2-VoLTE-SWB-AC | MAX | -3.7 | TRUE |
|  | MIN | 21.0 | FALSE |
|  | NOM | 2.3 | TRUE |
| DUT2-VoLTE-SWB-BT | NOM | 0.1 | TRUE |
| DUT3-VoLTE-SWB-A | MAX | -13.2 | TRUE |
|  | MIN | 6.8 | TRUE |
|  | NOM | -0.1 | TRUE |

## Idle channel noise

### SND

Results for idle noise levels in send are provided in Table 8 to Table 10. The provisional requirements for send direction are -64 dBm0(p) (for NB) / -64 dBm0(A) (for WB/SWB/FB).

Table 8: Results for idle noise level in send (NB)

| DUT | Avg. idle noise level [dBm0(p/A)] | Requirement pass? |
| --- | --- | --- |
| DUT1-3G-NB-AC | -55.2 | FALSE |
| DUT1-3G-NB-BT | -89.7 | TRUE |
| DUT2-3G-NB-AC | -52.6 | FALSE |
| DUT2-3G-NB-BT | -89.7 | TRUE |
| DUT3-3G-NB-A | -52.7 | FALSE |
| DUT3-3G-NB-BT | -89.9 | TRUE |
| DUT3-3G-NB-USB | -89.6 | TRUE |
| DUT4-3G-NB-AC | -76.0 | TRUE |
| DUT4-3G-NB-BT | -89.8 | TRUE |
| DUT4-3G-NB-USB | -87.4 | TRUE |
| DUT5-3G-NB-AC | -76.0 | TRUE |
| DUT5-3G-NB-BT | -89.1 | TRUE |
| DUT7-3G-NB-A | -72.8 | TRUE |
| DUT7-3G-NB-BT | -89.7 | TRUE |
| DUT8-3G-NB-A | -59.1 | FALSE |
| DUT8-3G-NB-AC | -42.0 | FALSE |
| DUT8-3G-NB-ADC | -42.2 | FALSE |
| DUT8-3G-NB-BT | -87.3 | TRUE |
| DUT8-3G-NB-USB | -82.3 | TRUE |
| DUT9-3G-NB-A | -47.4 | FALSE |
| DUT9-3G-NB-AC | -52.1 | FALSE |
| DUT9-3G-NB-BT | -89.5 | TRUE |

Table 9: Results for idle noise level in send (WB)

| DUT | Avg. idle noise level [dBm0(p/A)] | Requirement pass? |
| --- | --- | --- |
| DUT1-3G-WB-AC | -57.9 | FALSE |
| DUT1-3G-WB-BT | -78.9 | TRUE |
| DUT2-3G-WB-AC | -69.3 | TRUE |
| DUT2-3G-WB-BT | -77.7 | TRUE |
| DUT3-3G-WB-A | -74.8 | TRUE |
| DUT3-3G-WB-USB | -77.4 | TRUE |
| DUT4-3G-WB-AC | -69.4 | TRUE |
| DUT4-3G-WB-BT | -78.2 | TRUE |
| DUT4-3G-WB-USB | -77.8 | TRUE |
| DUT5-3G-WB-AC | -52.9 | FALSE |
| DUT5-3G-WB-BT | -77.8 | TRUE |
| DUT6-3G-WB-ADC | -71.1 | TRUE |
| DUT7-3G-WB-A | -69.4 | TRUE |
| DUT7-3G-WB-BT | -77.5 | TRUE |
| DUT8-3G-WB-A | -48.3 | FALSE |
| DUT8-3G-WB-AC | -42.0 | FALSE |
| DUT8-3G-WB-ADC | -42.9 | FALSE |
| DUT8-3G-WB-BT | -77.7 | TRUE |
| DUT8-3G-WB-USB | -78.0 | TRUE |
| DUT9-3G-WB-A | -51.2 | FALSE |
| DUT9-3G-WB-AC | -49.4 | FALSE |
| DUT9-3G-WB-BT | -77.4 | TRUE |

Table 10: Results for idle noise level in send (SWB)

| DUT | Avg. idle noise level [dBm0(p/A)] | Requirement pass? |
| --- | --- | --- |
| DUT2-VoLTE-SWB-AC | -79.7 | TRUE |
| DUT2-VoLTE-SWB-BT | -91.8 | TRUE |
| DUT3-VoLTE-SWB-A | -64.9 | TRUE |
| DUT3-VoLTE-SWB-BT | -86.9 | TRUE |

NOTE:	Results for single frequency disturbances identical regarding pass/fail status and are not shown here for sake of clarity. Each DUT that passes average idle noise floor also passes single frequency disturbance. There is no device that fails the average idle noise level but passes the single frequency disturbance.

### RCV

Results for idle noise levels in receive are provided in Table 11 to Table 13. For all bandwidth modes, the common provisional requirements apply:

- for analogue electrical interface UE:

  - -84 dBV(A/p) for MAX volume setting

  - -87 dBV(A/p) for NOM volume setting

- for digital electrical interface UE:

  - -64 dBm0(A/p) (same as for SND).

Table 11: Results for idle noise level in receive (NB)

| DUT | Volume Setting | Avg. idle noise level [dBm0(p) / dBV(A)] | Requirement pass? |
| --- | --- | --- | --- |
| DUT1-3G-NB-AC | MAX | -96.6 | TRUE |
|  | NOM | -103.5 | TRUE |
| DUT1-3G-NB-BT | NOM | -90.0 | TRUE |
| DUT2-3G-NB-AC | MAX | -95.7 | TRUE |
|  | NOM | -102.1 | TRUE |
| DUT2-3G-NB-BT | NOM | -114.3 | TRUE |
| DUT3-3G-NB-A | NOM | -84.9 | FALSE |
| DUT3-3G-NB-BT | NOM | -80.8 | TRUE |
| DUT3-3G-NB-USB | NOM | -72.8 | TRUE |
| DUT4-3G-NB-AC | MAX | -96.4 | TRUE |
|  | NOM | -103.0 | TRUE |
| DUT4-3G-NB-BT | NOM | -88.6 | TRUE |
| DUT4-3G-NB-USB | NOM | -85.1 | TRUE |
| DUT5-3G-NB-AC | MAX | -91.8 | TRUE |
|  | NOM | -111.5 | TRUE |
| DUT5-3G-NB-BT | NOM | -77.5 | TRUE |
| DUT7-3G-NB-A | MAX | -95.6 | TRUE |
|  | NOM | -101.7 | TRUE |
| DUT7-3G-NB-BT | NOM | -88.5 | TRUE |
| DUT8-3G-NB-A | MAX | -86.9 | TRUE |
|  | NOM | -97.3 | TRUE |
| DUT8-3G-NB-AC | MAX | -84.7 | TRUE |
|  | NOM | -91.5 | TRUE |
| DUT8-3G-NB-ADC | MAX | -80.8 | FALSE |
|  | NOM | -94.1 | TRUE |
| DUT8-3G-NB-BT | NOM | -81.9 | TRUE |
| DUT8-3G-NB-USB | NOM | -72.6 | TRUE |
| DUT9-3G-NB-A | MAX | -107.0 | TRUE |
|  | NOM | -115.4 | TRUE |
| DUT9-3G-NB-AC | MAX | -97.2 | TRUE |
|  | NOM | -104.1 | TRUE |
| DUT9-3G-NB-BT | NOM | -88.5 | TRUE |

Table 12: Results for idle noise level in receive (WB)

| DUT | Volume Setting | Avg. idle noise level [dBm0(p) / dBV(A)] | Requirement pass? |
| --- | --- | --- | --- |
| DUT1-3G-WB-AC | MAX | -97.2 | TRUE |
|  | NOM | -97.1 | TRUE |
| DUT1-3G-WB-BT | NOM | -89.4 | TRUE |
| DUT2-3G-WB-AC | MAX | -95.9 | TRUE |
|  | NOM | -95.3 | TRUE |
| DUT2-3G-WB-BT | NOM | -93.2 | TRUE |
| DUT3-3G-WB-A | MAX | -103.4 | TRUE |
|  | NOM | -108.4 | TRUE |
| DUT3-3G-WB-USB | NOM | -76.1 | TRUE |
| DUT4-3G-WB-AC | NOM | -96.4 | TRUE |
| DUT4-3G-WB-BT | NOM | -89.0 | TRUE |
| DUT4-3G-WB-USB | NOM | -85.8 | TRUE |
| DUT5-3G-WB-AC | MAX | -102.0 | TRUE |
|  | NOM | -110.7 | TRUE |
| DUT5-3G-WB-BT | NOM | -77.3 | TRUE |
| DUT6-3G-WB-ADC | MAX | -73.4 | FALSE |
|  | NOM | -91.4 | TRUE |
| DUT7-3G-WB-A | MAX | -94.6 | TRUE |
|  | NOM | -95.6 | TRUE |
| DUT7-3G-WB-BT | NOM | -84.8 | TRUE |
| DUT8-3G-WB-A | MAX | -80.6 | FALSE |
|  | NOM | -89.9 | TRUE |
| DUT8-3G-WB-AC | MAX | -84.7 | TRUE |
|  | NOM | -91.7 | TRUE |
| DUT8-3G-WB-ADC | MAX | -80.9 | FALSE |
|  | NOM | -89.2 | TRUE |
| DUT8-3G-WB-BT | NOM | -86.3 | TRUE |
| DUT8-3G-WB-USB | NOM | -74.0 | TRUE |
| DUT9-3G-WB-A | MAX | -107.5 | TRUE |
|  | NOM | -107.5 | TRUE |
| DUT9-3G-WB-AC | MAX | -96.8 | TRUE |
|  | NOM | -96.8 | TRUE |
| DUT9-3G-WB-BT | NOM | -88.0 | TRUE |

Table 13: Results for idle noise level in receive (SWB)

| DUT | Volume Setting | Avg. idle noise level [dBm0(p) / dBV(A)] | Requirement pass? |
| --- | --- | --- | --- |
| DUT2-VoLTE-SWB-AC | MAX | -95.7 | TRUE |
|  | NOM | -95.7 | TRUE |
| DUT2-VoLTE-SWB-BT | NOM | -90.7 | TRUE |
| DUT3-VoLTE-SWB-A | MAX | -83.3 | FALSE |
|  | NOM | -83.2 | FALSE |
| DUT3-VoLTE-SWB-BT | NOM | -104.3 | TRUE |

NOTE:	Results for single frequency disturbances identical regarding pass/fail status and are not shown here for sake of clarity. Each DUT that passes average idle noise floor also passes single frequency disturbance. There is no device that fails the average idle noise level but passes the single frequency disturbance.

## Sensitivity/frequency characteristics

### SND

Results for send frequency response are provided separately for NB, WB and SWB in Figure 1 to Figure 5, separated for analogue and digital electrical interfaces. The tolerance masks in the graphs are provisional and are based on Recommendation ITU-T P.381 [6].

Figure 1: Results for send frequency response (NB, analogue)

Figure 2: Results for send frequency response (NB, digital)

NOTE 1:	Larger mask violations are observed for several devices and are mainly due to the upper tolerance mask between 100 Hz and 200 Hz, which specifies a high-pass character. However, it seems not reasonable that the flat response of this terminal in this frequency range does not meet the requirements. The -9 dB tolerance at 100 Hz should be set to at least ~3 dB.

Figure 3: Results for send frequency response (WB, analogue)

Figure 4: Results for send frequency response (WB, digital)

NOTE 2:	A similar issue as described in note 1 can be observed here: the upper tolerance mask between 100 Hz and 200 Hz shows a roll-off, which leads to larger violations of the mask.

Figure 5: Results for send frequency response (SWB)

### RCV

Results for receive frequency response are provided separately for NB, WB and SWB in Figure 6 to Figure 10, separated for analogue and digital electrical interfaces.. The tolerance masks in the graphs are provisional and based on Recommendation ITU-T P.381 [6].

Figure 6: Results for receive frequency response (NB, analogue)

Figure 7: Results for receive frequency response (NB, digital)

Figure 8: Results for receive frequency response (WB, analogue)

Figure 9: Results for receive frequency response (WB, digital)

Figure 10: Results for receive frequency response (SWB)

## Sidetone characteristics

Results for sidetone characteristics are provided in Table 14 to Table 16 by STMR and in Table 17 by sidetone delay. The following provisional requirements apply for the sidetone characteristics:

- STMR > 20 dB (performance objective: STMR < 35 dB)

- Sidetone delay < 5 ms

Table 14: Results for sidetone masking rating (STMR, NB)

| DUT | STMR [dB] | Requirements pass? |
| --- | --- | --- |
| DUT1-3G-NB-AC | 79.6 | TRUE |
| DUT1-3G-NB-BT | 83.4 | TRUE |
| DUT2-3G-NB-AC | 78.3 | TRUE |
| DUT2-3G-NB-BT | 200.0 | TRUE |
| DUT3-3G-NB-A | 44.2 | TRUE |
| DUT3-3G-NB-BT | 200.0 | TRUE |
| DUT3-3G-NB-USB | 69.5 | TRUE |
| DUT4-3G-NB-AC | 49.4 | TRUE |
| DUT4-3G-NB-BT | 82.9 | TRUE |
| DUT4-3G-NB-USB | 35.9 | TRUE |
| DUT5-3G-NB-AC | 88.7 | TRUE |
| DUT5-3G-NB-BT | 71.4 | TRUE |
| DUT7-3G-NB-A | 74.5 | TRUE |
| DUT7-3G-NB-BT | 200.0 | TRUE |
| DUT8-3G-NB-A | 70.6 | TRUE |
| DUT8-3G-NB-AC | 79.3 | TRUE |
| DUT8-3G-NB-ADC | 71.6 | TRUE |
| DUT8-3G-NB-BT | 200.0 | TRUE |
| DUT8-3G-NB-USB | 83.4 | TRUE |
| DUT9-3G-NB-A | 84.7 | TRUE |
| DUT9-3G-NB-AC | 79.3 | TRUE |
| DUT9-3G-NB-BT | 82.5 | TRUE |

Table 15: Results for sidetone masking rating (STMR, WB)

| DUT | STMR [dB] | Requirements pass? |
| --- | --- | --- |
| DUT1-3G-WB-AC | 73.5 | TRUE |
| DUT1-3G-WB-BT | 81.0 | TRUE |
| DUT2-3G-WB-AC | 69.4 | TRUE |
| DUT2-3G-WB-BT | 200.0 | TRUE |
| DUT3-3G-WB-A | 40.4 | TRUE |
| DUT3-3G-WB-USB | 67.4 | TRUE |
| DUT4-3G-WB-AC | 44.8 | TRUE |
| DUT4-3G-WB-BT | 79.9 | TRUE |
| DUT4-3G-WB-USB | 31.3 | TRUE |
| DUT5-3G-WB-AC | 82.2 | TRUE |
| DUT5-3G-WB-BT | 69.0 | TRUE |
| DUT6-3G-WB-ADC | 59.1 | TRUE |
| DUT7-3G-WB-A | 66.3 | TRUE |
| DUT7-3G-WB-BT | 200.0 | TRUE |
| DUT8-3G-WB-A | 62.9 | TRUE |
| DUT8-3G-WB-AC | 66.6 | TRUE |
| DUT8-3G-WB-ADC | 66.5 | TRUE |
| DUT8-3G-WB-BT | 200.0 | TRUE |
| DUT8-3G-WB-USB | 78.3 | TRUE |
| DUT9-3G-WB-A | 78.3 | TRUE |
| DUT9-3G-WB-AC | 73.7 | TRUE |
| DUT9-3G-WB-BT | 80.1 | TRUE |

Table 16: Results for sidetone masking rating (STMR, SWB)

| DUT | STMR [dB] | Requirements pass? |
| --- | --- | --- |
| DUT2-VoLTE-SWB-AC | 18.5 | FALSE |
| DUT2-VoLTE-SWB-BT | 200.0 | TRUE |
| DUT3-VoLTE-SWB-A | -18.5 | FALSE |
| DUT3-VoLTE-SWB-BT | 179.8 | TRUE |

Table 17: Results for sidetone delay

| DUT | Sidetone Delay [ms] | Requirements Pass |
| --- | --- | --- |
| DUT1-3G-NB-AC | 1.1 | TRUE |
| DUT2-3G-NB-AC | 1.0 | TRUE |
| DUT3-3G-NB-A | 2.1 | TRUE |
| DUT4-3G-NB-AC | 17.8 | FALSE |
| DUT5-3G-NB-AC | 6.4 | FALSE |
| DUT7-3G-NB-A | 10.8 | FALSE |
| DUT8-3G-NB-A | 1.1 | TRUE |
| DUT8-3G-NB-AC | 1.1 | TRUE |
| DUT8-3G-NB-ADC | 22.6 | FALSE |
| DUT9-3G-NB-A | 5.2 | FALSE |
| DUT9-3G-NB-AC | 1.1 | TRUE |
| DUT1-3G-WB-AC | 1.1 | TRUE |
| DUT2-3G-WB-AC | 3.8 | TRUE |
| DUT3-3G-WB-A | 1.7 | TRUE |
| DUT4-3G-WB-AC | 18.1 | FALSE |
| DUT5-3G-WB-AC | 2.0 | TRUE |
| DUT6-3G-WB-ADC | 1.4 | TRUE |
| DUT7-3G-WB-A | 44.3 | FALSE |
| DUT8-3G-WB-A | 22.3 | FALSE |
| DUT8-3G-WB-AC | 11.2 | FALSE |
| DUT8-3G-WB-ADC | 44.0 | FALSE |
| DUT9-3G-WB-A | 44.5 | FALSE |
| DUT9-3G-WB-AC | 1.1 | TRUE |
| DUT2-VoLTE-SWB-AC | 1.0 | TRUE |

NOTE 1:	For electrical interface BT, the RCV signal is expected to be digitally zero, which causes a singular STMR calculation (indicated here as 200 dB).

NOTE 2:	Since a sidetone is not desired for digital interfaces, results for sidetone delay are only provided for analogue electrical interfaces.

NOTE 3:	Several fails for analogue interface may be explained by the connection via USB-C interface, which seems to have an unintended, but rather high cross-talk than a traditional jack plug.

NOTE 4:	For handset/headset UE, the sidetone delay requirement is defined as the difference between measured and mouth-to-ear delay, which is not reflected in these results. Since the loopback delay for analogue electrical interface can be assumed as constant, the requirement might be increased by 1.5-2.0 ms, which would let two additional devices pass this test.

## Acoustic echo control

Results for (acoustic) echo control characteristics are provided in Table 18 to Table 20. The following requirements apply:

- TCL > 46 dB for MAX volume setting

- TCL > 55 dB for NOM volume setting

Table 18: Results for echo loss (NB)

| DUT | Volume Setting | TCL [dB] | Requirements pass? |
| --- | --- | --- | --- |
| DUT1-3G-NB-AC | MAX | 11.6 | FALSE |
|  | NOM | 16.2 | FALSE |
| DUT1-3G-NB-BT | NOM | 33.9 | FALSE |
| DUT2-3G-NB-AC | MAX | 41.7 | FALSE |
|  | NOM | 43.9 | FALSE |
| DUT2-3G-NB-BT | NOM | 66.2 | TRUE |
| DUT3-3G-NB-A | MAX | 47.8 | TRUE |
|  | NOM | 46.8 | FALSE |
| DUT3-3G-NB-BT | NOM | 72.3 | TRUE |
| DUT3-3G-NB-USB | NOM | 49.5 | FALSE |
| DUT4-3G-NB-AC | MAX | 5.4 | FALSE |
|  | NOM | 12.5 | FALSE |
| DUT4-3G-NB-BT | NOM | 33.3 | FALSE |
| DUT4-3G-NB-USB | NOM | 72.3 | TRUE |
| DUT5-3G-NB-AC | MAX | 78.9 | TRUE |
|  | NOM | 78.8 | TRUE |
| DUT5-3G-NB-BT | NOM | 71.9 | TRUE |
| DUT7-3G-NB-A | MAX | 55.8 | TRUE |
|  | NOM | 72.3 | TRUE |
| DUT7-3G-NB-BT | NOM | 71.9 | TRUE |
| DUT8-3G-NB-A | MAX | 38.0 | FALSE |
|  | NOM | 33.2 | FALSE |
| DUT8-3G-NB-AC | MAX | -1.6 | FALSE |
|  | NOM | -0.9 | FALSE |
| DUT8-3G-NB-ADC | MAX | -0.6 | FALSE |
|  | NOM | 11.9 | FALSE |
| DUT8-3G-NB-BT | NOM | 72.2 | TRUE |
| DUT8-3G-NB-USB | NOM | 6.8 | FALSE |
| DUT9-3G-NB-A | MAX | 36.3 | FALSE |
|  | NOM | 34.2 | FALSE |
| DUT9-3G-NB-AC | MAX | 2.0 | FALSE |
|  | NOM | 1.0 | FALSE |
| DUT9-3G-NB-BT | NOM | 72.3 | TRUE |

Table 19: Results for echo loss (WB)

| DUT | Volume Setting | TCL [dB] | Requirements pass? |
| --- | --- | --- | --- |
| DUT1-3G-WB-AC | MAX | 19.7 | FALSE |
|  | NOM | 34.9 | FALSE |
| DUT1-3G-WB-BT | NOM | 35.4 | FALSE |
| DUT2-3G-WB-AC | MAX | 60.3 | TRUE |
|  | NOM | 64.7 | TRUE |
| DUT2-3G-WB-BT | NOM | 32.4 | FALSE |
| DUT3-3G-WB-A | MAX | 64.3 | TRUE |
|  | NOM | 64.3 | TRUE |
| DUT3-3G-WB-USB | NOM | 65.3 | TRUE |
| DUT4-3G-WB-AC | NOM | 4.3 | FALSE |
| DUT4-3G-WB-BT | NOM | 33.5 | FALSE |
| DUT4-3G-WB-USB | NOM | 64.9 | TRUE |
| DUT5-3G-WB-AC | MAX | 75.5 | TRUE |
|  | NOM | 75.5 | TRUE |
| DUT5-3G-WB-BT | NOM | 64.6 | TRUE |
| DUT6-3G-WB-ADC | MAX | 75.5 | TRUE |
|  | NOM | 75.5 | TRUE |
| DUT7-3G-WB-A | NOM | 64.7 | TRUE |
| DUT7-3G-WB-BT | NOM | 64.6 | TRUE |
| DUT8-3G-WB-A | MAX | 39.8 | FALSE |
|  | NOM | 42.0 | FALSE |
| DUT8-3G-WB-AC | MAX | -0.1 | FALSE |
|  | NOM | 9.5 | FALSE |
| DUT8-3G-WB-ADC | MAX | -0.7 | FALSE |
|  | NOM | 8.1 | FALSE |
| DUT8-3G-WB-BT | NOM | 64.6 | TRUE |
| DUT8-3G-WB-USB | NOM | 15.8 | FALSE |
| DUT9-3G-WB-A | MAX | 45.4 | FALSE |
|  | NOM | 38.2 | FALSE |
| DUT9-3G-WB-AC | MAX | 7.3 | FALSE |
|  | NOM | 1.9 | FALSE |
| DUT9-3G-WB-BT | NOM | 64.6 | TRUE |

Table 20: Results for echo loss (SWB)

| DUT | Volume Setting | TCL [dB] | Requirements pass? |
| --- | --- | --- | --- |
| DUT2-VoLTE-SWB-AC | MAX | 75.6 | TRUE |
|  | NOM | 75.3 | TRUE |
| DUT2-VoLTE-SWB-BT | NOM | 80.8 | TRUE |
| DUT3-VoLTE-SWB-A | MAX | 66.0 | TRUE |
|  | NOM | 62.5 | TRUE |

NOTE 1:	The requirement fails for BT interface may be explained by non-operational signal processing (even though it was signalled correctly from the reference interface when establishing the connection).

NOTE 2:	The clear requirement fails for analogue interface may be explained by the connection via USB-C interface, which seems to have a unintended, but rather high cross-talk (and in general, a lower circuit quality) than a traditional jack plug.

## Distortion

### SND

Results for sine distortion ratio (SDR) in send direction are provided in Table 21 to Table 23. As provisional requirements, the same values as currently specified for handset/headset UE were assumed. Since numerous measurements and requirements apply, only the single values of the closest passing/failing sub-test are reported here.

Table 21: Results for send distortion (NB)

| DUT | Requirement pass? | SDR [dB] | Limit [dB] | Comment |
| --- | --- | --- | --- | --- |
| DUT1-3G-NB-AC | TRUE | 42.4 | 27 | Closest pass: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT1-3G-NB-BT | TRUE | 58.5 | 35 | Closest pass: Avg. SDR for 0.0 dBPa @ 1020 Hz |
| DUT2-3G-NB-AC | FALSE | 31.6 | 33 | Closest fail: Avg. SDR for -10.0 dBPa @ 1020 Hz |
| DUT2-3G-NB-BT | TRUE | 57.1 | 35 | Closest pass: Avg. SDR for 0.0 dBPa @ 1020 Hz |
| DUT3-3G-NB-A | TRUE | 37.7 | 27 | Closest pass: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT3-3G-NB-BT | FALSE | 23 | 30 | Closest fail: Avg. SDR for 5.0 dBPa @ 1020 Hz |
| DUT3-3G-NB-USB | TRUE | 53.6 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT4-3G-NB-AC | TRUE | 52.4 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT4-3G-NB-BT | TRUE | 58.7 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT4-3G-NB-USB | FALSE | 21.1 | 30 | Closest fail: Avg. SDR for 5.0 dBPa @ 1020 Hz |
| DUT5-3G-NB-AC | TRUE | 55.9 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT5-3G-NB-BT | FALSE | 23.3 | 27 | Closest fail: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT7-3G-NB-BT | TRUE | 42.4 | 35 | Closest pass: Avg. SDR for 0.0 dBPa @ 1020 Hz |
| DUT8-3G-NB-A | TRUE | 27.2 | 27 | Closest pass: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT8-3G-NB-AC | FALSE | 32.8 | 33 | Closest fail: Avg. SDR for -10.0 dBPa @ 1020 Hz |
| DUT8-3G-NB-BT | FALSE | 23.7 | 30 | Closest fail: Avg. SDR for 5.0 dBPa @ 1020 Hz |
| DUT8-3G-NB-USB | TRUE | 58.4 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT9-3G-NB-A | TRUE | 32.2 | 27 | Closest pass: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT9-3G-NB-AC | TRUE | 35.8 | 27 | Closest pass: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT9-3G-NB-BT | TRUE | 56.8 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |

Table 22: Results for send distortion (WB)

| DUT | Requirement pass? | SDR [dB] | Limit [dB] | Comment |
| --- | --- | --- | --- | --- |
| DUT1-3G-WB-AC | TRUE | 36.6 | 28 | Closest pass: Avg. SDR for -4.7 dBPa @ 408 Hz |
| DUT1-3G-WB-BT | TRUE | 47.3 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT2-3G-WB-AC | FALSE | 28.4 | 33 | Closest fail: Avg. SDR for -10.0 dBPa @ 1020 Hz |
| DUT2-3G-WB-BT | TRUE | 40.9 | 28 | Closest pass: Avg. SDR for -4.7 dBPa @ 408 Hz |
| DUT3-3G-WB-A | TRUE | 43.8 | 28 | Closest pass: Avg. SDR for -4.7 dBPa @ 408 Hz |
| DUT3-3G-WB-USB | TRUE | 54.7 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT4-3G-WB-AC | TRUE | 52.3 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT4-3G-WB-BT | TRUE | 49.3 | 28 | Closest pass: Avg. SDR for -4.7 dBPa @ 408 Hz |
| DUT4-3G-WB-USB | FALSE | 26.4 | 30 | Closest fail: Avg. SDR for 5.0 dBPa @ 1020 Hz |
| DUT5-3G-WB-AC | FALSE | 8.3 | 27 | Closest fail: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT5-3G-WB-BT | FALSE | 27.8 | 28 | Closest fail: Avg. SDR for -4.7 dBPa @ 510 Hz |
| DUT6-3G-WB-ADC | TRUE | 43.6 | 28 | Closest pass: Avg. SDR for -4.7 dBPa @ 408 Hz |
| DUT7-3G-WB-A | FALSE | 30.5 | 35 | Closest fail: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT7-3G-WB-BT | TRUE | 39.5 | 35 | Closest pass: Avg. SDR for 0.0 dBPa @ 1020 Hz |
| DUT8-3G-WB-A | FALSE | 31.4 | 35 | Closest fail: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT8-3G-WB-AC | FALSE | 33 | 35 | Closest fail: Avg. SDR for 0.0 dBPa @ 1020 Hz |
| DUT8-3G-WB-ADC | FALSE | 34.9 | 35 | Closest fail: Avg. SDR for 0.0 dBPa @ 1020 Hz |
| DUT8-3G-WB-BT | TRUE | 37.3 | 30 | Closest pass: Avg. SDR for 5.0 dBPa @ 1020 Hz |
| DUT8-3G-WB-USB | TRUE | 55.3 | 35 | Closest pass: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT9-3G-WB-A | TRUE | 32.9 | 27 | Closest pass: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT9-3G-WB-AC | TRUE | 30 | 27 | Closest pass: Avg. SDR for -20.0 dBPa @ 1020 Hz |
| DUT9-3G-WB-BT | TRUE | 48 | 28 | Closest pass: Avg. SDR for -4.7 dBPa @ 408 Hz |

Table 23: Results for send distortion (SWB)

| DUT | Requirement pass? | SDR [dB] | Limit [dB] | Comment |
| --- | --- | --- | --- | --- |
| DUT2-VoLTE-SWB-AC | FALSE | 26.4 | 35 | Closest fail: Avg. SDR for -4.7 dBPa @ 1020 Hz |
| DUT2-VoLTE-SWB-BT | TRUE | 32.7 | 32 | Closest pass: Avg. SDR for -4.7 dBPa @ 408 Hz |
| DUT3-VoLTE-SWB-A | FALSE | 31.4 | 32 | Closest fail: Avg. SDR for -4.7 dBPa @ 408 Hz |

NOTE:	In the comment column, the pass/fail status is described for a certain level and frequency. The level information is provided in terms of "dBPa", referring to a (virtual) acoustic level at the MRP. For sake of simplicity and to have a better comparison between analogue and digital electrical interface, these levels are reported here instead of the corresponding electrical levels.

### RCV

Results for sine distortion ratio (SDR) in receive direction are provided in Table 24 to Table 26. As provisional requirements, the same values as currently specified for handset/headset UE were assumed. Since numerous measurements and requirements apply, only the single values of the closest passing/failing sub-test are reported here.

Table 24: Results for receive distortion (NB)

| DUT | Requirement pass? | SDR [dB] | Limit [dB] | Comment |
| --- | --- | --- | --- | --- |
| DUT1-3G-NB-AC | TRUE | 40.9 | 22.5 | Closest pass: Avg. SDR for -40.0 dBm0 @ 1020 Hz |
| DUT1-3G-NB-BT | TRUE | 47 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT2-3G-NB-AC | TRUE | 47.1 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT2-3G-NB-BT | TRUE | 35.7 | 22.5 | Closest pass: Avg. SDR for -40.0 dBm0 @ 1020 Hz |
| DUT3-3G-NB-A | TRUE | 19.7 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT3-3G-NB-BT | FALSE | 24.2 | 25.5 | Closest fail: Avg. SDR for 0.0 dBm0 @ 1020 Hz |
| DUT3-3G-NB-USB | TRUE | 32.6 | 22.5 | Closest pass: Avg. SDR for -40.0 dBm0 @ 1020 Hz |
| DUT4-3G-NB-AC | TRUE | 44.8 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT4-3G-NB-BT | TRUE | 48.3 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT4-3G-NB-USB | TRUE | 46.8 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT5-3G-NB-AC | TRUE | 29.3 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT5-3G-NB-BT | FALSE | 27.9 | 28 | Closest fail: Avg. SDR for -16.0 dBm0 @ 510 Hz |
| DUT7-3G-NB-A | TRUE | 30 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT7-3G-NB-BT | TRUE | 46.6 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT8-3G-NB-A | TRUE | 31.4 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT8-3G-NB-AC | TRUE | 47.4 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT8-3G-NB-BT | TRUE | 47.2 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT8-3G-NB-USB | TRUE | 52.8 | 33 | Closest pass: Avg. SDR for -20.0 dBm0 @ 1020 Hz |
| DUT9-3G-NB-A | TRUE | 49.6 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT9-3G-NB-AC | TRUE | 49.1 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT9-3G-NB-BT | TRUE | 49.6 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |

Table 25: Results for receive distortion (WB)

| DUT | Requirement pass? | SDR [dB] | Limit [dB] | Comment |
| --- | --- | --- | --- | --- |
| DUT1-3G-WB-AC | TRUE | 35.4 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT1-3G-WB-BT | TRUE | 48.3 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT2-3G-WB-AC | TRUE | 21.8 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT2-3G-WB-BT | TRUE | 46.2 | 28 | Closest pass: Avg. SDR for -16.0 dBm0 @ 408 Hz |
| DUT3-3G-WB-A | TRUE | 47.2 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT3-3G-WB-USB | TRUE | 40.3 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT4-3G-WB-AC | TRUE | 43.3 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT4-3G-WB-BT | TRUE | 45.4 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT4-3G-WB-USB | TRUE | 43.3 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT5-3G-WB-AC | TRUE | 50.3 | 33 | Closest pass: Avg. SDR for -20.0 dBm0 @ 1020 Hz |
| DUT5-3G-WB-BT | TRUE | 36.9 | 28 | Closest pass: Avg. SDR for -16.0 dBm0 @ 408 Hz |
| DUT6-3G-WB-ADC | TRUE | 43.6 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT7-3G-WB-A | TRUE | 47.4 | 28 | Closest pass: Avg. SDR for -16.0 dBm0 @ 510 Hz |
| DUT7-3G-WB-BT | TRUE | 46.6 | 28 | Closest pass: Avg. SDR for -16.0 dBm0 @ 408 Hz |
| DUT8-3G-WB-A | TRUE | 19.3 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT8-3G-WB-AC | TRUE | 19 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT8-3G-WB-ADC | TRUE | 24.3 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT8-3G-WB-BT | TRUE | 46 | 28 | Closest pass: Avg. SDR for -16.0 dBm0 @ 510 Hz |
| DUT8-3G-WB-USB | TRUE | 25.8 | 17.5 | Closest pass: Avg. SDR for -45.0 dBm0 @ 1020 Hz |
| DUT9-3G-WB-A | TRUE | 45.4 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT9-3G-WB-AC | TRUE | 40.9 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |
| DUT9-3G-WB-BT | TRUE | 41.8 | 30.5 | Closest pass: Avg. SDR for -30.0 dBm0 @ 1020 Hz |

Table 26: Results for receive distortion (SWB)

| DUT | Requirement pass? | SDR [dB] | Limit [dB] | Comment |
| --- | --- | --- | --- | --- |
| DUT2-VoLTE-SWB-AC | TRUE | 37.8 | 22.5 | Closest pass: Avg. SDR for -40.0 dBm0 @ 1020 Hz |
| DUT2-VoLTE-SWB-BT | FALSE | 19.4 | 20 | Closest fail: Avg. SDR for -16.0 dBm0 @ 315 Hz |
| DUT3-VoLTE-SWB-A | TRUE | 23.1 | 22.5 | Closest pass: Avg. SDR for -40.0 dBm0 @ 1020 Hz |

## Delay

Results for the round-trip delays (send + receive delay) are provided in Table 27 to Table 29. The following provisional requirements apply for the delay D:

- AMR-NB: D < 220.0 ms

- AMR-WB: D < 220.0 ms

- EVS-SWB: D < 197.0 ms

Table 27: Results for round trip delay (NB)

| DUT | RT-Delay [ms] | Requirements pass? |
| --- | --- | --- |
| DUT1-3G-NB-AC | 171.2 | TRUE |
| DUT1-3G-NB-BT | 232.8 | FALSE |
| DUT2-3G-NB-AC | 191.3 | TRUE |
| DUT2-3G-NB-BT | 233.5 | FALSE |
| DUT3-3G-NB-A | 150.8 | TRUE |
| DUT3-3G-NB-BT | 131.9 | TRUE |
| DUT3-3G-NB-USB | 146.4 | TRUE |
| DUT4-3G-NB-AC | 174.9 | TRUE |
| DUT4-3G-NB-BT | 180.3 | TRUE |
| DUT4-3G-NB-USB | 160.8 | TRUE |
| DUT5-3G-NB-AC | 163.3 | TRUE |
| DUT5-3G-NB-BT | 370.4 | FALSE |
| DUT7-3G-NB-A | 190.8 | TRUE |
| DUT7-3G-NB-BT | 211.6 | TRUE |
| DUT8-3G-NB-A | 184.3 | TRUE |
| DUT8-3G-NB-AC | 221.4 | FALSE |
| DUT8-3G-NB-ADC | 211.7 | TRUE |
| DUT8-3G-NB-BT | 231.0 | FALSE |
| DUT8-3G-NB-USB | 245.8 | FALSE |
| DUT9-3G-NB-A | 185.3 | TRUE |
| DUT9-3G-NB-AC | 173.3 | TRUE |
| DUT9-3G-NB-BT | 211.2 | TRUE |

Table 28: Results for round trip delay (WB)

| DUT | RT-Delay [ms] | Requirements pass? |
| --- | --- | --- |
| DUT1-3G-WB-AC | 187.1 | TRUE |
| DUT1-3G-WB-BT | 218.3 | TRUE |
| DUT2-3G-WB-AC | 185.8 | TRUE |
| DUT2-3G-WB-BT | 272.5 | FALSE |
| DUT3-3G-WB-A | 160.5 | TRUE |
| DUT3-3G-WB-USB | 150.9 | TRUE |
| DUT4-3G-WB-AC | 175.6 | TRUE |
| DUT4-3G-WB-BT | 174.6 | TRUE |
| DUT4-3G-WB-USB | 160.2 | TRUE |
| DUT5-3G-WB-AC | 165.8 | TRUE |
| DUT5-3G-WB-BT | 401.9 | FALSE |
| DUT6-3G-WB-ADC | 194.1 | TRUE |
| DUT7-3G-WB-A | 180.4 | TRUE |
| DUT7-3G-WB-BT | 235.1 | FALSE |
| DUT8-3G-WB-A | 161.8 | TRUE |
| DUT8-3G-WB-AC | 212.2 | TRUE |
| DUT8-3G-WB-ADC | 209.4 | TRUE |
| DUT8-3G-WB-BT | 215.8 | TRUE |
| DUT8-3G-WB-USB | 237.4 | FALSE |
| DUT9-3G-WB-A | 184.1 | TRUE |
| DUT9-3G-WB-AC | 173.9 | TRUE |
| DUT9-3G-WB-BT | 208.5 | TRUE |

Table 29: Results for round trip delay (SWB)

| DUT | RT-Delay [ms] | Requirements pass? |
| --- | --- | --- |
| DUT2-VoLTE-SWB-AC | 143.7 | TRUE |
| DUT2-VoLTE-SWB-BT | 176.5 | TRUE |
| DUT3-VoLTE-SWB-A | 395.6 | FALSE |
| DUT3-VoLTE-SWB-BT | 110.8 | TRUE |

NOTE 1:	Results for the analysis of time-variant impairments for VoLTE are not yet available.

NOTE 2:	While almost all available devices pass the requirement for the analogue interface, several fails can be observed for BT and USB interfaces. When comparing the differences between analogue and BT for identical combinations of device/connection/codec, the additional "delay budget" for the introduced BT link varies between 30 ms and 90 ms. For the USB interface, up to 20 ms can be observed – in most cases it is even lower than for analogue interface.
In general, it seems technically possible to meet the default requirement for BT and USB without any additional budget.

## Send speech quality and noise intrusiveness in the presence of ambient noise

Results for S-MOS and N-MOS analysis (average across eight noise types) are provided in Table 30 and Table 31. The following provisional requirements apply (based on handset/headset UE):

NB:

- S-MOS ≥ 3.0

- N-MOS ≥ 2.3

WB:

- S-MOS ≥ 3.0

- N-MOS ≥ 2.3

SWB:

- S-MOS ≥ 3.7

- N-MOS ≥ 2.7

Table 30: Results for speech quality and noise intrusiveness (NB)

| DUT | S-MOS | Pass | N-MOS | Pass |
| --- | --- | --- | --- | --- |
| DUT1-3G-NB-AC | 3.7 | TRUE | 2.9 | TRUE |
| DUT1-3G-NB-BT | 3.7 | TRUE | 3.0 | TRUE |
| DUT2-3G-NB-AC | 3.5 | TRUE | 2.8 | TRUE |
| DUT2-3G-NB-BT | 4.0 | TRUE | 2.7 | TRUE |
| DUT3-3G-NB-A | 3.7 | TRUE | 2.5 | TRUE |
| DUT3-3G-NB-BT | 3.9 | TRUE | 1.9 | FALSE |
| DUT3-3G-NB-USB | 3.9 | TRUE | 2.6 | TRUE |
| DUT4-3G-NB-AC | 3.9 | TRUE | 2.9 | TRUE |
| DUT4-3G-NB-BT | 3.9 | TRUE | 2.5 | TRUE |
| DUT4-3G-NB-USB | 3.9 | TRUE | 2.2 | FALSE |
| DUT5-3G-NB-AC | 4.0 | TRUE | 2.7 | TRUE |
| DUT5-3G-NB-BT | 3.2 | TRUE | 1.8 | FALSE |
| DUT7-3G-NB-A | 3.5 | TRUE | 3.3 | TRUE |
| DUT7-3G-NB-BT | 4.0 | TRUE | 1.7 | FALSE |
| DUT8-3G-NB-A | 3.5 | TRUE | 2.5 | TRUE |
| DUT8-3G-NB-AC | 3.3 | TRUE | 2.4 | TRUE |
| DUT8-3G-NB-ADC | 3.2 | TRUE | 2.4 | TRUE |
| DUT8-3G-NB-BT | 3.8 | TRUE | 1.9 | FALSE |
| DUT8-3G-NB-USB | 3.8 | TRUE | 2.3 | TRUE |
| DUT9-3G-NB-A | 3.3 | TRUE | 2.7 | TRUE |
| DUT9-3G-NB-AC | 3.3 | TRUE | 2.8 | TRUE |
| DUT9-3G-NB-BT | 3.7 | TRUE | 2.9 | TRUE |

Table 31: Results for speech quality and noise intrusiveness (WB+SWB)

| DUT | S-MOS | Pass | N-MOS | Pass |
| --- | --- | --- | --- | --- |
| DUT1-3G-WB-AC | 3.4 | TRUE | 3.6 | TRUE |
| DUT1-3G-WB-BT | 3.6 | TRUE | 3.3 | TRUE |
| DUT2-3G-WB-AC | 3.4 | TRUE | 4.0 | TRUE |
| DUT2-3G-WB-BT | 3.9 | TRUE | 3.4 | TRUE |
| DUT3-3G-WB-A | 3.9 | TRUE | 3.0 | TRUE |
| DUT3-3G-WB-USB | 3.9 | TRUE | 3.0 | TRUE |
| DUT4-3G-WB-AC | 3.9 | TRUE | 3.1 | TRUE |
| DUT4-3G-WB-BT | 3.9 | TRUE | 2.9 | TRUE |
| DUT4-3G-WB-USB | 3.9 | TRUE | 2.4 | TRUE |
| DUT5-3G-WB-AC | 3.8 | TRUE | 3.2 | TRUE |
| DUT5-3G-WB-BT | 3.7 | TRUE | 2.4 | TRUE |
| DUT6-3G-WB-ADC | 3.9 | TRUE | 3.0 | TRUE |
| DUT7-3G-WB-A | 3.3 | TRUE | 3.6 | TRUE |
| DUT7-3G-WB-BT | 3.7 | TRUE | 3.7 | TRUE |
| DUT8-3G-WB-A | 3.2 | TRUE | 3.2 | TRUE |
| DUT8-3G-WB-AC | 3.2 | TRUE | 3.2 | TRUE |
| DUT8-3G-WB-ADC | 3.2 | TRUE | 3.2 | TRUE |
| DUT8-3G-WB-BT | 3.8 | TRUE | 2.2 | FALSE |
| DUT8-3G-WB-USB | 3.8 | TRUE | 2.8 | TRUE |
| DUT9-3G-WB-A | 3.0 | TRUE | 3.5 | TRUE |
| DUT9-3G-WB-AC | 2.9 | FALSE | 3.4 | TRUE |
| DUT9-3G-WB-BT | 3.6 | TRUE | 3.3 | TRUE |
| DUT3-VoLTE-SWB-BT | 4.2 | TRUE | 2.7 | TRUE |

# Conclusion

For testing the performance of electrical (headset) interface according to the latest draft CR to TS 26.131/132 [4] [5] and latest proposals [7], initial results were obtained in a test series with several recent mobile phones.

The most relevant measurements were evaluated, reviewed and available results were reported. The preliminary agreed setup and test descriptions seem to work as expected. Also, the provisional requirements are already in a reasonable range, only some values of the tolerance masks of send frequency response should be adapted.

# References

| [1] | 3GPP S4-200918, „New WID on Extension for headset interface tests of UE (HInT),“ HEAD acoustics GmbH, Orange, Sony Mobile Communications, Samsung Electronics Co., Ltd, Qualcomm Incorporated, Online, 20 May 2020. |
| --- | --- |
| [2] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release-16. |
| [3] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release-16. |
| [4] | 3GPP S4aQ210164, „dCR 26.131 Extension for headset interface tests of UE,“ Orange, April 2021. |
| [5] | 3GPP S4-210651, „DraftCR TS26.132 on Headset Interface Description (update),“ HEAD acoustics GmbH, April 2021. |
| [6] | Recommendation ITU-T P.381, „Technical requirements and test methods for the universal wired headset or headphone interface of digital mobile terminals,“ 10/2020. |
| [7] | 3GPP S4-210825, „Proposals for HInT,“ HEAD acoustics GmbH, 05/2021. |