3GPP TSG-SA4-e (AH) Video SWG post 131	S4aV250022

# online, , 04th Mar 2025 – 26th Mar 2025									revision of S4aV250016

Source:	Qualcomm Germany

Title:	Pseudo-CR on [VOPS] Updates to System Integration

Spec:	3GPP TS26.265v0.6.0

Agenda item:	3.5

Document for:	Decision

1. Introduction

At the last meeting the system integration was simplified.

2. Reason for Change

At the last meeting the system integration was simplified, but not completed.

A collection of common system related metadata is provided in the following:

- ISO/IEC 14496-15 defines the decoder configuration record

- Web Codecs API refers to the decoder configuration record

- DASH Initialization and CMAF Initialization Segments contain the decoder configuration record for MSE based configuration. In addition, width and height may be specified in track header.

- RRC7798 defines the payload related parameters: https://datatracker.ietf.org/doc/html/rfc7798#section-7

- To configure the Android media codec, see here, https://developer.android.com/reference/android/media/MediaCodec#configure(android.media.MediaFormat,%20android.view.Surface,%20android.media.MediaCrypto,%20int). For HEVC it is said to use for CSD#0 VPS, SPS and PPS.

Systems integration parameters of relevance are:

- 	The codecs string begins with the prefix "hev1." or "hvc1.", with a suffix of four dot-separated fields as described in Section E.3 of [iso14496-15].

-	Chunks/Access units

-	Random access

-	Decoder Configuration parameters see section 8.3.3.1 of [iso14496-15].

-	profile, tier, level

-	constraints flags

-	chroma format

- 	bit depth chroma and luma

-	frame rates, average or constant

-	layering structure

-	NAL units

-	VPS (Video Parameter Set): Contains parameters that apply to the entire video sequence.

-	SPS (Sequence Parameter Set): Contains parameters that apply to a sequence of pictures.

-	PPS (Picture Parameter Set): Contains parameters that apply to individual pictures.

-	declarative SEI NAL unit, as specified in ISO/IEC 23008-2. When one or more SEI NAL units containing an SEI manifest SEI message and/or an SEI prefix indication SEI message are available, they should be stored as instances of nalUnit.

3. Conclusions

Let’s discuss the content as baseline and have proper resolution by the April meeting.

4. Proposal

It is proposed to agree the following changes to 3GPP TS26.265v0.6.0 at SA4-131-bis-e.

5. Revision

| S4aV250016 | [VOPS] Updates to System Integration | Qualcomm Germany | Thomas Stockhammer |
| --- | --- | --- | --- |

Presenter: Thomas Stockhammer

Online Discussion: (March 4 2025)

- Gilles: Would you expect to point to CMAF profiles?

  - Thomas: No, we keep this independent of the encapsulation.

  - Gilles: OK, this will be applicable to RTP, CMAF, …

- Waqar: I didn’t see this Tdoc. Can we endorse it?

  - Thomas: OK, I plan to revise it.

Decision: OK to use it as a way forward. Revised to 22.

S4aV250016 is revised to S4aV250022.

* * * First Change * * * *

# 7	Common System Integration

## 7.1	Introduction

This clause documents general functionalities that are relevant for integration of video codecs into delivery systems to support common APIs on encoders and decoders.

## 7.2	Functional Definitions

### 7.2.1	General

This clause defines functional definitions for system integration in Table 7.2.1-1.

Editor’s Note:

- See here for guidelines: https://www.w3.org/TR/webcodecs-hevc-codec-registration/

- Codecs String

- Random Access point

- Chunk

- Decoder Configuration Record

Table 7.2.1-1	Functional Definitions

| Term | Definition |
| --- | --- |
| Codec String | A single value identifying the codec indicated to render the content in the Bitstream as defined in IETF RFC 6381. |
| Decoder Configuration | a data structure storing essential parameters needed for decoding and rendering a video stream. |
| Random Access Point | A bit position in the Bitstream, for which in combination with the Decoder Configuration, the Bitstream can be randomly accessed, i.e. in decoding order the Bitstream carries sufficient information to access the media in the stream. |
| Chunk | bits that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain exactly one coded picture. The decoding of each access unit results in one decoded picture. |
| Random Access Chunk | A chunk that starts with a random access point |

### 7.2.2	AVC

Editor’s Note: This needs to be completed.

### 7.2.3	HEVC

Editor’s Note: This needs to be completed.

* * * End of Changes * * * *