3GPPSA4-(AH) Video SWG post 132						S4aV250057

Online, 17 June – 8 July 2025

Source:	Xiaomi

Title:	[VOPS] pCR on conformance work for bitstream validation tooling

Agenda item:	3.5

Document for:	DISCUSSION

# Introduction

At the 3GPP SA4 meeting #132, the VOPS Permanent Document on conformance has been published [1]. As reminder, this document describes a platform for bitstream validation composes of four components:

- A bitstream validator

- A bitstream database

- A bitstream hosting server

- A public portal

# Proposal

This contribution relates to the component 1) bitstream validator and proposes to agree on:

- Agree on that the bitstream validation tooling is best-effort and it should not prevent the VOPS Work Item from closing.

- Agree on using the BSD 3-Clause Clear License copyrighted as “3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC)” for developing this tool.

- Agree on using the 3GPP Forge (Gitlab instance) [2] to develop the tool.

- Agree on using the proposed repository [3] as starting point for the bitstream validator and move it to the SA4 namespace on the 3GPP Forge.

# Motivation

# On the proposed starting point repository

The proposed starting point for the bitstream validator can be found here [3]:

https://forge.3gpp.org/rep/thomasem/sa4-bitstream-validator

It most notably contains the validator as a Python module, an external reference to the HTM project (using submodule), a license. The goal of the script is perform a two-step validation on an HEVC input bitstream:

- Validating that the bitstream is an HEVC compliant bitstream.

- Validating that the bitstream is compliant to an operation point defined in VOPS.

For the first validation step, it is expected to reuse the HTM project developed by JVET.

For the second validation step, it is proposed to express the validation of VOPS bitstream rules using XML schema. This XML schema can then be used to validate a bitstream which has been parsed and described in a XML format. This approach is document in VOPS Permanent Document v 0.1.0 clause 3.1.2.1.3.

The propose script has currently two commands ‘dump’ and ‘validate’. The ‘dump’ command generates an XML document describing an input bitstream. The ‘validate’ command takes such XML description as well as an XML schema expressing conformance for a given operation point defined in VOPS.

(env) thomase@LAPTOP-1CLM2OCG:~/workspace/sa4-bitstream-validator$ python -m sa4_bitstream_validator dump MVHEVCS_K_Apple_2.bit test.xml

Start dumping MVHEVCS_K_Apple_2.bit in test.xml

Here below is the generated XML document:

<?xml version="1.0" ?>

<HEVCBitstream xmlns:bs1="urn:mpeg:mpeg21:2003:01-DIA-BSDL1-NS" bs1:bitstreamURI="MVHEVCS_K_Apple_2.bit">

<NALUnit>

<startCode>00000001</startCode>

<forbidden_zero_bit>0</forbidden_zero_bit>

<nal_unit_type>32</nal_unit_type>

<nuh_layer_id>0</nuh_layer_id>

<nuh_temporal_id_plus1>1</nuh_temporal_id_plus1>

<payload>6 56</payload>

<VideoParameterSet>

<vps_video_parameter_set_id>23</vps_video_parameter_set_id>

<vps_base_layer_internal_flag>0</vps_base_layer_internal_flag>

<vps_base_layer_available_flag>0</vps_base_layer_available_flag>

<vps_max_layers_minus1>35</vps_max_layers_minus1>

<vps_max_sub_layers_minus1>7</vps_max_sub_layers_minus1>

<vps_temporal_id_nesting_flag>1</vps_temporal_id_nesting_flag>

<vps_reserved_0xffff_16bits>65504</vps_reserved_0xffff_16bits>

</VideoParameterSet>

</NALUnit>

<NALUnit>

<startCode>00000001</startCode>

<forbidden_zero_bit>0</forbidden_zero_bit>

<nal_unit_type>33</nal_unit_type>

<nuh_layer_id>0</nuh_layer_id>

<nuh_temporal_id_plus1>1</nuh_temporal_id_plus1>

<payload>68 49</payload>

</NALUnit>

<NALUnit>

<startCode>00000001</startCode>

<forbidden_zero_bit>0</forbidden_zero_bit>

<nal_unit_type>34</nal_unit_type>

<nuh_layer_id>0</nuh_layer_id>

<nuh_temporal_id_plus1>1</nuh_temporal_id_plus1>

<payload>123 6</payload>

</NALUnit>

<NALUnit>

<startCode>000001</startCode>

<forbidden_zero_bit>0</forbidden_zero_bit>

<nal_unit_type>19</nal_unit_type>

<nuh_layer_id>0</nuh_layer_id>

<nuh_temporal_id_plus1>1</nuh_temporal_id_plus1>

<payload>134 10565</payload>

</NALUnit>

...

</HEVCBitstream>

In the folder ‘bitstream_rules’, one can write an XML schema for each operation point. Here is below an starting point example for the 3GPP-MV-HEVC-Stereo operation point. Note that the schema requires the presence of a VPS as the first NAL unit and that the vps_max_layers_minus1 shall be equal to 1.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

elementFormDefault="qualified"

xmlns:vc="http://www.w3.org/2007/XMLSchema-versioning"

vc:minVersion="1.1">

<xs:complexType name="VideoParameterSetType">

<xs:sequence>

<xs:element name="vps_video_parameter_set_id" type="xs:integer"/>

<xs:element name="vps_base_layer_internal_flag" type="xs:integer"/>

<xs:element name="vps_base_layer_available_flag" type="xs:integer"/>

<xs:element name="vps_max_layers_minus1" type="xs:integer"/>

<xs:element name="vps_max_sub_layers_minus1" type="xs:integer"/>

<xs:element name="vps_temporal_id_nesting_flag" type="xs:integer"/>

<xs:element name="vps_reserved_0xffff_16bits" type="xs:integer"/>

</xs:sequence>

</xs:complexType>

<xs:element name="NALUnit">

<xs:complexType>

<xs:sequence>

<xs:element name="startCode" type="xs:string"/>

<xs:element name="forbidden_zero_bit" type="xs:integer"/>

<xs:element name="nal_unit_type" type="xs:integer"/>

<xs:element name="nuh_layer_id" type="xs:integer"/>

<xs:element name="nuh_temporal_id_plus1" type="xs:integer"/>

<xs:element name="payload" type="xs:string"/>

<xs:element name="VideoParameterSet" type="VideoParameterSetType" minOccurs="0"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:element name="HEVCBitstream">

<xs:complexType>

<xs:sequence>

<xs:element ref="NALUnit" maxOccurs="unbounded"/>

</xs:sequence>

<xs:assert test="exists(NALUnit[1]/VideoParameterSet)"/>

<xs:assert test="NALUnit[1]/VideoParameterSet/vps_max_layers_minus1 == 1"/>

</xs:complexType>

</xs:element>

</xs:schema>

Then the command ‘validate’ can be invoked:

(env) thomase@LAPTOP-1CLM2OCG:~/workspace/sa4-bitstream-validator$ python -m sa4_bitstream_validator validate MVHEVCS_K_Apple_2.xml bitstream_rules/vops_3gpp-mv-hevc-stereo.xsd

Start validation of MVHEVCS_K_Apple_2.xml against bitstream_rules/vops_3gpp-mv-hevc-stereo.xsd

Validation succeeded.

# On the license

The proposed license to use for this project is the BSD 3-Clause Clear License. The advantage is that it is commonly used in other standardisation bodies and has clear statement on patent rights.

https://opensource.org/license/BSD-3-clause

# On the copyright author

The copyright in the  license should be aligned with 3GPP policies. Based on current work on the forge, it appears that electronic work is copyrighted with the following statement:

Copyright © 2005, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved.

For example, here is XML schema containing such copyright:

https://forge.3gpp.org/rep/all/5G_APIs/-/blob/REL-19/TS26346_AssociatedDeliveryProceduresDescription.xsd

# On using the 3GPP forge

The 3GPP Forge is a Gitlab instance maintained by 3GPP for developing projects in the context of standardisation. One key benefits of using it is the user management which is linked to EOL account and alleviate the handling of access control by SA4 delegate.

Since the 3GPP Forge is getting tracking in 3GPP wide in terms of usage, it appears to be a futureproof choice to opt for this platform as opposed to external alternative, e.g. public Gitlab, public GitHub, etc...

# References

- S4-251144, VOPS Permanent Document v0.1.0, 3GPP TSG SA WG4#132, Fukuoka, JP, 19th – 23rd May 2025

- 3GPP Forge, https://forge.3gpp.org/rep/

- Emmanuel Thomas / sa4-bitstream-validator · GitLab, https://forge.3gpp.org/rep/thomasem/sa4-bitstream-validator