SA4-(AH) Video SWG post 132	S4aV250058

# Online, 8 July 2025

Source:	China Mobile Com. Corporation

Title:	[FS_Beyond2D] Evaluation Results for the Scenario 1 Using MV-HEVC and Simulcast HEVC

Agenda item:	3.7

Document for:	Agreement

1. Introduction

This document provides input on the evaluation of Scenario 1: UE-to-UE Stereoscopic Video Live Streaming (TR 26.956 v1.0.0, Section 7.2), including evaluation results using MV-HEVC and  Simulcast HEVC.

2. Proposal

The proposal is to add the provided information to a new section x.y of the PD.

3. Attachments

- Configuration files

- Encoder log files

- Shell scripts

- Metrics

x.y	Evaluation of Stereoscopic Video

x.y.1	Build Environment

The compilation was performed in the following environment:

-	Operating System: Windows 11 Subsystem for Linux (WSL), Ubuntu 22.04.2 LTS

-	CPU: 13th Gen Intel(R) Core(TM) i9-13900H

-	Memory: 15 GB

-	Make Version: GNU Make 4.3

The 6 test sequences used are taken from TR 26.956  Annex C.3.5, C.3.6, C.3.7, C.3.2, C.3.3, and C.3.4.

x.y.2	Software

The source code was downloaded from the official SVN repository, targeting the HTM-16.3-fixes directory (https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/branches/HTM-16.3-fixes/).

After compiling the source code, the following executable files were generated:

TAppEncoderStatic / TAppEncoderStaticd: The encoder application. It converts raw video data (e.g., YUV) into an HEVC bitstream. It implements all encoding tools as defined by the HEVC standard, configurable via command-line arguments.

TAppDecoderStatic / TAppDecoderStaticd: The decoder application. It decodes an HEVC bitstream back into raw video data (YUV). It is primarily used for verifying the correctness of the encoded bitstream.

TAppDecoderAnalyserStatic / TAppDecoderAnalyserStaticd: An enhanced decoder with analysis capabilities. In addition to decoding, it provides detailed statistics on the decoding process, such as information per CU/PU/TU, distribution of motion vectors, prediction modes, and quality metrics (e.g., PSNR, SSIM).

TAppExtractorStatic / TAppExtractorStaticd: A tool for extracting specific information from an HEVC bitstream, such as NAL unit information, parameter sets (VPS, SPS, PPS), or data for specific frames/slices.

TAppRendererStatic / TAppRendererStaticd: A tool to render decoded raw video data directly to the screen for visual inspection, often using graphics APIs to convert YUV to RGB for display.

annexBbytecountStatic / annexBbytecountStaticd: A utility to analyze and count bytes of NAL units within an Annex B formatted HEVC bitstream. Useful for validating stream structure and gathering NAL unit size statistics.

convert_NtoMbit_YCbCrStatic / convert_NtoMbit_YCbCrStaticd: A utility for converting the bit-depth of YCbCr data (e.g., from 8-bit to 10-bit) or for converting between chroma subsampling formats (e.g., YUV420 to YUV422).

The script for calculating MV-HEVC bitrate and Y-PSNR is under  ./metrics/analyze_mvhevc.py

The script for calculating Simulcast HEVC bitrate and Y-PSNR is under  ./metrics/analyze_hm_logs.py

The script for calculating BD-Rate is under  ./metrics/compare.py

x.y.2	Encoding of bitstreams

x.y.2.1	MV-HEVC

- Run HM TAppEncoderstatic to encode each YUV file using a fixed QP.

All sequences have been encoded using the configurations in ./cfg/MV-HEVC/baseCfg_2view. The configuration files are attached to this document.

Encoding was performed by running the shell script with appropriate parameters.  The shell scripts are attached to this document. For example:

echo "dog_ai_generated: QP=17"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=17 -wdt 1920 -hgt 1080 -fr 30 -f 150

echo "dog_ai_generated: QP=22"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=22 -wdt 1920 -hgt 1080 -fr 30 -f 150

echo "dog_ai_generated: QP=27"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=27 -wdt 1920 -hgt 1080 -fr 30 -f 150

echo "dog_ai_generated: QP=32"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=32 -wdt 1920 -hgt 1080 -fr 30 -f 150

echo "dog_ai_generated: QP=37"

./TAppEncoderStatic -c ./cfg/baseCfg_2view_movegirl_ai_generated.cfg --QP=37 -wdt 1920 -hgt 1080 -fr 30 -f 150

x.y.2.2	Simulcast HEVC

- Run HM TAppEncoderstatic to encode each YUV file using a fixed QP.

All sequences have been encoded using the configurations in ./cfg/HM/encoder_lowdelay_main_rext.cfg The configuration files are attached to this document.

Encoding was performed by running the shell script with appropriate parameters. The shell scripts are attached to this document. For example:

echo "streetView_captured_left: QP=17"

./TAppEncoderStatic -c ~/HTM-16.3/cfg/HM/encoder_lowdelay_main_rext.cfg -i ~/yuvfiles/streetView_captured_left.yuv -o output0703.bin --SourceWidth=1920 --SourceHeight=1080 --FrameRate=30 --FramesToBeEncoded=150 --QP=17

echo "streetView_captured_right: QP=17"

./TAppEncoderStatic -c ~/HTM-16.3/cfg/HM/encoder_lowdelay_main_rext.cfg -i ~/yuvfiles/streetView_captured_right.yuv -o output0703.bin --SourceWidth=1920 --SourceHeight=1080 --FrameRate=30 --FramesToBeEncoded=150 --QP=17

x.y.3 Results

Based on the test condition define in section 7.2.8 of TR 26.956, a bitstream for each rate point and codec configuration was obtained, as given in the table below:

| Test Sequence | Rate points | Simulcast HEVC |  | MV-HEVC |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Bitrate [kbps] | PSNR [dB] | Bitrate [kbps] | PSNR [dB] |
| Street View - captured | R1 (17) | 82597.98 | 45.46 | 77934.46 | 42.79 |
|  | R2 (22) | 40906.65 | 41.19 | 29811.63 | 37.89 |
|  | R3 (27) | 12828.06 | 36.93 | 9759.37 | 35.08 |
|  | R4 (32) | 4591.49 | 34.35 | 3882.06 | 32.86 |
|  | R5 (37) | 1655.49 | 31.69 | 1568.76 | 30.38 |
| Street View - Generated | R1 (17) | 79587.76 | 45.59 | 63793.59 | 42.05 |
|  | R2 (22) | 37996.06 | 41.31 | 22728.87 | 38.38 |
|  | R3 (27) | 11355.45 | 37.19 | 7199.21 | 35.87 |
|  | R4 (32) | 3988.24 | 34.69 | 3105.62 | 33.83 |
|  | R5 (37) | 1506.74 | 32.03 | 1387.66 | 31.37 |
| Cute Dog - Captured | R1 (17) | 60282.85 | 46.58 | 50309.66 | 43.54 |
|  | R2 (22) | 31296.69 | 42.86 | 21219.06 | 40.21 |
|  | R3 (27) | 13854.47 | 39.02 | 8549.64 | 37.45 |
|  | R4 (32) | 7691.15 | 35.89 | 4030.95 | 34.97 |
|  | R5 (37) | 4361.23 | 32.99 | 2146.72 | 32.38 |
| Cute Dog - Generated | R1 (17) | 64623.85 | 46.53 | 40674.14 | 43.26 |
|  | R2 (22) | 32782.40 | 42.69 | 16506.89 | 40.33 |
|  | R3 (27) | 14509.13 | 38.79 | 6686.76 | 37.67 |
|  | R4 (32) | 7981.78 | 35.69 | 3274.28 | 35.22 |
|  | R5 (37) | 4504.21 | 32.82 | 1771.73 | 32.65 |
| Moving Girl - Captured | R1 (17) | 9100.29 | 51.28 | 8944.59 | 49.49 |
|  | R2 (22) | 3319.19 | 48.92 | 2661.69 | 47.36 |
|  | R3 (27) | 1485.50 | 46.75 | 947.79 | 45.72 |
|  | R4 (32) | 818.86 | 44.34 | 475.70 | 43.77 |
|  | R5 (37) | 480.74 | 41.72 | 250.63 | 41.41 |
| Moving Girl - Generated | R1 (17) | 9788.79 | 51.19 | 7,297.06 | 49.41 |
|  | R2 (22) | 3630.03 | 48.73 | 2300.78 | 47.26 |
|  | R3 (27) | 1523.01 | 46.49 | 822.27 | 45.59 |
|  | R4 (32) | 825.61 | 44.14 | 422.32 | 43.75 |
|  | R5 (37) | 478.95 | 41.56 | 250.63 | 41.58 |

The table below shows the Bjøntegaard Delta (BD) bitrates reductions obtained by MV-HEVC in comparison to Simulcast HEVC.

| Test Sequence | BD-rate reduction of MV-HEVC [%] relative to Simulcast HEVC |
| --- | --- |
| Street View - captured | 61.580523 |
| Street View - Generated | 22.565416 |
| Cute Dog - Captured | -8.714002 |
| Cute Dog - Generated | -37.240038 |
| Moving Girl - Captured | -2.813581 |
| Moving Girl - Generated | -20.596453 |