Source:	China Mobile Com. Corporation

Title:	[FS_AI4Media] Super-Resolution-assisted Object Recognition

Agenda item:	3.6

Document for:	Discussion and Agreement

- Introduction

In the current FS_AI4Media PD, we discussed an object recognition use case for images and video streams. However, current object detection frameworks have unsatisfactory recognition accuracy for small objects in vast background. For example, when input size is 416x416 pixels, the small object recognition accuracy of YOLOv3 on the COCO dataset was only 15%. To tackle this problem, we propose a super-resolution-assisted object recognition use case that aims to improve the recognition performance by addressing the challenges caused by small targets or low resolution images.

- 2 	Proposed Updates

- 4.1.3 Scenario: Super-Resolution-assisted Object Recognition

- 4.1.3.1 Motivation and use case relevance

The existing object recognition frameworks require a minimum number of pixels on the target (e.g., YOLO needs at least 20x20 pixels on the target [1]), and most of them are not specifically trained to identify objects from blurring, noise and compressed images. Therefore, in certain scenarios, a Super-Resolution (SR) model could be used as a pre-processing step to discriminate small objects with low-resolution (LR) inputs, leading to the improvement in recognition accuracy.

One example is the “Person identification in security surveillance system” use case (defined in TR 22.874 [2] clause 6.1.6). In this scenario, the existing face recognition frameworks demand a minimum face resolution of 32x32 as input. However, in surveillance applications, objects are often far away from cameras, resulting in LR images. The objects therefore lack sufficient information for recognizing human faces and behavior [3]. A SR model can be used to enhance the detail of the objects and significantly improve recognition accuracy.

Another example is the “AR loT control” use case (defined in TR 26.998 [4] A.5). In this scenario, the Image & Object Recognition functionality is provided by AR glasses to track loT devices positions and add textual overlays for UIs. However, identifying small-size objects in complex backgrounds (e.g.:industrial inspection) is a challenging task. A SR model can be integrating for ensuring accurate position tracking in diverse and intricate environments.

The general steps for a super-resolution-assisted object recognition includes:

Figure 4.x.x.x  Super-Resolution-assisted Object Recognition use case

- Real-time image/video input: the UE sends a stream of LR images or frames that contain small objects of interest.

- Generator: the network uses a pre-trained SR model to enhances the information contained in small objects. The specific method used for the generation is outside the scope of this use case. For example, the SR model can generate the SR image of input LR image to compensate for the details of small object, or it could use feature enhancement methods to transform the poor features of LR images to high quality features [5].

- Detector: the network then uses object recognition (OR) frameworks, such as YOLOv3, SSD, to recognize and classify objects from the enhanced image generated in step 2.

- Real-time output: the network sends the results of the object recognition process back to the UE in real-time, which may include bounding boxes around detected objects, class labels, and other relevant information. This output can be used for various applications, such as video surveillance, autonomous vehicles, or real-time analysis.

- 3	Proposal

We propose to include section 2 of this document into the FS_AI4Media PD as a relevant scenario for object recognition.

- 4	References

- [1]  Christoph Borel-Donohue and S. Susan Young "Image quality and super resolution effects on object recognition using deep neural networks", Proc. SPIE 11006, Artificial Intelligence and Machine Learning for Multi-Domain Operations Applications, 110061M (10 May 2019); https://doi.org/10.1117/12.2518524

- [2]  3GPP TR 22.874 V18.2.0 (2021-12): "Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS".

- [3]  X. Yang, W. Wu, K. Liu, P. W. Kim, A. K. Sangaiah and G. Jeon, "Long-Distance Object Recognition With Image Super Resolution: A Comparative Study," in IEEE Access, vol. 6, pp. 13429-13438, 2018, doi: 10.1109/ACCESS.2018.2799861.

- [4]  3GPP TR 26.998 V18.0.0 (2022-12): "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

- [5]  Xi, Yue, Jiangbin Zheng, Wenjing Jia, Xiangjian He, Hanhui Li, Zhuqiang Ren and Kin-Man Lam. “See Clearly in the Distance: Representation Learning GAN for Low Resolution Object Recognition.” IEEE Access 8 (2020): 53203-53214.